3GPP TSG-RAN WG1 Meeting #109-e	R1-2203223

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.9

Source:	Huawei, HiSilicon

Title:	On use of DwPTS for 16QAM NPDSCH in NB-IoT

Document for:	Discussion and Decision

- Introduction

This paper discusses the text proposal on use of DwPTS for 16QAM NPDSCH in NB-IoT with TDD operation.

In RAN1#108-e, the following agreement has been agreed.

Agreement: - FDD/TDD differentiation is needed for FGs 1-1/1-2 - DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured.

In this contribution, we propose a text proposal to implement the agreement.

- Text proposal

Reason for change: In RAN1#108e meeting, the following was agreed, which has not been implemented.

Agreement:

- FDD/TDD differentiation is needed for FGs 1-1/1-2

  - DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when 16QAM is configured.

Summary of change: DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM.

Consequences if not approved: DwPTS in special subframe configuration 9 for normal cyclic prefix may be used for NPDSCH transmission with 16QAM, which exceeds the maximum coding rate for NPDSCH.

Proposal 1: Endorse the following text proposal to TS 36.211.

#### 10.0.1.2 Frame structure type 2 Frame structure type 2 is applicable to TDD operation only. The following restrictions apply: - Uplink-downlink configuration 0 and 6 are not supported. - UpPTS is not used for NPUSCH or NPRACH. - DwPTS and UpPTS in special subframe configuration 10 is not used for transmissions. - On an NB-IoT carrier for which higher-layer parameter operationModeInfo indicates inband-SamePCI or inband-DifferentPCI, or higher-layer parameter inbandCarrierInfo is present, or on an NB-IoT carrier for SystemInformationBlockType1-NB for which sib1-carrierInfo indicates non-anchor and the value of the higher layer parameter sib-GuardbandInfo is set to sib-GuardbandInbandSamePCI or sib-GuardbandinbandDiffPCI, DwPTS in special subframe configuration 0 and 5 for normal cyclic prefix is not used for NPDCCH and NPDSCH transmission. DwPTS in special subframe configuration 9 for normal cyclic prefix is not used for NPDSCH transmission with 16QAM, when NPDSCH-16QAM-Config-NB is configured. - Higher-layer parameter symbolBitmap does not apply to special subframes.

- Conclusion

In this contribution, we proposed a text proposal to implement the agreement on use of DwPTS.

Proposal 1: Endorse the text proposal in section 2 to TS 36.211.

- References

RP-201306, “WID revision: Additional enhancements for NB-IoT and LTE-MTC”, Huawei, HiSilicon, RAN#88e, E-meeting, June 2020.