3GPP TSG RAN WG1 Meeting#109-e		R1-2203227

e-Meeting, May 9th – May 20th , 2022

Agenda item:	8.12.1

Source:	TD Tech, Chengdu TD Tech

Title:	Open issues on reliability for NR MBS

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. In the contribution, some open issues on the reliability for NR MBS are discussed with some proposals suggested for approval.

- Open issues on reliability for NR MBS

In the last RAN1 meeting, the following agreements were made with some open issues or FFS items left for further discussion. These open issues or FFS items are discussed in the section.

Agreement 1

For Type-1 codebook generation,

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, FFS between the 3 alternatives below:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

  - Other alternatives are not precluded

Agreement 2

If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

  - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

Agreement 3

When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

  - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

  - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

- FFS: details of the UL DAI field

Agreement 4

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

For agreement 1, we think Alt 2 is better because it can provide the true ACK/NACK information for all G-RNTIs compared with Alt1 and has less configuration limitation on the different G-RNTIs compared with Alt3.

Therefore, the following proposal is suggested.

Proposal 1: For Type-1 codebook generation, the following solution is suggested.

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, Alt 1 is used:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

For agreement 2，there are three alternatives for counting the total number of HARQ-ACK bits for all configured G-RNTIs. We think Alt1-1 is better than the other alternatives because it can provide the entire ACK/NACK information for all G-RNTIs. Therefore, the following proposal is suggested.

Proposal 2: If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted with Alt1-1 used to indicate the sum of DL-DAIs.

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

  - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

For agreement 3, the UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. But how to count the total number of HARQ-ACK bits for all configured G-RNTIs is not decided yet. There are three alternatives. We think Alt1-1 is better because it can provide the entire ACK/NACK information for all G-RNTIs. Therefore, the following proposal is suggested.

Proposal 3: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. Furthermore for the FFS item, Alt1-1 is suggested to count the total number of HARQ-ACK bits for all configured G-RNTIs.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

  - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

  - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

- FFS: details of the UL DAI field

For agreement 4, because the HARQ feedback is disabled for multicast SPS scheduling, we think there’s no need for UE to report any HARQ-ACK information for G-CS-RNTI. Therefore, the following proposal is suggested.

Proposal 4: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, for the FFS item with five alternatives, Alt4 is suggested.

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

- Conclusion

Based on the discussion in the above section, the following proposals are suggested for approval.

Proposal 1: For Type-1 codebook generation, the following solution is suggested.

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, Alt 1 is used:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

Proposal 2: If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted with Alt1-1 used to indicate the sum of DL-DAIs.

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

  - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

Proposal 3: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. Furthermore for the FFS item, Alt1-1 is suggested to count the total number of HARQ-ACK bits for all configured G-RNTIs.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

  - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

  - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

- FFS: details of the UL DAI field

Proposal 4: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, for the FFS item with five alternatives, Alt4 is suggested.

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

- Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e

- R1-2101637, Study on the reliability for RRC_CONNECTED UEs, Chengdu TD Tech, TD Tech, RAN1 Meeting #104-e

- R1-2107951, Reliability related discussion for RRC_CONNECTED UEs, Chengdu TD Tech, TD Tech, RAN1 Meeting #106-e