3GPP TSG RAN WG1 Meeting#109-e		R1-2203228

e-Meeting, May 9th – May 20th , 2022

Agenda item:	8.12.4

Source:	TD Tech, Chengdu TD Tech

Title:	Discussion on typical configuration for broadcast mode

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. Based on the agreements on NR MBS, broadcast mode is used to provide a broadcast session. Before the broadcast sessions can be provided in the predefined area covered by the intra-frequency network, the MBS related network planning for broadcast mode will be done. In the contribution, the discussion on the MBS related network planning for broadcast mode is made and the typical configuration for broadcast mode is suggested.

- MBS related network planning for broadcast mode

Before broadcast sessions can be provided in the predefined area covered by the intra-frequency network, the MBS related network planning for broadcast mode shall be executed as below.

- Determine the bandwidth requirement for broadcast sessions which is applied in each cell within the predefined area.

- Configure one CFR for broadcast mode according to the bandwidth requirement for broadcast sessions, with the CFR applied in each cell with the predefined area.

  - The CFR can be CORESET 0, the initial DL BWP or contain the initial DL BWP. The CFR has the same numerology as the initial DL BWP if the CFR contains the initial DL BWP.

  - Determine the PDCCH configuration information for the CFR where the PDCCH configuration information is used to configure the CORESETs and search spaces for the CFR. The CORESETs/search spaces for the CFR are used to schedule broadcast sessions.

  - Determine the PDSCH configuration information for the CFR where the PDSCH configuration information is used to configure the PDSCH specific parameters for the CFR. The PDSCH specific parameters for the CFR are used to transmit the PDSCH which carries a broadcast session.

Based on the above MBS related network planning for broadcast mode, the CFR for broadcast mode can be area specific. Furthermore, the PTM configuration information of a broadcast session may be area specific, which means the MBS session has the same PTM configuration information in each cell providing the MBS session. If the PTM configuration information of a broadcast session is area specific, the UE receiving the MBS session can use the PTM configuration information acquired in the source cell to receive the MBS session in the target cell, which can reduce the broadcast session reception interruption time in the target cell.

In general, before receiving the broadcast session in the target cell, the UE needs to read MIB, SIB1, SIB2, SIB x carrying MCCH configuration information and MCCH in the target cell, which will lead to the broadcast session reception interruption in the target cell. If the source cell and target cell have the same PTM configuration information, UE only needs to read MIB, SIB1 and SIB2 in the target cell and then continue to receive the broadcast session in the target cell using the PTM configuration information acquired in the source cell. In order to save the radio resource in Uu, the SIBs other than SIB 2 usually have the period of 320ms/640ms in the commercial network, therefore the broadcast session reception interruption time will be reduced greatly if the source cell and target cell have the same PTM configuration information. Based on the MBS related network planning, it’s easy to make the PTM configuration information of a broadcast session area specific.

Based on the above discussion, the following proposal is suggested for broadcast mode.

Proposal 1: Support the following typical configuration for broadcast mode to simplify the broadcast session reception during the UE mobility:

- The PTM configuration information of a broadcast session can be area specific. In other word, the source cell and target cell can have the same PTM configuration information for a broadcast session.

Proposal 2: Send an LS to RAN2 with the following information included:

- In order to reduce the broadcast session reception interruption time, for a broadcast session in the source cell and for each neighbour cell providing the broadcast session, RAN1 expects an additional bit is added to indicate whether or not the neighbour cell has the same PTM configuration information as the source cell.

- Conclusion

Based on the discussion in the above sections, the following proposals are suggested.

Proposal 1: Support the following typical configuration for broadcast mode to simplify the broadcast session reception during the UE mobility:

- The PTM configuration information of a broadcast session can be area specific. In other word, the source cell and target cell can have the same PTM configuration information for a broadcast session.

Proposal 2: Send an LS to RAN2 with the following information included:

- In order to reduce the broadcast session reception interruption time, for a broadcast session in the source cell and for each neighbour cell providing the broadcast session, RAN1 expects an additional bit is added to indicate whether or not the neighbour cell has the same PTM configuration information as the source cell.

- Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e.

- R1-2107953, Other aspects for NR MBS, Chengdu TD Tech, TD Tech, RAN1 Meeting#106-e