3GPP TSG-RAN WG1 Meeting #109-e	R1-2203229

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.1.2

Source:	Ericsson

Title:	On CSI enhancements for Rel-18 NR MIMO evolution

Document for:	Discussion, Decision

# 1	Introduction

In [1], the Rel-18 work item for NR MIMO evolution was agreed. The following two objectives of the work item concern CSI enhancements:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

…

4. Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss the CSI enhancement for high and medium UE velocities and coherent JT.

# 2	TRS-based time-domain channel property reporting

It has been observed by measurements and performance logging in real deployments that downlink MU-MIMO precoding performance degrades when one or more of the co-scheduled UEs start to move faster than a few km/h. The reason is that the information of the channels, used to compute the precoding, becomes outdated rather soon when this occurs.

Although the Rel-18 WID includes an objective on Rel-16/Rel-17 Type-II codebook refinement for high/medium UE velocities, this may only be applicable to advanced, higher complexity UEs that support further Type-II extensions beyond what is supported in Rel-16/Rel-17.  It should be noted that Type-II CSI computation is highly complex already in current releases up to Rel-17, including measurements of up to 32 CSI-RS ports, and adding also time domain information extraction on top of this makes the complexity for such UEs even higher.

The motivation for TRS-based time-domain channel property reporting is to give gNB important information about the variablity of the channel without costing too much computational effort from the UE side (i.e., as opposed to the case of UEs supporting Rel-16/Rel-17 Type-II codebook refinement discussed above). Doppler spread changes slowly over time, and hence, reporting of doppler information may not need to be so frequent. The goal is to utilize the TRS and extract TRS measurement information from UE side with low overhead and low complexity comparing to type II CSI report.

The UE reporting of time-domain channel property measured based on TRS can have multiple uses on the network side:

- Using the UE report, the network may determine the periodicity for configuration or time for triggering of CSI-RS, CSI report or SRS.

- Using the UE report, the network determines whether to configure Type I or Type II CSI report.

- Using the UE report, the network can determine whether to use reciprocity based CSI acquisition or type II based CSI feedback for MU-MIMO scheduling

- Using the UE report, the network can determine the number of additional DMRS symbols needed.

- The network uses the report as an input to open loop link adaptation algorithms to adjust the selection of robust MCS, also for URLLC cases.

- The network uses the report as an input to AI/ML algorithm in the higher layers of the network or for beam management.

Doppler measurements may in principle be performed by the gNB based on the uplink. Phase incoherency of the SRS across different slots does, however, prohibit high accuracy measurements. The link-budget is worse for the uplink, which also reduce the measurement accuracy.  In addition the configuration of an UL SRS consumes precious uplink resources. The use of UE Doppler measurements is therefore the best solution. In fact, for some use-cases only UE based measurements can be expected to give sufficient measurement accuracy.

Doppler information report can have multiple uses on the network side, and is not only used to support DL MIMO precoding.

Hence, the information obtained by the TRS based reporting should be possible to acquire even if SRS based DL precoding is used or if this information is not used at all for a DL transmission (as exemplified above). Hence, this report quantity should not be tied to a codebook such as eType-II since it is in most cases unnecessary to configure an eType-II report only for acquiring the new TRS based report quantity as the use of this measurement is not related to DL precoding.

The TRS based time domain channel property is a stand alone report quantity that is not necessarily tied to any other report quantity, such as DL codebook based PMI reporting.

Study what report quantity to adopt for the TRS based stand-alone report, based on the use cases enlisted above.

## 2.1	On Doppler reporting quantities

Different doppler reporting quantity or quantities should be evaluated as part of the study. Here we give a few possibilities

Doppler spread: Here there may be multiple alternative definitions based on the Doppler power spectrum, such as e.g. some moment of the Doppler power spectrum or max Doppler shift minus min Doppler shift.

Autocorrelation: The UE measures and reports the autocorrelation of the channel for one or more fixed delays. This is a more direct measure of the variability of the channel than the Doppler spread.

Peak reporting: The UE reports a number of identified peaks in the channel impulse response, including for  each peak the relative Doppler shift (e.g. relative to the Doppler shift of the strongest peak), and relative peak power (e.g. relative to the peak power of the strongest peak) and potentially also additional peak measures such as the peak delay. Note that relative doppler shifts and peak powers give the Doppler power spectrum.

We propose that measurement accuracies for these quantities as well as the resulting performance gains resulting from the use of these quantities are studied and evaluated.

## 2.2	Evaluation assumption

Evaluations should be performed for 3.5GHz with 30kHz subcarrier spacing with 50MHz and 100MHz system bandwidth. The TRS bandwidth should be 20MHz or the full system bandwidth.

Measurement accuracies should be evaluated in LLS. For the LLS evaluation of measurement accuracies a single antenna may be used at both UE and gNB side since the TRS is anyway single port. For the same reason, simple TDL channels can be used. All TDL channels (TDL-A/B/C/D/E) may be used, but with priority on TDL-A. Velocities should range from 0 to 120kmph. Note that some mode switching points can be expected to be at rather low velocities. Delay spread should range from 10ns to 1000ns.

Evaluation criteria for measurement accuracies should be

- RMS error

- Standard deviation

- Bias

The performance of Doppler based mode selection (e.g. Doppler based selection of CSI feedback mode or Reciprocity mode) can also be evaluated based on LLS as described in subsection 2.2.1 below.

Some further proposed link level evaluation assumptions are given in Table 1.

Table 1 Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 100MHz |
| TRS bandwidth | 20MHz, 100MHz |
| Channel model | Alt. 1: TDL channels with uncorrelated antenna elements with first priority on TDL-A while the use of other TDL channels isn’t precluded Alt. 2: CDL channels with first priority on CDL-A while the use of other CDL channels isn’t precluded |
| Delay spread | 10ns, 30ns, 100ns, 300ns, and 1000ns |
| UE velocity | 3km/h, 10km/h, 20km/h, 30km/h, 60km/h, 120km/h |
| Antennas at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) For TRS based Doppler accuracy evaluations a single UE antenna may also be used Other configurations are not precluded. |
| Antennas at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ For TRS based Doppler accuracy evaluations a single gNB port may also be used. Other configurations are not precluded. |
| Link adaptation | For TRS based Doppler accuracy: Not applicable For mode selection performance: Adaptation of both MCS and rank. |
| Evaluation metrics for measurement accuracies | RMS error, Standard deviation, Bias |
| Evaluation metric for Doppler based mode selection | User throughput |

If needed SLS could also be used to evaluate performance gains from the use of TRS based Doppler measurements. The scenario may here depend on what the TRS based Doppler measurement is used for. As a baseline we propose, however, the use of the UMa scenario with 500m ISD and 1000m ISD. For large velocities the evaluations should be restricted to outdoor UEs only.

Some further proposed system level evaluation assumptions are given in Table 2.

Table 2 System level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 100MHz |
| TRS bandwidth | 20MHz, 100MHz |
| Scenario | UMa, ISD =500m or ISD=1000m, Outdoor UEs only |
| Antennas at UE | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configurations are not precluded. |
| Antennas at gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ For TRS based Doppler accuracy evaluations a single gNB port may also be used. Other configurations are not precluded. |

### 2.2.1 LLS based evaluation of Doppler based mode selection

The use cases for TRS based Doppler estimation can typically be viewed as selecting one of two modes based on the reported Doppler quantity relative to some threshold, i.e.:

Doppler_quantity>threshold →switch to mode 2

Doppler_quantity<threshold→switch to mode 1

The pair of modes switching is performed between may be e.g.

- SRS based reciprocity mode – CSI feedback based mode

- No additional DMRS symbols – One additional DMRS symbol

- Two additional DMRS symbols – Three additional DMRS symbols

For a given channel characterized by UE velocity v, we can use LLS to estimate the probability pv,CSI of selecting the CSI feedback mode P(CSI) and the probability pv,SRS of selecting the SRS reciprocity mode P(SRS). Obviously, we have

pv,CSI=1-pv,SRS

We can of course also use LLS to estimate the user throughput thptvL,mode k, for different UE velocities v, and modes k.

Let vB be the UE velocity for which the mode 1 (e.g. CSI feedback mode)  and mode 2 (e.g. SRS reciprocity mode) give the same user throughput and let vL <vB<vH , i.e.

thptvL,mode 1<thptvL,mode 2

thptvB,mode 1=thptvB,mode 2

thptvH,mode 1>thptvH,mode 2

For mode selection, we get the user throughput as

thptvL,mode selection=pvL,mode 1∙thptvL,mode 1+pvL,mode 2∙thptvL,mode 2

thptvH,mode selection=pvH,mode 1∙thptvH,mode 1+pvH,mode 2∙thptvH,mode 2

Alternatively, mode selection can be implemented in the link simulator to give the user throughput using mode selection directly.

To get a measure of how well the mode selection performs we can calculate the average throughput for the two UE velocities

average_thptmode selection=thptvL,mode selection+thptvH,mode selection2=12pvL,mode 1∙thptvL,mode 1+pvL,mode 2∙thptvL,mode 2+pvH,mode 1∙thptvH,mode 1+pvH,mode 2∙thptvH,mode 2

This can be used to compare different mode selection methods.

The different mode selection methods may also be compared to ideal mode selection

average_thptideal mode selection=thptvL,mode 1+thptvH,mode 22

as well as to random mode selection

average_thptrandom mode selection=thptvL,mode 1+thptvL,mode 2+thptvH,mode 1+thptvH,mode 24

or fixed mode average throughputs

average_thptmode 1=thptvL,mode 1+thptvH,mode 12

average_thptmode 2=thptvL,mode 2+thptvH,mode 22

This evaluation scheme could in principle be generalized to more than two UE velocities and/or to a probability distribution over UE velocities, but we think that two velocities with equal probability should do fine.

# 3	Rel-16/Rel-17 Type II codebook refinement for high/medium UE velocities

The Rel-16 Type II regular codebook has the following structure

W=W1 W2WfH,

where W1 is a spatial domain (SD) compression matrix, Wf  is a frequency domain (FD) compression matrix, and W2 is a matrix of linear combining coefficients. The Rel-16 and Rel-17 Type II port selection codebooks have the same structure but, in this case, W1 is a port selection matrix.

The WID proposes ‘Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis’. In our view, this means that the W1 and Wf  matrices should be considered fixed, i.e., they should not change over the time instances that are used for exploiting the time/Doppler-domain information. This can be motivated by the fact that the angles and delays of channel clusters change relatively slowly with time. Hence, we regard the codebook refinement should capture the small-scale fading within the channel stationarity time.

The below are a few example codebook refinements that can be studied in Rel-18:

- Introduction of Doppler domain (DD) or time domain (TD) basis and extending the structure of the W2 coefficient matrix such that the elements correspond to the weights of the spatial beams, the FD bases and the introduced DD/TD bases to model the time evolution of W.

- reporting of multiple W2, each of them estimated using different CSI-RS instances suitably separated in time.  W2 can further be approximated as reporting of single  W2 and delta information of the subsequent W2.

- reporting of Doppler information or time domain information along with a single W2

Study the following enhancements on rel-16 and rel-17 type II feedback targeting high/medium UE velocities:

- (a) introduction of Doppler domain or time domain basis and extending structure of W2.

- (b) reporting of a single W2  and delta information corresponding to subsequent W2’s

- (c) reporting of Doppler information or time domain information along with a single W2

We note that the motivation of this enhancement is to use the enhanced feedback to predict the PMI at a future time at which the gNB will schedule the UE potentially MU-MIMO paired with other UEs.  As for the prediction, it can be performed either at the gNB or at the UE, and both these possibilities can be considered in Rel-18 evaluations.

Consider both gNB based and UE based CSI prediction for Rel-18 Type II Doppler evaluations

The current CSI-RS configuration may not be suitable for deriving time domain/Doppler information related to Rel-16/Rel-17 Type II codebook refinement.  This is because only a single CSI-RS sample is possible per resource within a CSI-RS periodicity or when CSI-RS is triggered aperiodically in the current CSI-RS resource configuration. In order for the UE to compute time domain/Doppler information and extend Rel-16/Rel-17 Type II codebook for high/medium UE mobility scenarios, CSI-RS configurations allowing multiple CSI-RS samples may need to be studied.  The number of samples needed also needs to be investigated for deriving accurate Doppler information (e.g., for deriving the Doppler domain basis vectors).  It can be further studied whether the multiple samples are contained within a slot or spread across multiple slots.

Study multiple sample CSI-RS configuration for accurate derivation of doppler information needed for rel-16 and rel-17 type II codebook refinement targeting high/medium UE velocities.

## 3.1	Evaluation assumption

### 3.1.1 System level simulations

For SLS, the Rel-16 and Rel-17 EVM assumptions are appropriate to evaluate improvements of the Rel-16 and Rel-17 Type II codebooks, i.e. using Rel-16 eType II and Rel-17 PS feType II codebooks as baseline, respectively. In addition, UE distributions with only outdoor UEs with medium to high velocity, e.g., 3 km/h, 10 km/h, 20km/h, 30km/h, 60 km/h and 120 km/h, may be appropriate to add to the Rel-16/Rel-17 EVM assumptions listed below.

The Rel-16/Rel-17 EVMs does not need to be enhanced with spatially consistent channel modelling and physical UE movement as this would require changing spatial and frequency basis which is contrary to what is stated in the WID.

Table 3 EVM assumptions for Rel-16 eType II codebook refinement for high/medium UE velocities (Modifications to Rel-16 EVM assumptions shown in green)

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 4GHz. |
| Inter-BS distance |  | 200m |
| Channel model |  | According to the TR 38.901 Note: Spatial consistency modelling with physical UE movement is not considered |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configuration is not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) - 100% outdoor (3 km/h,10km/h, 20 km/h, 30 km/h, 60km/h,120km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 eType II Codebook is the baseline for performance and overhead evaluation for overhead reduction. (Type I Codebook can be considered at least for performance evaluation) - Companies are encouraged to compare the proposed overhead reduction scheme with Rel-16 overhead reduction scheme, Rel-15 Type I Codebook is the baseline for performance and overhead evaluation for higher rank codebook. |

Table 4 EVM assumptions for Rel-17 PS feTypeII codebook refinement for high/medium UE velocities (Modifications to Rel-17 EVM assumptions shown in green)

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) is a baseline. Other scenarios (e.g. UMi@4GHz 2GHz, Urban Macro) are not precluded. |
| Frequency Range |  | FR1 only, 2GHz with duplexing gap of 200MHz between DL and UL, optional for 4GHz |
| Inter-BS distance |  | 200m |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded. |
| BS Tx power |  | 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz for 15kHz as a baseline (optional for 10 MHz with 15KHz), and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | For low RU, SU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 70% for SU/MU-MIMO with rank adaptation - 20% for SU-MIMO with rank adaptation Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) - 100% outdoor (3 km/h,10km/h, 20 km/h, 30 km/h, 60km/h,120km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-17 PS feTypeII Codebook is the baseline for performance and overhead evaluation. (Type I Codebook can be considered at least for performance evaluation) - Note that it is encouraged to disclose further details of beamforming mechanism/ordering over CSI-RS ports/resources. |

### 3.1.2 Link level simulations

System level simulations provide an insight into the benefits of Type II precoder prediction on a system level by capturing the diversity in spatial-temporal properties across many links. However, prediction entails measuring time variations of the channel between a TRP and a UE within a small timescale (e.g., 10 slots). Therefore, to understand the limitations of the enhancements, impact of SNR on parameter estimation and prediction we propose to include link level simulations using a few representative geometric channel models, for instance, CDL channel models of 3GPP TR 38.901. This allows us to understand the influence of delay and spatial separation of clusters, and angular and delay spreads of clusters on the quality of prediction. Additionally, link level simulations of a single link over a CDL channel can be used for calibration of results.

Throughput vs. SNR results using different mobility can be used to assess the benefits of the enhancements.

Consider link level simulations to study the enhancements on rel-16 and rel-17 type II feedback targeting high/medium UE velocities.

Table 5 Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplexing | FDD |
| Carrier frequency | 2 GHz, 3.5 GHz |
| Sub-carrier spacing (SCS) | 15 kHz, 30 kHz |
| Allocation | 10 MHz, 20 MHz bandwidth |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 symbols |
| CSI-RS | Periodic CSI-RS or a periodic bursts of CSI-RS |
| DMRS | Type 1, 1+1 additional or 1+2 additional configuration FDM with data on unused subcarriers |
| TRS | Periodicity of 20 ms, 2-slot pattern |
| Channel model | CDL channel models in 38.901 |
| Vehicle speed | 3 km/h, 10 km/h, 20km/h, 30km/h, 60 km/h and 120 km/h |
| gNB antenna | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded. |
| UE antenna | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configurations are not precluded. |
| Rank | 1–4 |
| Modulation and coding scheme | Adaptive with 256-QAM MCS table |
| HARQ retransmissions | Maximum of 3 retransmissions |
| Channel estimation | Realistic |

# 4	Rel-15/Rel-16 Type II codebook refinement for CJT multi-TRP

## 4.1  Extension of Rel-16/17 Type II CB to  CJT

To further enhance multi-TRP PDSCH transmission, coherent joint transmission (CJT) from multiple TRPs will be studied in NR Rel-18, in which signals from multiple TRPs are supposed to be coherently combined at a UE through proper joint antenna precoding at the multiple TRPs.  This is to be achieved by extension of Rel-16 and Rel-17 type II CSI feedback in which the UE measures the channels associated with the multiple TRPs and reports back either a joint precoder W=W1W2WfH per layer across the multiple TRPs.  In case of joint precoder, co-phasing is naturally achieved through W2 .

For CJT CSI feedback, the UE can be configured with S CSI-RS resources where the UE measures the channel corresponding to each of S TRPs using one of the S CSI-RS resources.  An example with S=3 is shown in Figure 1.  The channel measurements corresponding to CSI-RS resources 1, 2, and 3 are respectively denoted as H1, H2, and H3.  Note that measured channel Hs corresponds to the channel measured between the UE and a TRP.

For rel-16 and rel-17 type II feedback extension for CJT, consider multiple CSI-RS resources (i.e., one resource per TRP) for channel measurement.

Figure 1: An example showing different channels being measured on different configured NZP CSI-RS resources for CJT CSI feedback

For Rel-16 Type-II codebook For each MIMO layer, the precoder matrix is given by a size-P×N3 matrix

W=w0 …w(N3-1)=W1W2WfH,

where

- P=s=1SPs  is the total number of CSI-RS ports in all the S NZP CSI-RS resources, where Ps  is the number of CSI-RS ports in the sth NZP CSI-RS resource.

- W1=diag(W11,…, W1S) is size-P×2L  block diagonal spatial compression matrix, where L is the total number of selected beams associated with all the S NZP CSI-RS resources, where

  - W1s=v1s,…,vLss 00v1s,…,vLss (s=1,…,S) is a size Ps×2Ls precoding matrix associated with the sth NZP CSI-RS resource,

  - {v1s,…,vLss} is a set of size Ps/2×1 selected orthogonal spatial DFT vectors associated with the sth NZP CSI-RS resource,

  - and L=s=1SLs.

- Wf is size-N3×Mtotal  frequency compression matrix, where Mtotal is the total number of selected FD basis vectors out of the N3 orthogonal FD DFT basis vectors {f0 f1…fN3-1} for all S NZP CSI-RS resources, where fk is a size-N3×1 frequency domain DFT vector

As part of the extended Rel-16 Type-II feedback, W1=diag(W11,…, W1S) is reported by the UE.  The feedback of W1S s=1,…,S comprises feeding back the Ls precoding vectors {v1s,…,vLss}.  It can be further studied whether the number of precoding vectors Ls need to be TRP specific or common for all TRPs (i.e., Ls=L).

For Type-II codebook extension, study whether the number of spatial beams or ports selected need to be TRP-specific or common for all TRPs.

For the frequency compression matrix Wf, there are two possiblities:

- Option 1:  separate FD DFT basis vectors per TRP (i.e., Wf=fk0(1) fk1(1) …fkM-1(1) …fk0,l(S) fk1,l(S) …fkM-1,l(S), where fk0(s) fk1(s) …fkM-1(s) are the FD DFT basis vectors corresponding to the sth TRP.

- Option 2:  common FD DFT basis vectors for all TRPs (i.e., Wf=fk0 fk1 …fkM-1).

Performance-overhead tradeoffs can be studied for both the above options.

For Type-II codebook extension, study whether the FD DFT basis vectors need to be TRP-specific or common for all TRPs.

## 4.2  Practical Issues for CJT

There are a number of challenges in supporting CJT in practice.  Firstly, propagation delays between different TRPs and a UE can be different. These large delay differences would result in a large frequency selective composite channel. In existing NR Type II CSI feedback, a precoding matrix per PMI subband is reported.  The PMI subband size can vary between 2 RBs to 32RBs as specified in TS38.214.  Figure 2 shows the maximum delay spreads that can be handled by Rel-16/17 type II CSI as a function of PMI subband size and SCS. Therefore, with a few us delay difference between TRPs, it cannot be handled even with the minimum PMI subband size of 2 RBs.

The maximum channel delay spread that can be handled by Rel-16/17 type II CSI is a function of PMI subband size and SCS

Even at the minimum PMI subband size, the maximum delay spread that can be handled by Rel-16/17 type II is rather small.

- 15kHz SCS

- 30kHz SCS

Figure 2: Maximum channel delay spreads that can be handled by Rel-16/17 type II CSI.

Figure 3 shows a simple example where a UE receives a signal transmitted from two TRPs with various delay differences with co-phasing being done per sub-carrier, per RB, per PMI subband of 2 RBs, and without co-phasing (SFN). It can be seen that with per PMI subband co-phasing, the signal degrades as the delay difference increases.  CJT does not work at all when the delay difference approaches the limit of 2.78us.

Figure 3: An example of CJT performance with co-phasing being done per subcarrier, per RB, per PMI subband, and without co-phasing (SFN) for various delay differences between two TRPs.

CJT does not work well in presence of some modest delay differences between TRPs even with the minimum PMI subband size of 2RBs.

Secondly, even though a same nominal transmit frequency may be assumed at multiple TRPs, due to local oscillator stability, there will be some actual transmit frequency difference between the multiple TRPs.  In 3GPP RAN4, the maximum transmit frequency error for a base station is specified in TS38.104 and is copied below. For the most stringent +/-0.05ppm requirement, there will be some residual frequency errors.  These frequency errors means that the relative phase of the signal received from different TRPs will change over time.   Figure 5 shows phase variations over 5ms time period with +/-0.05ppm at different nominal carrier frequencies. It can be seen that even at 1GHz carrier frequency, the relative phase can change up to 180 degrees over 5ms time period. In other words, the time between a CSI is measured and the CSI is applied needs to much less than 5ms. This is not practical in most cases.

With existing RAN4 BS frequency error minimum requirement, CJT may not be achieved even at 1GHz carrier frequency due to phase change over time as a result of frequency error

Figure 4: 3gpp minimum requirements on base station transmit frequency error.

Figure 5: phase change due frequency errors (+/-0.05ppm)

Furthermore, for CJT to work, base station needs to maintain phase coherency over time.  Therefore, we have the following proposals

Study on the impact of practical delay differences in the links between a UE and the different TRPs on CJT and possible mitigation schemes.

Study on the impact of practical frequency differences between TRPs on CJT and possible mitigation schemes.

## 4.3	Evaluation assumption

Table 6 EVM assumptions for CJT with Rel-16/17 type II CB extension

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD (TDD is not precluded), OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | - Dense Urban (Macro only) - Urban Macro - Indoor - Rural Intra- and inter-site with up to 4 TRPs |
| Frequency Range |  | FR1 only 4GHz for Dense Urban, Urban Macro, and Indoor 700MHz for Rural |
| Inter-BS distance |  | 200m for Dense Urban 500m for Urban macro 1.732km for Rural 20m Indoor |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 16 ports: (4,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 4 ports: (4,2,2,1,1,1,2), (dH,dV) = (0.5, 0.8)λ Other configuration is not precluded. |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Type II overhead reduction Other configuration is not precluded. |
| BS Tx power |  | Dense urban: 41+x dBm Urban macro: 44 +x dBm Rural: 46dBm Indoor: 24dBm x = 10log10(BW (in MHz)/10) |
| BS antenna height |  | 25m for Dense Urban and Urban macro 35m for Rural 3m for Indoor |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Number of RBs |  | 52 for 15 kHz SCS |
| Simulation bandwidth |  | 10 MHz for 15kHz as a baseline, and configurations which emulate larger BW, e.g., same sub-band size as 40/100 MHz with 30kHz, may be optionally considered. |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline for overhead reduction. For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed for higher rank extension. For medium/high RU, SU/MU-MIMO with rank adaptation is assumed for higher rank extension. |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12) |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback) : 5 ms, - Scheduling delay (from CSI feedback to time to apply in scheduling) : 4 ms |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes Other FTP model is not precluded. |
| Traffic load (Resource utilization) |  | - 50/70 % for CSI overhead reduction - 20/50 % for high rank extension Companies are encouraged to report the MU-MIMO utilization. |
| UE distribution |  | - 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic |
| Evaluation Metric |  | Throughput and CSI feedback overhead as baseline metrics. Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used. Maximum overhead (payload size for CSI feedback)for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics. |
| Baseline for performance evaluation |  | Rel-16 or Rel-17 Type II Codebook based per TRP CSI feedback and single TRP scheduling is the baseline for performance. |

# 4.4 Initial SLS results for CJT

Some initial evaluation results for CJT for the indoor scenario is given in Table 6.  Note that these are our initial evaluation results only as these results assume ideal CSI.  The evaluation results are intended to show the upper bound potential of CJT under ideal assumptions (assumptions for these results are shown in Table 7).

Table 6. Initial CJT evaluation results for Indoor scenario

| User throughput gains of CJT over single TRP transmission |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20% RU | 50% RU | 70% RU | Full-Buffer |
| Mean | 1% | 11% | 28% | 27% gains |
| Cell-edge | 5% | 42% | 80% | 57% gains |
| 50% | 0% | 13% | 35% | - |
| 95% | 0% | 1% | 2% | - |

Table 7. Evaluation assumptions for initial CJT evaluation results for Indoor scenario

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Indoor |
| Frequency Range | FR1 @ 4GHz |
| Antenna setup and port layouts at gNB | 8TX: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.5)λ |
| Antenna setup and port layouts at UE | 8RX: (2,2,2,1,1,2,2), (dH,dV) = (0.5, 0.5)λ |
| BS antenna height | 3m |
| Simulation bandwidth | 10 MHz with 15KHz of scs |
| Cluster size for CJT | Cluster 4 sectors in 4 sites |
| MIMO layers | 60 maximum MU-MIMO layers |
| CSI feedback | Ideal CSI feedback |
| Traffic model | Full buffer and FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) | 50/70% for SU/MU-MIMO |

# 3.3	Conclusion

In the previous sections we made the following observations:

Observation 1	Doppler information report can have multiple uses on the network side, and is not only used to support DL MIMO precoding.

Observation 2	The maximum channel delay spread that can be handled by Rel-16/17 type II CSI is a function of PMI subband size and SCS

Observation 3	Even at the minimum PMI subband size, the maximum delay spread that can be handled by Rel-16/17 type II is rather small.

Observation 4	CJT does not work well in presence of some modest delay differences between TRPs even with the minimum PMI subband size of 2RBs.

Observation 5	With existing RAN4 BS frequency error minimum requirement, CJT may not be achieved even at 1GHz carrier frequency due to phase change over time as a result of frequency error

Based on the discussion in the previous sections we propose the following:

Proposal 1	The TRS based time domain channel property is a stand alone report quantity that is not necessarily tied to any other report quantity, such as DL codebook based PMI reporting.

Proposal 2	Study what report quantity to adopt for the TRS based stand-alone report, based on the use cases enlisted above.

Proposal 3	Study the following enhancements on rel-16 and rel-17 type II feedback targeting high/medium UE velocities:

(a) introduction of Doppler domain or time domain basis and extending structure of W2.

(b) reporting of a single W2 and delta information corresponding to subsequent W2’s

(c) reporting of Doppler information or time domain information along with a single W2

Proposal 4	Consider both gNB based and UE based CSI prediction for Rel-18 Type II Doppler evaluations

Proposal 5	Study multiple sample CSI-RS configuration for accurate derivation of doppler information needed for rel-16 and rel-17 type II codebook refinement targeting high/medium UE velocities.

Proposal 6	Consider link level simulations to study the enhancements on rel-16 and rel-17 type II feedback targeting high/medium UE velocities.

Proposal 7	For rel-16 and rel-17 type II feedback extension for CJT, consider multiple CSI-RS resources (i.e., one resource per TRP) for channel measurement.

Proposal 8	For Type-II codebook extension, study whether the number of spatial beams or ports selected need to be TRP-specific or common for all TRPs.

Proposal 9	For Type-II codebook extension, study whether the FD DFT basis vectors need to be TRP-specific or common for all TRPs.

Proposal 10	Study on the impact of practical delay differences in the links between a UE and the different TRPs on CJT and possible mitigation schemes.

Proposal 11	Study on the impact of practical frequency differences between TRPs on CJT and possible mitigation schemes.

# References

RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021