# 3GPP TSG RAN WG1 #109-e		 	                                   R1-2203231

# e-Meeting, May 9th – May 20th, 2022

# Source:	ZTE

# Title:	Remaining issues on NR-NTN

# Agenda Item:	8.4

# Document for:  Discussion

Introduction

In RAN#95e [1], the following remaining issues are identified for RAN1, which need to be discussed in maintenance phase.

- Details on UE behaviour w.r.t Validity timer expiry

- Support of negative TACommonDriftVariation values for GEO

- Resolving the ambiguity in interpretation of SFN indicating Epoch time

In this contribution, the above issues are elaborated with corresponding analysis.

UE behaviour w.r.t Validity timer expiry

  - Validity time definition

              - In RAN1#108e [2], the validity time of common TA and ephemeris was discussed and following two types of definitions are considered.

- The epoch time tepoch is the start of validity duration. The UL synchronization is thought kept only in the duration 0≤t-tepoch<∆t, where ∆t is the validity duration length.

- The epoch time is the middle point of validity time. The UL synchronization is thought kept in the duration 0≤t-tepoch<∆t, where ∆t is the validity duration length.

              - For definition (1), the backward propagation from epoch time is not considered so that the epoch time should not be set at far future. For definition (2), the epoch time should be at future to enable enough time for backward propagation.

              - In above two definitions, definition (1) should be adopted if fitting is considered in determination of common TA parameters. In our understanding, the motivation of definition (2) is that the common TA error is assumed to be minimized at epoch time and will increase when a time instant is far from epoch time. From this point of view, backward propagation should have similar validity duration as forward propagation. However, with fitting, the indicated common TA parameters are not exactly equal to the real common TA parameters at epoch time. The residual TA error is not minimized at middle point of validity time and will not monotonously increase when the time instant is far from the middle point. Therefore, there is no benefit to define the epoch time as the middle point of the validity time. For example, consider the case shown in Table 1, the common TA, common TA drift rate, and common TA drift rate variation are obtained by fitting the real common TA curve in [0 s, 30 s] to reduce the residual error (i.e., extend the validity duration). With the obtained common TA parameters, the approximated common TA calculated at UE is evaluated as shown in Figure 1.

              - Table 1 Parameters for evaluation

| - Parameter | - Value |
| --- | --- |
| - Orbit | - LEO-600 |
| - Initial satellite position | - Above Gateway |
| - Time period for common TA fitting | - [0, 30] s |
| - Fitting method | - Least square fitting |

- Real and approximated common TA           (b) Residual error of approximated common TA

Figure 1 Evaluation of common TA fitting

              - From the Figure 1, it can be easily found that the fitted common TA parameters are not equal to the real ones at t=0. Therefore, the validity of common TA parameters cannot be guaranteed for backward propagation from t=0. In this case, the definition (1) should be adopted. Of course, one may argue that the epoch time can be set at middle point of fitting time period (i.e., t=15 in above case) to enable the definition (2), where forward and backward propagation can have same validity duration length. However, because of SFN wrap around, the epoch time cannot be set at a time that is 10.24 s far from the reception time. That is, the validity duration for backward propagation is at most 10.24 s. If the total validity time is longer than 20.48 s (e.g., 30 s in above case), setting the epoch time at the middle point will reduce the overall validity duration (e.g., reduced to 25.24s in above case). Overall, the epoch time should be set as the start of validity time period at least with consideration of following aspect:

- When curve fitting is adopted to extend validity duration of common TA, the validity of backward propagation cannot be guaranteed.

- The validity duration of backward propagation is limited by SFN wrap around.

      - Proposal 1: The epoch time tepoch should be set as the start of validity time period. The UL synchronization is thought kept only in the time period 0≤t-tepoch<∆t, where ∆t is the validity duration length.

  - UE behavior

              - Since the epoch time is the start of validity time period and no backward propagation is guaranteed, UL synchronization cannot be ensured between expiry of previous validity timer and new epoch time even if the new assistance information has already been received before the expiry. Hence, there is no need to define the new behaviour of suspending validity timer when new assistance information is received, which introduced to maintain UL synchronization before new epoch time. And the epoch time should not be set at far future.

              - Based on above analysis, the legacy UE behaviors are enough, i.e., the validity timer start/restart at epoch time and UL synchronization is thought lost when validity timer is expired. And UE shall apply new assistance information after epoch time. It is up to UE implementation to re-acquire and apply new assistance information before expiry of UL validity timer.

      - Proposal 2: UL synchronization should not be maintained after validity timer expiry.

      - Proposal 3: The UE shall re-acquire and apply new assistance information before expiry of UL validity timer.

Negative TACommonDriftVariation values for GEO

              - In GEO scenario, UEs can be served by the same satellite for a long time, which may require longer validity duration of SI to minimize the overhead for SI update. Therefore, the long validity duration, i.e., 900s as agreed in previous meeting is needed. Meanwhile, during the long period of satellite’s movement, the satellite may experience figure 8 motion if the orbit inclination is nonzero as shown in Figure 2. As a result, there could be negative TACommonDriftVariation values. However, in current agreement, only positive TACommonDriftVariation is defined. If the value range of TACommonDriftVariation is not extended to include negative values, the validity duration of common TA may be much shorter since only linear propagation is ensured.

- GEO with and without inclination           (b) Trajectory of GEO with inclination in ECI

Figure 2 Illustration of figure 8 motion

              - For example, when the inclination angle is 36 degree, the maximum Doppler is about 0.94 ppm, which aligns the 0.93 ppm defined in TR 38.821 Table 4.2-2 [3]. In this case, by assuming the GEO satellite fly over the equator at 0 degree longitude and the NTN-GW locates at (70°N, 0°), the common TA residual error with different indication period is simulated as shown in Figure 3. From the figure, it can be observed that TACommonDriftVariation is necessary to ensure the validity duration of common TA can be up to 900s. Hence, negative TACommonDriftVariation values should be supported. And a simple method is to add an additional bit for TACommonDriftVariation to extend its value range to include negative values.

Figure 3 Evaluation of common TA residual error

      - Proposal 4: Negative TACommonDriftVariation values should be supported to handle the figure 8 motion in GEO.

Interpretation of SFN indicating Epoch time

              - As discussed in previous section, the epoch time should be set as the start of whole validity time period with consideration of common TA curve fitting. That is, backward propagation from epoch time cannot be guaranteed. Therefore, the epoch time should be set at the reception time of assistance information or near future to avoid loss of UL synchronization. In such case, the legacy approach for SIB9 can be reused, i.e., interpreting the subframe indicated by SFN and subframe number as the nearest one to the subframe where the message indicating epoch time is received. With this interpretation, the network can either set the epoch time at past to ensure the UL synchronization is not lost, or set the epoch time at near future to fully utilize the validity duration.

              - Proposal 5: If indicated explicitly by a SFN and subframe number, the Epoch time t_epoch is the sub-frame which is nearest to the sub-frame where the message indicating the Epoch time is received.

Conclusions

In this contribution, analysis on remaining issues for NTN is conducted with following proposals and observations:

      - Proposal 1: The epoch time tepoch should be set as the start of validity time period. The UL synchronization is thought kept only in the time period 0≤t-tepoch<∆t, where ∆t is the validity duration length.

      - Proposal 2: UL synchronization should not be maintained after validity timer expiry.

      - Proposal 3: The UE shall re-acquire and apply new assistance information before expiry of UL validity timer.

      - Proposal 4: Negative TACommonDriftVariation values should be supported to handle the figure 8 motion in GEO.

              - Proposal 5: If indicated explicitly by a SFN and subframe number, the Epoch time t_epoch is the sub-frame which is nearest to the sub-frame where the message indicating the Epoch time is received.

- Reference

- RP-220952 SR WID NR-NTN solution for RAN95-e, RAN#95e

- R1-2202908 FL Summary #4: Maintenance on UL time and frequency synchronization for NR NTN, RAN1#108e

- TR 38.821 Solutions for NR to support non-terrestrial networks (NTN), V16.1.0