# 3GPP TSG RAN WG1 #109-e		 	                                   R1-2203232

# e-Meeting, May 9th – May 20th, 2022

# Source:	ZTE

# Title:	Remaining issues on IoT-NTN

# Agenda Item:	8.14

# Document for:  Discussion

Introduction

In RAN1#108e [1], the following issues are discussed without agreement.

- Details of uplink segment gaps as agreed in RAN1#107-e

- Ambiguity issue of implicit epoch time indication

In this contribution, views on the above issues are elaborated with corresponding analysis.

Uplink segment gap

              - In RAN1#107e [2], how to apply different TAs for different segments was discussed with multiple methods as shown in the agreement below.

Agreement

UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation

1. UE may drop / Insert samples / Puncture OFDM symbols

2. UE may blank subframes / slots where UE skip a slot or a subframe

The total transmission time is not changed

UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot

The method used for the UE pre-compensation is known to the eNB by a single UE capability

UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing)

FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

  - Segmented pre-compensation

              - Firstly, it should be noted that there is following statement in legacy specification.

[3GPP TS 36.133 V16.8.0, Section 7.20.2]

When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

              - That is, the applied TA is not allowed to be adjusted during repetition of same PUSCH or NPUSCH. However, the TA may drift fast in NTN due to high satellite mobility. If the repetition number is large, the TA error may exceed tolerance before finishing the ongoing repetition period, which leads to loss of UL synchronization. Hence, UE should be allowed to adjust the pre-compensated TA per segment to avoid too large timing error. And the details of how to apply the segment pre-compensation should be captured in the specification.

      - Proposal 1: The details of how to apply the segment pre-compensation should be captured in the specification.

              - In order to ensure the segment pre-compensation is supported in RAN1 spec, the TPs in the following proposal should be captured in TS 36.213 to illustrate how UE perform segment pre-compensation for (N)PUSCH and (N)PRACH in NB-IoT and eMTC.

      - Proposal 2: Adopt following TPs in TS 36.213 to ensure the support of segment pre-compensation.

<TP1, Section 4.2.3, TS 36.213>

For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPRACH-r17, the  [36.211] shall be adjusted per segment during the transmission of physical random access preamble.

For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPUSCH-r17, the  [36.211] shall be adjusted per segment during the transmission of physical uplink shared channel.

              - <END TP1>

<TP2, Section 16.1.2, TS 36.213>

If a segment duration is configured by higher layer parameter TransmissionDurationNPRACH-NB-r17, the  [36.211] shall be adjusted per segment during the transmission of narrowband physical random access preamble.

If a segment duration is configured by higher layer parameter TransmissionDurationNPUSCH-NB-r17, the  [36.211] shall be adjusted per segment during the transmission of narrowband physical uplink shared channel.

              - <END TP2>

              - Moreover, as described above, TS 36.133 restricts that UE shall not adjust UL timing during an ongoing repetition period, which blocks the application of segment pre-compensation. Hence, an LS to RAN4 is needed to inform the agreement in RAN1 and the requirement of updates on TS 36.133.

      - Proposal 3: An LS to RAN4 is needed to inform the RAN1 required updates on TS 36.133 to support segment pre-compensation, i.e., allow UE to adjust UL timing during an ongoing repetition period at least in NTN.

  - Capability indication

              - Regarding the capability discussion, based on the approaches defined above, two options can be considered:

- Option-1: Drop samples

In the legacy spec, the UE is assumed to be able to adjust the transmission in case of overlapping as shown below:

[Section 16.1.2, TS 36.213]

When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

However, in NTN, although the above legacy behavior can be applied, the updates on the specification as mentioned 2.1 is also needed. Otherwise, dropping samples to update the transmission timing within the repetition will not be applicable.

- Option-2: Blank symbol/slot/subframe

In implementation, since some UEs are not able to handle the TA adjustment as mentioned in option-1, blanking symbol/slot/subframe may be considered due to lower requirement on UE capability. In this way, potential impacts on the eNB's implementation should be defined with clearly indication (e.g., UE capability). Then eNB can correctly perform UL detection.

              - In RAN1#108e [1], the details of above methods were discussed but no agreement is achieved since there are different understandings on whether to include option-1 in the UE capability report:

- Option-1 is regarded as the default behaviour, i.e., no capability report is needed if UE is able to follow legacy TA adjustment method. If the UE cannot support option-1, it will adopt option-2 and report its capability to network.

- Option-1 is regarded as one type of capability, i.e., UE should report capability even if it can follow the legacy TA adjustment method.

              - In above two understandings, we prefer the first one since it minimizes the impact on legacy specification. There is no need of signaling if UE can perform segment pre-compensation with the method in option-1, which is similar to the legacy behaviour as illustrated above. Only the specification impact of option-2, which is for the UEs without enough high capability, need be focused.

      - Proposal 4: UE should follow legacy behavior at slot boundaries due to TA adjustment if single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH and NPUSCH, when the UE performs segmented pre-compensation, is not indicated.

  - UE behaviour

              - For the methods of blanking symbol/slot/subframe, the detailed UE behaviors on how to achieve blanking should be specified since the resource mapping will be impacted. The blanked symbol/slot/subframe is actually applied in resource mapping but not used for transmission. In legacy specification, such behaviour is generally specified as “a transmission gap shall be counted for the PUSCH resource mapping but not used for transmission”. Therefore, we support the TPs on TS 36.211 proposed by the feature leader of synchronization issue in RAN1#108e [1].

      - Proposal 5: Support the TPs on TS 36.211 proposed by the feature leader of synchronization issue in RAN1#108e, which are as follows:

<TP3, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensationis one subframe, one slot, or one symbol may be  configured by network based on the   quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

<END TP3>

<TP4, Section 10.1.3.6, TS 36.211>

For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the PUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot or 2 slots, or one symbol may be  configured by network based on the quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

<END TP4>

<TP5, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot, or one symbol may be  configured by network based on the quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

              - <END TP5>

Implicit epoch time indication

In RAN1#107e [2], how to indicate the epoch time is discussed and the methods used in NR-NTN is agreed to be reused. And the implicit epoch time indication method is further revised in RAN1#108e [3] as follows:

Agreement

Modify second bullet of RAN1#107-e agreement on Epoch time as follows:

Otherwise, when Epoch time is not explicitly indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the NTN-specific SIB SI message is transmitted.

However, in legacy NB-IoT, UE is allowed to combine SI across multiple SI windows. In such case, if implicit epoch time indication is applied, UE cannot figure out which SI window is used to indicate the epoch time. Hence, in order to avoid such ambiguity without significant specification impact, it is preferred to let the NTN-specific SIB to be updated per SI window.

      - Proposal 6: The NTN-specific SIB is expected to be updated per SI window in IoT-NTN.

Conclusions

In this contribution, analysis on the remaining issues for IoT-NTN is conducted with following proposals and observations:

      - Proposal 1: The details of how to apply the segment pre-compensation should be captured in the specification.

      - Proposal 2: Adopt following TPs in TS 36.213 to ensure the support of segment pre-compensation.

<TP1, Section 4.2.3, TS 36.213>

For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPRACH-r17, the  [36.211] shall be adjusted per segment during the transmission of physical random access preamble.

For a BL/CE UE, if a segment duration is configured by higher layer parameter TransmissionDurationPUSCH-r17, the  [36.211] shall be adjusted per segment during the transmission of physical uplink shared channel.

              - <END TP1>

<TP2, Section 16.1.2, TS 36.213>

If a segment duration is configured by higher layer parameter TransmissionDurationNPRACH-NB-r17, the  [36.211] shall be adjusted per segment during the transmission of narrowband physical random access preamble.

If a segment duration is configured by higher layer parameter TransmissionDurationNPUSCH-NB-r17, the  [36.211] shall be adjusted per segment during the transmission of narrowband physical uplink shared channel.

              - <END TP2>

      - Proposal 3: An LS to RAN4 is needed to inform the RAN1 required updates on TS 36.133 to support segment pre-compensation, i.e., allow UE to adjust UL timing during an ongoing repetition period at least in NTN.

      - Proposal 4: UE should follow legacy behavior at slot boundaries due to TA adjustment if single UE capability that governs UE behavior w.r.t gaps between segments for PUSCH and NPUSCH, when the UE performs segmented pre-compensation, is not indicated.

      - Proposal 5: Support the TPs on TS 36.211 proposed by the feature leader of synchronization issue in RAN1#108e, which are as follows:

<TP3, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensationtime units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensationis one subframe, one slot, or one symbol may be  configured by network based on the   quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

<END TP3>

<TP4, Section 10.1.3.6, TS 36.211>

For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a transmission gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the PUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot or 2 slots, or one symbol may be  configured by network based on the quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

<END TP4>

<TP5, Section 5.3.4, TS 36.211>

For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUCCH resource mapping but not used for transmission of the PUCCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot, or one symbol may be  configured by network based on the quantity  supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps.

              - <END TP5>

      - Proposal 6: The NTN-specific SIB is expected to be updated per SI window in IoT-NTN.

- Reference

- R1-2202915 Summary #4 of AI 8.14.1	Enhancements to time and frequency synchronization, RAN1#108e

- R1-2112803 Summary #4 of AI 8.15.1	Enhancements to time and frequency synchronization, RAN1#107e

- R1-2202908 FL Summary #4: Maintenance on UL time and frequency synchronization for NR NTN, RAN1#108e