3GPP TSG RAN WG1 #109-e                                     R1-2203234

e-Meeting, May 9th – May 20th, 2022

Source:	ZTE

Title:	Discussion on UE features for IoT over NTN

Agenda Item:	8.16.14

Document for:   Discussion

- Introduction

The latest version of UE features list for IoT-NTN is provided in [1]. In this contribution, our views on some features are elaborated with corresponding updates.

- Discussion on the UE feature

- As lists in Table 1, FG 2-1, 2-1b, 2-1a, 2-1c, 2-2, 2-2a, 2-3, and 2-3a are changed.

W.r.t the type column of FG 2-1, 2-1b, 2-1a, 2-1c, 2-2, 2-2a, 2-3 and 2-3a, “per UE” is preferred since more than one bands will be supported for NTN in implementation and it’s strange that UE only support NTN feature for one of them. Therefore, the type column should be updated to [Per UE/per band]

Moreover, in legacy eMTC and NB-IoT, TA adjustment during an ongoing repetition period is not allowed as shown below:

[3GPP TS 36.133 V16.8.0, Section 7.20.2]

When a repetition period is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission as defined above.

However, the motivation of segmented UL transmission is mainly to let UE handle the TA drift during an repetition period. According to the agreements in RAN1#107e [2], the UE is expected to adjust the value for pre-compensation for a segment if a segment duration is configured. Hence, for FG 2-1a and 2-1c, an addition component “Different values (e.g., TA) for pre-compensation may be used per segment” should be added. Otherwise, the benefit of the FG cannot be introduced.

Proposal-1: The updates on the UE features for IoT-NTN listed in Table-1 should be supported.

Table 1 UE features for IoT-NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements 1-1. UE reports the information associated with the validity duration of GNSS 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements 1-1. UE reports the information associated with the validity duration of GNSS 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | [per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission for eMTC | - UE applies segmented UL transmission according to duration configuration by the network - Different values (e.g., TA) for pre-compensation may be used per segment | 2-1 | Yes | N/A | [Release 17 eMTC UE cannot communicate via GEO and NGSO NTNs] | [per UE/per band] | No | No | [For UEs supporting communication via GEO and NGSO NTNs, it must indicate this FG is supported.] | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | - UE applies segmented UL transmission according to duration configuration by the network - Different values (e.g., TA) for pre-compensation may be used per segment | 2-1b | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | [per UE/per band] | No | No | For UEs supporting communication via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset/K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1b | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |

# References

- R1-2202926 Updated RAN1 UE features list for Rel-17 LTE after RAN1 #108-e including remaining RAN1 issues, RAN1#108e

- Chair’s notes RAN1#107-e final.