3GPP TSG RAN WG1 #109-e		 	                                   R1-2203241

e-Meeting, May 9th – May 20th, 2022

Source:	ZTE

Title:	Discussion on disabling of HARQ feedback for IoT-NTN

Agenda Item:	9.12.2

Document for: Discussion

Introduction

In RAN#94e [1], the WID of Rel-18 IoT-NTN was approved and the performance enhancement objective in terms of throughput is listed as below.

- Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1,RAN2]

In this contribution, the above issue is elaborated with corresponding analysis.

Discussion on HARQ feedback disabling

              - In Rel-17 SI for IoT-NTN, the study to support the HARQ feedback disabling is conducted without normative works in Rel-17 due to the limited time. Similar to the NR-NTN, the benefits for such mechanism is to mitigate the impact of HARQ stalling with potential improvement on the date rate for IoT case to enable more service. In this release, as shown in the endorsed WI [1], specify the mechanism to enable this functionality in IoT-NTN is expected. Regarding on the solutions to configure the HARQ feedback enabling/disabling, in general, following two options can be considered:

- Option-1: Reuse the Rel-17 NR-NTN solution for HARQ feedback configuration

- Option-2: Design additional mechanism to accommodate repetition transmission in IoT-NTN

              - W.r.t option-1, since disabling of HARQ feedback has already been supported to mitigate the throughput loss caused by HARQ stalling in Rel-17 NR-NTN, the agreed solution can be considered as baseline. For Rel-17 NR-NTN [2], there are following agreements for disabling on HARQ feedback (HARQ codebook related agreements are omitted):

Agreement

              - Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

Agreement

              - For a DL HARQ process with disabled HARQ feedback, the UE is not expected to receive another PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process that starts until X after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process.

- Working assumption: X = T_proc,1

- FFS: Whether X should be changed to X = max(T_proc,1, K1) where K1 is the minimum k1 if it is configured, otherwise k1 = 0

- Note: The TB of the two PDSCHs can be either same or different

Agreement

              - Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

Agreement

              - For HARQ feedback of each SPS PDSCH, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process, except for the first SPS PDSCH after activation if HARQ feedback for SPS activation is additionally enabled.

              - Based on above agreements, it can be observed that enabling/disabling on HARQ feedback is configurable per HARQ process via UE specific RRC signaling in NR-NTN. Moreover, when HARQ feedback is disabled, the enhancement on PDCCH monitoring and SPS activation are also investigated and agreed. The solutions illustrated by the agreements can be reused in IoT-NTN with minor revision to accommodate the specification of eMTC and NB-IoT. For example, there is no definition of T_proc,1 in legacy NB-IoT and eMTC as in NR so that the time gap for PDCCH monitoring need to be modified.

      - Proposal 1: Consider to reuse the HARQ feedback disabling solution agreed in Rel-17 NR-NTN, i.e., enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signaling. The enhancement on PDCCH monitoring and SPS activation may also be reused with minor necessary modification.

      - Although option-1, i.e., reuse Rel-17 NR-NTN solution, can minimize the spec effort, option-2 can also be considered since IoT-NTN has different features compared with NR-NTN. In IoT-NTN, the duration of one (N)PDSCH transmission may be long because of the repetitions. For example, for NPDSCH transmission in legacy NB-IoT, the maximum repetition number can be as large as 2048. With maximum repetition number, the duration of one NPDSCH transmission will be longer than 2s. While in NTN, the RTT cannot be longer than 600 ms even in GEO scenario. That is, HARQ stalling will not happen if more than one HARQ process are applied and repetition number is very large. Hence, whether to disable HARQ feedback can be determined by repetition number. For example, a repetition number threshold can be configured to UE. UE and network will achieve consensus on disabling the HARQ feedback when the repetition number is below the threshold since HARQ stalling is more likely to happen. With this method, unnecessary disabling of HARQ feedback can be avoided in time.

      - Proposal 2: The enabling/disabling HARQ feedback in IoT-NTN based on repetition number for each transmission can also be supported.

Reduced NPDCCH/MPDCCH monitoring

In NTN, the propagation delay can be large because of high altitude of the satellite. As a result, when HARQ feedback is enabled, UE may not able to receive the new scheduling information immediately after transmitting HARQ-ACK due to long RTT. In IoT communication, unnecessary NPDCCH/MPDCCH monitoring should be reduced as much as possible to reduce power consumption. Hence, UE should not monitor NPDCCH/MPDCCH for the given HARQ process until RTT after the transmission of HARQ-ACK when HARQ feedback is enabled since it is not possible to receive a new scheduling information during this period. While if HARQ feedback is disabled, the monitoring restriction has been discussed in previous section.

      - Proposal 3: When HARQ feedback for a HARQ process is enabled, the UE is not expected to receive another NPDCCH/MPDCCH carrying a DCI scheduling a NPDSCH/PDSCH scheduled for the given HARQ process that starts until round trip propagation delay after the end of the transmit of HARQ-ACK.

Conclusions

In this contribution, analysis on HARQ feedback disabling for IoT-NTN is conducted with following proposals and observations:

      - Proposal 1: Consider to reuse the HARQ feedback disabling solution agreed in Rel-17 NR-NTN, i.e., enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signaling. The enhancement on PDCCH monitoring and SPS activation may also be reused with minor necessary modification.

      - Proposal 2: The enabling/disabling HARQ feedback in IoT-NTN based on repetition number for each transmission can also be supported.

      - Proposal 3: When HARQ feedback for a HARQ process is enabled, the UE is not expected to receive another NPDCCH/MPDCCH carrying a DCI scheduling a NPDSCH/PDSCH scheduled for the given HARQ process that starts until round trip propagation delay after the end of the transmit of HARQ-ACK.

- Reference

- RP-213596 New WID on IoT NTN enhancements, RAN#94e

- R1-2112890 3GPP TSG-RAN WG1 Agreements under 8.4 up to eMeeting RAN1#107-e, RAN1#107e