# 3GPP TSG RAN WG1 Meeting #109-e	R1-2203254

# e-Meeting, May 9th – 20th, 2022

# Source:	ZTE

# Title:	Discussion on other use cases for AI/ML

# Agenda Item:	9.2.5

# Document for:	Discussion and Decision

- Introduction

In RAN#94, a new Rel-18 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved[1]. The objective of the new SID is to study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. And the use cases focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Apart from the three use cases mentioned above, we think AI/ML for channel estimation is a promising use case. In this contribution, we will present our initial evaluations on AI/ML based channel estimation

- AI for channel estimation

  - Introduction

Artificial Intelligence (AI)/Machine Learning (ML) is prevalent in many areas such as wireless communication system, as detailed in Figure 1. Besides, AI/ML is expected to be the core functionality with standard support in 5G advanced networks, where 5G communication systems will be enhanced for better support of AI/ML based services.

Figure 1 Diagram of AI applied to wireless communication system

Currently, 3GPP experts are identifying related use cases for possible applicability of AI/ML techniques, such as CSI feedback, beam management, and positioning.  Apart from these three use cases, one of the promising use cases is RS overhead reduction for channel estimation. AI-based channel estimation requires only a small number of RS REs (Resource Elements), which can be achieved by the following two methods. The first method, as shown in Figure 2(a), reduces the RS(e.g. DMRS, SRS, PRS) overhead by reducing the number of REs corresponding to each port, e.g., by increasing the comb value. The second method, as shown in Figure 2(b), uses the CSI-RS resources configured on N ports to predict the channels corresponding to M ports, in which N<M.

Figure 2 Diagram of the two AI-based channel estimation methods

AI-based channel estimation significantly reduces the RS overhead, which on the one hand allows more RS REs to be used for data transmission to improve the system transmission capacity in frequency domain, and on the other hand allows more RS ports to be multiplexed in the same RB in spatial domain. Since RS overhead for different kinds of reference signals are similar, the following study and description focus on the DMRS-specific use case merely.

  - AI for DMRS-based channel estimation

In the initial releases of NR, two DMRS types are supported, named as DMRS type 1 and DMRS type 2. Figure 3(a) shows the DMRS pattern for DMRS type 1 within one PRB in the case when one front loaded DMRS symbol is configured by RRC signaling or indicated by DCI signaling. There are two DMRS CDM groups, each of which corresponds to a comb of 6 REs. More specifically, DMRS port 0 and 1 multiplexed in CDM group#0 correspond to the even REs. DMRS port 2 and 3 multiplexed in CDM group#1 correspond to the odd REs. Figure 3(b) shows the DMRS pattern for DMRS type 2 within one PRB in the case when one front loaded DMRS symbol is configured by RRC signaling or indicated by DCI signaling, where two adjacent frequency REs form one DMRS CDM group. Specifically, DMRS port 0 and 1 are multiplexed in CDM group#0, i.e. port 0 and port1 are multiplexed in RE#0 and RE#1 in CDM manner, and port 0 and port 1 are also multiplexed in RE#6 and RE#7 in CDM manner. So CDM group#0 are repeated twice where one is in RE#0 and #1, and the other one is in RE#6 and #7. Similar mapping is done for other DMRS ports. In summary, 6 DMRS ports are supported in the case of one front loaded DMRS symbol, and the density of each DMRS port is 4 REs per PRB per symbol.

Figure 3 Diagram of the two traditional DMRS types with 1 front loaded DMRS symbol

With the application of AI technology, the number of REs per port can be reduced considerably. One of the feasible ways is shown in Figure 4. More specifically, Figure 4(a) shows the enhanced DMRS pattern for DMRS type 1 within one PRB in the case when one front loaded DMRS symbol is configured. There are two DMRS CDM group, each of which occupies 2 REs in one RB. DMRS port 0 and 1 are multiplexed in RE#0 and RE#5 in CDM manner. DMRS port 2 and 3 are multiplexed in RE#1 and RE#6 in CDM manner. Thus, the remaining REs can be used for the transmission of data and other DMRS ports. Figure 4(b) shows the enhanced DMRS pattern for DMRS type 2 within one PRB in the case when one front loaded DMRS symbol is configured, where two adjacent frequency REs form one DMRS CDM group. Specifically, DMRS port 0 and 1 are multiplexed in CDM group#0, i.e. port 0 and port1 are multiplexed in RE#0 and RE#1 in CDM manner. Similar mapping is done for other DMRS ports. In summary, 6 DMRS ports are supported in the case of one front loaded DMRS symbol, and the density of each DMRS port is 2 REs per PRB per symbol. Similarly, the remaining REs can be used for the transmission of data and other DMRS ports.

Figure 4 Diagram of the two enhanced DMRS types with 1 front loaded DMRS symbol

  - Simulation results

Two channel estimation methods are simulated and compared in this contribution, that is, the conventional LMMSE interpolation-based method and the AI-based method. As shown in Table I in the Appendix, the CDL-A channel model of the 3GPP protocol is adopted, with the delay being 30/300ns and  the carrier frequency being 3.5G Hz. The mobile speed of UE is set to 3 km per hour. Both the base station and UE are configured with one omni-antenna. There are 52 RBs in the frequency domain, where each RB consists of 12 subcarriers and 14 OFDMA symbols. The following two cases are simulated: the traditional DMRS type 1 includes 2 symbols in the time domain with a comb equals to 2 (i.e., 6 REs  in the frequency domain per port), and the enhanced DMRS type 1 includes 2 symbols in the time domain with a comb equals to 6  (i.e., 2 REs in the frequency domain per port). In the AI-based method, the number of training sets and test sets are 50,000 and 3,000, respectively. The loss function is the MSE between the AI output and the realistic channel.

Figure 5 below shows the MSE performance comparisons of the conventional LMMSE interpolation-based method and the AI-based method, where the conventional method is achieved by the LMMSE interpolation in the time-frequency domain. As can be seen, the AI-based method performs better than the conventional LMMSE interpolation-based method regardless of whether the traditional DMRS type 1 or enhanced DMRS type1 is adopted. Besides, the AI-based method yields an MSE performance of 4.8e-4 at a comb equals to 6  (i.e., 2 REs per port), which outperforms the 7e-4 MSE performance of the conventional LMMSE interpolation-based method at a comb equals to 2  (i.e., 6 REs per port).

Figure 5 Performance evaluation of  the traditional DMRS type1 and the enhanced DMRS type1

Observation 1: LMMSE interpolation-based method has huge performance loss over Enhanced DMRS type1 when compared to Traditional DMRS type 1. However, the AI-based method shows significant performance gain over Enhanced DMRS type1, where the performance is superior to Traditional DMRS type 1 conditioned on LMMSE interpolation-based method.

- Conclusion

In this contribution, we provide our initial views on Rel-18 AI-based channel estimation. We have following observation,

Observation 1: LMMSE interpolation-based method has huge performance loss over Enhanced DMRS type1 when compared to Traditional DMRS type 1. However, the AI-based method shows significant performance gain over Enhanced DMRS type1, where the performance is superior to Traditional DMRS type 1 conditioned on LMMSE interpolation-based method.

- References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- Appendix

Table I  Simulation parameters for DMRS channel estimate

| Simulation parameters | value |
| --- | --- |
| Carrier frequency | 3.5GHz |
| Channel model | CDL-A |
| Delay spread | 30ns/300ns |
| UE speed | 3km/h |
| BS antenna configuration | 1 |
| UE antenna configuration | 1 |
| DMRS pattern | Symbol (3,12) Frequency domain: Comb=2, and Comb=6 |
| Bandwidth | 20M |
| Subcarrier spacing | 30kHz |
| RB Number | 52 |