3GPP TSG RAN WG1 #109-e						        R1-2203258

e-Meeting, May 9th –  20th, 2022

Agenda item:	8.1.1

Source: 	Moderator (ZTE)

Title: 	Moderator Summary for Rel.17 NR FeMIMO maintenance: Item 1: multi-beam

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.17 NR_FeMIMO maintenance is given below.

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select four issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Editorial (E): this includes editorial issues that will be handled as editorial CRs

- Maintenance issues

The issues are summarized in the following table:

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | FL assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| Sub-Item 1 – Unified TCI Framework |  |  |  |  |
| 1-1 | Clarify UE behavior in BFR for determining UL power control parameter for PUSCH/PUCCH/SRS after receiving gNB response in unified TCI framework (TP2/3/4 in R1-2203257, P10 in R1-2203505, P3 in R1-2203948, P3/P4 in P1-2204169, P1 in R1-2204682, P1 in R1- 2204976) FL note 1: The issue identified in the problem is valid and essential. FL note 2: This issue has NOT been discussed. | QC, MTK, vivo, ZTE, OPPO, Lenovo, | H (H:13, N:1) | Apple: OK to discuss Google: Agree with FL MediaTek: Okay to discuss, and the X symbols after NW response should be defined as well Ericsson: not essential – Rel-15 rules apply OPPO: ok to discuss Nokia: OK to discuss QC: Agree to discuss Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss CATT: OK to discuss Xiaomi: OK to discuss vivo: Agree with FL Intel: Ok to discuss |
| 1-2 | Clarify UE behavior of PL-RS determination for CA case (TP1 in R1-2203257, P5 in R1-2203505, P1 in R1-2204137, P5 in R1-2204976) FL note 1: The issue identified in the problem is valid, otherwise cross-CC PL-RS indication may be precluded in unified TCI framework. FL note 2: This issue has NOT been discussed. | QC, vivo, ZTE, LGE | H (H:14, N:1) | Apple: OK to discuss Google: Support discussing MediaTek: Okay to discuss Ericsson: not needed. There is no ambiguity. The UE uses the TCI states from the reference CC, and interprets them for the serving cell. For parameters other than the PL RS, the parameters will be the same: there is no other option. For the PL RS, it would be Alt2. OPPO: ok to discuss Nokia: OK to discuss QC: Agree to discuss Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss CATT: OK to discuss LG: Agree to discuss Xiaomi: OK to discuss vivo: Agree with FL Intel: OK to discuss |
| 1-3 | To support that BFR procedure can be performed per CC group in unified TCI framework (Proposal-1 in R1-2203257, P11 in R1-2203505). FL note 1: Technically speaking, this enhancement is valid in unified TCI framework. But considering that it can be achieved by per-CC BFR procedure, whether this issue is essential should be justified. FL note 2: This issue has been discussed for one meeting. | vivo, ZTE | N (H:4, N:9) | Apple: OK to discuss MediaTek: Fine to discuss Langbo: OK to discuss Ericsson: not essential OPPO: do not support to discuss. It is not essential issue and it might not comply with the principle of beam failure detection. The beam failure is per CC and different CC might experience different interference. Nokia: agree with FL recommendation, we need to focus on the essential issues and in this case the spec is not broken! Futurewei: agree with FL recommendation. SS: Agree with FL recommendation. No need to discuss. Since this has been discussed before, propose to conclude no consensus and no support DCM: Not essential. CATT: no need to discuss. Similar view as OPPO that different CC may have different interference. LG: Agree with FL assessment as ‘N’ Xiaomi: Agree with FL recommendation. vivo: OK to discuss Intel: This is an enhancement. Agree with FL |
| 1-4 | Clarifying UE behavior of beam indication for AP CSI-RS based on already agreement (Section 2.1.2 in R1-2203421, P4 in R1-2204682, TP2 in R1-2203301) FL note 1: The issue as raised in Section 2.1.2 in R1-2203421 was discussed during editor-spec review, and, if my understanding is correct, it may be implicitly derived according to the following: ‘If configured, the UE may assume that a CSI-RS resource in an aperiodic CSI-RS resource set configured without trs-Info is quasi co-located with the RS(s) in the indicated TCI state.’ in 5.2.1.5.1. Then the further refinement for above as proposed in P4 in R1-2204682 may not be needed. Look forward to other companies’ inputs. FL note 2: This issue has been discussed for one meeting. | CATT, MTK, Spreadtrum | N (H:0, N:15) | Apple: Agree with FL Google: Agree with FL MediaTek: If the common understating is that current spec can implicitly indicate that UE follows the indicated TCI state when qcl-info is absent for aperiodic CSI RS instead of configuring by a higher layer parameter, we are fine with FL’s assessment. Ericsson: The information is in 38.331, that’s enough. OPPOL Agree with FL Nokia: we think the specification is clear enough, no need to discuss, hence N. QC: Same view as FL Futurewei: Agree with FL SS: Agree with FL recommendation. Since this has been discussed before, propose to conclude no consensus and no support Spreadtrum: We are OK with the implicit way. DCM: Agree with FL. CATT: If the implicit way is common understanding, we are OK with FL. LG: Agree with FL’s assessment Xiaomi: Agree with FL. vivo: Agree with FL Intel: Agree with FL |
| 1-5 | In section 7 of TS 38.213, update PC parameter setting of SRS resource set (Section 2.2.1 in R1-2203421, P1 in R1-2203771), i.e., removing ‘and for a first SRS resource from the SRS resource set’ in Clause 7 Uplink Power control in TS 38.213 or just remove ‘first’, etc. FL note 1: If my understanding is correct, the wording of ‘a first SRS resource from the SRS resource set’’ is to align with ‘an SRS transmission occasion’ for Clause 7.1.3 for UL power control. However, some further editorial clarification may not be hurt. FL note 2: This issue has NOT been discussed. | CATT, xiaomi | E | Apple: Agree with FL Google: Support MediaTek: Agree with FL Ericsson: agree with FL Nokia: agree with FL Futurewei: Agree with FL SS: Spec is OK, fine to discuss wording refinement in spec. DCM: Agree with FL. CATT: support to discuss LG: Fine to discuss the refinement of wording Xiaomi: support to remove it based on the agreement. And we also prefer to remove the ‘indicated’ in ‘…is provided by PL-RS associated with or included in the indicated DLorJoint-TCIState or UL-TCIState of an SRS resource…’ vivo: OK to clarify Intel: OK |
| 1-6 | Update the description of PC parameters in section 7 of TS 38.213. (Section 2.2.2 in R1-2203421) FL note 1: The issue identified in the problem is valid and editorial. FL note 2: This issue has NOT been discussed. | CATT | E | Apple: OK to discuss Google: Looks reasonable to us. Support discussing. MediaTek: Fine to update Ericsson: Support as editorial OPPO: Ok to discuss Nokia: agree with FL Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss. CATT: OK to discuss LG: Fine to discuss vivo: OK to clarify the understanding of ‘associated with’ and ‘included in’ Intel: OK to discuss |
| 1-7 | In TS38.213 section 7, clarify that the PCI information in Rel-17 TCI state configured by RRC is applied to the PL-RS associated with the TCI state (P1 in R1-2203505). FL note 1: The issue identified in the problem is valid. However, some further justification may be needed. FL note 2: This issue has NOT been discussed. | vivo | H (H:13, N:0) | Apple: OK to discuss Google: Support having discussion MediaTek: Okay to discuss Ericsson: OK to discuss OPPO: ok to discuss Nokia: agree with FL Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss. CATT: OK to discuss LG: Fine with FL’s assessment vivo: Agree with FL Intel: Ok to discuss |
| 1-8 | In TS38.213 section 7, clarify that UL PC parameter setting associated with the unified TCI state is not applied for a PUSCH (re)transmission configured by ConfiguredGrantConfig. (TP2 in R1-2203505) FL note 1: The issue identified in the problem is valid. However, some further justification may be needed. FL note 2: This issue has NOT been discussed. | vivo | N (H:4, N:8) | Apple: It seems this is not essential. Google: Agree with FL MediaTek: Prefer current spec Ericsson: Prefer current spec. OPPO: ok to discuss. Nokia: not essential SS: Non-essential issue. CG should follow the unified TCI state. DCM: OK to discuss. CATT: Not essential. LG: Seems non-essential vivo: The clarification is needed Intel: OK with N |
| 1-9 | In TS38.213 section 7, clarify which QCL-Type of the RS in the TCI state is used as the PLRS. (TP4 in R1-2203505) - For FR2, if UE supports beam alignment, the QCL-TypeD RS can be used as PLRS. But for FR1, if the QCL-TypeD RS is absent in joint or UL TCI state, other QCL-type RS other than QCL-TypeD RS is used for pathloss estimation for joint TCI. FL note 1: The issue identified in the problem may not be essential and can be handled by implementation. FL note 2: This issue has NOT been discussed. | vivo | N (H:1, N:13) | Apple: Agree with FL MediaTek: Not essential Ericsson: not essential OPPO: Agree with the assessment of FL Nokia: not essential QC: Agree with FL Futurewei: not essential SS: Agree with FL, this is a non-essential issue. DCM: OK to discuss. CATT: Not essential LG: Agree with FL’s assessment Xiaomi: Agree with FL vivo: Similar clarification exists also for BFD RS in TS38.213 section 6. For implicit BFD RS determination, the description is as follows: ‘If there are two RS indexes in a TCI state, the set q0 or q0,0, or q0,1 includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states.’ It can be seen that the QCL-TypeD RS in TCI state is used as BFD RS if there are two RSs. Thus, in Rel-17 unified TCI, if UE supports beam alignment, it should be clarified which QCL-Type of the RS included in the TCI state is used as the PLRS, i.e. QCL-TypeD RS for FR2 or other QCL-Type RS for FR1. Intel: OK with N |
| 1-10 | In TS38.213 section 6, exclude the new beam application for FR1. (P12 in R1-2203505) FL note 1: The editorial issue identified in the problem may not be essential, considering that spatial relation/filter is only applicable in FR2 by default. FL note 2: This issue has NOT been discussed. | vivo | N (H:0, N:13) | Apple: Agree with FL Google: Agree with FL MediaTek: Not essential Ericsson: not essential OPPO: Agree with the assessment of FL Nokia: not essential QC: Agree with FL Futurewei: Not essential SS: Agree with FL, this is a non-essential issue. DCM: Agree with FL. CATT: Not essential LG: Agree with FL’s assessment Intel: Agree with FL |
| 1-11 | In section 5.1.5 in TS 38.214, clarifying the UE behavior when only one TCI state is activated. (P1 in R1-2203673, P7 in R1-2204199, P2-2 in R1-2204335, P1 in R1-2204680) FL note 1: The issue identified in the problem is valid and editorial. But, proponents’ TPs are a little bit different, and some detailed discussion may be needed. FL note 2: This issue has NOT been discussed. | NEC, Apple, DOCOMO, Google | E (E:10, N:3) | Apple: Agree with FL Google: Support MediaTek: Not essential Ericsson: Agree with the FL assessment. Editorial (Apple,NEC), discussion needed (Google). OPPO: ok to discuss Nokia: not essential SS: OK to discuss DCM: OK to discuss. CATT: OK to discuss LG: Non-essential Xiaomi: OK to discuss. vivo: OK to clarify the UE behavior after MAC CE activation Intel: OK to discuss |
| 1-12 | Clarify that beam failure recovery procedure should be updated based on the Rel-17 TCI framework, for example, BFI_COUNTER should also be set to 0 based on DCI based TCI state update. (R1-2203673, P5 in R1-2204199) - RAN1 may send an LS to RAN2 for their information. FL note 1: Technical speaking, the issue identified as above is valid, but look forward to more views from other companies. FL note 2: This issue has NOT been discussed. | NEC, Apple | N (H:7, N:7) | Apple: Agree with FL Google: Support having discussion MediaTek: Okay to discuss Langbo: Agree with FL Ericsson: not needed. In current 38.321, the BFI_COUNTER is reset when “…any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell.” “upper layers” means a layer above MAC, i.e., RRC. OPPO: That is not essential issue and the proposal could even cause more problems. For example, the beam failure is based on all the PDCCH but the DCI-indicated TCI state only applies to UE-dedicated PDCCH (if the CORESET-C is not configured to follow). So the proposal of resetting BFI_COUNTER might cause problem Nokia: agree with FL QC: Seems not critical Futurewei: Agree with FL SS: Not needed. This could have impact on RAN2 (too late at this stage). DCM: OK to discuss. CATT: Not needed. We share similar view as Samsung. LG: Non-essential based on the current reset criterion of BFI_COUNTER Xiaomi: Agree with FL. vivo: OK to discuss Intel: OK with N |
| 1-13 | In Section 6 in TS 38.213, clarify that the resources that can share a same indicated TCI state as for the PDCCH and PDSCH are defined as “aperiodic CSI-RS in a resource from a CSI-RS resource set”, or “aperiodic CSI-RS resource in a CSI-RS resource set”. (R1- 2203764) FL note 1: The issue identified in the problem is just editorial. Hopefully it may not be controversial. FL note 2: This issue has NOT been discussed. | Langbo | E (E:9, N:3) | Apple: This looks to be non-essential MediaTekL Fine to clarify Langbo: Agree with FL that the problem is just editorial. RAN1 has confirmed in the agreed LS R1-2202720 that “QCL per an aperiodic resource set is currently configured i.e. all resource within NZP-CSI-RS resource set follow unified TCI state in DCI”. However, we think this agreement is not accurately captured in Clause 6 of 3GPP TS38.213 v17.1.0, e.g. “receives …… aperiodic CSI-RS in a resource from a CSI-RS resource set”. Ericsson: good editorial correction. Nokia: agree with FL Futurewei: agree with FL Samsung: OK to clarify DCM: Agree with FL. CATT: not essential LG: Agree with FL’s assessment vivo: Not essential Intel: OK with E |
| 1-14 | In Section 5.1.5 in TS 38.214, clarify that when CORESET 0 has been configured by RRC to follow the unified TCI state (or not configured to no follow the unified TCI state), the TCI state/quasi-co-location is determined by the indicated (unified) TCI state, or the most recent random access procedure if no unified TCI state has been indicated after the most recent random access procedure. (P1 in R1-2203855) FL note 1: The issue identified in the problem may be valid, but it seems that there were some opponent views last meeting. Let’s check companies views again. FL note 2: This issue has been discussed for one meeting. | Samsung | H (H:11, N:2) | Apple: Support to discuss Google: Fine with discussion MediaTek: Fine to discuss Ericsson: not essential OPPO: It seems that the proposal is just what is supported in current spec. Nokia: OK to discuss, the issue might be more 38.213 though. QC: Seems not critical Futurewei: OK to discuss SS: OK to discuss and should conclude in this meeting (one way or another). DCM: OK to discuss. CATT: OK to discuss vivo: OK to clarify Intel: OK to discuss |
| 1-15 | For the Rel-17 unified TCI framework, update 38.214 to describe cross carrier beam indication based on the “carrier indicator field” in DCI Format 1_1 or DCI Format 1_2. (P2 in R1-2203855, P1 in R1-2204680) FL note 1: The issue identified in the problem may be valid and seems to capture the already agreement. FL note 2: This issue has been discussed for one meeting. | Samsung, Google, | H(H:7, N:5) | Apple: Support to discuss Google: Support (at least to make a conclusion) MediaTek: Not essential Ericsson: not essential Nokia: OK to discuss QC: Seems no issue. The TCI is configured on the CC indicated by CIF, same as R16 SS: OK to discuss. In spec it is not clear that a carrier indicator field for a DCI Format applies to the indicated carrier by the carrier indicator field. We should conclude in this meeting (one way or another). SS: OK to discuss. In spec it is not clear that a carrier indicator field for a DCI Format applies to the indicated carrier by the carrier indicator field. We should conclude in this meeting (one way or another). DCM: OK to discuss. CATT: OK to discuss. LG: Non-essential Xiaomi: OK to discuss. vivo: Agree with FL Intel: not essential |
| 1-16 | Clarify that the DCI format 1_1/1_2 with PDSCH assignment in CORESET #0 or CORESET C not provided with followUnifiedTCIstate does not indicate the rel-17 TCI state (P1 in R1-2203948). FL note 1: Technically speaking, this proposal can be assumed as an optimized one (no spec broken). It may be also relevant to co-existence discussion for Rel-15/16 beam and Rel-17 unified TCI indication. Look forward to other companies’ views. FL note 2: This issue has NOT been discussed. | OPPO | N (H:1, N:14) | Apple: Agree with FL Google: Agree with FL MediaTek: Not essential Ericsson: not essential OPPO: that is an essential problem. As agreed, if the CORESET C does not following rel-17 DCI-indicated TCI state, the PDCCH and corresponding PDSCH shall reuse the rel15/16 mechanism. So we need to clarify whether the TCI field in a DCI format 1_1 in CORESET C indicates a TCI state by following the rel15/16 mechanism or rel17 mechanism. Nokia: agree with FL QC: Agree with FL Futurewei: Agree with FL SS: Agree with FL DCM: OK to discuss. CATT: Agree with FL. LG: Agree with FL’s assessment Xiaomi: Agree with FL. vivo: Agree with FL Intel: OK with N |
| 1-17 | To support URLLC power control in Rel-17 Unified TCI framework, DCI format 0_1 or 0_2 can indicate the UE to apply the P0 configured for URLLC or the P0 associated with the indicated Rel-17 TCI state. (P4 in R1-2203948, R1-2204192). FL note 1: The issue identified in the problem may be valid. However, whether spec update is needed should be justified. FL note 2: This issue has been discussed for one meeting. | OPPO, ASUSTeK | N (H:5, N:7) | Apple: Support to discuss. DCI field is already there, which needs to be clarified. MediaTek: Fine to discuss Ericsson: not essential OPPO: We need discuss and conclude this issue. Otherwise, the DCI field in DCI format 0_1 or 0_2 will break in Rel-17 Nokia: agree to discuss QC: Seems optimization SS: Seems to be an optimization. Since this has been discussed before, propose to conclude no consensus and no support DCM: Seems not essential. CATT: Fine to discuss. LG: Non-essential vivo: Agree with FL Intel: OK with N |
| 1-18 | To clarify that the maximum number of pathloss RSs for unified TCI is 64. Then, when the NW configures Uplink-powerControl-r17, the corresponding power control parameters should not be provided in PUCCH-PowerControl, PUSCH-PowerControl or SRS-Config. (P1 and P2 in R1-2204031) FL note 1: The issue identified in the problem may be valid and can be assumed as a remaining issue about the configuration of PL-RS pool. It may have RRC impacts FL note 2: This issue has been discussed for one meeting. | Ericsson | (To be discussed in LS response) | Apple: It can be discussed in LS response. MediaTek: Can be discussed in the LS response to RAN2 Ericsson: fine to discuss as part of LS response Nokia: discuss in LS response Futurewei: Can be discussed in LS response SS: This is being discussed in the LS response (no need to discuss in this agenda item) DCM: this can be discussed in LS response. CATT: Can be discussed in the LS response to RAN2. vivo: discuss in RAN2 LS reply Intel: discuss in RAN2 LS response |
| 1-19 | In Section 6 in TS 38.213, clarify that the UE may determine the set q0 from a DLorJointTCIState in addition to a TCI-State. (P4 in R1-2204031) FL note 1: The issue identified in the problem may be valid and editorial. FL note 2: This issue has NOT been discussed. | Ericsson | E | Apple: Agree with FL Google: Support MediaTek: Support the proposal Langbo: Agree with FL Ericsson: agree with FL OPPO: Ok Nokia: agree with FL Futurewei: Agree with FL SS: Agree with FL DCM: OK to discuss. CATT: OK LG: Fine with FL’s assessment vivo: Agree with FL Intel: Agree with FL |
| 1-20 | Clarify for virtual PUSCH-PHR, the UE uses the pathloss reference signal associated with the activated TCI state. (P6 in R1-2204031, P2 in P1-2204169, P1 in R1-2204199) FL note 1: The issue identified in the problem is valid. FL note 2: This issue has been discussed for one meeting. | Ericsson, Lenovo, Apple | H (H:9; N:4) | Apple: Agree with FL Google: Support having discussion MediaTek: Fine to discuss Ericsson: agree with FL OPPO: not essential issue. Nokia: agree with FL QC: Seems not critical. Unified TCI reuses the PUSCH PL RS, which always exists to our understanding SS: Not essential (optimization). Since this has been discussed before, propose to conclude no consensus and no support DCM: seems not essential. CATT: Agree with FL’s proposal. LG: Fine to discuss vivo: OK to discuss Intel: OK to discuss |
| 1-21 | Clarify that CCs in a CC list for common TCI activation and common TCI ID update should be configured with the same TCI mode, i.e. separate or joint TCI states (R1-2204199) - Send an LS to RAN2 FL note 1: The issue identified in the problem seems non-essential and can be well handled with a reasonable configuration. Look forward to hear views from other companies. FL note 2: This issue has NOT been discussed. | Apple | N (H:1; N:10) | Apple: This could have impact on MAC CE design. RAN1 should identify whether there is any use case for this configuration. MediaTek: Not essential Ericsson: not essential Nokia: not essential. QC: It may be resolved by NW implementation Futurewei: Not essential SS: Agree with FL DCM: Not essential. CATT: Not essential. LG: Non-essential Intel: OK with N |
| 1-22 | Clarify UE behaviour if TCI state is not indicated for P/SP-CSI-RS (R1-2204199, P1/2 in R1-2204274, P1 in R1-2204535) FL note 1: The issue identified in the problem seems valid and has been highlighted as FFS in the existing agreement. But, it seems fine without spec update/conclusion. FL note 2: This issue has been discussed for one meeting. | Apple, CMCC, Nokia, | N (H;2; N:11) | Apple: This was agreed as a FFS. It is reasonable to have a consensus for this FFS at least from procedure perspective. MediaTek: Not essential Ericsson: we prefer option 2 in P8 in R1-2204199, but it is not essential – it’s just like R15 OPPO: not essential issue. If TCI state in not indicated, it is up to UE implementation. Nokia: the FFS points should be solved, propose to discuss. QC: Seems not critical. Legacy behavior can be reused Futurewei: It seems not essential SS: Seems to be non-essential issue. Since this has been discussed before, propose to conclude no consensus and no support DCM: seems not essential. CATT: Seems not essential issue, legacy behavior can be reused. LG: Similar view with OPPO vivo: Agree with FL Intel: OK with N |
| 1-23 | Clarify UE behavior for the following aspects (P2-1 in R1-2204335): - For CC-common TCI state pool/configuration, “a reference BWP of a reference CC” in Sect. 5.1.5 of TS38.214 is a BWP/CC where DLorJointTCIState or UL-TCIState is configured in the same band. - If bwp-id or cell for QCL-Type A/D RS is absent, QCL-Type A/D RS is CSI-RS or SSB, with the resource ID in the QCL-Info, configured in the CC/DL BWP where TCI state applies. FL note 1: The spec update identified in the contribution could improve the clarity of the specifications but does not seem to be essential. FL note 2: This issue has NOT been discussed. | DOCOMO | N (H:4 ; N:7) | Apple: OK to discuss MediaTek: OK to discuss Ericsson: not essential. If any change is needed, it could be to say that the reference BWP/CC is provided as an alternative to the explicit list. RAN2 is implementing this as a CHOICE, so it should work fine. Nokia: agree with FL QC: Seems not critical SS: Agree with FL CATT: OK to discuss. DCM: Support to discuss. RRC configured CC-list (simultaneousU-TCI-UpdateList1~4) is for MAC CE based TCI state ID indication only. The issue here is for RRC based CC-common TCI state pool, there is neither configuration of CC list, nor definition of “reference BWP/CC”. The issue is illustrated in the following figure. It is not clear which BWP/CC to be followed for CCs without TCI state configuration. LG: Agree with FL’s assessment vivo: Agree with FL Intel: Agree with Ericsson’s comment above |
| 1-24 | Clarify that for UE with activated with more than one TCI state, if the symbols of paging/short message/SI from serving cell are not overlapped with the symbols of PDCCH/PDSCH/CSI-RS from non-serving cell, UE receives both. FL note 1: The issue identified in the problem seems valid based on existing agreements. FL note 2: This issue has been discussed for one meeting. | DOCOMO | N (H: 3; N: 5) | Apple: We do not think this needs to be discussed again. MediaTek: Not essential Ericsson: this is supported. The UE should monitor all configured search spaces, unless there is an exception. QC: Seems not critical SS: OK to discuss and should conclude in this meeting (one way or another) DCM: Support to discuss. We believe whether UE can receive paging/short message is critical for system operation. CATT: Not essential. LG: Non-essential and discussed already Intel: OK with N |
| 1-25 | In Section 5.1.5, clarify the following rules as agreed in previous meeting (P2 in R1-2204682) - One source RS across CCs can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs - One source RS per CC can be determined from the indicated common TCI state ID to provide QCL Type-D indication and to determine UL TX spatial filter for the set of configured CCs, and the CC-specific source RSs are further associated with a same QCL-TypeD RS FL note 1: The issue identified in the problem seems valid based on already agreements. However, some companies don't think this agreement has any impact on current spec based on the previous discussion. Let’s try it again. FL note 2: This issue has been discussed for one meeting. | MTK | N (H: 5; N:7) | Apple: OK to discuss MediaTek: Prefer to discuss Ericsson: There should be no such restriction in the normative spec. May be part of a UE feature discussion. OPPO: ok to discuss Nokia: no need to discuss QC: Seems not critical SS: This seems to be a non-essential DCM: Agree with Ericsson. CATT: OK to discuss LG: Seems non-essential vivo: OK to clarify Intel: OK with N |
| 1-26 | In section 5.1.5, clarify determination of beam application time for the case when common TCI state ID update across CCs is configured and when not configured is not differentiated. (Section 3.1.1 in R1-2204763) FL note 1: From spec perspective, the CC list information should be clear, if considering the following paragraph together. ‘… The UE can assume that when the UE is configured with TCI-State in any CC in the CC list configured by simultaneousTCI-UpdateList1-r16, simultaneousTCI-UpdateList2-r16, simultaneousSpatial-UpdatedList1-r16, or simultaneousSpatial-UpdatedList2-r16, the UE is not configured with DLorJointTCIState or UL-TCIState in any CC within the same band in the CC list.’ Then, after previous discussion, regardless of CC-group or CC-level beam indication, BAT should be configured per BWP and is also aligned with current RRC spec. FL note 2: This issue has NOT been discussed. | Intel | N | Apple: Agree with FL Google: Agree with FL MediaTek: Agree with FL Ericsson: agree with FL Nokia: agree with FL QC: Agree with FL SS: Agree with FL DCM: Agree with FL. CATT: Agree with FL. LG: Agree with FL’s assessment vivo: Agree with FL |
| 1-27 | In Section 5.1.5, clarify configuration of CC list for Common TCI state ID Update and Activation (Section 3.2.1 in R1-2204763) FL note 1: Considering that the corresponding paragraphs are also applied to Rel-16 procedure, keeping what we have in the current spec seems better. Look forward to hearing views from other companies. FL note 2: This issue has NOT been discussed. | Intel | N (E:2;N:11) | Apple: OK to discuss Google: Agree with FL MediaTek: Agree with FL Ericsson: this seems a good editorial clarification, that the MAC CE activation uses the same CC lists. OPPO: agree with the assessment of FL Nokia: agree with FL QC: Agree with FL SS: Agree with FL DCM: Agree with FL. CATT: Agree with FL. LG: Agree with FL’s assessment vivo: Agree with FL Intel: This should be an editorial correction which would make the spec more accurate. |
| 1-28 | In section 5.1.5 clarify that, when the bwp-id or cell for QCL-TypeD source RS in a QCL-Info of the TCI state configured with DLorJointTCIState is configured, UE assumes that the QCL-TypeD source RS is configured in the CC/DL BWP indicated by bwp-id or cell. (R1-2204763) FL note 1: The above update improve the clarity of the specifications but does not seem to be essential. Let’s check other companies’ views. FL note 2: This issue has NOT been discussed. | Intel | N (H:1;N:11) | Apple: OK to discuss MediaTek: Not essential Ericsson: not essential OPPO: seems not necessary Nokia: agree with FL QC: Seems not critical Futurewei: agree with FL SS: Seems to be non-essential issue. DCM: Seems not essential. CATT: not essential. LG: Seems non-essential vivo: Not essential |
| 1-29 | For DCI formats 1_1 and 1_2 without DL assignment, the UCI carrying the HARQ feedback should be mapped to high priority HARQ codebook and PUCCH resources associated with priority index 1 when the UE is configured with two priority indexes. If UE is configured with single priority index, the UCI carrying the HARQ feedback for beam indication should be prioritized over other UCI. (P1 in R1-2204763) FL note 1: Technically speaking, this proposal can be assumed as an optimized one. Look forward to other companies’ views. FL note 2: This issue has been discussed for one meeting. | Intel | N | Apple: Agree with FL that this is not essential. Google: Seems an optimization MediaTek: Not essential Ericsson: up to NW configuration OPPO: not essential issue Nokia: agree with FL QC: It seems optimization Futurewei: agree with FL SS: Agree with FL this is an optimization. Since this has been discussed before, propose to conclude no consensus and no support DCM: Not essential. CATT: Agree with FL’s proposal LG: Agree with FL’s assessment vivo: Not essential |
| 1-30 | To clarify UE behavior for ensuring same PC parameters for the two SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2, if at least one of them is not provided with useIndicatedTCIState. FL note 1: The issue identified in the problem seems valid, since the two SRS resource sets/groups are individually configured by srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2. However, if further reviewing the current spec in 212: ‘… the SRS resource set is composed of the first NSRS, 0_2 SRS resources together with other configurations in the SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList ’, we may need to further check whether this clarification in the spec is sufficient. FL note 2: This issue has NOT been discussed. | QC | H (H:10; N:2) | Apple: This seems to be non-essential. MediaTek: OK to discuss Ericsson: OK to discuss OPPO: ok to discuss Nokia: OK to discuss QC: Prefer to clarify such that identical PC parameters for the two SRS resource sets for DCI 0_1 and 0_2 can be maintained as in R16 Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss. CATT: OK to discuss. LG: Fine to discuss vivo: Seems the spec is clear. Intel: Ok to discuss |
| 1-31 | Clarify that default PDSCH beam also shares the indicated unified TCI. (P3 in R1-2204976) FL note 1: The issue identified in the problem seems valid and can be assumed as editorial. FL note 2: This issue has NOT been discussed. | QC | E (E:12, N:2) | Apple: OK to discuss Google: Support having discussion. But we think it may not be editorial in consideration of inter-cell beam management case. MediaTek: OK to discuss Ericsson: good to clarify – not sure if it’s editorial. Related to 2-3. OPPO: ok to discuss but not editorial change Nokia: no need to discuss QC: Support to clarify Futurewei: OK to discuss SS: Seems to be a non-essential issue no need to discuss DCM: Agree to discuss. CATT: OK to discuss LG: Fine to discuss Xiaomi: OK to clarify. vivo: OK to clarify the default PDSCH beam following the legacy rule or the indicated unified TCI state Intel: Ok to clarify. Should be E |
| 1-32 | Clarify whether the corresponding PC adjustment state should be reset when the associated PC parameters are updated for an UL or joint TCI state used for UL transmission (P4 in R1-2204976) FL note 1: The issue identified in the problem may not be essential. In principle as in Rel-15, beam update as SRI indication should not reset the closed loop FL note 2: This issue has NOT been discussed. | QC | N (H: 2; N: 10) | Apple: Agree with FL Google: Agree with FL MediaTek: Agree with FL Ericsson: this should be discussed. It seems we have two conflicting behaviors: the PC adjustment state is reset at RRC reconfiguration, but not at SRI indication. It is not clear to us which rule is applicable for a MAC CE activation, but we lean towards QCs proposal Nokia: agree with FL QC: Prefer to clarify. A clear conclusion is needed Futurewei: Agree with FL SS: Agree with FL DCM: Agree with FL. CATT: Agree with FL. LG: Agree with FL’s assessment Intel: Ok with N |
| Sub-Item 2 – L1/L2 Centric Inter-Cell Mobility |  |  |  |  |
| 2-1 | Clarify that when a serving cell is configured with inter-cell BM operation and includes only a single BFD RS set, the single BFD RS set should include RS of the serving cell only and should not include RS associated with the additional PCI (R1-2203064) FL note 1: The issue identified in the problem is valid. However, whether specification change is needed might need some discussion. Alternatively, it can be handled in Item 2c-mTRP-BM. FL note 2: This issue has NOT been discussed. | Futurewei | N (H:2; N:10) | Apple: This seems to be non-essential. MediaTek: OK to discuss Ericsson: with the implicit rule, the BFD RS set would by contain the QCL source of the PDCCH, which is associated with the other cell. This is what is captured in the spec now, and it correct. OPPO: seems not essential issue. QC: This can be resolved by NW implementation Futurewei: This should be discussed. If the single BFD RS set includes RS associated with the additional PCI, when this RS is working but the RS associated with the serving cell is not, there will be no BFR triggered. The consequence will then be that the UE is not able to receive non-UE dedicated channels/signals from the serving cell. SS: Seems to be non-essential no need to discuss DCM: Not essential. We agree with Ericsson, this proposal is different from our understanding. CATT: not essential issue. LG: Similar understanding with QC for configuring the BFD RS set Xiaomi: Agree with Ericsson. And it seems that this is more relevant to BM for mTRP. vivo: OK to discuss. Intel: Ok with N. Agree with Ericsson |
| 2-2 | Clarifying that, for inter-cell BM operation, rate matching information (e.g., rate match pattern for PDCCH/PDSCH in inter-cell L1 measurement) can be configured for each additional PCI (P17 in R1-2203505, P2 in R1-2203257, P7 in R1-2204031, P3-4 in P1-2204335). - Besides, in P7 in R1-2204535, the additional issue for DMRS scrambling ID is raised. FL note 1: The issue identified in the problem is valid, and it may have some RRC impacts. FL note 2: This issue has NOT been discussed. | vivo, ZTE, Ericsson, DOCOMO, Nokia | H | Apple: Support to discuss rate matching part. DMRS scrambling ID seems to be non-essential. Google: Support having discussion. MediaTek: OK to discuss the rate matching issue Ericsson: this should be discussed OPPO: OK to discuss the rate matching part. Nokia: Support to discuss QC: Support to discuss Futurewei: OK to discuss SS: OK to discuss DCM: OK to discuss. CATT: OK to discuss Xiaomi: The rate matching behavior has been discussed in inter-cell mTRP operation, which can be a reference. vivo: Share the similar views with Apple that except rate matching part, other aspects seem to be non-essential. Intel: Ok with H |
| 2-3 | Clarifying UE behavior whether/when both Rel-15/16 default beam determination scheme and Rel-17 unified TCI scheme in the inter-cell beam management are applied, e.g., in the case that the indicated TCI is associated with additional PCI. (Proposal 3 in R1-2203257, P15 in R1-2203505, P5 in R1-2204031, P3 in R1-2204199, P3-3 in P1-2204335, P6 in R1-2204535). FL note 1: The issue identified in the problem is valid. FL note 2: This issue has been discussed for one meeting. | vivo, ZTE, Ericsson, Apple, DOCOMO, Nokia | H (H: 12 ,N: 1) | Apple: Support to discuss Google: Agree with FL Ericsson: support to discuss OPPO: ok to discuss Nokia: Support to discuss QC: Support to discuss Futurewei: OK to discuss SS: Not essential since Rel-17 unified TCI doesn’t require default beam. DCM: OK to discuss. CATT: OK to discuss Xiaomi: OK to discuss. vivo: Support to discuss Intel: Ok with H |
| 2-4 | In section 5.1.5 of TS 38.214, update the parameter NumberOfAdditionalPCI, based on latest RRC spec (Section 2.1.4 in R1-2203421). FL note 1: The issue identified in the problem is valid and editorial. FL note 2: This issue has NOT been discussed. | CATT | E | Apple: Agree with FL Google: We suggest discussion can be started after this May meeting since RAN2 is still working on finalizing ICBM related parameters. MediaTek: OK to discuss Ericsson: agree with FL Futurewei: Agree with FL SS: Agree with FL DCM: Agree with E. CATT: OK to discuss LG: OK Xiaomi: OK to discuss. vivo: Agree with FL Intel: Agree with FL |
| 2-5 | In TS38.214 section 5.2.1.4.3, clarify the mapping rule between SSB index and PCI index for inter-cell L1-RSRP measurement based on latest RRC spec. (P13 in R1-2203505) FL note 1: The issue identified in the problem is valid and editorial. FL note 2: This issue has NOT been discussed. | vivo | E | Apple: OK to discuss Google: Support MediaTek: OK to discuss Ericsson: agree with FL Futurewei: Agree with FL SS: Agree with FL DCM: Agree with FL. CATT: OK to discuss Xiaomi: OK with FL vivo: Support to discuss Intel: OK |
| 2-6 | Clarify that in case of beam indication for PCI different from serving cell, the CORESET B TCI state does not follow the indicated TCI state and can be indicated separately. (P5 in R1-2204535) FL note 1: The issue identified in the problem is valid but whether additional spec update is needed should be justified. FL note 2: This issue has NOT been discussed. | Nokia | N | Apple: This seems to be non-essential MediaTek: Not essential Ericsson: not essential OPPO: agree with the assessment of FL. Futurewei: Agree with FL SS: Seems to be non-essential no need to discuss DCM: Not essential. CATT: Not essential LG: Agree with FL’s assessment Xiaomi: Ok to discuss vivo: Agree with FL Intel: Ok with N |
| 2-7 | In Section 5.1.5, clarify that SSB having a PCI different from the serving cell can be a source RS for periodic TRS. (P3 in R1-2204682, Section 2.1 in R1-2204763) FL note 1: The issue identified in the problem seems valid. FL note 2: This issue has been discussed for one meeting. | MTK, Intel, | H | Apple: OK to discuss Google: Support having discussion MediaTek: Prefer to discuss Ericsson: this needs to be discussed Futurewei: OK to discuss SS: OK to discuss DCM: Agree to discuss. CATT: OK to discuss Xiaomi: Ok to discuss vivo: OK to discuss |
| 2-8 | Clarify that inter-cell beam measurement/reporting is not supported for group-based reporting (P4 in R1-2204682). FL note 1: The issue identified in the problem may be valid and can be assumed as editorial. FL note 2: This issue has been discussed for one meeting. | MTK | E (E:11; N:2) | Apple: OK to discuss Google: Agree with FL MediaTek: Prefer to discuss Ericsson: agree with FL OPPO: Ok to discuss QC: Open to discuss, but the proposal seems not match the conclusion to our understanding. The MB conclusion only says inter-cell measurement cannot be applied to non-group based beam report defined in MB session. It does not forbid the application to group-based beam report Futurewei: OK to discuss SS: Support the proposal as editorial. This issue has been concluded so there is no need to be discussed. DCM: Agree with FL. CATT: Agree with FL LG: Agree with FL’s assessment Xiaomi: Agree with SS. No need to further discuss. vivo: Agree with FL Intel: OK with E |
| 2-9 | At least for UE only supporting 1 active TCI state, the UE can be configured to preform pre-determined periodic beam switch between the common channel on serving cell and the dedicated channel on the non-serving cell. (P7 in R1-2204976) FL note 1: The issue identified in the problem can be assumed as an optimization one. However, look forward to other companies views. FL note 2: This issue has NOT been discussed. | QC | N (Pls pay attention to QC’s reply) | Apple: This seems to be non-essential Google: Seems an optimization MediaTek: Not essential Ericsson: optimization OPPO: not essential issue QC: Can we clarify a more fundamental question: Does current inter-cell BM work for UE supporting 1 active TCI? To our understanding, it seems not work, since UE has to reserve 1 active TCI for CORESET 0. It would be good to have common understanding. Futurewei: Agree with FL SS: Agree with FL, seems to be an optimization. No need to discuss DCM: Not essential. CATT: This is an optimization issue, not essential. LG: Agree with FL’s assessment Xiaomi: Ok to discuss vivo: This seems a potential optimization. Intel: OK with N. We think current inter-cell can work for single active TCI state case. Since CORESET#0 should be received only from serving cell PCID, we don’t see an issue when UE is switched to non-serving cell PCID using MAC-CE. |
| Sub-Item 3 – Dynamic TCI Update Signalling |  |  |  |  |
| 3-1 | Clarify the UE behavior when spatial domain transmit filter provided by TCI-State configurations is mismatched with the configured spatialRelationInfo for SRS resource indicated by SRI (R1-2203105, TP5 in R1-2203257, P2 in R1-2203673, P2 in R1-2203948). FL note 1: The issue identified in the problem is valid. However, there may be several candidate solutions (e.g., priority rule, new timeline, and/or only support single-layer PUSCH transmission, etc.) after reviewing proponent’s proposals, and some further discussion seems to be needed. FL note 2: This issue has NOT been discussed. | Huawei, HiSilicon, ZTE, NEC, OPPO, | H (H: 8; N:5) | Apple: OK to discuss Google: Support having discussion MediaTek: OK to discuss Ericsson: no need to discuss. Under no circumstances will a PUSCH transmission follow the spatial relation of an SRS. OPPO: support to discuss, this is an essential issue. QC: We think NW implementation can ensure they are aligned Futurewei: OK to discuss SS: No need to discuss, seems to be an optimization. DCM: Not essential. We don’t understand why gNB configures spatialRelationInfo for SRS resource indicated by SRI, when gNB configures/indicates unified TCI state for PUSCH. CATT: Agree with QC. This issue could be resolved by NW implementation. LG: OK to discuss Xiaomi: From our understanding, there will be no spatialrelationinfo in R17, and it is replaced by UL TCI state or joint TCI state. vivo: OK to discuss Intel: Not essential. Rel-17 unified TCI framework is designed with UL TCI which provides spatial filter for PUSCH and it does not follow spatial filter of SRS. We also don’t think gNB can configure spatialRelationInfo and unified TCI for the same PUSCH based on agreement that these frameworks cannot be configured together. |
| 3-2 | Clarifying that for TCI update by DCI without DL assignment, the virtual PDSCH is assumed in the same slot as the DCI format, or the value of K1-K0 shall be among the “set of slot timing values K1” (TP6 in R1-2203257, P10 in R1-2204976). FL note 1: The issue identified in the problem is valid. Otherwise, K1 for determining candidate PDSCH reception becomes ‘PDSCH-to-HARQ_feedback timing - K0’. If so, there may be some risks that this value may be out of candidate list for semi-static HARQ-ACK codebook generation. FL note 2: This issue has been discussed for one meeting | ZTE, QC | N (H: 3; N:6) | Apple: This seems to be non-essential MediaTek: OK to discuss Ericsson: OK to discuss OPPO: not essential issue. QC: Support to discuss. Otherwise, the ACK may not be sent in the codebook. SS: Agree with Apple/OPPO/Ericsson, not essential DCM: Not essential. CATT: not essential. Intel: OK with N |
| 3-3 | Clarifying, on Rel-17 DCI-based beam indication, whether the TCI update signaling is applied to all configured BWP(s) or active BWP, and whether the BAT should count the BeamAppTime_r17 in all configured BWP(s) or in active BWP only. (TP7 in R1-2203257, R1- 2203301, Section 2.1 in R1-2203421, P6 in R1-2203505, P9 in R1-2204199, P3/4 in R1-2204274). FL note 1: The issue identified in the problem is valid and is highlighted by FFS in the last meeting agreement. FL note 2: This issue has been discussed for one meeting | Spreadtrum, ZTE, CATT, vivo, Apple | H | Apple: Support to discuss Google: Support having discussion MediaTek: OK to discuss Ericsson: OK to discuss. Then it feels strange that a TCI state can even be activated in a non-active BWP: this may require that the UE performs measurements on the corresponding RS. How about inactive cells? OPPO: ok to discuss SS: OK to discuss Spreadtrum: OK to discuss DCM: OK to discuss. CATT: OK to discuss LG: Fine with FL’s assessment Xiaomi: OK to discuss. vivo: OK to discuss Intel: OK to discuss |
| 3-4 | Clarifying that, for NCB PUSCH transmission, whether the associated NZP-CSI-RS is the NZP-CSI-RS in the indicated TCI or precluded to be configured (R1-2203257, Section 2.3 in P1-2204169) FL note 1: The issue identified in the problem is valid, otherwise how to handle NCB PUSCH transmission is unclear in unified TCI framework. FL note 2: This issue has NOT been discussed. | ZTE, Lenovo, | H (H:7;N:3) | Apple: Support to discuss MediaTek: OK to discuss Langbo: Agree with FL Ericsson: No need to discuss. As previously stated, there are no exceptions for PUSCH: under no circumstances will the UE derive the spatial relation for PUSCH from SRS. QC: Seems not critical. This can be solved by gNB implementation to configure the NZP CSI-RS to use the indicated TCI as well. SS: Seems to be an optimization. No need to discuss. DCM: OK to discuss. CATT: OK to discuss. LG: OK to discuss vivo: OK to clarify Intel: Agree with Ericsson. Should be N |
| 3-5 | In TS38.214 section 5.1.5, clarify that the indicated Rel-17 TCI state should be based on the activated TCI states in each slot. (P7 in R1-2203505, P6 in R1-2203948) FL note 1: The issue identified in the problem is valid. But, to be honest, I am not sure whether this issue can be assumed as an editorial issue. If not and still a little bit controversial, we may highlight it as ‘H’. Then, P8 in R1-2203505 and P8 in R1-2204031 may be treated later after this issue is stable. FL note 2: This issue has NOT been discussed. | vivo, OPPO, Ericsson, | H | Apple: Support to discuss Google: Support having discussion. Seems not editorial. MediaTek: OK to discuss Ericsson: OK to discuss. OPPO: this is an essential problem. Support to discuss. QC: No need discussion. Same legacy rule should be reused. SS: Agree this is editorial although OK to discuss, too DCM: OK to discuss. CATT: OK to discuss LG: Similar view with QC vivo: This issue is not editorial. There is ambiguity in current specification regarding which TCI state is used for DCI based beam indication when there is MAC CE update of active TCI state list. Similar to the PDSCH and AP-SRS in Rel-15/16, the clarification is needed. In addition, for multi-slot transmission and reception, there is ambiguity whether all the transmission occasions should use the same TCI across multiple slots or updated per occasion basis based on indicated beams. This issue will affect the UE behavior in 52.6GHz, so it is necessary to be discussed. Intel: Ok to discuss. Agree that there may be some ambiguity for multi-slot case. |
| 3-6 | In TS38.214 section 5.1.5, clarify the beam application time in CA case with different TAGs. (P9 in R1-2203505) FL note 1: By default, frame timeline between DL and UL has already specified in TS 38.211. So for a given UL channel/RS or HARQ procedure, BAT should be considered based on the timeline accordingly. The necessity of spec update is unclear. FL note 2: This issue has NOT been discussed. | vivo | N | Apple: This seems to be non-essential Google: Agree with FL MediaTek: Not essential Ericsson: not essential OPPO: not essential problem. QC: Agree with FL SS: Seems to be a non-essential issue. No need to discuss. DCM: Not essential. CATT: Agree with FL. LG: Agree with FL’s assessment vivo: Could the moderator point out where to refer regarding whether TA is considered or not based on 38.211. [Mod]: Section 4.3.1 in TS 38.211, and details can be found in Section 4.2 in TS 38.213. Intel: OK with N |
| 3-7 | Clarifying that only ACK of the PDSCH scheduled by the DCI carrying the beam indication can be used as an ACK also for the DCI (P5 in R1-2203948, P5 in R1-2204274, P8 in R1-2204976). Besides, in case of HARQ-ACK multiplexing, the TCI state(s) indicated in a DCI corresponding to last position with ACK value in the HARQ-ACK codebook is applied after application timing (P5 in R1-2203673, P6 in R1-2203771, P2 in R1-2204535). Then, in P3 in R1-2203855, another solution is proposed, considering the case that whether gNB can distinguish a NACK from a DTX or not. FL note 1: The issue identified in the problem is valid based on the already agreement, and let’s give a second chance for further clarification. FL note 2: This issue has been discussed for one meeting | NEC, Xiaomi, Samsung, OPPO, CMCC, Nokia, | H (H: 11; N: 3 ) | Apple: Support to discuss Google: Support having discussion MediaTek: Not essential, this has been discussed for more than one meeting Langbo: Agree with FL Ericsson: No spec impact. How to interpret that ACK is up to NW implementation. OPPO: essential problem. Support to discuss QC: Support to discuss SS: OK to discuss. Leaving it to NW implementation would lead to different outcomes (for TCI state application between UE and gNB) hence ambiguous. DCM: Support to discuss the first two sets of proposals. The first two sets of proposals are essential. For the last proposal (P3 in R1-2203855) is not essential, and it was discussed multiple times. CATT: OK to discuss. LG: Support to discuss Xiaomi: Support to discuss. vivo: Not essential Intel: OK to discuss |
| 3-8 | In Section 5.1.5 in TS 38.214, clarification of TCI update action delay (P6 in R1-2204199): ‘…and if the indicated TCI-State is different from the one being applied,’ FL note 1: The issue identified in the problem is valid and can be assumed as an editorial issue. FL note 2: This issue has been discussed for one meeting | Apple | E | Apple: Agree with FL Google: Support MediaTek: Support the TP Langbo: Agree with FL SS: Agree that this is editorial, but current wording is fine. DCM: fine with E, but current wording is also fine. CATT: Fine with FL. LG: Fine with FL’s assessment Xiaomi: Support the TP. vivo: Agree with FL Intel: Current wording seems fine |
| 3-9 | Clarify that for downlink signals associated with a non-serving cell, if virtual cell ID is not configured, the default ID should be PCI for the non-serving cell (P10 in R1-2204199) FL note 1: The issue identified in the problem is valid. FL note 2: This issue has NOT been discussed. | Apple | N (H:5, N:6) | Apple: Agree with FL Google: Support having discussion MediaTek: Not essential Ericsson: No need to discuss. The UE should under no circumstances update its RRC configuration autonomously. QC: Support to discuss SS: Non-essential, no need to discuss. DCM: Support to discuss. CATT: Agree with FL. LG: Not essential issue vivo: Seems no problem exists if properly configured. Intel: Non essential |
| 3-10 | Clarify that for Beam Application Timing (BAT) of Rel.17 DCI-based the beam indication, BAT is counted from the last symbol of PUSCH with HARQ-ACK, as well as PUCCH with HARQ-ACK. (P4-1 in R1-2204335) FL note 1: Technically speaking, this issue identified in the problem seems valid, but from spec perspective, the piggyback issue can be handled together in TS 38.213. So current spec seems clear. FL note 2: This issue has NOT been discussed. | DOCOMO | H (H:7, N:4) | Apple: Support to discuss Google: Support having discussion MediaTek: Prefer to discuss Ericsson: no need to discuss. The statement “When the UE would transmit a PUCCH with HARQ-ACK” would seem accurate, even if the ACK is multiplexed on PUSCH OPPO: not essential issue. QC: Agree with FL SS: OK to discuss. DCM: the issue is that current specification only covers “UCI on PUCCH”. Based on the current specification, if HARQ-ACK is transmitted on PUSCH, it looks “indicated TCI” is not updated. Is this the intended behavior? If companies has no technical concern, we suggest to update, or simply remove “PUCCH” from the specification. CATT: Non-essential issue. LG: Agree with FL’s assessment vivo: OK to clarify Intel: OK to discuss |
| 3-11 | Send LS to RAN2 to clarify MPE-Resource-r17 is either an SSB resource or a periodic CSI-RS (P7 in R1-2204682) FL note 1: Technically speaking, this issue identified in the problem seems valid. Hopefully, it is not controversial. FL note 2: This issue has NOT been discussed. | MTK | E (H/E:6 ,N:3) | Apple: This is non-essential MediaTek: Prefer to discuss, however, it seems this issue should be put in sub-item 5? Ericsson: somewhat editorial in nature. But since it should be captured in 331, fixing it is a little more complicated. QC: Seems not critical SS: This should be E (agreeable, no need to discuss), not H. This is also MPE related, so prefer to discuss in issue 5. DCM: not critical, but fine to discuss. CATT: Prefer to discuss in issue 5. LG: Seems non-essential issue vivo: Fine to clarify. Intel: Ok to clarify. Should be E not H |
| Sub-Item 4 – MP-UE |  |  |  |  |
| 4-1 | In section 5.2.1.4.3 of TS 38.214, update the parameter cri-RSRP-Capability[Set]Index, based on latest RRC spec (Section 2.1.3 in R1-2203421, R1-2204031, R1-2204763). Besides, in R1-2204031, they also propose to use reportQuantity-r17, to avoid confusion with the Rel-15 parameter. FL note 1: The issue identified in the problem is valid and editorial. FL note 2: This issue has NOT been discussed. | CATT, Ericsson, Intel | E | Apple: Agree with FL MediaTek: Support FL’s assessment Ericsson: agree with FL SS: Agree to align parameters with RRC spec, however, we don’t think introducing a new RRC parameter is essential (e.g. we don’t introduce new RRC parameter when we introduce a new codebook, we rather just add a new value for R15 parameter codebookType) NTT DOCOMO: agree with FL CATT: Support FL’s assessment. LG: OK to discuss vivo: Agree with FL |
| 4-2 | Introduce an RRC parameter to provide the indication of enabled UE capability index(es) (P11 in R1-2204199) - The bitwidth and interpretation of the capability index reported in beam report should be based on the configured UE capability index(es) instead of UE capability report FL note 1: The issue identified in the problem is valid and essential. FL note 2: This issue has NOT been discussed. | Apple | H (H: 8; N:2) | Apple: Agree with FL MediaTek: Okay to discuss Ericsson: OK to discuss the bit width of the index. The window for new RRC parameters is closed, and the straightforward solution is that the UE always reserves 2 bits. OPPO: seems not needed. Why is the UE capability report not used? QC: Seems not critical SS: Agree with Ericsson/OPPO (we can’t add a new RRC parameter) NTT DOCOMO: agree with Ericsson CATT: Agree with FL. LG: Open to clarify the bitwidth of the index, e.g. always 2 bits as suggested by Ericsson or associated to UE capability report as OPPO. Maybe making a conclusion would be enough? vivo: OK to discuss. Intel: Ok discuss |
| 4-3 | Before the first report of the correspondence index, the SRS resource(s) with the lowest capability in terms of the SRS antenna ports is assumed by the UE. (P8/9 in R1-2204535) FL note 1: The issue identified in the problem seems valid but may be an optimization one. Look forward to other companies’ views. FL note 2: This issue has NOT been discussed. | Nokia | N | Apple: Agree with FL MediaTek: Not essential Ericsson: not essential. We note that “max rank” still applies. QC: Agree with FL SS: Seems to be non-essential. No need to discuss. NTT DOCOMO: Agree with FL. CATT: Not essential. LG: Support to discuss this issue vivo: Not essential Intel: OK with N |
| Sub-Item 5 – MPE |  |  |  |  |
| 5-1 | To enable MPE report in inter-cell beam management, e.g., an explicit PCI, or AdditionalPCIIndex-r17, is needed for the MPE resource configuration. (P3 in R1-2204031) FL note 1: The issue identified in the problem seems valid but may be an optimization one. Look forward to hearing views from other companies FL note 2: This issue has NOT been discussed. | Ericsson | To be handled in LS | Apple: This is under discussion in LS response MediaTek: Can be discussed in the LS response to RAN2 Ericsson: fine to discuss as part of the LS response SS: Being discussed in the LS response. NTT DOCOMO: fine to discuss in LS response CATT: Ok to discuss in LS response. vivo: discuss in RAN2 LS reply |
| 5-2 | Support UL beam resetting based on the reported UL beam in the enhanced P-MPR report, after UE receives gNB’s response to the report. (P12 in R1-2204976) FL note 1: The issue identified in the problem seems valid but may be an optimization one. Look forward to other companies’ views. FL note 2: This issue has NOT been discussed. | QC | N | Apple: Agree with FL Google: Agree with FL MediaTek: Agree with FL Ericsson: We don’t think the UE should reset the beam. OPPO: not essential issue. SS: Non-essential issue. No need to discuss NTT DOCOMO: Agree with FL. CATT: Agree with FL. LG: Agree with FL’s assessment vivo: Good to have some discussions. Intel: OK with N |

- Observation

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issue can be handled as E (a part of editorial CR): 1-5, 1-6, 1-11, 1-13, 1-19, 1-31, 2-4, 2-5, 2-8, 3-8, 3-11, 4-1.

- The following issues can be designated as H (requiring discussion and additional agreements/conclusions): 1-1, 1-2, 1-7, 1-11, 1-15, 1-20, 1-30, 2-2, 2-3, 2-7, 3-1, 3-3, 3-4, 3-5, 3-7, 3-10, 4-2.

- The following issues can be designated as N (non-essential) but can be discussed again in future meetings: 1-8, 1-9, 1-10, 1-12, 1-16, 1-21, 1-23, 1-26, 1-27, 1-28, 1-32, 2-1, 2-6, 2-9, 3-6, 3-9, 4-3, 5-2.

- The following issues can be designated as N (non-essential) and have been discussed in previous meeting(s): 1-3, 1-4, 1-17, 1-22, 1-24, 1-25, 1-29, 3-2.

# References

| 1 | R1-2203064 | Enhancement on multi-beam operation | FUTUREWEI |
| --- | --- | --- | --- |
| 2 | R1-2203105 | Remaining issues on multi-beam operation in Rel-17 | Huawei, HiSilicon |
| 3 | R1-2203257 | Remaining issues on multi-beam enhancements | ZTE |
| 4 | R1-2203301 | Remaining issues on multi-beam enhancements | Spreadtrum Communications |
| 5 | R1-2203421 | Maintenance issues on Rel-17 multi-beam operation | CATT |
| 6 | R1-2203505 | Maintenance on multi-beam enhancement | vivo |
| 7 | R1-2203673 | Discussion on remaining issues on multi-beam operation | NEC |
| 8 | R1-2203764 | Maintenance of Enhancements on Multi-beam Operation | Langbo |
| 9 | R1-2203771 | Remaining issues on multi-beam operation enhancement | xiaomi |
| 10 | R1-2203855 | Maintenance on Rel-17 multi-beam | Samsung |
| 11 | R1-2203948 | Remaining Issues of Enhancements on Multi-Beam Operation | OPPO |
| 12 | R1-2204031 | Maintenance of multi-beam enhancements | Ericsson |
| 13 | R1-2204137 | Text proposal on multi-beam operation | LG Electronics |
| 14 | R1-2204169 | Remaining issues on muiti-beam operation | Lenovo |
| 15 | R1-2204192 | Maintenance issue of unified TCI power control | ASUSTeK |
| 16 | R1-2204199 | Remaining Issues on Beam Management Enhancement | Apple |
| 17 | R1-2204274 | Remaining issues of enhancements on multi-beam operation | CMCC |
| 18 | R1-2204335 | Remaining issues on multi-beam operation | NTT DOCOMO, INC. |
| 19 | R1-2204447 | Remaining issues on multi-beam enhancements | Spreadtrum Communications |
| 20 | R1-2204535 | Maintenance of enhancements on Multi-beam Operation | Nokia, Nokia Shanghai Bell |
| 21 | R1-2204680 | Maintenance of enhancements on multi-beam operation | Google Inc. |
| 22 | R1-2204682 | Maintenance of Rel-17 multi-beam operation | MediaTek Inc. |
| 23 | R1-2204763 | Enhancements on Multi-Beam Operations | Intel Corporation |
| 24 | R1-2204976 | Enhancements on Multi-beam Operation | Qualcomm Incorporated |