3GPP TSG RAN WG1 #109-e                                                   R1-2203259

e-Meeting, May 9th – 20th, 2022

Source:         ZTE

Title:            Remaining issues on multi-TRP deployment

Agenda item:    8.1.2

Document for:	Discussion and Decision

# Introduction

Overall, the basic aspects on Rel-17 multi-TRP related enhancements, i.e. MTRP PDCCH, MTRP PUCCH/PUSCH, MTRP inter-cell, BM for MTRP and MTRP HST, have been fulfilled so far, and most of the fundamental features have been captured in the current specifications. In this contribution, we provide our views on the remaining but essential issues related to Rel-17 MTRP related enhancements.

# Discussion

## Remaining issues on MTRP inter-cell

### 2.1.1	Text proposal on symbols overlapping between UL channel/signal and non-serving cell SSB in 38.213

Reason for change:l

With respect to the rule of symbol overlapping between UL channel/signal and non-serving cell SSB, the following agreement was reached in RAN1#108-e meeting [1]. Correspondingly, the current wording in TS 38.213 captures this agreement as “ ...by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states”. Meanwhile, according to the following agreement in RAN1#106-e meeting, it should be noticed that the active additional PCI should be associated with the activated TCI states for PDSCH/PDCCH only, not any other channels/signals [2]. Hence, the definition of the active TCI states for PDSCH/PDCCH should be clarified in the current specification accordingly.

Agreement (in RAN1#108-e) For inter-cell mTRP, UE does not transmit PUCCH/PUSCH/PRACH in a slot or SRS in the symbols if in time domain the PUCCH/PUSCH/PRACH/SRS overlaps with an SSB of a serving cell PCI or an SSB associated with the active additional PCI. Agreement (in RAN1#106-e) - For inter-cell mTRP, one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex, another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex - FFS: The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

Summary of change:

Clarifying that the activated TCI states of the active additional PCI is dedicated to PDSCH or PDCCH only when MTRP inter-cell operation.

Consequence if not approved:

The meaning of the activated TCI states of the active additional PCI will be unclear in Rel-17 MTRP inter-cell operation, which may incorrectly be interpreted as the TCI states activated for A/SP CSI-RS, for instance.

According to the above, we have the following text proposal for TS 38.213 V17.1.0.

Text Proposal 1:

<Start of the text proposal> ## 9.2.6 PUCCH repetition procedure <Unchanged parts are omitted> A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index corresponding to the SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDSCH or PDCCH. <Unchanged parts are omitted> ## 11.1 Slot configuration For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDSCH or PDCCH, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. <Unchanged parts are omitted> If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, for a set of symbols of a slot that are indicated to the UE for reception of SS/PBCH blocks in a first cell of the multiple serving cells by ssb-PositionsInBurst in SystemInformationBlockType1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDSCH or PDCCH, the UE does not transmit PUSCH, PUCCH, or PRACH in the slot if a transmission would overlap with any symbol from the set of symbols, and the UE does not transmit SRS in the set of symbols of the slot in - any of the multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - any one of the cells corresponding to the same band as the first cell, irrespective of any capability indicated by simultaneousRxTxInterBandCA. <Unchanged parts are omitted> ## 11.1.1 UE procedure for determining slot format For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indices corresponding to the SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, as described in clause 4.1 or, if the UE is not provided DLorJoint-TCIState or followUnifiedTCIstate, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDSCH or PDCCH, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink. <Unchanged parts are omitted> <End of the text proposal>

## Remaining issues on BM for MTRP

### 2.2.1	Text proposal on repetition parameter in different CSI Resource Sets in 38.214

Reason for change:

In RAN1#108-e meeting, RRC parameter repetition configuration in different CSI Resource Sets was agreed as follows [1]:

Agreement When the higher layer parameter groupBasedBeamReporting-r17 in CSI -ReportConfig is configured, the UE can be configured with the same or different value(s) of repetition in different CSI Resource Sets. FFS: UE reports CRI regardless of whether repetition is set to ‘on’ or not. Agreement When the higher layer parameter groupBasedBeamReporting-r17 in CSI -ReportConfig is configured, support the configuration of the same or different CSI -RS triggering offset values for two different CSI Resource Sets.

In Rel-16, if a UE is configured with a NZP-CSI-RS-ResourceSet configured with the higher layer parameter repetition set to 'on', the UE may assume that the CSI-RS resources within the NZP-CSI-RS-ResourceSet are transmitted with the same downlink spatial domain transmission filter, and CRI is not reported. However, multiple resource sets configured for group-based beam report are used for measuring the channel quality and simultaneous transmission. If the higher layer parameter repetition in one resource set is set to ‘on’, and in the other resource set is set to ‘off’ , that will confuse the UE whether to refine its Rx beam or scan the Tx beam from gNB side. To simplify the understanding of the UE, UE reports CRI regardless of whether repetition is set to 'on'.

Summary of change:

Clarifying that the report behavior of UE when the higher layer parameter repetition in one resource set is set to ‘on’.

Consequence if not approved:

Though same or different value(s) of repetition can be configures in different CSI Resource Sets. Once different values are configured, ambiguity in UE behavior of reporting RSRP/SINR will happen according to current spec.

According to the above, we have the following text proposal for TS 38.214 V17.1.0.

Text Proposal 2:

<Start of the text proposal> #### 5.2.1.4 Reporting configurations <Unchanged parts are omitted> When the UE is configured with higher layer parameter NZP-CSI-RS-ResourceSet and when the higher layer parameter repetition is set to 'off', the UE shall determine a CRI from the supported set of CRI values as defined in Clause 6.3.1.1.2 of [5, TS 38.212] and report the number in each CRI report. When the higher layer parameter repetition is set to 'on', CRI is not reported. When the higher layer parameter groupBasedBeamReporting-r17 in CSI-ReportConfig is configured, the UE shall report CRI for each of corresponding CSI resource sets regardless that repetition in the set is set to 'on'. CRI reporting is not supported when the higher layer parameter codebookType is set to 'typeII', 'typeII-PortSelection', 'typeII-r16', to 'typeII-PortSelection-r16', or 'typeII-PortSelection-r17'. <Unchanged part omitted> <End of the text proposal>

### 2.2.2 Text proposal on beam and power control update for PUCCH in 38.213

Reason for change:

In RAN1#107-e, there was a proposal in email discussion on beam management for multi-TRP, which is shown as follows [3].

Updated Proposal 6: After 28 symbols from receiving the BFR response, if a BFD-RS set on a spCell/PUCCH -SCell fails, the UL spatial filter for PUCCH resource(s) associated with the failed BFD-RS set on the spCell/PUCCH -SCell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) , and the power control parameter for the corresponding PUCCH transmission is set to [qu=0], qd= qnew , and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set, if 1. the PUCCH resource is provided with spatial relation info, [2. SR is not transmitted or is transmitted on PUCCH not associated with this failed BFR -RS set, and] 3. the failed BFD –RS set on the SpCell/PUCCH -SCell is indicated in the TRP –specific BFR MAC-CE FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set, E.g. the PUCCH group for group-based PUCCH -spatial relation info update is associated with each BFD -RS set.

In our views, PUCCH-related beam and power control update should be supported for facilitating TRP-specific uplink recovery as in PCell and SCell BFR procedure.

Summary of change:

Clarifying that PUCCH-related beam and power control update for facilitating TRP-specific uplink recovery should be as in PCell and SCell BFR procedure.

Consequence if not approved:

The meaning of the PUCCH-related beam and power control update will be unclear to Rel-17 MTRP BFR operation, which may cause confusion on update rules.

According to the above, we have the following text proposal for TS 38.213 V17.1.0.

Text Proposal 3:

<Start of the text proposal> #### 6 Link recovery procedures <Unchanged part omitted> For serving cells associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, and having radio link quality worse than Qout,LR, after 28 symbols from a last symbol of a first PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for transmission of the second PUSCH and having a toggled NDI field value, the UE assumes antenna port quasi-collocation parameters - corresponding to qnew from q1,0, if any, for the first CORESETs, - corresponding to qnew from q1,1, if any, for the second CORESETs and transmits PUCCH associated with the q0,0, or q0,1 on the PCell, or PSCell, or PUCCH-SCell using a same spatial domain filter as the one corresponding qnew, if any, and using a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, if - the UE is provided PUCCH-SpatialRelationInfo for the PUCCH, and - the index(es) of set q0,0, or q0,1 with radio link quality worse than Qout,LR on the PCell, or PSCell, or PUCCH-SCell are indicated by the MAC-CE where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the serving cells. <Unchanged part omitted> <End of the text proposal>

### 2.2.3 Simultaneously reporting BFR of Cell-Specific and TRP-Specific of different cells

Reason for change:

There is following agreement in RAN1#104b-e [4]:

Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs

In current 38.321, the BFR information of a first serving cell with cell-specific BFR and a second serving cell with TRP-specific BFR can be reported in a same enhanced MAC-CE, but it can’t be reported according to 38.213.

Summary of change:

It should be clarified that the second PUSCH MAC CE can also include BFR information of a serving cell with cell-specific BFR configuration. In addition, the condition to use the second PUSCH MAC-CE is that there is at least one serving cell configured with TRP-Specific BFR.

Consequence if not approved:

BFR information for a first serving cell with cell-specific BFR configuration and a second serving cell with TRP-specific BFR configuration has to be reported in different MAC-CEs. It leads to unnecessary overhead of MAC-CE. In addition, it doesn’t match with the BFR reporting method in 38.321.

According to the above, we have the following text proposal for TS 38.213 V17.1.0.

Text Proposal 4:

<Start of the text proposal> # 6. Link recovery procedures <Unchanged part omitted> If there is at least one serving cell associated with sets q0,0 and q1,0, and with sets q0,1 and q1,1, the UE can provide in a second PUSCH MAC CE index(es) for cell(s) with q0, or with at least one of q0,0 and q0,1 having radio link quality worse than Qout,LR, the index(es) of those q0,0 and/or q0,1, and indication(s) of presence of qnew and of index(es) qnew, if any, from corresponding sets q1,0 and/or q1,1 for the serving cells. <Unchanged part omitted> <End of the text proposal>

### 2.2.4	CFRA should also be supported when both TRPs of SpCell fail

There are following agreements about TRP-Specific BFR reached in RAN1#108-e meeting and RAN1#106b-e meeting [1, 5].

Agreement For multi-DCI based MTRP inter-cell, if Rel-17 per-TRP BFR is configured, SSB associated with an additional PCI can be configured as NBI-RS in one NBI -RS set, and the NBI-RS set is associated with the BFD-RS set associated with the additional PCI. Agreement Simultaneous configuration of “Rel-15/16 BFR” (i.e., BeamFailureRecoveryConfig/BeamFailureRecoverySCellConfig-r16) and “TRP-specific BFR” on one CC is not supported. Agreement For RACH-based transmission, at least when all BFD-RS sets fail in SPCell, CBRA is supported Agreement RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

If SpCell is configured with two BFD-RS sets, each of which is respectively associated with one different PCI, CBRA is triggered when both TRPs of SpCell fail, and then the UE only transmits PRACH to the serving cell instead of the neighboring cell by default. Otherwise, the UE needs to get the PRACH parameter of the neighboring cell and process cell selection. In Rel-15/16, if BFR is detected on SpCell, CFRA can be initiated. If only CBRA is supported in Rel-17, the link recovery procedure of Rel-17 is to be worse than Rel-15/Rel-16. So we propose that the CFRA should also be supported to keep same BFR speed as in Rel-15/16. Alternatively, we have to additionally introduce the mapping between NBI-RS in the NBI-RS set associated with non-serving cell PCI and PRACH resources.

Proposal 1: CFRA should be also supported when both TRPs of SpCell fail

- CFRA procedure can be triggered on a SpCell only based on a new candidate beam associated with the serving cell PCI.

- Note: the mapping between PRACH resources for BFR and NBI-RSes in the NBI-RS set associated with serving cell PCI can be reused.

### 2.2.5 MAC-CE based BFD-RS set update

In RAN1#108-e meeting, MAC-CE based BFD-RS set update was agreed as follows [1]:

Agreement Support to configure/update explicit BFD -RS set by RRC signaling and MAC CE signaling.

In beam recovery response, the UE will monitor the PDCCH by using the new beam. However, BFD-RS sets are only reconfigured by RRC signaling, which may cause the RS in BFD-RS sets to be inconsistent with the current PDCCH beam. Consequently, in RAN1#108-e, the MAC-CE based BFD-RS update has been agreed and then the specific update method should be been further clarified.

The UE can be activated with BFD-RS(s) per BFD-RS set by a MAC-CE directly. In other words, MAC-CE can further indicate N indexes from multiple BFD-RSs configured by RRC, wherein N is the maximum number of RSs supported in a BFD-RS set.

For Multi-TRP (e.g., TRP-0 and TRP-1), BFD-RS(s) should be updated per set. Specifically, UE can be configured two RRC TCI state pools corresponding to TRP-0 and TRP-1 respectively. The MAC-CE should select RS(s) from each pool and further include the TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index) to indicate the RS(s) are used for updating which BFD-RS set.

Proposal 2：Send a LS to RAN2 with the following:

- For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation can activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index).

## Remaining issues on MTRP HST

### 2.3.1	Text proposal on default TCI states for PDSCH scheduled by DCI format 1_0 in 38.214

Reason for change:

In RAN1#108-e meeting, default TCI states for SFN based PDSCH was agreed as follows [1]:

Agreement - When SFN is configured for PDSCH and not configured for PDCCH, TCI field should be always present in DCI formats 1_1 and 1_2 for SFN PDSCH transmission with scheduling offset larger than threshold timeDurationForQCL if applicable - FFS whether the above assumption is applicable for UE not capable of dynamic switching

In Rel-15/16, due to there is no TCI field in DCI format 1_0, single TRP based PDSCH should be scheduled. In Rel-17, DCI format 1_0 should also be used to schedule single TRP based PDSCH no matter SFN PDSCH is configured or not, i.e. the function of DCI format 1_0 should be the same in Rel-15/16/17. Hence if SFN PDSCH is configured and then it is scheduled by DCI format 1_0, dynamic switching between single TRP and SFN should be supported.

Summary of change:

Clarifying that when SFN is configured for PDSCH and not configured for PDCCH, dynamic switching between single TRP and SFN should be supported.

Consequence if not approved:

The meaning of the SFN based PDSCH transmission scheduled by DCI format 1_0 will be unclear to Rel-17 MTRP HST-SFN operation, which may cause confusion on TCI states acquisition.

Based on above analysis, we have the following text proposal for TS 38.214 V17.1.0.

Text Proposal 5:

<Start of the text proposal> ### 5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> When a UE is configured with both sfnSchemePdcch and sfnSchemePdsch scheduled by DCI format 1_0 or by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable: - if the UE supports DCI scheduling without TCI field, the UE assumes that the TCI state(s) or the QCL assumption(s) for the PDSCH is identical to the TCI state(s) or QCL assumption(s) whichever is applied for the CORESET used for the reception of the DL DCI within the active BWP of the serving cell regardless of the number of active TCI states of the CORESET. If the UE does not support dynamic switching between SFN PDSCH and non-SFN PDSCH, the UE should be activated with the CORESET with two TCI states. - else if the UE does not support DCI scheduling without TCI field, the UE shall expect TCI field present when scheduled by DCI format 1_1/1_2. When a UE is configured with sfnSchemePdsch scheduled by DCI format 1_0, dynamic switching between SFN PDSCH and non-SFN PDSCH is supported. <Unchanged part omitted> <End of the text proposal>

### 2.3.2	Text proposal on default TCI states for PDSCH when time offset equal or larger than threshold in 38.214

Reason for change:

In current specification, if only one TCI codepoint is activated for PDSCH by MAC CE, TCI field is not needed in DCI format 1_1 and 1_2. If dynamic switching between single TRP and SFN is not supported by the UE, MAC CE should active two TCI states for each codepoint, but if only one codepoint with two TCI states is activated by MAC CE, TCI field should be absent in the DCI.

When SFN PDCCH is not configured and SFN PDSCH is configured, if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal or larger than the threshold timeDurationForQCL, and if TCI field is absent in DCI format 1_1 and 1_2, and if a UE is not capable for dynamic switching between single TRP and SFN, the UE applies the two TCI states of the codepoint activated by MAC CE.

Agreement (in RAN1#108-e) - When SFN is configured for PDSCH and not configured for PDCCH, TCI field should be always present in DCI formats 1_1 and 1_2 for SFN PDSCH transmission with scheduling offset larger than threshold timeDurationForQCL if applicable - FFS whether the above assumption is applicable for UE not capable of dynamic switching

Summary of change:

Clarifying that when SFN is configured for PDSCH and not configured for PDCCH, if dynamic switching between single TRP and SFN is not supported, TCI field can be absent in DCI format 1_1 and 1_2, and the UE should apply the TCI states activated for PDSCH as the default TCI states when the scheduling offset is larger than threshold timeDurationForQCL if applicable.

Consequence if not approved:

When dynamic switching between SFN and non-SFN PDSCH transmission is not supported, two TCI states should be applied to the UE. However, based on the current specification, only one TCI state can be applied, so that will cause deviation on TCI states acquisition of SFN PDSCH transmission when the time offset is equal or larger than the threshold.

Based on above analysis, we have the following text proposal for TS 38.214 V17.1.0.

Text Proposal 6:

<Start of the text proposal> ### 5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, and if the UE supports dynamic switching between SFN PDSCH and non-SFN PDSCH, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present. When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, and if the UE does not support dynamic switching between SFN PDSCH and non-SFN PDSCH, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, if the TCI field is absent, the UE assumes that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the two TCI states in a codepoint activated for the PDSCH. <Unchanged part omitted> <End of the text proposal>

### 2.3.3	Text proposal on default TCI states for PDSCH when time offset less than threshold in 38.214

Reason for change:

If dynamic switching is not supported by a UE, and if SFN PDSCH is configured and SFN based PDCCH is not configured, the UE should apply two TCI states for the PDSCH reception. If single-TRP PDCCH and SFN PDSCH is configured, and enableTwoDefaultTCI-States is not configured for PDSCH scheduled by DCI Formats 1_0/1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, the UE should apply the activated TCI state of the CORESET, considering two TCI states should be applied by the UE in the case that dynamic switching between SFN and non-SFN PDSCH is not supported. Specifically, the TCI states of the first and the second lowest CORESET should be applied as the default TCI states.

Summary of change:

Clarifying that when SFN is configured for PDSCH and not configured for PDCCH, if dynamic switching between single TRP and SFN is not supported, the UE should apply the TCI states of the first and the second lowest CORESET as the default TCI states when the scheduling offset is less than threshold timeDurationForQCL if applicable.

Consequence if not approved:

When dynamic switching between SFN and non-SFN PDSCH transmission is not supported, two TCI states should be applied by the UE. However, based on current specification, only one TCI state can be applied, so SFN PDSCH transmission may be precluded incorrectly when the time offset is less than the threshold.

Based on above analysis, we have the following text proposal for TS 38.214 V17.1.0.

Text Proposal 7:

<Start of the text proposal> ### 5.1.5 Antenna ports quasi co-location <Unchanged parts are omitted> When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, when scheduled by DCI format 1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH of a serving cell is equal to or greater than a threshold timeDurationForQCL if applicable, the UE shall expect TCI field present. When a UE is configured with sfnSchemePdsch and sfnSchemePdcch is not configured, dynamic switching between SFN and non-SFN PDSCH is not supported, and enableTwoDefaultTCI-States is not configured for PDSCH scheduled by DCI Formats 1_0/1_1/1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, UE applies the activated TCI states of the CORESETs with the first and the second lowest CORESET IDs in the latest slot when receiving the PDSCH. <Unchanged part are omitted> <End of the text proposal>

### 2.3.4	Both two TCI states of CORESET 0 are used to monitor CSS type 0/0A/2

The following agreement is reached in RAN1#108-emeeting [1]. There is some discussion about which TCI states of CORESET 0 is used for monitoring CSS type 0/0A/2 of CORESET 0. Some companies support only one of two TCI states of CORESET 0 is used to monitor CSS type 0/0A/2. Firstly, TCI states are configured at CORESET level. If two TCI states are configured for CORESET 0 of a UE, then the UE can receive the two beams corresponding to the two TCI states. If only one of the two TCI states is used for CSS, the performance of USS is better than CSS. It is unreasonable. Secondly, CCE of USS and CSS type 0/0A/2 may be overlapped. If two TCI states are used for USS and one TCI state is used for CSS type 0/0A/2, the CCE shared by the CSS type 0/0A/2 and USS needs to be demodulated twice and the shared CCE should be counted at least two times.

Agreement For the response to RAN2 LS (in R1-2200886), the following is agreed Question: RAN2 would like to ask whether “Enhanced TCI state indication for UE specific PDCCH MAC CE” can be applied to CORESET zero or not.. - RAN1 response: There is no restriction in RAN1 on whether enhanced TCI state indication for UE specific PDCCH MAC CE can be applied to CORESET zero.

Correctly receiving the CSS type 0/0A/2 is important to keep link between UE and gNB. In current specification, only one monitoring occasion associated with one SSB is used for CSS, the reliability of CSS can’t be guaranteed at same level as USS which can be transmitted by multiple CORESETs. So two monitoring occasions of CSS can be determined by two SSB of the two TCI states.

Observation 1: If CORESET#0 is configured with two TCI state and only one of the two TCI states are used for CSS type 0/0A/2, the CCE shared by USS and CSS should be counted two times.

Proposal 3: If CORESET#0 is configured with two TCI states, both of the two TCI states should apply to the reception of a given CSS of the CORESET#0, and two monitor occasions of the given CSS can be individually determined according to the respective SSBs associated with those two TCI states.

# Conclusion

In this contribution, we discuss the remaining and essential issues on Rel-17 MTRP related enhancements and provide observation and proposals as follows. In addition, total seven text proposals with detailed updates of the current specifications are given in Section 2.

Observation 1: If CORESET#0 is configured with two TCI state and only one of the two TCI states are used for CSS type 0/0A/2, the CCE shared by USS and CSS should be counted two times.

Proposal 1: CFRA should be also supported when both TRPs of SpCell fail

- CFRA procedure can be triggered on a SpCell only based on a new candidate beam associated with the serving cell PCI.

- Note: the mapping between PRACH resources for BFR and NBI-RSes in the NBI-RS set associated with serving cell PCI can be reused.

Proposal 2：Send a LS to RAN2 with the following:

- For a given BWP in a CC, TRP-specific MAC-CE for BFD-RS activation should activate up to 2 BFD-RS(s) from respective RRC BFD-RS pools (including up to 64 BFD-RSs) for a TRP-ID (e.g. CORESETPoolIndex, BFD-RS set index).

Proposal 3: If CORESET#0 is configured with two TCI states, both of the two TCI states should apply to the reception of a given CSS of the CORESET#0, and two monitor occasions of the given CSS can be individually determined according to the respective SSBs associated with those two TCI states, i.e., a total of 2 monitoring occasions in such case.

# References

- Chair's Notes RAN1#108-e v27, February 21st - March 3rd, 2022, 3GPP

- Chair's Notes RAN1#106-e v24, August 16th - 27th, 2021, 3GPP

- Chair's Notes RAN1#107-e v18, November 11th - 19th, 2021, 3GPP

- Chair's Notes RAN1#104b-e final, April 12th - 27th, 2021, 3GPP

- Chair's Notes RAN1#106b-e v19, October 11th - 19th, 2021, 3GPP