3GPP TSG RAN WG1 #109-e						R1-2203261

e-Meeting, May 9th – May 20th, 2022

Agenda item:	8.1.3

Source: 	Moderator (ZTE)

Title: 	Moderator Summary for Rel.17 NR FeMIMO maintenance: Item 3: SRS

Document for:	Discussion and Decision

- Introduction

The moderator summary of the maintenance-related issues raised in the submitted contributions for Rel.17 NR_FeMIMO maintenance is given below.

An initial assessment on each of the issues is given (but can be revised based on the outcome of the discussion during the preparation week). The assessment will be used as a basis to select four issues (per chairman instruction) for further discussion in the upcoming weeks.

- High priority (H): this includes high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N): this includes all other purposes such as spec optimization and low priority issues

- Editorial (E): this includes editorial issues that will be handled as editorial CRs

- Maintenance issues

The issues are summarized in the following table:

Table 1 Summary

| # | Issue (summary of CR proposal) | Companies | FL assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| Sub-item 1: Flexibility enhancements |  |  |  |  |
| 1-1 | Collision handling for aperiodic SRSs In 38.214 section 6.2.1, clarify that whether/how to introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs. (R1-2203303, R1-2203423, R1-2203507, R1-2204171, R1-2204337) FL note 1: This issue has been discussed in the past, proponents had strong views of supporting it but opponents thought it can be handled by gNB scheduling even in a strict way. Given this issue is relevant to flexibility enhancements, it deserves further discussion. FL note 2: This issue has been discussed for > 2 meetings. | Spreadtrum, CATT, vivo, Lenovo, NTT DOCOMO | H (H:4, N:5) | Apple: We do not prefer to introduce any new SRS dropping rule on top of the current specification. OPPO: Not support. It is not preferred to introduce new functionality in the maintenance stage. Moreover, the collision can be addressed by gNB implementation Samsung: Not support. ZTE: We prefer introduce dropping rule when collision happens among multiple aperiodic SRS resource sets in a same CC or different CCs, the dropping rule can be the priority rules of Usage > CC ID > Set ID. QC: “N”, should be handled by gNB implementation. Ericsson: Same view as ZTE, dropping rule needed to avoid ambiguity issues. Intel: Ok for discussion. CATT: Support FL’s assessment. We have strong views on introducing dropping rule when collision happens. Without collision rule, gNB implementation would be complicated. Huawei, HiSilicon: Not support. It’s a new feature and should not be discussed in maintenance phase. vivo: need to resolve, should be H |
| 1-2 | TP on SRS configuration when T=R antenna switching In 38.214 section 6.2.1.2, clarify that the the legacy configurations of 2 periodic, or 2 aperiodic, or 1 periodic+1 aperiodic for Rel-17 UE with the capability of “Maximum 2 SP and 1 periodic SRS sets for antenna switching”. (R1-2204904, R1-2203423) FL note 1: This TP has been discussed in the past quite fully (four rounds in the last meeting), this was the compromise for companies but failed due to lack of time. It may be better to try it again to make progress. Companies are encouraged to check whether this TP can be endorsed directly. FL note 2: This issue has not been discussed before. | Huawei/HiSilicon, CATT | E | Apple: We are fine to discussion, but it appears to be very minor editorial change. OPPO: We support to discuss it Samsung: Support FL’s assessment. LGE: OK to discuss, which is editorial correction. ZTE: Support FL’s assessment QC: open for discussion Ericsson: open for discussion Intel: We are fine to discuss. But it should be technical change (H) instead of editorial. CATT: OK with E or H. Huawei, HiSilicon: Support FL’s assessment. Intel2: In RAN1 #106-e meeting, below agreement was reached. Agreement For antenna switching SRS, support maximum one SRS resource set for periodic SRS and maximum 2 SRS resource sets for semi-persistent SRS. - Note: the two SP-SRS resource sets are not activated at the same time - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Applies for all supported xTyR where y<=8 - For each xTyR antenna switching (except for 4T6R if supported), each periodic or semi-persistent resource set contains y/x resources. This feature is UE optional: For UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS It can be seen that the feature applies to all the xTyR configurations, including 1T1R/2T2R/4T4R. Following the TPs from companies, the previous agreement will be changed. For example, the UE (with xTyR and x=y) can be configured with 2 periodic SRS resource sets or 2 semi-persistent SRS resource sets which can be active at the same time when the UE indicates supporting the feature. Therefore, it should be technical change instead of editorial. We understand the issue and we are ok to discuss as long as it is H instead of E. In addition, we think the TP also has impact on the corresponding UE feature (FG 23-8-4) which has a note capturing the agreement from RAN1 #106-e, as copied below Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time FL: @Intel, according to the situation that almost all companies tend to treat issues#1-2 as “E”, this should be truth. For your concern above with reasonable motivation, actually, you still have the chance to address it at the stage of CR. Hope that can clarify. |
| 1-3 | TP on flexible DCI format In 38.214 section 6.1, introduce that AP-SRS triggering with DCI format 0_1/0_2 without data but with a non-zero "CSI request" where the associated "reportQuantity" in CSI-ReportConfig set to "none" for all CSI report(s) triggered by "CSI request" in this DCI format 0_1 or 0_2. (R1-2203423) FL note 1: The issue has been discussed in the past, which is a new feature but not an essential correction. FL note 2: This issue has been discussed for 1 meeting. | CATT | N (H:2, N:6) | Samsung: Support FL’s assessment. LGE: Agree with FL’s assessment. ZTE: Support FL’s assessment. QC: Agree with “N” Ericsson: N Intel: Open to discuss. We think it’s a valid point. CATT: We think it’s a valid point. Suggest to discussing. Huawei, HiSilicon: Support FL’s assessment. |
| 1-4 | TP on available slot for flexible SRS triggering In 38.214 section 6.2.1, change “the first symbol carrying the SRS request DCI” to “the first symbol of the PDCCH carrying the triggering command”. (R1-2203423) FL note 1: This TP seems technically correct and to be a good clarification. FL note 2: This issue has not been discussed before. | CATT | E (E:3, N:7) | Apple: We are fine to discuss, but the current specification is also clear OPPO: We failed to see what other interpretation it can be. The current is clear. Samsung: Not support. Spec. is already clear. LGE: Not essential. ZTE:We are fine with the TP which is clearer QC: We think it is different wording that has same interpretation as current spec Ericsson: N. We don’t see what the ambiguity is. Intel: The current spec seems clear. CATT: Which symbol is the first symbol carrying DCI is not clear. OK with E. Huawei, HiSilicon: Not support. The current spec seems clear enough. |
| 1-5 | TP on available slot for flexible SRS triggering In 38.214 section 6.2.1, remove K and clarify that for SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with same number of values of availableSlotOffset parameter. (R1-2203423) FL note 1: The use case of same number of values of availableSlotOffset parameter is unclear, more clarification is needed. Otherwise, there is no impact on the current spec. FL note 2: This issue has been discussed for 1 meeting. | CATT | H or N? (H:9, N:2) Count “E” as “H” since different TPs. | Apple: We are fine to discuss whether K shall be the same for all AP SRS resource set OPPO: We support to make the spec clearer Samsung: We think this issue is “N” since there is no agreement on using same number of values of availableSlotOffset parameter. DCM: Agree with Samsung. Also, according to the agreement below, we believe different number of values of availableSlotOffset across SRS-ResourceSet would work. We think it is RAN1’s common understanding to allow different number of values. Agreement (#107-e) For a CC with t value configured, SOI bit width depends on the maximum number of t values configured for all the resource sets across all configured BWPs in a CC for SRS transmission. - For the CCs without any t value configured, follow Rel-15/16 mechanism to determine the SRS slot offset, where SOI bit width is 0 LGE: OK to discuss but we think it is editorial correction. The K should be the same within a CC according to the previous agreement. Agreement Bit width of SOI depends on the maximum number of “t” values configured for any of the aperiodic SRS resource sets (FFS: across all CCs or across a CC/BWP) - The SOI field is 0 bit if the maximum number of ‘t’ values is one - If at least one resource set has “t” configured - For the resource sets with “t” value configured, each of them is configured with K values of “t”, where 1<=K<=4 - t=0 applies for the resource set(s) without “t” configured in RRC - If none of the resource sets is configured with “t” values, follow Rel-15 approach to determine slot offset ZTE: We support to make the spec clearer and make this issue to be ‘E’. In addition, according to above agreement given by DCM and following description in 38.212, - SRS offset indicator – 0, 1 or 2 bits. otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of AvailableSlotOffset configured for all aperiodic SRS resource set(s) in the scheduled cell;. we suggest following TP can be considered. For SRS resource set configured with availableSlotOffset parameter, each of resource set is configured with up to 4 values of availableSlotOffset parameter QC: open for discussion. Ericsson: ok to discuss this issue Intel: Agree with LGE’s view that following agreement, K should be the same for those SRS resource sets configured with ‘t’ values. But we are fine with CATT’s TP if companies think it’s clearer. CATT: Support with H. Since companies have different understanding on whether same number of values of availableSlotOffset should be configured for SRS resource sets, discussion is needed. Huawei, HiSilicon: Support to discuss it as “H”. vivo: ok to discuss |
| Sub-item 2: Antenna switching up to 8Rx |  |  |  |  |
| 2-1 | TP on inter-set Guard period In 38.214 section 6.2.1.2, clarify that whether/how to utilize inter-set GP to handle the case when the interval between SRS resources is larger than Y. (R1-2203507, R1-2203857, R1-2204139, R1-2204337, R1-2204537, R1-2204765, R1-2204978) FL note 1: Although this issue has been discussed multiple times in the past, this identified in the problem is valid and essential. Otherwise, UE behavior for antenna switching in this case will be ambiguous. FL note 2: This issue has been discussed > 2 meetings. | vivo, Samsung, LG, NTT DOCOMO, Nokia/NSB, Intel, Qualcomm | H (H:11, N:0) | Apple; We are fine to discuss OPPO: We are open to discuss it Samsung: Support FL’s assessment. DCM: Support FL’s assessment. LGE: support to discuss. ZTE: We are fine to discuss. QC: Agree with “H”. Ericsson: ok to discuss further Intel: Support to discuss CATT: Support FL’s assessment. Huawei, HiSilicon: Support FL’s assessment. vivo: support FL assessment |
| 2-2 | TP on intra-set Guard period In 38.214 section 6.2.1.2, clarify that the wording “guard period” is aimed to “intra-set guard period” only, except “inter-set guard period”. (R1-2204904) FL note 1: This TP seems a reasonable clarification but not urgent. It would be better to process this TP together with the result of issue#1 later. FL note 2: This issue has not been discussed before. | Huawei/HiSilicon | N (H:2, N:7) | OPPO: We are open to discuss it Samsung: Support FL’s assessment. LGE: Agree with FL’s assessment. ZTE:Support FL’s assessment. QC: Agree with “N”. Ericsson: Support FL plans for this issue. “N” Intel: Agree with FL’s assessment. CATT: Support FL’s assessment. Huawei, HiSilicon: Need to be treated as “H” or “E”. Without this TP, whether the collision handling rule of R15 should be used for inter-set guard period is unclear. Misunderstanding violating the reached agreement as well as causing conflict may occur. Agreement For inter-set guard period, UE does not transmit any other signal on any symbols of the interval if the interval between SRS resource sets is Y symbols. - When both the SRS resource on all of the corresponding symbols prior to the gap and the SRS resource on all of the corresponding symbols after the gap are dropped due to collision handling, the gap period is also dropped with same priority and can be used for UL transmission. - The above is the only collision handling rule to be introduced in Rel-17 for antenna switching guard period |
| 2-3 | TP on antenna switching for 1T2R/2T4R In 38.214 section 6.2.1.2, clarify that up to 3 SRS resource sets can be configured with different resource types for 1T2R/2T4R if only one antenna switching enhancement feature relative to 1T2R/2T4R is supported. (R1-2203507, R1-2204978) FL note 1: This TP seems technically correct and to be a good clarification. FL note 2: This issue has not been discussed before. | vivo, Qualcomm | H (H:9, N:0) | Apple: We are fine to discuss Samsung: Support FL’s assessment. LGE: Open to discuss. ZTE: We are fine to discuss QC: Agree with “H”. Ericsson: Agree with “H” Intel: More discussion is needed, and we are fine to discuss. CATT: OK to discuss. Huawei, HiSilicon: Ok to discuss. vivo: support FL assessment |
| 2-4 | TP on antenna switching for 1T2R/2T4R In 38.214 section 6.2.1.2, clarify that for this type of UE, aperiodic SRS for 1T2R/2T4R can only be configured when the UE is configured with zero or one peridoc/semi-persistent SRS resource set for antenna switching. (R1-2203950) FL note 1: This TP seems technically correct and to be a good clarification. FL note 2: This issue has not been discussed before. | OPPO | H (H:9, N:0) Discuss with issue 2-3 together, if approved. | Apple: We are fine to discuss OPPO: The issues 2-3 and 2-4 can be discussed together Samsung: Support FL’s assessment and fine with OPPO’s suggestion. LGE: This can be discussed together with issue 2-3. ZTE: We are fine to discuss QC: Agree with “H”. Ericsson: “H” vivo: ok to discuss Intel: More discussion is needed, and we are fine to discuss. Agree with OPPO and LGE that 2-3 and 2-4 could be discussed together. CATT: OK to discuss. Huawei, HiSilicon: The issues 2-3 and 2-4 can be discussed together. |
| 2-5 | TP on SRS for antenna switching In 38.214 section 6.2.1.2, change “'1t8r' for 1T8R, '2t6r' for 2T6R, '2t8r' for 2T8R, '4t8r' for 4T8R” to “'t1r8' for 1T8R, 't2r6' for 2T6R, 't2r8' for 2T8R, 't4r8' for 4T8R” in TS 38.214. FL note 1: Direct correction and this is editorial. FL note 2: This issue has not been discussed before. | CATT | E | Apple: Agree with the TP OPPO: Support Samsung: Support FL’s assessment. LGE: OK. QC: Agree with “E” Ericsson: “E” Intel: Fine with the TP. CATT: Support Huawei, HiSilicon: Support FL’s assessment. vivo: ok |
| 2-6 | TP on SRS for antenna switching In 38.214 section 6.2.1.2, clarify in TS 38.214 that for 1T2R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, and 4T8R, when multiple AP-SRS resource sets are configured, the multiple AP-SRS resource sets are configured with the same values of alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet. And the value of aperiodicSRS-ResourceTrigger or the value of an entry in AperiodicSRS-ResourceTriggerList in each SRS-ResourceSet is the same, and the value of slotOffset in each SRS-ResourceSet is different. (R1-2203423) FL note 1: This issue can be up to gNB implementation. If no strong motivation here, spec change is not needed. FL note 2: This issue has not been discussed before. | CATT | N (H:5, N:3) | OPPO: We are open to discuss it since similar description is used from R15 for 1T4R. Samsung: Support FL’s assessment. ZTE: Support FL’s assessment. QC: Open to discuss. Intel: Support to discuss and it should be H. We agree with CATT’s view since similar spec text exists for legacy 1T4R CATT: OK with H or E. We think such restriction is needed. Ericsson: Agree with Intel. This should be “H” and thus be discussed. Huawei, HiSilicon: Support FL’s assessment. vivo: ok to discuss |
| Sub-item 3: Coverage and capacity enhancements |  |  |  |  |
| 3-1 | Maximum number of CSs In 38.211 section 6.4.1.4.2, clarify that nSRSCS,max=6 for comb-2 and comb-4 in case of the length of SRS sequence is 6. (R1-2203260, R1-2203675) FL note 1: This issue identified in the problem is valid, because in the case of sequence length is 6, the current spec does not work well to use RPFS and Comb 2 or 4 for 4 ports. FL note 2: This issue has been discussed for 1 meeting. | ZTE, NEC | H (H:4, N:6) | Apple: We do not support to discuss this. RFPS does not have to work with all the SRS configuration, in fact, it does not work for all the SRS configuration as of the current specification. OPPO: We are open to discuss it Samsung: Not support. We do not prefer to make another maximum CS values for Comb-2 and Comb-4. DCM: We support to discuss this, since gNB’s configurability can be degraded without this. LGE: similar view with Apple. ZTE: We support to discuss this QC: This is non-essential. gNB can use subset of CSs or even don’t use this configuration at all. Ericsson: “H”, needs to be discussed. Intel: Similar view as Apple and Samsung Huawei, HiSilicon: Not support. This can be solved by implementation. vivo: seems not essential |
| 3-2 | TP on partial frequency hopping In 38.211 section 6.4.1.4.3, clarify that the candidate values of PF including 2 and 4 in a explicit way. (R1-2204171) FL note 1: Not critical (may be a good clarification but not needed). FL note 2: This issue has not been discussed before. | Lenovo | N (E:4, N:2) | Samsung: Support FL’s assessment. LGE: It seems editorial. ZTE:Support FL’s assessment. It will be clarified in 38.331. Ericsson: Clarification is useful. Intel: Open to discuss. And it could be editorial. Huawei, HiSilicon: Support to be treated as “E”. |
| 3-3 | TP on SRS frequency hopping In 38.214 section 6.2.1.1, change “pair” to “set” to avoid the confliction between “pair” and R adjacent symbols which can be more than 2 OFDM symbols. (R1-2203950) FL note 1: This TP seems technically correct and to be a good clarification. FL note 2: This issue has not been discussed before. | OPPO | E | Apple: Fine with TRP OPPO: Support Samsung: Support FL’s assessment. LGE: Open to discuss. ZTE: Agree with the TP QC: Open to discuss. Ericsson: Ok to discuss Intel: Ok with the TP Huawei, HiSilicon: Support FL’s assessment. |
| 3-4 | TP on parameter alignment for start RB hopping In 38.214 section 6.2.1, clarify that the parameter name for start RB hopping pattern should be khop to keep alignment with 38.211. (R1-2204904) FL note 1: Necessary correction and this is editorial. FL note 2: This issue has not been discussed before. | Huawei/HiSilicon | E | Apple: Fine OPPO: Support Samsung: Support FL’s assessment. LGE: OK ZTE: Support FL’s assessment. QC: Agree with “E” Ericsson: “E” Intel: Fine with FL’s assessment Huawei, HiSilicon: Support FL’s assessment. |
| 3-5 | TP on increase repetition In 38.214 section 6.2.1.1, clarify that the the condition of Ns>R is needed if Ns should be divisible by R. (R1-2203773) FL note 1: Not critical (may be a good clarification but not needed). FL note 2: This issue has not been discussed before. | xiaomi | N (H:3, N:4) | Samsung: Support FL’s assessment. Xiaomi: Note that there are two issues to be clarified. One issue is that the parameter configuration with R ≥ 2, Ns≥ R is for aperiodic SRS resource with intra-slot frequency hopping in current specification, which is not consistent with the agreement of RAN1#108e. This should be modified. The other issue is the configuration with R ≥ 2 without the condition of Ns>R for periodic or semi-persistent SRS resource. We think the condition is necessary since the R may be equal or large than Ns, which results hopping cannot work for periodic or semi-persistent SRS resource . LGE: Agree with FL’s assessment. ZTE: Support FL’s assessment. Ericsson: “N”, clarification is useful. Intel: Open to discuss Huawei, HiSilicon: Support to be partially treated as “E”, since the agreement in 108 hasn’t been correctly captured. Agreement TP 3-1A and TP 4-2 in Section 3 of R1-2202625 are endorsed for the editor’s CR on TS38.214. TP 4-2 A UE may be configured Ns=2 or,4,8,10,12 or 14 adjacent symbol aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across symbols when frequency hopping is configured with R=1. A UE may be configured Ns=≥4 adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across NsR pairs of R adjacent OFDM symbols, when frequency hopping is configured with R≥2, Ns>R and Ns should be divisible by R. Each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each pair of R adjacent OFDM symbols of the resource. |
| 3-6 | Maximum value of available slot offset 't' Allow the UE to drop the triggered A-SRS if the available slot offset is not reached within a specific span of time. FL note 1: This issue identified in the problem is valid and needs to be discussed. FL note 2: This issue has not been discussed before. | Qualcomm | H (H:8, N:1) | Apple: We are open to discuss OPPO: This issue was discussed but no consensus was achieved. Samsung: We are fine to discuss. LGE: Open to discuss. ZTE: We are fine to discuss. QC: Agree with “H”. Ericsson: ok to discuss. Intel: Open to discuss. Huawei, HiSilicon: Ok to discuss. Vivo: ok to discuss |

- Observation

From the inputs shared by participating companies during the preparation phase, the following observation can be made:

- The following issue can be handled as E (a part of editorial CR): issues #1-1, #1-2, #2-5, #3-2, #3-3, #3-4

- The following issues can be designated as H (requiring discussion and additional agreements/conclusions): issues #1-5, #2-1, #2-3, #2-4, #3-6

- The following issues can be designated as N (non-essential) but can be discussed again in future meetings: issues #1-4, #2-2, #2-6, #3-5

- The following issues can be designated as N (non-essential) and have been discussed in previous meeting(s): issues #1-3, #3-1

- References

| 1 | R1-2203260 | Remaining issues on SRS and CSI enhancements | ZTE |
| --- | --- | --- | --- |
| 2 | R1-2203303 | Remaining issues on SRS in Rel-17 | Spreadtrum Communications |
| 3 | R1-2203423 | Maintenance issues on SRS | CATT |
| 4 | R1-2203507 | Miscellaneous correction on Rel-17 MIMO | vivo |
| 5 | R1-2203675 | Discussion on remaining issues on SRS enhancement | NEC |
| 6 | R1-2203773 | Remaining issues on M-TRP beam management, SRS and CSI enhancement | xiaomi |
| 7 | R1-2203857 | Maintenance on Rel-17 SRS and CSI | Samsung |
| 8 | R1-2203950 | Other maintenance issues on further MIMO enhancements | OPPO |
| 9 | R1-2204030 | Remaining issues on CSI enhancements for Multi-TRP | Ericsson |
| 10 | R1-2204139 | Text proposal on SRS enhancement | LG Electronics |
| 11 | R1-2204171 | Remaining issues on SRS enhancements | Lenovo |
| 12 | R1-2204337 | Remaining issues on SRS enhancements | NTT DOCOMO, INC. |
| 13 | R1-2204537 | Maintenance of misceleneus Rel-17 MIMO issues | Nokia, Nokia Shanghai Bell |
| 14 | R1-2204765 | Remaining details on SRS enhancement | Intel Corporation |
| 15 | R1-2204904 | Remaining issues on SRS enhancement in Rel-17 | Huawei, HiSilicon |
| 16 | R1-2204978 | Remaining issues for SRS enhancement | Qualcomm Incorporated |