3GPP TSG RAN WG1 Meeting #109-e	R1-2203263

e-Meeting, May 9th – 20th, 2022

Source:	ZTE

Title:	Enhancements on unified TCI framework extension for multi-TRP

Agenda Item:	9.1.1.1

Document for:	Discussion and Decision

- Introduction

In RAN#94, a new Rel-18 WID on MIMO evaluation for DL and UL was approved as in [1]. In such new WID, UL-MIMO related enhancement involving 8-Tx UL operation, simultaneous Tx cross multi-panel (STxMP), TA and power control enhancement, unified TCI and UL-DMRS are prioritized as essential feature(s) for 5G-Advanced; meanwhile, DL-MIMO related enhancement is also considered in Rel-18 MIMO evolution. In this contribution, we elaborate our initial views on Rel-18 extension for indication of multiple DL/UL TCI states, simultaneous multi-panel UL transmission, and UL power control for enabling UL multi-panel transmission.

In [1], the following objective is identified for enhancement on unified TCI framework extension for multi-TRP.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

- Scenarios and usages for unified TCI extension for multi-TRP

In order to achieve a flexible beam updating scheme for DL/UL data and control transmission/reception with low signaling overhead and latency, unified TCI framework was introduced in Rel-17. Due to the time limitation, unified TCI in Rel-17 only supports single-TRP scenario. Straightforwardly, in Rel-18, unified TCI framework for indication of multiple DL and UL TCI states will be enhanced mainly for multi-TRP use case.

To well move forward unified TCI extension for multi-TRP in Rel-18, we have the following suggestion:

- Firstly, we need to identify the scenario of this enhancement and determine the scope of this WID.

- Then, we will handle the detailed issues for supporting the TCI framework enhancement, e.g., signaling design (e.g., RRC, MAC-CE, DCI) and the association between indicated TCI states and target channels. In addition, CC-specific TCI indication in CA should be further studied.

- After that, we can try to address other issues with lower priority, such as the priority rules when there is a collision between multiple beams, dynamic UE indication for panel capability and mTRP-BFR procedure in the unified TCI framework.

It should be noticed that, as for the channel/RS-specific TCI state indication, it currently seems that the Rel-15/16/17 legacy signaling can be reused, so it will also be considered with a lower priority.

Proposal 1: The enhancement of unified TCI in the multi-TRP scenario, including signaling design (e.g., RRC, MAC-CE, DCI) and the association between indicated TCI states and target channels should be considered with higher priority.

In order to have a unified solution for associating a given TCI state with target channels as well as corresponding RS(s), if needed, the following cases/schemes should be considered:

- DL/UL separate/joint TCI state configuration

- In Rel-17, DL/UL common pool and separate pools have both been supported for unified TCI framework. Joint TCI states in joint/DL-TCI pool can be indicated for both downlink and uplink transmission; then, DL TCI states in joint/DL-TCI pool can be indicated for downlink transmission, and UL TCI states in UL separate pool can be only indicated for uplink transmission. Based on the flexible pools configuration in Rel-17 TCI state framework, considering the purpose of Rel-18 is to further improve the performance of unified TCI, both joint/DL and UL TCI state pools should be considered.

- S-DCI(URLLC-Repetition)/M-DCI operation

- For S-DCI operation, only one DCI is used to indicate the TCI states for two TRPs in the case of multi-TRP, or two separate TCI states for DL and UL, but cannot support both TCI states for multi-TRP. Therefore, the DCI indication method needs to be further enhanced. Besides, one DCI needs to schedule multiple target channels. In this case, the association between the indicated TCI state(s) and the target channel(s) need to be further enhanced, especially for the PDSCH.

- For M-DCI operation, the indications of the two TRPs are relatively independent, the unified TCI state can be determined according to the legacy rules in Rel-17. Therefore, there may be minor spec impact for supporting the M-DCI scenario.

- Simultaneous multi-panel UL transmission

- After the unified TCI state indication, one or more PUSCH/PUCCH transmissions can be transmitted to (received from) two different TRPs with different beams by multiple UE panels. More details will be analyzed in our companion contribution [2].

Proposal 2: In order to have a unified solution for TCI framework, the following aspects should be considered:

- Both joint and separate DL/UL-TCI indication.

- S-DCI and M-DCI operation.

- Simultaneous multi-panel UL transmission.

- Enhancement for unified TCI state framework

Based on R17 unified TCI framework, QCL assumption and power control parameters can be obtained through the corresponding TCI state(s), in order to achieve a unified solution for associating a given TCI state with target channels as well as corresponding RS(s), the association in S-DCI and M-DCI multi-TRP need to be discussed separately. The association between TCI states and target channels is shown as Figure 1.

- M-DCI: Some target channels/RSs with semi-static configuration and TCI states can be associated according to the RRC/MAC-CE configuration (RRC/MAC-CE level associartion), such as CORESET/PDCCH, PUCCH and P/SP CSI-RS or SRS. And some dynamically triggered/scheduled channels/RSs and TCI states can be associated according to CORESET/PDCCH scheduling (DCI-level association), such as scheduled PDSCH/PUSCH, aperiodic CSI-RS/SRS.

- S-DCI: Same as M-DCI, CORESET/PDCCH, PUCCH and P/SP CSI-RS or SRS can still be associated with TCI states according to RRC/MAC-CE level association. For aperiodic CSI-RS/SRS and scheduled PUSCH, they can obtain the association according to the CSI-RS/SRS set index indicated by DCI. However, for PDSCH, there is no clear association indication. We will further discuss this issue in Section 3.3.

Figure 1 The association between TCI states and target channels

In order to implement the above association between target channels and TCI states, for multi-TRP, the following flexible framework should be considered. For sake of presentation, one example is shown in Figure 2.

- RRC pool configuration: To apply unified TCI for multi-TRP, RRC pool can be configured per TRP, which implies that, in RRC level, configured TCI state(s) are associated with corresponding TRP accordingly.

- MAC-CE activation: One TCI state codepoint can be activated with one or more TCI states, e.g., joint TCI states or DL/UL-TCI state pairs.

- DCI indication: Joint or DL/UL TCI states in the indicated TCI state codepoint can apply to all channel/RSs associated with respective TRPs accordingly.

Figure 2 Signaling structure for unified TCI framework for multi-TRP

  - RRC pool configuration

In general, there are independent transmission links between multiple TRPs and a single UE, so the QCL assumption and power control parameters should be determined per link. Thus, for RRC level, to apply the unified TCI framework to multi-TRP scenario, gNB can configure two TCI state sub-pools/pools corresponding to two TRPs respectively.

- Considering the number of configured TCI states for a UE is 128 for downlink and 64 for uplink in Rel-17, in order not to increase the complexity of UE, we think the total number of TCI states configured in the two sub-pools/pools should be kept the same as Rel-17.

- For joint TCI state pool configuration, gNB can configure 2 joint TCI state pools (can also be sub-pools) corresponding to 2 TRPs respectively. Each TCI states in the pool can be indicated for both downlink and uplink transmission for one TRP, as shown in Figure 3(a).

- For separate TCI state pool configuration, gNB can configure 4 separate TCI state pools (can also be sub-pools) (2 DL pools and 2 UL pools) corresponding to 2 TRPs respectively. Each TCI states in DL separate pool can be only indicated for downlink transmission for one TRP. TCI states in UL separate pool can be only indicated for uplink transmission for one TRP, as shown in Figure 3(b).

- Note: whether joint and DL TCI state pools can share a same pool from RRC signaling perspective as in Rel-17 can be discussed later after the above framework is stable.

(a)

(b)

Figure 3 TCI state sub-pool/pool configuration for multi-TRP operation

Proposal 3: Regarding unified TCI framework for multi-TRP, multiple TCI state pools corresponding to respective TRPs can be configured in RRC.

To indicate the association between TCI state pools and TRPs, we can introduce a TRP-ID for each pool.

- Considering multi-DCI scenario, UE is provided with two coresetPoolIndex values 0 and 1 for the first and second CORESETs, or is not provided with coresetPoolIndex value for the first CORESETs and is provided with coresetPoolIndex value of 1 for the second CORESETs, respectively. Therefore, coresetPoolIndex can be used as the TRP-ID.

- In single-DCI scenario, UE is not provided coresetPoolIndex for the first and second CORESETs, a new RRC parameter, such as TCI state pool ID, can be introduced as the TRP-ID. Furthermore, the new RRC parameter can also be used for a multi-DCI scenario.

To simplify the RRC parameter design, some existing indices can be also considered for achieving the association (used as TRP-ID) indirectly, such as UE capability index, physical cell index (PCI), CSI-RS resource set index, SRS resource set index.

- Note: UE capability index can be associated with the number of antenna ports (e.g., SRS antenna ports), the number of layers (e.g., for PUSCH, or PDSCH), or the UE panel ID/index.

Proposal 4: Regarding unified TCI framework for multi-TRP, TRP-ID can be introduced to indicate the association between TCI state pools and TRPs.

- For M-DCI, coresetPoolIndex can be used as TRP-ID.

- For S-DCI, TCI state pool ID can be used as TRP-ID.

- In order to strive to have a unified solution, TCI state pool ID can be considered for M-DCI also.

  - MAC-CE activation

In Rel-17, a unified TCI framework includes both joint indication and separate DL/UL indication. For joint indication, a TCI state in a TCI codepoint can be used for both DL and UL transmissions. For a separate indication, a TCI state in a TCI codepoint can be used for DL transmissions only or UL transmissions only, or a pair of TCI states in a TCI codepoint can be used for DL and UL transmissions, respectively. Mode of joint or separate indications can be configured by RRC, as in Rel-17.

For MAC-CE level, we have the following suggestion:

- If joint indication is configured by RRC, one codepoint can be associated with up to two TCI states corresponding to 2 TRPs respectively.

- If the separate indication is configured by RRC, one codepoint can be associated with up to four TCI states corresponding to 2 TRPs, and each TRP related to a pair of DL and UL TCI states.

Based on the unified TCI state activation/deactivation MAC CE in Rel-17, D/U field still can be used for indicating whether the TCI state is for joint/downlink or uplink TCI state. For unified TCI framework in multi-TRP scenario, TRP-ID should be further included in the MAC-CE to indicate the TCI state is activated for which TRP.

Proposal 5: Regarding unified TCI framework for multi-TRP, one codepoint can be associated with one or more joint/separate TCI states in a unified TCI state activation/deactivation MAC CE.

- TRP-ID should be included in the MAC-CE.

  - DCI indication

For DCI level, one TCI state codepoint can be indicated by a single TCI field. Since DCI format 1_1/1_2 with or without DL assignment has been supported for unified TCI indication, we can reuse the DCI format 1_1/1_2 for dynamical TCI indication in Rel-18.

Proposal 6: On unified TCI framework for multi-TRP, DCI format 1_1/1_2 with or without DL assignment can be reused for dynamic TCI indication.

As described above, one codepoint can be associated with one or more TCI states. If the codepoint contains multiple TCI states corresponding to multi-TRP, UE can apply the TCI states to the target transmission related to TRPs accordingly. If the codepoint only contains TCI state(s) corresponding to one TRP, such as a joint TCI state for TRP-0 or a DL/UL separate TCI state pair for TRP-0, UE can perform the following operations:

- Option 1: The scheduled PDSCH can switch from multi-TRP mode to single-TRP mode in SDCI-mTRP.

- Option 2: UE keeps multi-TRP mode and only updates TCI state for associated TRP.

Proposal 7: For DCI indication for unified TCI in multi-TRP, the following should be supported:

- Indicated TCI state(s) can be applied to each of TRP-link, respectively.

- Indicated TCI state(s) can be applied to one TRP-link.

- Support dynamic switching between multi-TRP and single-TRP modes for scheduled PDSCH.

In Rel-16/17 multi-TRP with single DCI case, one DCI can schedule multiple PDSCH corresponding to one or two TRPs. However, in dynamic switch scenario, if the gNB schedules a single PDSCH corresponding to one TRP, how to indicate one or more TCI states for scheduled PDSCH in the multi-TRP scenario should be further considered.

- For dynamic switch mode configuration for scheduling PDSCH reception, it can be configured by RRC explicitly according to a new parameter, such as DynamicSwitch-r18.

- When the dynamic switch mode is enabled, gNB needs to further indicate one or more TCI states for scheduled PDSCH from multiple indicated TCI states. In order to indicate TCI states more flexibly, we consider introducing TCI state selection function in DCI. For example, a new field (e.g., dynamic switch indicator) similar to “SRS resource set indicator” in DCI can be considered.

- For instance, the definition for each codepoint is shown as Table 1.

Table 1 Dynamic switch indicator to scheduled PDSCH

| Codepoint | Usage |
| --- | --- |
| 00 | Only used TCI state associated with TRP1 |
| 01 | Only used TCI state associated with TRP2 |
| 10 | TRP1,TRP2 transmission order |
| 11 | TRP2,TRP1 transmission order |

For multi-TRP, Figure 4 shows an example of dynamic switch. The UE is configured to receive multiple PDSCH at slot n. When slot n+1, gNB wants to schedule PDSCH2 from TRP2 only, the codepoint “01” will be indicated by the new field in DCI, then UE can determine TCI state associated with TRP2 for subsequent transmission. Therefore, at slot n+1, the UE only uses TCI state 2 to receive PDSCH2 from TRP2.

Figure 4 Dynamic switch process

Proposal 8: For dynamic switch mode on unified TCI framework for multi-TRP, the following should be supported:

- A new field (e.g., dynamic switch indicator) similar to “SRS resource set indicator” in DCI should be introduced for accommodating flexible TCI indication for scheduled PDSCH.

- UL power control for UL multi-panel transmission

In Rel-17, the enhancement on per TRP power control for single DCI based multi-TRP PUSCH repetition via TDM scheme has been specified, where second power control parameter set, second TPC command indication, and second OLPC set indication are introduced. In Rel-18, In order to support the enhancement of simultaneous multi-panel UL transmission, UL power control for simultaneous uplink signals from multiple panels should be further considered.

Basically, the transmission power of an uplink channel/signal is determined based on Tx power calculation and equally power scaling. Regarding Tx power calculation, the transmission power of PUSCH in the current specification can be calculated as an example,

,

wherein closed loop parameter(), open loop parameter(, ), pathloss compensation(), resource assignment() and maximum power limitation() are the main factors for calculating the transmission power.

Due to well supporting the association between UL power control setting and unified TCI state, UL transmission power can be determined directly in SDCI-mTRP. But, for SDCI-mTRP, in the case of single DCI scheduling uplink simultaneous transmission, signals/channels from multiple panels can be transmitted simultaneously. So, uplink transmission power calculation should be studied. In our views, we have the following two methods.

- Method 1: Transmission power is calculated per panel.

The method is to separately calculate the transmission power of an uplink signal/channel transmitted by each panel.

- Regarding open loop parameter, i.e. P0 and alpha, the separate configuration of alpha and P0 for multiple panels can bring flexibility, so that separate P0-alpha configuration for different panels should be supported.

- Regarding PCMAX, it is defined for a carrier of a serving cell in a transmission occasion in the current specification. If transmission power is calculated per panel, it is required to redefine PCMAX for each panel respectively or define rules to allocate the PCMAX for two panels, more details can be referenced from our contribution of AI 9.1.5 [3].

- Note: the antenna port architecture of multi-panel UE should be taken into account in this method, e.g., the number of ports configured for each panel can be different, and hence equally split of power among ports or panels (port groups) may need further study.

- Method 2: Transmission power is calculated per channel.

The method is to calculate the total transmission power of an uplink signal/channel transmitted by multiple panels.

- Open loop parameters separately or jointly configured to two panels should be determined.

- Regarding pathloss compensation, each transmission link can calculate a respective pathloss associated with a PL-RS, so that how to handle two values of pathloss in power calculation should be studied.

- Regarding PCMAX, the total transmission power for two panels is obtained, whether and how to further allocate the total power for panels should be considered.

Actually, the above two methods are raised for different PA architectures of multi-panel UE. That is, method 1 aims for the multi-panel UE with two separate PAs/Tx chains on two panels (e.g., in FR2), and method 2 aims for the multi-panel UE that one single PA/Tx chain shared to two panels (e.g., in FR1). We think these two PA architectures can be implemented in the future commercial market, and hence both of method 1 and method 2 should be considered.

Based on analysis above, the following proposal is raised.

Proposal 9: On UL power control for UL multi-panel transmission, the following methods should be considered for facilitating uplink power control in single DCI,

- Method 1: Transmission power is calculated per panel.

- Individual open loop parameters (i.e. P0 and alpha) and pathloss RS are determined for each panel.

- FFS: PCMAX definition for each panel and how to allocate PCMAX to each panel.

- Method 2: Transmission power is calculated per channel.

  - FFS: In terms of power scaling, whether and how to split the total Tx power for each ports/panels, rather than equally splitting.

  - FFS: How to configure/determine respective open loop parameters (i.e. P0 and alpha) to each panel or unified open loop parameters to all panels.

  - FFS: How to process pathloss calculation by each panel.

- Second TPC command and second SRS resource indicator can be reused for handling UL power control corresponding to the second panel.

In addition, according to the contribution we proposed in [2], several single DCI based simultaneous uplink transmission schemes can be considered, and the corresponding resource assignment of PUSCH/PUCCH can be different in each transmission scheme, therefore we prefer to postpone the discussion of uplink power control until such transmission schemes are determined in AI 9.1.4.1. Consequently, we have the following observation.

Observation 1: Power control for UL single DCI may should be discussed later until the simultaneous transmission schemes are determined in AI 9.1.4.1.

- Conclusion

In this contribution, we provide our initial views on Rel-18 unified TCI framework. Observations/ proposals are listed as follows.

Observation 1: Power control for UL single DCI may should be discussed later until the simultaneous transmission schemes are determined in AI 9.1.4.1.

Proposal 1: The enhancement of unified TCI in the multi-TRP scenario, including signaling design (e.g., RRC, MAC-CE, DCI) and the association between indicated TCI states and target channels should be considered with higher priority.

Proposal 2: In order to have a unified solution for TCI framework, the following aspects should be considered:

- Both joint and separate DL/UL-TCI indication.

- S-DCI and M-DCI operation.

- Simultaneous multi-panel UL transmission.

Proposal 3: Regarding unified TCI framework for multi-TRP, multiple TCI state pools corresponding to respective TRPs can be configured in RRC.

Proposal 4: Regarding unified TCI framework for multi-TRP, TRP-ID can be introduced to indicate the association between TCI state pools and TRPs.

- For M-DCI, coresetPoolIndex can be used as TRP-ID.

- For S-DCI, TCI state pool ID can be used as TRP-ID.

- In order to strive to have a unified solution, TCI state pool ID can be considered for M-DCI also.

Proposal 5: Regarding unified TCI framework for multi-TRP, one codepoint can be associated with one or more joint/separate TCI states in a unified TCI state activation/deactivation MAC CE.

- TRP-ID should be included in the MAC-CE.

Proposal 6: On unified TCI framework for multi-TRP, DCI format 1_1/1_2 with or without DL assignment can be reused for dynamic TCI indication.

Proposal 7: For DCI indication for unified TCI in multi-TRP, the following should be supported:

- Indicated TCI state(s) can be applied to each of TRP-link, respectively.

- Indicated TCI state(s) can be applied to one TRP-link.

- Support dynamic switching between multi-TRP and single-TRP modes for scheduled PDSCH.

Proposal 8: For dynamic switch mode on unified TCI framework for multi-TRP, the following should be supported:

- A new field (e.g., dynamic switch indicator) similar to “SRS resource set indicator” in DCI should be introduced for accommodating flexible TCI indication for scheduled PDSCH.

Proposal 9: On UL power control for UL multi-panel transmission, the following methods should be considered for facilitating uplink power control in single DCI,

- Method 1: Transmission power is calculated per panel.

- Individual open loop parameters (i.e. P0 and alpha) and pathloss RS are determined for each panel.

- FFS: PCMAX definition for each panel and how to allocate PCMAX to each panel.

- Method 2: Transmission power is calculated per channel.

  - FFS: In terms of power scaling, whether and how to split the total Tx power for each ports/panels, rather than equally splitting.

  - FFS: How to configure/determine respective open loop parameters (i.e. P0 and alpha) to each panel or unified open loop parameters to all panels.

  - FFS: How to process pathloss calculation by each panel.

- Second TPC command and second SRS resource indicator can be reused for handling UL power control corresponding to the second panel.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- R1-2203268, Enhancements on UL precoding indication for multi-panel transmission, ZTE

- R1-2203270, Evaluation assumptions for CSI, simultaneous multi-panel UL transmission and 8-Tx UL operation, ZTE