3GPP TSG RAN WG1 Meeting #109-e	R1-2203270

e-Meeting, May 9th – 20th, 2022

Source:	ZTE

Title:	Evaluation assumptions for CSI, simultaneous multi-panel UL transmission and 8-Tx UL operation

Agenda Item:	9.1.5

Document for:	Discussion and Decision

- Introduction

In RAN#93, a new WID for MIMO evolution for downlink and uplink was approved for Rel-18 [1]. Among items in this WID for DL and UL MIMO, several items need to be justified by comprehensive evaluation before specifying corresponding enhancements. Therefore, in this contribution, we elaborate our views on SLS evaluation assumptions for CSI (involving Doppler related CSI and CJT), simultaneous multi-panel UL transmission and 8-Tx UL operation.

- Evaluation assumptions for CSI

In this subsection, we discuss the individual evaluation assumptions for Doppler related CSI enhancement and coherent joint transmission (CJT).

  - On issue 1: Doppler related CSI enhancement

Among items in this WID for DL and UL MIMO, the aspects for Doppler related CSI enhancement are listed as below.

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking

    - Scenario and evaluation methodology

For high/medium UE velocities (e.g., 60 km/h or 120km/h), UE should be traveled across the high way or avenue, and UE trajectory should be specified rather than randomly dropped. In our views, the high-way scenario with linear UE trajectory as in R17 multi-beam operation can be considered as a starting point.

  - For example, the diagram for UE mobility and trajectory handling for Doppler CSI can be found in Figure 1. The UE is randomly dropped in the high way (e.g., line PQ), and moves along the 120-deg line downward to Q. Then, the UE should be constrained within one cell (inter-cell mobility is not included). Then, trajectory sampling should be spaced within the de-correlation distance, and the distance between gNB and trajectory should be specified, such as 30m.

As a basic requirement for this enhancement, the spatial and frequency domain basis should not be modified. Therefore, we need to evaluate the time efficiency of given spatial and frequency domain basic vector(s). Clearly, if still reusing traditional drop-based approach, we can not evaluate the corresponding time efficiency, and then the evaluation results to prove the benefits of Doppler-CSI enhancement may also be challenged. In our views, the channel model of spatial consistency should be used, besides traditional drop based.

  - In TR 38.901, there are two alternative spatial consistency procedures – Procedure A and Procedure B. For the former, cluster specific delay and angle is updated using a cluster-wise transformation of the UT velocity vector (like linear extrapolation); for the latter, the above parameter are interpolated within the coherence length or time of the respective parameter. In our views, the realistic channel variation (e.g., sporadic change) can be better emulated in Procedure-B.

Figure 1 UE mobility and trajectory handling for Doppler CSI related evaluation

After that, in order to save the CSI-RS and CSI overhead, CSI-RS burst should be considered in the simulation, and the CSI report should be performed after measuring a whole CSI-RS burst and can be available for the whole period. For instance, the periodicity of CSI-RS burst is N4, and then the duration for the burst is NT, as shown in Figure 2. The length of N4 should be evaluated based on the above high-way scenario with spatial consistency procedure. Due to Doppler domain prediction, CSI-RS transmission for CSI between two CSI-RS burst can be saved.

Figure 2 Diagram for CSI-RS burst transmission

Proposal-1: Regarding Rel-16/17 Type-II codebook refinement for Doppler-CSI prediction/compression,

  - For emulating high/medium-speed UE mobility, high-way scenario with linear UE trajectory should be specified.

  - Channel model of spatial consistency (e.g., procedure-B in TS 38.901) should be considered besides traditional drop based.

  - The duration and periodicity of CSI-RS burst is relevant to CSI prediction/compression and can be up to proponent companies.

Regarding time-domain channel properties measured via CSI-RS for tracking, in the wireless communication systems, as a basic requirement for time/frequency-domain synchronization, UE should always estimate the real-time Doppler shift and compensate it accordingly, via SSB and CSI-RS for tracking. Besides for UE mobility as mentioned in above objectives, the Doppler shift is mainly contributed by a slight deviation of centric frequency between gNB and UE oscillators. Therefore, from UE perspective, it is impossible to identify the respective Doppler shift introduced by UE mobility and imbalance between gNB and UE oscillators. Therefore, if UE always need to compensate the mixed Doppler shift impact for synchronization, the report of Doppler-shift estimate based on either a single CSI-RS (i.e., CSI-RS for tracking) or a unique spatial basic vector becomes meaningless.

Observation 1: From the perspective of UE receiver, Doppler shift is contributed by both UE mobility and deviation of centric frequency between gNB and UE oscillators and can be compensated for synchronization in UE, and therefore the necessity of the report of a single Doppler-shift estimate based on either a single CSI-RS or a unique spatial basic vector (e.g., Type I codebook) is unclear.

Therefore, a potential usage for UE reporting of time-domain channel properties measured via CSI-RS for tracking may be much relevant to the mTRP-HST scenario as discussed in Rel-17, rather than sTRP. If so, the agreed evaluation assumption for Rel-17 mTRP-HST can be considered as a starting point.

    - Simulation assumption

For Rel-16/17 Type-II codebook refinement for Doppler-CSI prediction/compression in Rel-18, we have the following SLS assumption as in Table-1.

Table-1 SLS evaluation assumption for Doppler related Type-II codebook refinement

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2GHz |
| Channel Model | According to the TR 38.901 3D UMa |
| Scenario | Dense urban |
| Antenna setup and port layouts at gNB | (M, N, P, Mg, Ng, Mp, Np) =(4,2,2,1,1,4,2). (dH, dV) = (0.5, 0.8) |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) (dH, dV) = (0.5, 0.5) |
| UE mobility and trajectory handling | The UE is randomly dropped in the high way (e.g., line PQ), and moves along the 120-deg line downward to Q. Then, the UE should be constrained within one cell (inter-cell mobility is not included). For emulating UE movement, the following two options can be considered: - Option-1: Drop-based - Option-2: Spatial consistency Procedure-B in TS 38.901 Notes: Trajectory sampling should be spaced within the de-correlation distance |
| BS Tx power | 44 dBm（38.802 TableA2.1-1） |
| UE Tx power | 23dBm |
| BS antenna height | 25 m（38.802 TableA2.1-1） |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Numerology | 14 OFDM symbol slot, 30kHz SCS |
| Bandwidth | 20MHz |
| UE receiver | MMSE-IRC |
| Network Layout | 10 UEs per cell (in a total of 7 cells) |
| PMI/CQI feedback | Subband |
| UE distribution | 100% outdoor (30km/h, 60km/h, or 120km/h) |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes. |
| Traffic load (Resource utilization) | 25%, 50% and 80% (38.802 Table A2.1-1) |
| MIMO scheme | SU-MIMO with rank adaptation Maximum rank = 4 per UE |
| Performance metrics | Average UPT and cell-edge/95%-ile UPT |

  - On issue 2: CJT

Among items in this WID for CJT, the aspects for enhancement on CJT are listed as below.

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

    - Scenario and evaluation methodology

Regarding the enhancement on CSI acquisitions for coherent-JT, we think that the legacy topology for TRP deployment should be considered as a starting point. Then, the deployment scenarios can be divided into the following three categories: a) intra-site, b) inter-site, and c) intra-&inter-site as shown in Figure 3. Then, if RSRP distance between the optimal TRP and the second optimal TRP (e.g., intra-site and/or inter-site) is smaller than a threshold (e.g., 6 dB), coherent joint transmission using the corresponding TRPs is performed; otherwise, sTRP based transmission is used.

Figure 3 Deployment scenarios for coherent JT: (a) intra-site, (b) inter-site, (c) intra-&inter-site

Besides, considering that real-time multi-UE paring and precoding determination in MU-MIMO is hard from the perspective of product implementation, especially for inter-site, we think that SU-MIMO should be considered as a starting point especially for inter-site case, and MU-MIMO can be evaluated with second priority.

Proposal-2: Regarding evaluation on CJT enhancement,

  - Legacy topology for TRP/cell as widely used in 3GPP should be considered firstly, and then we may further discuss other topologies.

  - Intra-site, inter-site, and intra-&inter-site scenarios can be considered for evaluation.

  - Compared with MU-MIMO, SU-MIMO should be considered with high priority due to realistic product deployment.

    - Simulation assumption

For CJT enhancement, we have the following SLS simulation assumption as in in Table-1.

Table-2 SLS evaluation assumption for CJT enhancement

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 2GHz |
| Channel Model | According to the TR 38.901 3D UMa |
| Scenario | Dense urban |
| Antenna setup and port layouts at gNB | (M, N, P, Mg, Ng, Mp, Np) = (4,2,2,1,1,4,2). (dH, dV) = (0.5, 0.8) |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng, Mp, Np) =(1,2,2,1,1,1,2) (dH, dV) = (0.5, 0.5) |
| Inter-BS distance | Mode 1 Intra-site C-JT Coherent joint transmission through different sectors of the same site. Mode 2 Inter_intra-site C-JT(200m/500m) Coherent joint transmission through different sectors of different or same sites (up to four TRPs). |
| BS Tx power | 44 dBm（38.802 TableA2.1-1） |
| UE Tx power | 23dBm |
| BS antenna height | 25 m（38.802 TableA2.1-1） |
| BS receiver noise figure | 5dB |
| UE receiver noise figure | 9dB |
| Traffic load (Resource utilization) | 25, 50 and 80%(38.802 TableA2.1-1) |
| Numerology | 14 OFDM symbol slot, 30kHz SCS |
| Bandwidth | 20MHz |
| UE receiver | MMSE-IRC |
| Network Layout | 20 UEs per cell |
| Precoding granularity | 2RB |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes. |
| MIMO scheme | SU-MIMO with rank adaptation Maximum rank = 4 per UE |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| Coordination assumptions | Step1: Selection According to the RSRP received by multiple TRPs, select N optimal TRPs for subsequent data transmission - If RSRP distance between the optimal TRP and the second optimal TRP is smaller than threshold (6dB), coherent joint transmission is used. - Otherwise, single-TRP is used for transmission. Step2: CSI measurement & report Ideal SVD-based or enhanced Type II based CSI acquisition can be used for CJT. |
| Performance metrics | Average UPT and cell-edge/95%-ile UPT |

- Evaluation assumptions for simultaneous multi-panel UL transmission

Among items in this WID for DL and UL MIMO, the aspects for enhancement on simultaneous multi-panel UL transmission are listed as below.

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

  - Scenarios and evaluation methodology

In this section, due to the enhancement is relevant to CPE/FWA/vehicle/industrial devices rather than the legacy handheld device, maximum Tx power (e.g., a total of 23 dBm, or 23dBm +23dBm) should be considered/specified in the evaluation assumption. Then, for the scenarios, both indoor hotspot and dense urban can be considered. In the setup, 4 panels at TRP side and two panels (back to back) at the UE side can be considered as a starting point.  Each panel contains two TXRUs for dual-polarization.

- Regarding simultaneous multi-panel UL transmission,

  - In UE side, one best Tx beams per UE panel is selected with the objective of maximizing RSRP according to UL beam measurement.

  - In gNB side, with the objective of maximizing RSRP, one best RX beam per UE-panel-specific selected Tx beam is determined to be used for different TRP panel set.

    - Note: A single TRP but multi-TRP-panel operation is also considered as for simultaneous multi-panel UL transmission.

- Regarding UE panel-selection for UL single panel operation as legacy approach,

  - In UE side, one best Tx beam and its corresponding panel is selected with the objective of maximizing RSRP according to UL beam measurement;

  - In gNB side, only one best Rx beam corresponding to the above selected UE Tx beam is selected to be used for all TRP panels in this simulation.

Proposal-3: Regarding evaluation on simultaneous multi-panel UL transmission,

  - Due to targeting CPE/FWA/vehicle/industrial devices, maximum Tx power, e.g., a total of 23 dBm, or 23dBm+23dBm, and two panels (back to back) at the UE side should be considered as a starting point.

  - For evaluation, Indoor hotspot and dense urban scenarios can be considered, especially for Indoor hotspot in FR2.

  - Simulation assumption

For simultaneous multi-panel UL transmission enhancement, we have the following SLS simulation assumption for indoor hotspot as in in Table-3.

Table-3 SLS evaluation assumption for simultaneous multi-panel UL transmission in indoor hotspot

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | UL only |
| Bandwidth | 50 MHz |
| Subcarrier spacing | 60 KHz |
| Channel Model | According to the TR 38.901 |
| Scenario | Indoor hotspot |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | All panels are attached with one TRP Step-1: Single TX/RX subelement - without TX/RX beamforming - The first sub-element per panel is used with single-pol. - The best TRP is selected cross all UE panels, in terms of coupling loss; Step-2: With analog TX/RX beamforming, one best Tx-Rx beam pair is selected per UE panel according to the TRP identified in step-a. |
| ISD | 20m |
| UE Tx power | Option 0: 23 dBm Option 1: 23 dBm + 23 dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1), (dV,dH) = (0.5, 0.5)λ Boresight direction is perpendicular to the ceiling |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| UE receiver type | MMSE-IRC |
| UL Codebook | SVD-based |
| MIMO scheme | SU-MIMO with rank adaptation Maximum rank = 4 per UE (up to 2 CWs) |
| Performance metrics | Average UPT and cell-edge/95%-ile UPT |

- Evaluation assumptions for 8-Tx UL operation

Among items in this WID for DL and UL MIMO, the aspects for enhancement on 8-Tx UL operation are listed as below.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

  - Evaluation methodology

For evaluating the performance of UL precoding, coherent type and UE antenna architecture for 8-Tx operation should be specified. For instance, we have the following suggested architecture as in Figure 4 for evaluation, especially for coherent case (2*2 and 4*1). Then, as for simultaneous multi-panel UL transmission, 8-TX UL operation is mainly targeted for CPE/FWA/vehicle/Industrial devices, based on the RAN4 requirement for CPE, the maximum Tx power should be assumed as 31 dB as a starting point. Finally, both indoor hotspot and dense urban can be considered, and if considering CPE in FR1 as the most attractive usage for 8-Tx operation based on NR market analysis, dense urban should be paid attention well.

Figure 4 8-Tx UE antenna architecture for: (a)/(b) full coherent with different N1/N2 config, (c)/(d) partial coherent

Proposal-3: Regarding evaluation on simultaneous multi-panel UL transmission,

  - Due to targeting CPE/FWA/vehicle/industrial devices, maximum Tx power in a total of 31 dBm and typical coherent/partial-coherent UE antenna architecture (as shown in Figure 4) should be considered as a starting point.

- For UL codebook evaluation, UE coherent-type (e.g., 1*4, or 2*2) should be considered with higher priority.

  - For evaluation, indoor hotspot and dense urban scenarios can be considered, especially for dense urban in FR1

  - Simulation assumption

For 8-Tx UL operation enhancement, we have the following SLS simulation assumption for dense urban as in Table-4.

Table-4 SLS evaluation assumption for simultaneous multi-panel UL transmission in dense urban

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Mode | UL only |
| Bandwidth | 20 MHz |
| Subcarrier spacing | 30 KHz |
| Channel Model | UMi in TR 38.901 |
| Scenario | Dense urban |
| Antenna setup and port layouts at gNB | (M, N, P, Mg, Ng, Mp, Np) = (4,4,2,1,1,4,4). (dH, dV) = (0.5, 0.8) |
| Antenna setup and port layouts at UE | (M, N, P, Mg, Ng, Mp, Np) =(2,2,2,1,1,2,2) or (M, N, P, Mg, Ng, Mp, Np) = (4,1,2,1,1,4,1) (dH, dV) = (0.5, 0.5) |
| UE Tx power | 31 dBm in total |
| ISD | 200m |
| Layer-to-CW mapping | 1 CW @ RANK1~4 2CWs @ RANK 5~8 |
| UL power control | Open loop TPC |
| power control parameter | P0 = -50, Alpha = 0.8 |
| Modulation | Up to 64QAM |
| Scheduling | Subband PF |
| Traffic model | Full buffer |
| UE distribution | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver type | MMSE-IRC |
| UL Codebook | Baseline: SVD-based or DL TypeI codebook Wideband |
| MIMO scheme | SU-MIMO with rank adaptation Maximum rank = 8 per UE (up to 2 CWs) |
| Performance metrics | Average UPT and cell-edge/95%-ile UPT |

- Conclusion

In this contribution, we discuss evaluation assumption for Rel-18 DL and UL MIMO, including SLS evaluation assumption for CSI (involving Doppler related CSI and CJT), simultaneous multi-panel UL transmission and 8-Tx UL operation. The observations, proposals, and detailed simulation assumption can be found in the above section 2~4.

- References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator)

- R1-2007151, R17 FeMIMO summary Item 1 EVM, Samsung