3GPP TSG RAN WG1 #109-e                                                                            R1-2203271

e-Meeting, May 9th – 20th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | xxxx | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on sequence hopping of PUSCH DMRS with mapping type B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In section 6.4.1.1.3 in 38.211, for PUSCH mapping type B, the symbol index‘’ refers to the symbol index relative to start of the scheduled PUSCH resources or the start of each hop as shown in the following, - for PUSCH mapping type B: - is defined relative to the start of the scheduled PUSCH resources if frequency hopping is disabled and relative to the start of each hop in case frequency hopping is enabled -. But the symbol index ‘’ used to generate the sequence in section 6.4.1.1.1.2 should refer to an absolute symbol index in a slot, in order to keep orthogonality between sequences of overlapped PUSCHs from different UEs. |
| Summary of change: |  | Clarifying the symbol index ‘’ used to generate the sequence in section 6.4.1.1.1.2 refers to an absolute symbol index in a slot. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If two PUSCHs of different UEs full/partial overlaps and with different mapping type or different frequency hopping configuration, those DMRS may not be orthogonal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: It is expected no isolated impact. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

If transform precoding for PUSCH is enabled, the reference-signal sequence  shall be generated according to

where ru,vα,δ(m) is given by clause 5.2.2 with  and  for a PUSCH transmission.

The sequence group , where  is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE and the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213].

-	nIDRS=NIDcell otherwise

where  and the sequence number v are given by:

-	if neither group, nor sequence hopping is enabled

-	if group hopping is enabled and sequence hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame

-	if sequence hopping is enabled and group hopping is disabled

where the pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame.

The hopping mode is controlled by higher-layer parameters:

-	for PUSCH transmission scheduled by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, sequence hopping is disabled and group hopping is enabled or disabled by the higher-layer parameter groupHoppingEnabledTransformPrecoding;

-	for all other transmissions, sequence hopping and group hopping are enabled or disabled by the respective higher-layer parameters sequenceHopping and sequenceGroupHopping if these parameters are provided, otherwise, the same hopping mode as for Msg3 shall be used.

The UE is not expected to handle the case of combined sequence hopping and group hopping.

The quantity l above is the OFDM symbol number in the slot except for the case of double-symbol DMRS in which case l is the OFDM symbol number in the slot ns,fμ of the first symbol of the double-symbol DMRS.