3GPP TSG RAN WG1 #109-e	R1-2203274

e-Meeting, May 9th – 20th, 2022

Agenda item:		8.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-17 URLLC / IIoT Intra-UE Multiplexing/Prioritization

Document for:		Discussion and Decision

# Introduction

In this contribution, we present our views on the remaining open points on UL intra-UE multiplexing and prioritization. Our discussion is built on top of the discussions, agreements and open issues related to Rel-17 IIoT/URLLC WI. The latest moderator summary from RAN1#108-e can be found in R1-2202770 & R1-2202830, respectively. The list of all agreements reached during the WI phase on URLLC/IIoT including all the agreements on Intra-UE multiplexing can be found in R1-2202776.

In Section 2, we discuss remaining open points on the overall Rel-17 intra-UE multiplexing framework. In Section 3, we discuss remaining open points on the enhancements related to intra-UE multiplexing/prioritization of traffic with different priorities.

# Discussion on the overall Rel-17 PHY multiplexing and prioritization framework

In this section, we discuss some of the remaining open points on the Rel-17 Intra-UE multiplexing/prioritization framework.

## Collision resolution involving LP PUCCH with HARQ-ACK and CSI/SR

For handling the overlap of more than two PUCCHs with different priorities including LP PUCCH with HARQ-ACK and CSI/SR, the following agreement was made in RAN1#108-e:

Agreement To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 2/3/4 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped. - FFS HP PUCCH with PUCCH format 0/1

As can be seen from the above agreement, one remaining open point is the case where the HP PUCCH carrying HP SR and HP HARQ-ACK is with PF 0/1. This is closely related to the pending issue on whether to support the multiplexing of HP HARQ-ACK, HP SR, and HP HARQ-ACK in this case, which is discussed in Sec. 3.1.1. Since we support such multiplexing, in our view the above agreement should be extended to also cover the case where the HP PUCCH is with PF 0/1.

Proposal 2.1: To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

# 3	Intra-UE multiplexing enhancements for overlapping channels with different priorities

In the following, we will separately discuss the scenarios depending on whether PUSCH is involved or not, i.e. control vs. control and control vs. data.

## 3.1	Control channel vs. control channel

### 3.1.1 PUCCH with high-priority HARQ-ACK and HP SR vs. PUCCH with low-priority HARQ-ACK

The handling of the scenario where a PUCCH carrying HP HARQ-ACK and HP SR with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK was agreed in RAN1#107bis-e as follows (R1-2200791):

Agreement When a PUCCH carrying HP SR and HP HARQ-ACK with PUCCH format 2/3/4 overlaps with a PUCCH carrying LP HARQ-ACK, information bits for K HP SRs are appended to HP HARQ-ACK bits, and treat them as HP UCI, where K (K≥1) PUCCHs semi-statically configured for K HP SRs overlap with the original PUCCH carrying the HP HARQ-ACK. - The number of HP UCI bits is , same as Rel-15; - FFS: PF0, PF1 - Reuse other procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e. separate coding, PRB determination, rate matching and power control. - If the HP HARQ-ACK is a dynamic HARQ-ACK, a PUCCH resource indicated by PRI is used for multiplexing. - If the HP HARQ-ACK is a SPS HARQ-ACK, a PUCCH resource determined from the PUCCH resource(s) provided by sps-PUCCH-AN-List is used for multiplexing.

As can be seen from this agreement, one remaining FFS point is the handling for the case where the PUCCH carrying HP HARQ-ACK and HP SR is with format 0/1. This point was extensively discussed in RAN1#108-e but without reaching a consensus, where the following proposal has been on the table (R1-2202830):

When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. - Option 2: 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

We still support Option 2 since unnecessary dropping of LP HARQ-ACK can be avoided with this Option, which is the main objective of the Rel-17 intra-UE enhancements, whereas with Option 0 the LP HARQ-ACK is always dropped. In addition, Option 2 is aligned with the solution agreed for the case where the PUCCH carrying HP SR and HP HARQ-ACK is with PUCCH format 2/3/4, where the multiplexing of HP HARQ-ACK, HP SR, and LP HARQ-ACK is supported (see agreement copied above).

Another point raised by some companies is when the HP HARQ-ACK is SPS HARQ-ACK. Specifically, if the sps-PUCCH-AN-List is provided, then the multiplexing mentioned above can be supported; otherwise, if (only) n1PUCCH-AN is provided, then the LP HARQ-ACK should be dropped in this case.

Based on the above, we think that the above proposal should be agreed considering Option 2. We thus propose:

Proposal 3.1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc.

  - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

## 3.2	Control channel vs. data channel (i.e. HARQ-ACK vs. PUSCH with different priorities)

The discussion in this subsection is focused on the remaining issues for the scenarios where the overlapping PUSCH(s) and PUCCH(s) are of different priorities in step 2-2.

### 3.2.1 UE behaviour when multiplexing HARQ-ACK on CG PUSCH of different priority carrying CG-UCI

Multiplexing LP HARQ-ACK on HP PUSCH without HP HARQ-ACK was discussed in RAN1#108-e meeting and the following agreement is achieved.

Agreement (RAN1#108-e) If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

Based on the agreement, when LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, UE follows the same behaviour as that in the case of PUSCH with HP HARQ-ACK assuming two dummy bits. One open point for further discussion is how to handle the scenarios with CG PUSCH carrying CG-UCI involved, i.e., for the operation over unlicensed band. In case with HP CG PUSCH carrying CG-UCI, when both multiplexing over different priorities and cg-UCI-Multiplexing are enabled, HARQ-ACK would be multiplexed in CG PUSCH of different priorities following the following agreement from RAN1#106bis-e meeting:

- Agreement (RAN1#106bis-e) - When performing Intra-UE multiplexing procedure, if a PUCCH with HARQ-ACK overlaps with a CG-PUSCH and the cg-RetransmissionTimer is configured: - If the HARQ-ACK and the CG-PUSCH have the same priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, CG-PUSCH would be dropped. - If the HARQ-ACK and the CG-PUSCH have different priority and the CG-PUSCH is selected for HARQ-ACK multiplexing: - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is not indicated, - The LP channel between PUCCH or CG-PUSCH would be dropped as in Rel-16. - If multiplexing HARQ-ACK on the CG-PUSCH with different priroity is indicated, - If cg-UCI-Multiplexing is enabled for that CG-PUSCH, HARQ-ACK would be multiplexed in CG-PUSCH. - Otherwise, the LP channel would be dropped.

In our opinion, in this case CG-UCI should be treated with the same PHY priority as HP CG PUSCH. Then following the specified procedure [e.g., Clause 6.3.2 in 38.212, Clause 9.3 in 38.213] of multiplexing HARQ-ACK onto CG-UCI PUSCH of the same priority, the UE jointly encodes the two-bit HP HARQ-ACK information and the CG-UCI. The number of resources for multiplexing is determined by using the beta-offset for HP HARQ-ACK (as in case of legacy operation). In case of the HP CG PUSCH carrying HP CSI including two parts, the LP HARQ-ACK needs to be dropped due to the number of available encoding chains.

Therefore, we have the following proposal:

Proposal 3.2: For multiplexing a LP HARQ-ACK without HP HARQ-ACK into a HP CG PUSCH carrying CG-UCI, UE follows the same behaviour as in case of PUSCH with HP HARQ-ACK assuming two bits.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK, HP CSI including a single part, and CG-UCI would be transmitted on the HP CG PUSCH,

  - Reuse Rel-16 procedure for handing HP HARQ-ACK and CG-UCI: joint coding, rate matching and RE mapping for jointly coded HP HARQ-ACK and CG-UCI.

  - Reuse Rel-15 CSI part 1 rate matching and RE mapping for the single part of HP CSI.

  - Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP HARQ-ACK.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK and HP CSI including two parts would be transmitted on the HP CG PUSCH,

  - LP HARQ-ACK is dropped.

  - Reuse Rel-16 procedure for handling HP HARQ-ACK, CG-UCI and two parts HP CSI on HP CG PUSCH.

It should be pointed out that it is rather straightforward to extend the above proposal to the scenario for multiplexing LP HARQ-ACK, HP HARQ-ACK into a HP CG PUSCH carrying CG-UCI by replacing the assumed 2-bit HARQ-ACK with the present HP HARQ-ACK bits.

Up to now we have discussed the multiplexing scenarios with HP CG PUSCH carrying CG-UCI. The scenarios with LP CG PUSCH carrying CG-UCI have not been discussed in RAN1 so far. To be more specific, how to handle CG-UCI when HP HARQ-ACK, LP HARQ-ACK and CSI (single part or both parts) would be multiplexed on the LP CG PUSCH. Following the same procedure as for licensed band operation, in case the number of available encoding chains is not sufficient to multiplex all UCIs, CSI part 2 should be dropped. Considering ways to handle CG-UCI, different options can be discussed further:

- Option 1: CG-UCI is treated as low priority (i.e., the same PHY priority as LP CG PUSCH) and jointly coded with LP HARQ-ACK (if present). With this option, legacy HARQ-ACK procedure of channel coding, rate matching and RE mapping can be reused to handle HP HARQ-ACK. The legacy procedure for handling CSI part 1 (rate matching and RE mapping) can be used for the jointly coded LP HARQ-ACK and CG-UCI where the joint coding can follow the specified procedure.

- Option 2: CG-UCI is treated as high priority (i.e. different PHY priority comparing to LP CG PUSCH) and jointly coded with HP HARQ-ACK. With this option, the legacy procedure of handling jointly coded HARQ-ACK and CG-UCI of the same priority can be reused for jointly coded HP HARQ-ACK and CG-UCI. The legacy procedure (rate matching and RE mapping) for handling CSI part 1 can be used for the LP HARQ-ACK.

- Option 3: Multiplexing HP HARQ-ACK on LP CG-UCI PUSCH is not supported in Rel-17. In another word, LP CG PUSCH with CG-UCI is always dropped/cancelled when overlapping with a PUCCH carrying HP HARQ-ACK.

To avoid unnecessary dropping of LP CG-UCI PUSCH, Option 3 should not be supported. Between Option 1 and Option 2, both options are feasible. Considering RAN1 agreements of separate coding for HARQ-ACKs with different PHY priorities, Option 1 is slightly preferred. Based on this, we propose:

Proposal 3.3: For multiplexing a HP HARQ-ACK into a LP CG PUSCH carrying CG-UCI in Rel-17,

- Reuse Rel-15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK.

- Reuse Rel-15 CSI part 1 rate matching and RE mapping for CG-UCI and LP HARQ-ACK (if present, jointly coded with CG-UCI).

- Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP CSI part 1 (if present).

- Drop LP CSI part 2 (if present).

# 4	Conclusions

In this contribution, firstly we discussed the overall Rel-17 PHY multiplexing and prioritization framework in Sec. 2, we have the following proposal:

- Proposal 2.1: To resolve overlapping between a HP PUCCH carrying SR and HP HARQ-ACK with PUCCH format 0/1 and a LP PUCCH carrying HARQ-ACK and CSI/SR, only LP HARQ-ACK is multiplexed with HP UCI (i.e., SR and HARQ-ACK) and LP CSI/SR are dropped.

For intra-UE multiplexing of overlapping channels of different priority on PUCCH (i.e. PUCCH versus PUCCH) in Sec 3.1, we have the following proposal:

- Proposal 3.1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive,

- In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource.

  - Note: The description of Step 1 here is only for information purpose, which has no spec impact.

- In Step 2,

  - if the HP HARQ-ACK is using a PUCCH resource provided by PUCCH-ResourceSet or SPS-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1

    - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4.

    - Reuse the (agreed) procedures for multiplexing of LP HARQ-ACK and HP HARQ-ACK on PUCCH resource with PF 2/3/4, i.e., PUCCH resource selection, separate coding, PRB determination, rate matching, power control, etc.

  - if HP HARQ-ACK is using a PUCCH resource provided by n1PUCCH-AN, the LP HARQ-ACK is dropped.

For intra-UE multiplexing of overlapping channels of different priority on PUSCH (i.e. PUCCH versus PUSCH) in Sec 3.2, we have the following proposals:

- Proposal 3.2: For multiplexing a LP HARQ-ACK without HP HARQ-ACK into a HP CG PUSCH carrying CG-UCI, UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK, HP CSI including a single part, and CG-UCI would be transmitted on the HP CG PUSCH,

  - Reuse Rel-16 procedure for handing HP HARQ-ACK and CG-UCI: joint coding, rate matching and RE mapping for jointly coded HP HARQ-ACK and CG-UCI.

  - Reuse Rel-15 CSI part 1 rate matching and RE mapping for the single part of HP CSI.

  - Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP HARQ-ACK.

- If the assumed (2-bit) HP HARQ-ACK, LP HARQ-ACK and HP CSI including two parts would be transmitted on the HP CG PUSCH,

  - LP HARQ-ACK is dropped.

  - Reuse Rel-16 procedure for handling HP HARQ-ACK, CG-UCI and two parts HP CSI on HP CG PUSCH.

- Proposal 3.3: For multiplexing a HP HARQ-ACK into a LP CG PUSCH carrying CG-UCI in Rel-17,

- Reuse Rel-15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK.

- Reuse Rel-15 CSI part 1 rate matching and RE mapping for CG-UCI and LP HARQ-ACK (if present, jointly coded with CG-UCI).

- Reuse Rel-15 CSI part 2 rate matching and RE mapping for LP CSI part 1 (if present).

- Drop LP CSI part 2 (if present).