3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203280

Online, May 16th – 27th, 2022

Agenda Item:	9.2.1

Source:	Ericsson

Title:	General aspects of AI PHY

Document for:	Discussion, Decision

# 1	Introduction

Artificial intelligence (AI) and machine learning (ML) is used in deployed radio access networks today, and we expect that it will play a large role in the years ahead. This study item aims to investigate if, and how, 3GPP standardization can further enhance AI/ML solutions for the physical layer (PHY). The findings and framework developed in this study item will be used as inputs for possible subsequent NR releases, and, we believe, shape how 3GPP addresses AI/ML in standardization for many years – even beyond NR. Hence, it is important to have a perspective in the discussion that not only focus on current capabilities and limitations, and it is useful if the developed framework(s) consider future, more advanced, AI/ML operation in PHY.

In general, Ericsson’s high-level views on AI/ML standardization for PHY can be summarized as follows:

- 3GPP is the right place to drive AI/ML PHY standardization and testing.

- AI/ML frameworks should maintain 3GPP’s competitive and open multi-vendor eco-system.

- AI/ML models and training data remain as each vendor’s proprietary assets.

- Implementation based solutions of AI/ML for PHY are prioritized; however, standardization may be discussed if there is a significant additional advantage.

- Scalable and robust AI/ML model life cycle management (LCM) solutions are essential. AI is a powerful technology for adapting to dynamic environments, so frequent model updates are likely desirable in the future. It is important to solve how to detect/handle/disable faulty AI/ML models in a scalable manner so that overall system complexity does not explode.

This contribution summarizes Ericsson’s views on the general aspects of introducing AI into the physical layer. Our discussion will largely follow the scope outlined in the study item description [1]; in particular, we will discuss the following aspects:

- Definitions and terminology for AI on PHY

- Datasets and the use of synthetic data for training

- UE-gNB collaboration, and corresponding definition of collaboration levels

- Life Cycle Management (LCM)

- UE processing capabilities relevant for AI PHY

# 2	Discussion

## 2.1	Defining stages, used notation and terminology

### 2.1.1	Terminology and notation for AI/ML on PHY

On the AI/ML model definition

An AI/ML model is, in the most general sense, an algorithm that has been trained to perform a certain task such as prediction, classification, or compression. The field of artificial intelligence and machine learning is vast, and the space of possible trainable algorithms is broad. It is, therefore, difficult to capture every AI/ML model corner case with single definition; indeed, there is no clear boundary between classical adaptive or tuneable algorithms (already widely used in 5G networks) and AI/ML models.

We propose to narrow the definition of an AI/ML model to something more specific for this 3GPP study item. In particular, the study should restrict attention to AI/ML models with specification impacts for RAN1/2/4. Purely proprietary AI/ML models, with no NR specification impact, are not in the scope of this study item.

Definition 1: An AI/ML model for the NR Air Interface is an algorithm residing in the NW and/or UE that, in order to be designed (trained), tested, and deployed, has one or more of the following specification impacts:

- PHY layer aspects:

  - New physical-layer measurements (including associated configurations and signalling) to obtain physical-layer data, where it is understood that the physical-layer data can be used to design, validate, and test the algorithm prior to deployment.

  - New signalling to monitor the algorithm’s performance.

  - New signalling to execute the algorithm (inference).

  - New signalling to train/retrain the algorithm (training)

- Protocol aspects:

  - New signalling to identify and configure the UE to use the algorithm (inference).

  - New signalling to download the algorithm to the UE.

  - New signalling to identify UE capabilities e.g. to execute/train/retrain/compile the algorithm.

- Interoperability and testing aspects:

  - New requirement, data, and testing frameworks to verify the algorithm’s performance.

On generation of an AI/ML model

We propose that the term AI/ML model generation should refer to the general research and development process of designing, training, validating, and testing AI/ML models before they are deployed in the field.

AI/ML model generation is primarily a highly iterative and somewhat heuristic process. In many cases (e.g., deep learning and deep reinforcement learning) it requires significant research and development time, compute power, and data resources.

In the 5G-advanced to 6G timeframe, we believe that AI/ML model design for physical layer features will primarily be an offline, in-house / development domain, research and development activity for vendors. However, proprietary fine tuning of an AI/ML algorithm upon collection of new data is of course possible in a proprietary manner. As this impose a risk of stability of the model performance, life cycle management (LCM) approaches for monitoring deployed models in the UE should be studied.

One can imagine more futuristic cases in which automatic RAN procedures and services collect data and perform advanced automated AI/ML algorithm design, training, and deployment of AI/ML model. However, given the performance critical nature of NR physical layer functionality, the relative immaturity of fully automated AI/ML solutions (to, e.g., design AI/ML algorithm architectures), the current state of physical-layer data collection, and the current state of interoperability device testing, we propose that this study item should focus on offline (development domain) AI/ML model generation. That is, the study item should initially focus on use cases where the AI/ML algorithms are trained outside the NW and UE.

In the study, prioritize AI/ML model generation under the assumption that AI/ML algorithms are trained offline.

The multi-vendor 3GPP ecosystem brings unique challenges to the standardization of physical layer AI/ML features. For example, it is a challenge to support AI/ML model interoperability in multivendor environments, while simultaneously supporting strong vendor differentiability (e.g., proprietary AI/ML models, training solutions, and data). This is further discussed in Section 2.4 below on collaboration.

### 2.1.2	Defining stages for AI/ML on PHY

The key defining stages of an AI/ML model are outlined below, divided into the stages of training and inference.

#### 2.1.2.1	Training stages

The flow of training of an AI/ML algorithm is shown in Figure 1.

Figure 1 The training stages for obtaining an AI/ML model for PHY

The steps are described in the following together with our initial view on the potential standard impact:

Data ingestion

In this stage, input data relevant to the use case is gathered, by gathering measurements collected in the network or from dedicated test or sounding devices. Once obtained, separation into training and validation datasets (and possibly test datasets) could be carried out at this stage.

- Potential standard impact - The data collection procedures needed for training may be specified. For example, defining new UE measurements, new events and reports or new reference signals and UE procedures for gNB measurements.

Data verification

Verify the validity of the data. For example, if the gathered data is sufficient for training and verifying the ML model.

- Potential standard impact – Currently no impact foreseen.

Data pre-processing

This step comprises, for example, imputing missing input data, filtering unnecessary data, and/or transforming the data. These steps are typically referred to as feature engineering, where the idea is to extract important information out of the raw data using domain knowledge. One example comprises, for the CSI compression use case, excluding the channel eigenvectors associated with the smallest eigenvalues since they are not likely to be useful.

- Potential standard impact - To save signalling overhead, some of the pre-processing can be located in the UE, hence necessary signalling to support such pre-processing might be needed. If the training data is pre-processed, parts of this pre-processing may be needed during inference as well, in which case it could require standardization.

Algorithm training

An ML-model usually represents a functional mapping f between a set of features x, which provide the input argument to the function represented by the model, and an output y representing the transformation of the input feature. Both the input feature x and the output y may be multidimensional tensors.

Training to obtain the model comprises finding the optimal mapping of x to y. Where optimal is defined according to a certain loss function, for example in linear regression problem, a typical loss function minimizes the mean squared error of the model output and the true value. Moreover, it is possible to include the importance of a certain sample in the loss function by including a sample weight term ws. Accordingly, an example of a loss function can be defined as minimizing the weighted mean squared error (MSE):

1Nswsfxs-ytrue_s2                                  (1)

Finding an appropriate model f depends on several factors such as amount of available data, computational power, and performance requirements associated to the model. There is a vast number of algorithms available, and the exact algorithm should be selected on a use case basis.

The term online/offline training refers to the process of how the model has been obtained. In online training, the model is updated (e.g. weights in neural network, or fingerprint database) in a sequential order by the arrival of new data. In offline training however, the model is obtained once after the whole dataset have been collected. This typically leads to more stable training in comparison to online training. Note that there is a grey zone between online/offline training, since in some cases online training also comprises collecting a batch of data before updating the model. Due to this ambiguity, SI discussions should include more specific descriptions than only referring to online/offline training.

- Potential standard impact – training of a dual sided joint ML-algorithms.

Model validation

Model validation comprises evaluating the trained model f by feeding unseen test data into the model. By unseen data we mean the data which is not part of model training, or does not have correlation properties with the training data that is different from data seen after deployment. The model can for example be validated by verifying that the computed loss metric according to equation 1 for the test data is below a certain threshold value. The model validation may also include evaluations related to business KPIs such as link/system throughputs.

- Potential standard impact – defining RAN4 test cases for the trained model.

Model Registration

The stage of assigning the model with a version identifier and make the model runnable via compilation to the specific hardware used in the inference stage.

- Potential standard impact – How to assign unique model version IDs for ML-models.

#### 2.1.2.2	Inference stages

The flow of inference of an AI/ML model is shown in Figure 2.

Figure 2: The inference stages of an AI/ML model for PHY

The steps are described in the following together on our initial view on the potential standard impact:

Data ingestion

The data needed for the ML-model for inference. In comparison to training, the data for inference is more time-restricted by the delay-constraints of the function using the model output. The data is typically a continuous flow of data, and not a large static dataset used during training.

- Potential standard impact – The data used for inference may be specified in case of UE sided AI/ML model. For example, defining new UE measurements, events, and reports. Also defining new reference signals may be needed. For gNB side AI/ML model, spec impact could be defining new reference signals and associated procedures.

Data pre-processing

If the training data is pre-processed, parts of this pre-processing may be needed during inference as well. For example, the process of normalizing, or excluding certain parts of the inference data. This processing stage is basically expected to be the same as in the corresponding stage for the training, illustrated in Figure 1.

- Potential standard impact – how the pre-processing has been performed may need to be informed to the other node, i.e. using control signalling over the Uu interface, for correct interpretation of the model output.

Model inference

Using the trained machine learning model to generate predictions with new, previously unseen, data.

- Potential standard impact –The accuracy, confidence interval and/or uncertainty to be included in addition to the model output need to be standardized. If the AI/ML model is dual sided joint ML, the output of the model of one side may need to have a standardized representation, e.g. quantization, payload and order of output bits etc.

Drift monitoring

The monitoring of drift in data or performance metrics after the model is deployed. The data drift, using information from the data ingestion stage as outlined in Figure 2, can for example be detected by comparing the data statistics during inference with the data collected during training. A large difference can indicate that a model that is not operating under the same assumptions during the training, and potentially needs to be retrained. Similarly, if performance metrics related to the model inference go below a given threshold it might trigger model retraining (including or not generation of a new training set). See Section 2.5 below on LCM for further description.

- Potential standard impact – See LCM Section 2.5

As mentioned earlier, it is useful to align and define terminology and notation. It is also mandatory to have such a section in the technical report [5]. In the Appendix, a dictionary is additionally given with corresponding definitions that are useful for discussing standardization of AI/ML for PHY in 3GPP.

## 2.2	Source of the datasets

Datasets are needed in different AI/ML stages like model training, validation, testing and inference. For AI on PHY studies in RAN1, datasets are needed for the following two purposes: 1) develop AI/ML algorithms for the considered use cases, and, using these algorithms to 2) evaluate the performance (including generalization) of the AI/ML-based approaches.

These evaluations aim to investigate whether there are benefits/gains of the ML-based solutions compared to legacy baseline schemes. These findings are then used, together with other considerations such as complexity and feasibility, to conclude on whether to proceed with normative standardization work in 3GPP.

In general, there are two types of datasets that can be considered for this AI on PHY SI work -- synthetic data and field data. Synthetic data can be generated primarily by using the 3GPP statistical channel models defined in [6]. In this case, a dataset can be defined via a set of parameters that represents the considered scenario(s).  There may be special needs on datasets for some specific sub use cases, e.g., beam tracking and fingerprinting-based positioning, where the datasets should include spatial consistency across UE positions. For the positioning use case, the specific scenarios with heavy NLOS defined in [7] could be used to generate data sets.

Although “real world data” (i.e., field data) is likely needed to train an AI/ML model to achieve the best possible performance, we believe that for 3GPP RAN1 standardization work on AI on PHY (i.e., defining new signalling and reporting/configuration content to support AI/ML-based solutions for the considered use cases) synthetic data generated based on 3GPP channel models is sufficient.

Although deployed AI/ML may likely benefit for being trained on field data, synthetically generated datasets (i.e., 3GPP channel models) are sufficient for the tasks that are within RAN1 standardization scope.

Moreover, we see many problems with ensuring data quality, fairness, diversity, and many challenges in obtaining representative field data for use in 3GPP evaluations and discussions. Field data can be very skewed to a specific scenario or system configuration. Field data can be biased due to the vendor specific software / hardware / implementation solutions. In addition, it may require large effort for 3GPP to ensure the usefulness of field data for standardization. For example, 3GPP would need to define data acquisition processes and the environmental settings for field data collection. Data formatting, cleaning, and pre-processing to construct a dataset based on the field data would also need to be defined. Hence, the need and practicality of using field data needs to be justified.

If field data is to be used in this SI, RAN1 need to discuss how to ensure data quality, fairness, and diversity, and how to avoid skewness to a specific scenario to obtain representative field data. How to share data among parties needs to be resolved. The amount of RAN1 work to achieve this seems excessive and the necessity shall be very strong to start such work.

Even when using the synthetic dataset for this SI evaluations and discussions, the dataset for training, validating, and testing should be carefully selected to evaluate the model generalization, e.g., by defining different sets of simulation parameters for different scenarios.

The details on dataset construction aspects should be discussed and decided per use case.

In this study item, synthetic datasets (based on TR.38.901 and TR38.857) are used including the possible option of spatial consistency for at least the beam management and positioning use cases.

## 2.3 On the need for an AI model for calibration

The SID [1] mentions the following in the objectives:

- Consider agreed-upon base AI model(s) for calibration

Inter-company calibration campaigns during LTE and NR standardization have been rather time consuming and tedious. A calibration was recently performed for the IMT-2020 submission [4]. A benefit of these previous efforts is that companies’ system level simulators (SLS) and link level simulators (LLS) are now rather well calibrated against each other: Bugs have been discovered, and there is a mutual understanding of modeling details. Simulation results provided for subsequent work and study items can now be considered as reasonably comparable.

The open issue is then whether there is a necessity to commence the rather large task to agree on AI models for calibration -- commencing yet another calibration campaign in RAN1. It is noted that AI-models can be calibrated against classical models (such as Type-II CSI reporting) already implemented in simulators and for which RAN1 has already performed the calibration campaign.

Note that the intent of the SI is not to standardize AI models (i.e., algorithms), which implies that there is no need to do algorithm or modeling details type calibrations – like traditional 3GPP calibration work. For AI based PHY, the improvements in classical KPIs such as overhead reduction, throughput gains, and compute requirements in application, can be compared against existing implementations of classical methods and “genie solutions” using genie information. In our initial assessment, these KPIs, together with KPIs/metrics to understand why/how the model improve performance are sufficient to fulfill the scope of the SI.

It is unclear what would be the value to spend the effort and time in RAN1 to agree on AI models for calibration

Based on this observation, we make the following proposal for agreement

RAN1 does not pursue any work in this SI aiming at agreeing AI-baseline models for calibration.

However, it should be noted that this doesn’t preclude the use of proprietary implemented AI models as baselines for comparison with standardized support for AI on PHY which is the target for 3GPP work. It is thus still interesting to understand how standard-transparent AI-models perform, as it is indicative of how large gains are expected by agreeing to changes in the standard.

The use of a standard-transparent AI models to enhance performance can be done on a per-company basis and can be used as a reference for comparison. If such results are included, then they should be shared as any other results with respect to e.g. AI model description and KPIs.

## 2.4	Definition of UE-gNB collaboration categories

Before considering various definitions of UE-gNB collaboration categories, it is useful reflect on the purpose of the collaboration. There are two main stages of AI/ML: training and inference, as described in Section 2.1.2. Inference refers to a ML model being used in a deployment (for example a UE). Considering the previous AI related work captured in [2] (TR 37.817), it outlines the operations for inference being performed in NG-RAN nodes, such as gNBs and associated behaviour.

For the study item at hand, we can see that the framework in [2] can be used as a starting point and be expanded to consider use cases addressing the Uu interface where ML functionality can also be in the UE.

For ML on Uu, UE-gNB/NW collaborations can be divided in three main categories: 1) single-sided ML functionality at the gNB/NW only, 2) single-sided ML functionality at the UE only and 3) dual-sided joint ML functionality at both the UE and gNB/NW that requires some joint operation.

Study the following three collaboration cases:

- Single-sided ML functionality at the gNB/NW only,

- Single-sided ML functionality at the UE only,

- Dual-sided joint ML functionality at both the UE and gNB/NW (joint operation).

Figure 3 illustrates the three proposed UE-gNB/NW collaboration categories. In sub figure c), the “ML report” means that the content of the report can only be interpreted by a ML-model at the gNB. The proposed collaboration levels are discussed in detail below with respect to training and inference.

It is noted that “no collaboration” case (i.e., implementation-based only AI/ML algorithms) is also possible for singled sided ML and can be used for comparison purpose. This is not included in the list of collaboration cases above since no change to the air interface is required.

Figure 3: Example of single sided and dual-sided joint ML functionality for the gNB and UE side. Note that action in b) can alternatively be that the UE triggers a report or feedback to the gNB.

The above collaboration category proposals with the single sided ML-functionality should not be seen as precluding that a ML based function e.g., at the UE side feed information into another single sided function at the gNB side that is based on ML. In this case, each of the ML functionalities in the UE and gNB are seen as separated entities that can be operated independently of each other, i.e., there would be no need for performing joint operations.

An example of such operation could be within the positioning use case wherein the UE uses ML to generate a measurement report, e.g., an enhancement to measure RSTD. The ML based information conveyed in the UE report may then be used by the network as input to a ML model at the network side to derive the position of the UE based on for example multiple RSTD reports and potentially other reports and information.

It is important to note that this case still describes the single-sided ML-functionality collaboration category since neither the UE nor the network is compelled to use ML-functionality. Hence, in the given example it is possible that the network side does not actually need to use ML-functionality but could have implemented a classical (non-ML) approach. The UE may not know whether the network uses an ML-model or not as that is transparent to the UE side. Hence, there is no need for a separate collaboration category to cover this scenario. The single sided ML-functionality is characterised by the possibility to run with and without ML-functionality in other entity, and furthermore ML algorithms at the UE and gNB can be trained independently.

At this point it is useful to differentiate and define the single-sided and dual-sided joint AI/ML models:

A single-sided AI/ML use case is one where the AI/ML algorithm training process does not include details of the air interface. For example, consider an AI/ML model for enhancing beam management operating in the UE. If the output of this AI/ML model is signalled over the Air Interface as part of a CSI report, then the gNB can interpret the meaning of the CSI report from a specification. The gNB does not need an AI/ML algorithm to be jointly trained with the UE’s AI/ML algorithm to decode the CSI report.

A dual-sided joint AI/ML use case is one where the AI/ML algorithm training needs to train algorithms for joint use in the UE and gNB, taking into account the air interface. For example, consider the CSI use case where AI/ML-based encoder in the UE compresses downlink CSI-RS based channel (feature) estimates and an AI/ML-based decoder in the gNB decompresses those estimates. The CSI report signalled over the uplink is only decodable by an appropriately trained AI/ML model in the gNB. The meaning of these bits is not explicitly written into a specification and may thus change when a model is retrained.

In dual-sided joint AI/ML use cases, we believe that it is essential that the training processes yield AI/ML models in UEs and gNBs that have good performance in 3GPP’s multi-vendor ecosystem. For example, to limit system complexity and cost of the taking the CSI compression use case, it is preferrable that

- A single gNB CSI decoder can achieve good performance with many different UE-side CSI encoders from different vendors, and

- A single UE CSI encoder can achieve good performance with many different gNB-side CSI decoders from different vendors.

Such AI/ML model generation challenges of multiple vendors operating in a joint manner, are unique to 3GPP, and there is very little experience in the ML industry of such collaboration. 3GPP discussions and specifications can play a role to help ensure the resulting solutions are suitable and scalable for the 3GPP multi-vendor ecosystem.

Study a multi-vendor framework including procedures and signalling for enabling dual-sided joint ML that ensures a single ML-model on the UE side that is independent from the gNB ML-model and a single ML model on the gNB side that is independent from the UE ML-model.

### 2.4.1 Algorithm training collaboration

Training collaboration aspects were considered in [2], including, for example, where training will be located (online/offline) and at which RAN node it will be performed. There is obviously a need to extend this framework to account for the fact that the AI models can be in the UE. That extension is also considered under the life cycle management, see section 2.5.

From a collaboration framework point of view, the starting point is no specification impact from training aspects for any of the single sided ML use cases since it is assumed that the gNB or UE/chipset vendor itself handles the training of its own ML algorithm.

This is further supported by the SID, as it states that this SI will not define the ML-models, i.e. they are proprietary, hence a training function would also need to be vendor specific, i.e. either gNB or UE vendor specific, therefore, some third part cannot train or retrain the model. Likely the ML models are executed on dedicated hardware (at least in case of large neural networks), which detail is not accessible to external parties.

However even if the ML models are proprietary, there may still be a need to identify a certain trained model in RAN, e.g. what identity and specific version of a model a UE has deployed. This is needed for the network to monitor a certain model’s performance of ML-models located within the UE.

At least for single sided ML models, the model training is assumed to be proprietary, hence no specification impact is foreseen for ML model training

For dual-sided joint ML functionality, where an ML model is used at both the UE and gNB and using joint operation, there is an additional challenge on how to jointly train the ML on both side of the Uu since this is a multi-vendor situation. This scenario is expected to occur in at least the AI-CSI use case for CSI compression. Several options on how to achieve joint training in this case is outlined below.

- Case A: [gNB vendor training, UE/chipset adoption] gNB vendors train both gNB and UE ML models, and UL ML model is adopted by UE for deployment and operation

  - Pros:

    - Only a single gNB ML model is implemented which handles any UE since all UEs use the same UE ML model (when connected to the gNB vendor)

    - Loss function can be optimized for e.g. MU-MIMO, C-JT or any other proprietary solution on network side

  - Cons:

    - UE needs to receive, store, and use a specific ML model for each of multiple gNB vendors.

    - ML model is not optimized for UE hardware, leading to decreased efficiency, power consumption etc as the UE hardware may need to be generic

- Case B: [UE/chipset vendor training, gNB adoption] UE/chipset vendors train both gNB and UE ML models, and gNB ML model is adopted by gNB for deployment and operation

  - Pros:

    - Only a single UE ML model is implemented which handles any gNB vendor.

    - ML model in UE can be optimized for its hardware.

  - Cons:

    - gNB needs to receive, store, and use a specific ML model for each of multiple UE vendor/chipset models,

    - In a slot, multiple such ML models may need to be executed in parallel in the gNB to receive AI CSI from multiple UEs.

    - ML model is not optimized for gNB hardware, leading to decreased efficiency, power consumption etc as the gNB hardware may need to be generic.

    - The used loss function for training doesn’t match the network implementation (MU-MIMO, C-JT, etc), e.g., training for SU-MIMO only.

- Case C: [Joint and bilateral UE/chipset and gNB vendor training and joint operation] A bilateral training event (likely offline, pre-deployment) between a UE/chipset and a gNB vendor to jointly train both sides ML models

  - Pros:

    - ML model in both UE and gNB can be optimized for it’s hardware leading to high efficiency and best possible performance

    - gNB side can select a loss function to optimized for e.g., MU-MIMO, C-JT or any other proprietary solution on network side

    - ML models can be kept proprietary even if jointly trained as only the backpropagation parameters needs to be shared with other vendor (see more discussion below)

  - Cons:

    - gNB/UE needs to store and configure a specific ML model respectively for each of multiple UE/gNB vendor/chipset models. Hence, there is an issue with scalability to handle many ML model pairs across both network and UE side vendors (here denoted the combinatorial problem)

    - In a given slot, multiple gNB ML models may need to be executed in the gNB to receive e.g. CSI from multiple UEs in parallel.

- Case D: [Separate UE/chipset and gNB vendor training and joint operation] UE/chipset & NW vendors train ML models separately using proprietary techniques and data while joint operation is still used. This may require the latent space bits sent from the first ML model can be interpreted by humans, or by the second receiving ML model, e.g. by preserving distance between features after compression in the first ML model. See Section 2.2.6 in [3].

  - Pros:

    - ML model in both UE and gNB can be optimized for it’s hardware leading to high efficiency and best possible performance for the given model

    - ML models are kept proprietary

    - Number of ML-models to support on the UE and gNB can be minimized

  - Cons:

    - Techniques to achieve this are unknown for this new area compared to “traditional” Auto Encoder/Auto Decoder frameworks for split ML models

    - Performance of the case is yet to be evaluated, but worth considering due to the attractive “Pros”.

It is important that the SI explores further the pros and cons of the dual-sided joint AI cases as listed above and possibly extend with additional cases or sub-cases.

As an example of the direction of such study, it may be possible to use transfer learning and only train a few layers of an ML model on either side to reduce the combinatorial issue. That could be a sub-case of e.g. Case C.

Study options for training collaboration of the dual-side joint AI, using Case A, B, C and D outlined above as a starting point, including at least feasibility of model adoption from external vendor and joint training frameworks in multi-vendor setups

A critical issue for case C, which needs to be specifically addressed is the combinatorial AI model compatibility problem; that is, the explosion in complexity in UE and gNB side due to a need to manage and use multiple AI models in parallel to support a range of different vendors at the collaboration node.

Study in particular the combinatorial problem for dual sided joint AI (Case C), to limit or remove the issue with having to implement/train one ML model for each collaborating vendor/device model/ML model version

Note that for Case C, there is a possibility to jointly train the ML models, without disclosing the actual model parameters. The figure below thus illustrates how joint training in a multi-vendor setup can be performed in Case C.

Figure 4: ML algorithm training illustration for dual-sided joint ML using two different vendors A and B (Case C). Note that the derivatives from the backpropagation in ML algorithm B propagate to vendor A so that it can continue the backpropagation of ML algorithm A.

Note that in Figure 4, it is expected that the UE side (Vendor A) share the dataset used for training with gNB side (Vendor B) so that Vendor B can compute the loss function and gradients through backpropagation.

### 2.4.2	Inference collaboration

For inference collaboration, we discuss single sided ML functionality and collaboration first, followed by the double-sided case.

We begin by expanding the concept around inference from [2] (TR 37.817) and consider different levels of collaborations. We observe that what is captured in TR 37.817 would be examples of single-sided ML models on the gNB side. This is of course possible to expand directly to single sided UE models that uses some input from the gNB and thereafter perform some action. An example of such a case would be if a UE receives PRS and the single sided ML-functionality at the UE takes the PRS as input and performs estimate/measurement on the PRS. The action the UE may further perform is to report that estimate to the gNB.

The single-sided ML-model located at the gNB is as previously described covered by [2], although the specific uses cases have not been covered. For this case, it is possible to use the TR 37.817 as a starting point when assessing specification impact and the corresponding needed functionality.

Note that the [2] describes that the high-level responsibility to ensure that the ML functionality operates properly resides at the network side (including the gNB).

The single sided ML-functionality at the UE can either be for a new functionality that is intended to targeting ML implementation (where no such “classical” case/feature exists currently within the NR specifications) or the ML functionality is implemented within a UE to enhance a “classical” functionality that was not originally intended to be supported with ML. For example, a UE’s algorithm to compute a recommended PMI for CSI feedback is a classical functionality already existing that may be enhanced using ML in the UE.

For both these two cases of new and classical, the gNB needs to be aware of that the UE performs ML functionality to execute the corresponding functionalities / operations / life cycle management adequately. This to ensure the network performance.

The gNB needs to be aware that the UE performs ML functionality to execute the corresponding functionalities/operations such as performance monitoring, to ensure the network performance.

A second collaboration level with respect to inference is dual sided joint ML functionality between the UE and gNB which is defined by that the ML functionality operates jointly at both the UE and the gNB. One such example is the UCI compressing functionality at the UE side and decompressing part at the gNB, where training collaboration is according to Case C in Section 2.4.1 Algorithm training collaboration. The data that is compressed could for example be some form of channel data that is used for MU-MIMO scheduling or for positioning.

By its very nature, the dual sided joint ML functionality for inference requires specification support and hence it is not possible to introduce support for a feature already been specified without specification change.

For the dual-sided joint ML-functionality the final user of the ML-functionality resides in the network and hence the network needs to be able to monitor the performance of both the UE and network. The network needs to be able to detect of the faulty ML model is in the network side or the UE side in case of the networks serves different UEs with different ML-model for the same functionality.

Study mechanism and signalling to enable to network to ensure performance of the ML-functionality in the UE, in case of single-sided ML-functionality in the UE and the UE part of the dual-sided joint ML functionality.

## 2.5	ML model lifecycle management

ML model lifecycle management (LCM) includes the stages for data preparation, model generation and deployment, as described earlier. In essence, LCM includes the stages from scoping the use case of a model to further replacing a deployed model with an updated model that performs better. Hence, there will evidently be a need to have an infrastructure to manage the lifecycle of ML models. Such infrastructure will however be vendor specific but UE-gNB collaborations within the ML model LCM can be needed to be standardized to ensure good network performance as well as enabling new or enhanced features that also require standardization efforts.

The iterations between ML algorithm training and model inference for improving the performance of a function characterizes the ML process, in which monitoring operational drifts in data and in model is an essential and integral part when a model has been deployed. Drifts in data distributions or/and in features or patterns captured by the model may degrade the end-performance of using the output of a model that tends to not generalize well to previously unseen data.

A detected drift in either data or model may trigger a decision to perform training by re-running the AI/ML algorithm on a new, or additional, set of data or by rolling back to a previous version of the model or by replacing the deployed model with a model that has been generated by using another type of AI/ML algorithm assuming that the hardware supports such model update. Further, a decision to discontinue the use of a deployed model may in NR refer to a decision to fallback to some legacy procedure when ML is used to enhance an existing feature, e.g., fall back to classical operation as defined in a previous release.

The RAN3 functional framework for RAN intelligence [2], depicted in Figure 5, represents a generic description of the iterative process between algorithm training and model inference that characterizes ML model LCM stages. This framework is used for model LCM for single-sided ML at the gNB only, where LCM can be transparent to a UE. When inference is done at the UE side, model LCM may not be transparent to the gNB side, implying that the RAN3 functional framework is in such case not directly applicable. This is discussed in the following section.

The RAN3 functional framework for RAN intelligence [2] can be used when model LCM is transparent across UEs and network nodes.

Figure 5: Functional Framework for RAN Intelligence [2].

### 2.5.1	ML model LCM for single-sided ML

Since the network has the overall responsibility for the RAN performance, LCM of a model used by a gNB can be transparent to a UE in the sense that drift monitoring, training and model updating are not visible at the UE other than the UE may have been configured to assist with e.g. measurement reports, or reference signal transmissions, or UE assistance data, that only have a meaning in the context of ML. This LCM scenario is illustrated in Figure 6 a) where a gNB uses assistance data for making inference which in turn leads to a certain action from the gNB side such as a scheduling action. As part of the model LCM, the gNB may feed data to a network node that collects data for the purposes of monitoring data/model drifts and performing offline training, or it may collect data for training and drift monitoring at the gNB. It can be noticed that for the RAN3 use case solutions captured in [2], training was considered to occur at either gNB or OAM. Note that drift monitoring (model performance feedback) may occur outside the gNB although inference at gNB.

For use case solutions with single-sided ML models at the gNB, studies are primarily within UE assistance in data collection for transparent LCM to the UE.

Figure 6 b) illustrates model LCM at the UE side where a gNB use a model-based report to perform an action, wherein ML at the UE side would only be visible if the UE has been configured to receive some NW assistance data, or transmit reference signals, or report predictions, or send messages, that only have purposes in the context of ML. Whether drift monitoring and training are within or outside the UE is in this case transparent to the network, but if model LCM is done e.g., in the cloud, a UE in the field would need to feedback data over-the-top for drift monitoring and offline training, possibly with permissions from the UE owner. Note that Figure 6b) assumes that a model at the UE side can be updated after deployment which may or may not be the case, see Section 2.6 for discussions on this model LCM related aspect. In this section, we assume that model updating is possible of an AI-PHY capable UE in the field.

Although LCM of single-sided ML models at the UE side could in principle be transparent to the gNB (network side), there could be a need to involve the gNB to prevent any performance degradation in the network when models and data drift from the training conditions, and to seamlessly update a model at the UE side without causing network disturbances. In the end, information used to base decisions at the network side, such as for making scheduling and mobility decisions, need to be reliable and therefore signaling to achieve robustness with respect to model operations at the UE side should be addressed.

In a use case where a model at the UE is to be deployed or updated, some testing may be needed for model validation that involves the network before the model can be deployed or activated. Certain performance tests of functionality are expected to be done before shipping out UEs and with data driven ML design it is foreseen that implemented function/model can be updated after production. Furthermore, a drift in data or in model that has been detected by the UE (or possibly by the gNB) may require some intermediate re-configuration actions until an updated model has been tested and fulfills some minimum performance requirements.

LCM stages of a single-sided ML model at a UE such as drift monitoring and model updating may not be fully transparent to a gNB or another network node.

Based on this we make the following proposal.

Study per use case solution how network LCM assistance can be introduced to assess network performance impacts due to drifts in ML model operations at the UE side.

Figure 6: Illustration of single-sided ML model LCM at gNB side a) and UE side b)

Figure 7 illustrates the LCM scenario where the network assists within e.g., validation and testing of deployed model to assess impacts of (either by network node/gNB detected or by UE indicated) operational drifts when the model-based information conveyed in a UE feedback or report is used by the gNB to perform some action. In this case, re-training and model updating are still within the model LCM at the UE side, whereas the assistance from the network side here relates to performance feedback and testing. The type of collaboration needed for such LCM assistance should be addressed per use case solution, specifically when the UE cannot assess/judge the network performance impact of drifts in data and/or in model.

Figure 7: Single-sided ML at the UE with assisted LCM at the gNB side for testing and performance feedback.

### 2.5.2	ML model LCM for dual-sided joint ML

LCM of dual-sided joint ML models will to large extent depend on how training/re-training will be done and what model dependencies that follow from the training such as that a model update at the UE side may or may not require a corresponding model update at the gNB side (and vice versa). LCM of dual-sided joint ML models in a multi-vendor scenario can be challenging and may require offline training interfaces across vendors development domains for training/re-training of deep learning algorithms.

For joint training in general, an ideal design procedure would be a joint ML design with training done on same datasets, basically as it would have been done in a single-vendor ML setup. See the discussion in Section 2.3.1 and Case C. LCM of dual-sided joint ML models could then be seen as a single-sided model LCM where UE-gNB collaborations are around data collection for joint training and drift monitoring. Furthermore, model deployments/updates can be done at the UE and gNB without any specific coordination needs on aligning models to work properly together.

However, single-sided model LCM in dual-sided joint ML may have performance benefits and LCM simplifications but requires standardization of ML deployment procedures for deploying or updating models. For ML models targeting baseband, such specification may lead to reduced implementation efficiency or may not even be feasible when tailored implementations of functionality are needed to meet real-time requirements in a cost and energy efficient manner, as pointed out previously.

For use cases solutions with dual-sided joint ML, focus is in this study on solutions that avoid standardization of the model deployment stage for updating a model.

## 2.6	Discussion on UE processing capabilities

Depending on the use case, ML model-based operation may impose different degrees of computational and procedural requirements on UEs. In order to ensure robust and efficient operation both from the UE and NW perspectives and an evolution path for progressive introduction of AI/ML in PHY, devices with different levels of ML functionality, performance, and support for different model update mechanisms should be supported. Capability reporting procedures for traditionally implemented functionalities, e.g. via IMEI software version (SV) and UE capability transfer procedures, are available today at connection establishment or upon network’s request. Such reporting works well for traditional, fixed capabilities.

However, ML model-based operation provides an opportunity for much more flexible adaptation of UE and NW functionalities, e.g. by enabling multiple different ML model versions for a given functionality during the lifetime of a device and possibly even dynamic updates of models during operation.

The SI should therefore study the motivation and opportunities for improved UE capability reporting that accounts for new aspects associated with ML models, like possible updates over time, the possible presence of multiple model versions, and model specificity and the need to rapidly adapt to certain scenarios or environments.

Study the need from the lower-layer perspective for improved UE capability reporting for conveying ML model-related information in PHY use cases, including enabling ML model version info provision and handling.

A UE may incorporate ML model-based support for multiple use cases. Since ML inference may be computationally expensive, this implies sufficient resource dimensioning for the use cases in question. In some cases, dynamic resource sharing is conceivable where a certain use case may not be able to operate ML-based if another use case is active and utilizes a certain resource.

It should be ensured that introducing new ML-based functions will not compromise executing other, conventional NR functionalities due to processing resource conflicts. If necessary, capability reporting solutions may be studied where UEs indicate ML model support depending on other functionality, conventional or ML-based, executed in parallel.

Enabling of ML capabilities/operations should not impede operations and behaviours per existing NR specs.

One aspect that strongly affects how completely the potential of ML-based PHY algorithms can be realized is the ability to adapt models to real-world environments and circumstances. It is also desirable to capture enhancements to ML-based algorithms during the lifespan of the device, so as to capitalize on performance improvements in the fast-developing ML field. While it might be possible to specify a fixed ML model to be used for the lifetime of the device, this would not nearly provide the advantages promised by applying AI/ML principles to PHY. In order to capitalize on these opportunities, the ML models should be updateable.

The ability to update models in the UE and the NW is essential for capitalizing on AI/ML potential for PHY.

For the UE, there may exist multiple mechanisms for updating its ML models. It is expected that the over-the-air firmware upgrade (FOTA) procedure can be adopted to allow ML model structure and/or ML parameter updates in the UE. It may also be possible to execute more dynamic updates where model updates are downloaded directly via Uu or via over the top (OTT) signaling. The different model update procedures also pose requirements on the UE’s HW and SW architectures, e.g. the need and ability to recompile or otherwise integrate the received model updates into the PHY platform, the need to restart the device vs. possibility to continue an ongoing session, remaining in RRC_CONNECTED or transitioning to RRC_INACTIVE / RRC_IDLE, etc. Aligned understanding of UE ML model update options is need for the NW to perform/assist LCM procedures, where at least dynamic updates may be an advanced capability. The SI should explore different model update options and clarify their implications on UE implementation.

In the study, deployed ML-capable UEs that support model update mechanism should be considered.

Robustness of ML model-based operation in a variety of environments may be achieved e.g. by employing a larger, generalized model or by adaptively switching between multiple smaller models already present in the UE. These choices imply certain minimum requirements on UE architecture, e.g. model storage capacity and latency of loading large parameter sets. Understanding the model size and dynamic switching trade-offs in different use cases is important since it largely determines how dynamically a UE can adapt to different environments – cell-specifically, NW-vendor-specifically, by environment type, etc.

Study adaptability measures to ensure UE ML model robustness across deployment scenarios in the three selected use cases.

From the perspective of guaranteeing UE and NW performance, ML model-based operation should in principle not be different from conventional functionality. To ensure predictable NW operation, ML model-based algorithms should similarly be subject to minimum performance requirements. Since the models may change during the device lifetime, the conformance testing may need to be performed after each model update, or if not possible, performance monitoring in live NW operation should be ensured.

Regardless of the update method of an ML model, performance assurance via conformance testing or live performance monitoring is necessary to ensure robust NW operation.

# 3 Conclusion

Proposal 1	In the study, prioritize AI/ML model generation under the assumption that AI/ML algorithms are trained offline.

Proposal 2	In this study item, synthetic datasets (based on TR.38.901 and TR38.857) are used including the possible option of spatial consistency for at least the beam management and positioning use cases.

Proposal 3	RAN1 does not pursue any work in this SI aiming at agreeing AI-baseline models for calibration.

Proposal 4	The use of a standard-transparent AI models to enhance performance can be done on a per-company basis and can be used as a reference for comparison. If such results are included, then they should be shared as any other results with respect to e.g. AI model description and KPIs.

Proposal 5	Study the following three collaboration cases:

	Single-sided ML functionality at the gNB/NW only,

	Single-sided ML functionality at the UE only,

	Dual-sided joint ML functionality at both the UE and gNB/NW (joint operation).

Proposal 6	Study a multi-vendor framework including procedures and signalling for enabling dual-sided joint ML that ensures a single ML-model on the UE side that is independent from the gNB ML-model and a single ML model on the gNB side that is independent from the UE ML-model.

Proposal 7	At least for single sided ML models, the model training is assumed to be proprietary, hence no specification impact is foreseen for ML model training

Proposal 8	Study options for training collaboration of the dual-side joint AI, using Case A, B, C and D outlined above as a starting point, including at least feasibility of model adoption from external vendor and joint training frameworks in multi-vendor setups

Proposal 9	Study in particular the combinatorial problem for dual sided joint AI (Case C), to limit or remove the issue with having to implement/train one ML model for each collaborating vendor/device model/ML model version

Proposal 10	Study mechanism and signalling to enable to network to ensure performance of the ML-functionality in the UE, in case of single-sided ML-functionality in the UE and the UE part of the dual-sided joint ML functionality.

Proposal 11	For use case solutions with single-sided ML models at the gNB, studies are primarily within UE assistance in data collection for transparent LCM to the UE.

Proposal 12	Study per use case solution how network LCM assistance can be introduced to assess network performance impacts due to drifts in ML model operations at the UE side.

Proposal 13	For use cases solutions with dual-sided joint ML, focus is in this study on solutions that avoid standardization of the model deployment stage for updating a model.

Proposal 14	Study the need from the lower-layer perspective for improved UE capability reporting for conveying ML model-related information in PHY use cases, including enabling ML model version info provision and handling.

Proposal 15	In the study, deployed ML-capable UEs that support model update mechanism should be considered.

Proposal 16	Study adaptability measures to ensure UE ML model robustness across deployment scenarios in the three selected use cases.

# References

RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

3GPP TR 37.817, Study on enhancement for Data Collection for NR and EN-DC (Release 17), RAN3, V17.0.0, 2022-04.

RP-2203282, Discussion on AI-CSI, Ericsson, May 2022.

RP-180524, Summary of calibration results for IMT-2020 self evaluation, Huawei, 3GPP TSG RAN Meeting #79, Mar. 19-22, 2018.

TR 38.843, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface.

TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

TR 38.857, Study on NR positioning enhancements

# Appendix

Table z: AI/ML terminology

| Term | Description |
| --- | --- |
| Machine learning model | The output, or result, of a machine learning algorithm that run on training data. It represents the learning obtained by the ML algorithm. |
| Training data | Set of data used by an AI/ML algorithm to generate an AI/ML model. |
| Validation data | Set of data used in algorithm training to mitigate overfitting to training data. |
| Test data | Set of unseen data used to evaluate performance of a model. It may be used for testing model generalization |
| Data preparation (pipeline) | A collective name representing the stages that provides the data used in either algorithm training or model inference |
| Data ingestion | Gathering raw data from locations where input data are stored. |
| Data verification | Verifying the validity of gathered raw data. |
| Data pre-processing | Data cleaning and feature engineering with featured data as output. |
| Data feature engineering | May refer to selecting features of data as feature inputs to an ML algorithm, transforming data into a desired feature input data domain, normalizing and formatting data to fit an AI/ML algorithm input interface. |
| Model generation | The development process leading to a model that is ready for deployment |
| Model inference | When a deployed model is used for making inference |
| Drift monitoring | A collective name for monitoring drifts in inference data and deployed model. |
| Supervised learning | Algorithm training relying on labelled input and output data |
| Unsupervised learning | Algorithm training relying on non-labelled input and output data |
| Split learning | A type of joint ML training where the training is a split between two or more entities, which for a neural network implies that feedforward data and backpropagation data (gradients) are exchanged between the entities. |
| Offline training | Algorithm training when a complete training dataset has been collected |
| Online training | Sequential update of ML model upon arrival of new training data |