3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203281

Online, May 16th – 27th, 2022

Agenda Item:	9.2.2.1

Source:	Ericsson

Title:	Evaluation of AI-CSI

Document for:	Discussion, Decision

# Introduction

In RAN#94e, the new study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. This is the first AI/ML study for 3GPP RAN1, and the intention is to explore the 3GPP framework for adopting AI/ML in the air interface. The study needs to investigate AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, life cycle management, etc. The investigation should also consider aspects such as performance, robustness, complexity, and potential specification impact.

One use case identified for the pilot study is CSI feedback enhancement [1]:

Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, the evaluation of the CSI use case and relevant sub use cases will be discussed.

# General aspects for AI-CSI use case study

## Key Performance Indicators (KPIs)

As discussed in the introduction of [6], some potential enhancement areas for introducing AI/ML for CSI:

- The use of SRS has drawbacks in coverage (compared to CSI Type-II feedback), requires antenna switching to be implemented in UE, and power imbalances between transmit antennas in the UE side as recently been discussed in RAN4 and RAN1.

- The use of CSI Type-II and eType-II have drawbacks in lack for spatial resolution (compared to SRS based).

- MU-MIMO performance has been observed to degrade at relatively low speeds, because the acquired CSI at the transmitter is instantaneous and is very sensitive to UE speed.

- The latency of the Type-II CSI reports is large, which contributes to the sensitivity to UE speed.

Hence, the relevant KPIs (for each network load point) should be as follows:

- User throughput gain over baseline for given UCI payload target (CSI report).

- UCI payload (CSI report) reduction relative to baseline for a target user throughput target .

- CSI reporting latency reduction using AI-CSI compared to baseline.

- User throughput difference compared to UE speed X=3 km/h at UE speeds X=10,20,30 km/h.

- The ability of the ML model to generalize to varying assumptions on gNB and UE implementation such as subarray size, directional UE antennas etc compared to the baseline.

- The ability of a dual-side AI models to cope with the multi-vendor issue (the combinatorial issue) compared to the baseline.

The baseline is suggested to be eType-II and is discussed in Section 3.

## On presenting results in this SI

We acknowledge that there is a balance between respecting each company’s right to its own intellectual property, while sharing enough to maintain trust and mutual understanding among companies and to enable some level of reproduction and cross-checking.

The set of tools and techniques for AI/ML model generation is vast, so it is difficult to construct a comprehensive checklist list. Instead, we propose the following high-level principle for presenting results.

Along with evaluation results, the proponent should provide sufficient details about the experiment (e.g., data generation, feature extraction, AI/ML model design, training, validation, and testing) so that the whole experiment can be reproduced by persons skilled in the art of AI/ML and 3GPP CSI evaluations.

To fulfill Proposal 1, we believe that the following details would typically need to be shared:

- Share a high-level, academic-paper style, description of the AI/ML model architecture. Sufficient details should be provided so that person(s) skilled in AI/ML can reimplement the AI/ML model.

- Share simulation parameters used to generate synthetic datasets for training, validation, and testing. Sufficient detail should be provided to enable persons skilled in 3GPP CSI evaluations to reproduce the most important characteristics of dataset(s).

- Share important data preprocessing / feature extraction steps.

- Share per-sample loss functions and loss functions (the overall objective function that was optimized), see [3] for details. This can include, for example, regularization terms in the loss function.

Optionally, companies can share useful hyperparameters including, for example, the learning rate, learning rate decay, mini-batch size, optimizer details (e.g., ADAM, Adagrad, RMSprop, momentum terms), and any training tricks (such as dropout regularization). The latter may, for example, be important when studying AI/ML model generalizability.

We believe that higher-level principle in Proposal 1 has proven successful for academic papers (e.g. IEEE contributions) discussing results of machine learning. Such input is valuable to assess the complexity of the proposed model and is important when capturing results in the TR so that a conclusion can make a right balance between performance and complexity.

# Evaluation methodology

Since the CSI enhancement is targeting serving cells where massive MIMO antenna arrays are deployed, and where MU-MIMO scheduling is likely to be enabled, the dataset for training of the ML models should at least be representative of the channels observed in such cells. That is, the datasets should cover macro and microcells, with massive MIMO deployed gNBs with a large number of TXRUs at the gNB.

We should strive to avoid experiments where the same dataset statistics are used for training, validation, and testing.  This is particularly important to avoid overfitting, so that the evaluations provide some meaningful indication of AI/ML model generalizability.

The dataset for AI model training should represent the large space of MIMO channels in cells where massive MIMO is likely to be deployed and where MU-MIMO is commonly used.

Note that this is different from our usual evaluation assumptions in RAN1, Hence, RAN1 need to discuss and agree on representative mix of channel model parameters for training of ML algorithms. A starting point can be to create a reference dataset training mix using these parameters

- A mix of 32 and 16 port CSI-RS

  - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ

  - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ

- A mix of Dense Urban (Macro only) and Urban Macro and Urban Micro

- A mix of 2 GHz and 4 GHz carrier frequencies using 15 and 30 kHz SCS respectively

- A mix of 200m and 500m ISD

- A mix of Indoor users (3 km/h), outdoor users (3 km/h) and outdoor users (30 km/h)

- 4RX UE: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ

In some settings, it will be necessary to further curate specific training datasets to achieve a desired outcome. Such curation should not be prohibited, provided the motivation well justified and the curation procedure well documented.

For evaluation on the other hand, (inference), a single evaluation assumption is used. We suggest to use the EVM that has been used for Rel-17 CSI enhancement work, i.e., section 4 in [2], can be re-used.

The dataset for evaluations (inference stage) uses a single parameter setting at a time and the previously used EVM for CSI enhancements can be re-used, e.g. section 4 in [2]

Models can certainly be evaluated in multiple scenarios, but as opposed to training, a single scenario is evaluated at a time. This also gives indication on how well a trained model can generalize to different scenarios.

Note that the UE speed in section 4 in [2] assumes 3 km/h and 30 km/h only. If the issue of degrading MU-MIMO performance at even more moderate speeds is to be evaluated, these evaluation assumptions needs to be modified in this aspect. For example, it could be useful to assume all UEs have the same speed in an evaluation, to thoroughly asses the benefit of a ML models that performs prediction of CSI.

If the AI/ML model is trained to perform CSI prediction, then consider revising the UE speed parameter from section 4 of [2], e.g. use a parameter sweep X=3,10,20,30 km/h where all UEs use the same speed X and all UEs are outdoor.

For the baseline to compare with, we suggest to use the Rel.16 eType-II CSI reporting as the classical, legacy reporting mode. We propose not to use Rel.17 eType-II CSI PS, because it is a special case that requires SRS and pre-compensation at the gNB. Hence, even if Rel-17 can give some modest gain over eType-II, we believe the eType-II is a cleaner baseline that relies on CSI-RS measurements only and full channel CSI reporting.

Use eType-II as the baseline for CSI use case in this SI.

# Initial performance evaluations of dual sided joint AI

In this section we provide some initial performance results related to the KPIs of

- User throughput gain over baseline for given UCI payload target (CSI report), and

- UCI payload (CSI report) reduction relative to baseline for a target user throughput target.

Note that these results are initial; for example, full buffer evaluations were used, which is not sufficient in RAN1 to make a decision on whether the benefits of an enhancement is realistic.

For this example, the simulation parameters are defined in Table 1. Specifically, the gNB is equipped with a uniform planar array with Na=32 antenna ports transmitting over N3=104 subbands to UEs with K=4 antenna port. According, the antenna-frequency domain ‘raw’ channel H have a dimension of 4×32×104 (K×Na×N3).

For the pre-processing step (see Figure 1) using domain knowledge, L = 8 spatial beams (per polarization) of the channel, i.e., 8 orthogonal DFT basis vectors per polarization, and M = 16 delay taps, i.e., 16 orthogonal DFT basis vectors, are selected.

Figure 1: Illustration of dual-sided AI CSI reporting. Note that X does not represent the full MIMO channel H but a representation of it, where pre-processing extract the relevant features X of the channel (e.g. dominant eigenvectors). Y is the output of the UE side autoencoder.

These are used to compress the ‘raw’ channel to the beam-domain channel Hmodel with a dimension of 1×16×16 (K×2L×M).  Note for the DFT basis for the spatial domain beams are for a selected 2D rotation of the oversampling pair (o1∊{1,…O1}, o2∊{1,…O2}), where the oversampling factor in both horizontal and vertical domain are taken as (O1=4, O2=4).

The UE signals in the UCI the selected oversampling pair as (o1, o2) with bos=4 bits to the gNB. The 8 orthogonal DFT basis vectors per polarization for spatial beams and 16 orthogonal DFT basis vectors for delay-taps can be encoded using bmodel= log2C816+ log2C16104=76 bits, where same spatial beams for both the polarization is considered. The beam-delay domain channel Hmodel is normalized and given as input to the AE, which generates a channel representation in latent space with 96 units, with compression or 0.0938 , respectively, where each element is encoded using 4 bits, generating  bAE= 384 bits at the output of the encoder.

Furthermore, around bnor=4 bits are required to signal the normalizing factor from UE to the gNB to reconstruct the channel at the output of the decoder. Accordingly, the total bits required is bmodel+ bos+ bAE + bnor=486 bits.

Table 1 Used SLS parameters for initial evaluation for dual sided joint AI for CSI compression

| System-level simulation parameters |  |
| --- | --- |
| Metric | DL mean User and cell edge throughput |
| Traffic model | Full Buffer |
| Number of sites | 19 |
| Number of UEs | 570 |
| UE distribution | 80% indoor and 20% outdoor |
| Handover margin | 0 dB |
| Carrier frequency | 2 GHz |
| Bandwidth | 20 MHz (104 subbands) |
| Subcarrier spacing | 15 kHz |
| Channel model | 38.901 |
| Scenario | UMa dense (200 m ISD) |
| BS antenna configuration | (M, N, P, Mg, Ng, Mp, Np) = (8, 8 ,2, 1, 1, 2, 8) with (dH, dV) = (0.5, 0.8)λ |
| UE antenna configuration | (M, N, P) = (1, 2, 2), 0.5λ element spacing, omni-directional elements |
| MIMO scheme | MU-MIMO enabled |
| Baseline scheme | Type-II codebook (parComb 6) |
| BS antenna height | 25 m |
| BS transmit power | 44 dBm |

We used autoencoder approach for the ML model, where the output of the encoder AEs has 432 bits and the total side information from the pre-processing and normalization steps use 84 bits in the UCI. The ML model used in this initial study is an inception style network with four branches and convolutional neural network (CNN) layers. See a brief description of the used ML models in UE and gNB in the Appendix. Approximately 1.6 million parameters are used in total for the UE and gNB side.

From these results, it seems both targeted KPIs can be enhanced. There is a performance increase and there is a CSI overhead reduction. However, the AE has not yet been optimized, the study will try other types of layers etc, enhanced pre-processing steps, and custom loss functions. And most importantly, these results are obtained for full buffer only, which cannot be used to judge whether AI can actually enhance the KPIs. In addition, here a single vendor training methodology was used, the study needs to consider the realistic case of multi-vendor training and the combinatorial problem of handling many trained models in UE and gNB side simultaneously, which may degrade performance benefit.

The results also shows the gap to ideal channel knowledge at the gNB, hence this sets an upper bound on how well the two considered KPIs can be improved using dual sided joint AI for CSI compression.

Figure 2 Cell edge user throughput (full buffer)

Figure 3 Mean user throughput (full buffer)

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Along with evaluation results, the proponent should provide sufficient details about the experiment (e.g., data generation, feature extraction, AI/ML model design, training, validation, and testing) so that the whole experiment can be reproduced by persons skilled in the art of AI/ML and 3GPP CSI evaluations.

Proposal 2	The dataset for AI model training should represent the large space of MIMO channels in cells where massive MIMO is likely to be deployed and where MU-MIMO is commonly used.

Proposal 3	The dataset for evaluations (inference stage) uses a single parameter setting at a time and the previously used EVM for CSI enhancements can be re-used, e.g. section 4 in [2]

Proposal 4	If the AI/ML model is trained to perform CSI prediction, then consider revising the UE speed parameter from section 4 of [2], e.g. use a parameter sweep X=3,10,20,30 km/h where all UEs use the same speed X and all UEs are outdoor.

Proposal 5	Use eType-II as the baseline for CSI use case in this SI.

# References

RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN Meeting #94e, December 6th – 17th, 2021

3GPP R1-2006973, Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity, Huawei, RAN1#102e, E-meeting, August 17th – 28th, 2020.

R1-2203282, Discussion on AI-CSI, Ericsson, 3GPP TSG-RAN WG1 Meeting #109-e, May 16th – 27th 2022.

# Appendix

The used autoencoder NN used for our initial evaluations is depicted in Figure 4 and Figure 5. It is a convolutional autoencoder with residual connections and a few dense layers, split into four branches. The branches are identical in terms of architecture, but may learn different parameters. On the encoder-side the input to one branch corresponds to the data from one of the four Rx-ports on the UE, and one the decoder side each branch is tasked with reconstructing the data corresponding to one UE Rx-port. The entire autoencoder has approximately 1.6 million trainable parameters and takes about 8 hours to train on a high-performance GPU.

In Figure 4 and Figure 5 only one of the branches are explicitly described, as the other three have identical architecture. A black box means a 2D convolution, with a dashed line it is with same padding and with solid line it is with valid padding, a blue box means a dense layer, a yellow box represents concatenation, and a green box is a 1D convolution with same padding, and the orange ovals is a residual connection realized through addition.

Figure 4: UE-side encoder

Figure 5: NW-side decoder