3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203283

Online, May 16th – 27th, 2022

Agenda Item:	9.2.3.1

Source:	Ericsson

Title:	Evaluation of AI-Beam Management

Document for:	Discussion, Decision

# 1	Introduction

In RAN#94e, the new study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. This is the first AI/ML study for 3GPP RAN1, and explores the 3GPP framework for adopting AI/ML in the air interface. The study needs to investigate AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, life cycle management, etc. The investigation should also consider aspects such as performance, robustness, complexity, and potential specification impact.

One use case identified for the pilot study is beam management:

RP-213599 (SID): Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

# 2	Use Cases and Scenarios

In the justification for the FeMIMO for Release 17 WID [2], it was highlighted that “FR2 require more aggressive reduction in latency and overhead – not only for intra-cell, but also for L1/L2 centric inter-cell mobility. This also includes reducing the occurrence of beam failure events”.  The goal of reducing the overhead and robustness were also reflected in the objectives. As an outcome of the work, for example a unified TCI state is one improvement solution for reducing the signaling complexity and thereby overhead.  However, the potential for further improving on robustness or overhead reduction using AI/ML should be explored.

One example AI/ML-model outlined in the AI for air-interface SID comprises predicting the channel in respect to a beam for a certain time-frequency resource. Radio channels in 3GPP network deployments exhibit correlations in time, frequency, and space. These correlations depend, to a large extent, on the radio equipment, local propagation environment, and UE mobility patterns. While it is not feasible to analytically model every deployment in detail, it may be feasible to collect and curate datasets to train AI/ML models that exploit such correlations to improve robustness, throughput, latency, and overhead. The expected performance of such AI/ML predictor model depends on several different aspects, for example spatial/time/frequency variation of channel due to UE mobility or changes in the environment. The spatial domain can comprise of different beams, where the correlation properties partly depend on the how the gNB antennas forms the different beams, and how UE forms the receiver beams. For AI/ML study, the goal should according to the justification for the Rel-17 WID on FeMIMO, focus on achieving improved robustness and reducing the overhead of the BM procedure. The reduced overhead can lead to energy savings or latency reductions.

Beam predictions are possible due to the inherent channel correlation in spatial or time-frequency domain

The study item should investigate how beam predictions can improve robustness and reduce overhead associated to beam management

The overhead should be studied in scenarios that were evaluated in the Rel-17 WI [3]. It is important to carefully design the scenario parameters to evaluate scenarios where the channel properties might experience high correlation in time/frequency/beam domain. It is also important to investigate more challenging conditions where the spatial correlation implies less trivial relations between beams. For the robustness use case, one should define scenario parameters that target outage events (lack of robustness). This can, for example, comprise of setting parameters causing large number of transitions from LOS->NLOS or blockers in the environment (fixed or dynamic blockers such as moving objects).

Example scenarios to be evaluated can be based on the intra-cell mobility and the Dense Urban scenario defined in [3]. It may be of interest to enrich these scenarios with variations/additions to facilitate for more diverse data sets in training/testing of AI/ML models. We note that baseline case for intra-cell only has a single line-of-sight (LOS) outdoor UE on fixed perpendicular trajectory with 60 km/h or 120km/h, 30 m in front of each sector antenna. Such scenario variations could include changes to the fixed trajectory (e.g. distance to the sector antenna), orientation of the trajectory compared to perpendicular and random starting position and direction along the trajectory. In addition, random initial UE positions with random direction (i.e. drop positions as in TR 38.901 but with only outdoor UEs) could be of interest for training/testing AI/ML models.

We also note that TR38.901 specifies that for physical UE mobility, an outdoor UE may change between line-of-sight (LOS) or not line-of-sight (NLOS) depending on its position and may thus transition between these two states due to movement, where the probability of LOS compared to NLOS is mainly dependent on the distance between the UE and gNB. If such transitions are modelled in a scenario, this should be highlighted.

As discussed in [6], we believe that the beam management use case is an important use case to study tradeoffs between AI/ML model specificity (trained for a small validity area, e.g., one site) and AI/ML model generality (trained for a large validity area, e.g., many sites or a scenario) For such comparisons, companies can report training, validation, and test dataset parameters, and describe the intended AI/ML model validity area/scenario.

In the appendix, the common parameters over multiple training/test scenarios are shown in Table 1, while specifics for variations of intra-cell mobility scenario is shown in Table 2. These tables are based on Table 1 and 2 in [3].

Beam management should be evaluated based on the scenarios defined in [3], which are summarized in the appendix

# 3	Beam prediction observations

An important first step, prior to training an ML-model, is to understand how a potential predictor could be trained (identify important features, variables that could be predicted etc.). Figure 1 shows an example where a mobile outdoor UE measures a set of beams over time and experiences a transition from a LOS to NLOS channel in slot number 10 000. The UE measures the path gains of the 20 best beam-pair-links (BPL), which are defined by narrow data beams from the BS to narrow beams to a UE with three antenna panels. The total number of candidate BPL is 1584, therefore it is useful to accurately predict which BPL candidates to measure.

More specifically, Figure 1 illustrates:

- Timeseries measurements of 20 strongest beam-pair-links, each depicted by a coloured line.

- LOS scenario has one beam that is strongest for a large time window, and monitoring the 2nd, 3rd beam trends over a certain time duration indicate possibility to forecast when their strength exceeds the currently strongest beam. Given the nature of the RSRP trend, such forecasting model can be of low complexity.

- NLOS have many beams of similar strength in comparison to LOS, indicating that it could be challenging to predict the strongest beam.  There are less obvious trends on correlations among beams in the NLOS in comparison to the LOS scenario.

- Large RSRP drop when transitioning from LOS to NLOS, could cause an outage

Figure 1: Illustration path gains for different beams as a UE transitions from a LOS channel to a NLOS channel.

Figure 1: Illustration path gains for different beams as a UE transitions from a LOS channel to a NLOS channel.

LOS -> NLOS

LOS -> NLOS

Prediction performance heavily depends on the simulation parameters (e.g. parameters that control LOS/NLOS or indoor/outdoor UE)

Large signal quality drop causing potential outage when transitioning from LOS to NLOS

# 4	Evaluation methodology

The information for ensuring comparable evaluation results is further discussed. Also, in the appendix, Table 3 describes ML-related information to be presented during evaluations based on the discussion below.

### 4.1	Baseline performance

One option outlined in the SID for comparable results is to introduce a reference model for calibrating results. If the simulation parameters are carefully described during the evaluation from each company, the need for such model is hard to motivate. The SI should first highlight the problem of comparable results before introducing a reference model.

No need to introduce a reference AI/ML model for calibration

### 4.2	Dataset construction

We believe that training, validation, and test datasets can be specified by simulation parameters. Companies should describe the training scenario/parameters with enough detail so that other companies can replicate the training set. For example, the number of training samples and specific dataset curation techniques (such as mixing data from different scenarios). We believe that training on data from mixed scenarios is important when quantifying the generalization performance of an ML-model. Similarly, another aspect on model generalization is whether it is trained for a specific cell, or valid for multiple cells, or all cells in the deployment. It can also be seen as the validity area of the model. This applies both for UE and gNB sided ML-models. As discussed in our contribution on general aspects [5], The models are assumed to be trained one-time using the entire training dataset (offline learning), hence no sequential training based on arrival off new data, commonly referred to as online learning.

Training/test dataset defined by simulation parameters. Dataset comprising a mix of scenarios should be possible

Model validity area should be described, if the AI/ML model is trained for a cell (cell-specific), multiple-cells, or for the entire scenario (valid for all cells).

Models are trained offline (no sequential update of the model)

On the aspect on using field data, we believe that a first approximation of potential performance gains can be made using the synthetic 3GPP channel models outlined in [3]. The vast number of potential parameters that could be explored in such simulation should be explored prior to evaluating any field data. A set of example scenarios to be evaluated are included in the appendix.

No need for field data for the study item BM use case

### 4.3	ML-Model description

An ML-model is more suitable in finding patterns in potentially high-dimensional data, due to the complex nature of these patterns, therefore it is important to describe the assumptions when providing performance results associated to such ML-model. It is also important to outline training methodology, for example the used optimization function, relevant hyperparameters and number of training/test samples which is used for training the model. A potential description for the ML-model input could outline the time-domain assumptions on beam measurement and prediction. This can comprise an observation time period, number of observations during the time period, for measurement. This would hence need to be presented in to make common assumptions on what observation time that could be assumed. It can also restrict when it is possible to start performing inference of such model. We need the same type of assumption in spatial domain e.g., number of the beams as well as their indices that are used for measurement to train the model, etc. . A potential description for the ML-model output could comprise the granularity of a potential predicted RSRP value, also what time-frequency resource and beam that is predicted. A potential description of the model could be a general description, for example the type of neural network, real valued vs complex valued, architecture, quantization, etc.

The training methodology impacts the ML-model performance evaluation, since the definition of a potential loss function will impact how the model adapts to various scenarios. For example, one loss function might be minimizing the mean absolute error, while another minimizing the mean squared error, each will result in different ML-models. Also, the model can be trained to prioritize certain output regions higher than others, that might lead to high accuracy for some regions, but less accurate for other regions. Another aspect related to training methodology is the number of samples in the test/training dataset.

Evaluation should provide a description of ML/AI model, model input/output, along with the training methodology

Training methodology could for example include the training optimization function, loss function and/or number of training/test samples

### 4.4	 Performance metrics

The common KPIs in our view comprises identifying the performance in common AI/ML-metrics, to understand the prediction potential of various solutions. Next, such AI/ML-specific KPIs should be translated into use-case specific KPIs and benchmarks.

#### 4.4.1	ML-performance metrics

One of the common KPIs can be associated to the accuracy in beam selection as outlined in the SID. There are however several potential metrics widely used for ML, for classification problems some of the mainly used metrics are

•	Confusion matrix

•	Classification Accuracy

•	F-Measure

•	Logarithmic Loss

•	Area under curve (AUC) for the Receiver operating characteristic (ROC) curve

For regression problems the main metrics in use are

•	R-Squared

•	Adjusted R-Squared

•	Mean Square Error (equation 1) / Root Mean Square Error

•	Mean Absolute Error

Classification problems for BM can comprise predicting the strongest beam index, or beam pair. Such prediction could be evaluated using an accuracy metric. It could also be evaluated by an accuracy metric that given selecting a certain number of beams based on strongest beam index probability, one of the selected beams are the strongest.

Regression problems for BM can comprise of predicting the RSRP in future time instance, or for a non-measured beam. Potential metrics could comprise of the mean-squared error evaluated on the test/training data set.

Evaluation results could include ML-performance metrics related to beam predictions, both comprising classification and regression performance metrics

#### 4.4.2	Use-case performance metrics

In addition to the common/ML performance KPIs. It is important to translate ML-specific KPIs into commonly used KPIs for beam management. More specifically, the study should translate into the performance metrics outlined in [3]. For example, use

- CDF of UE throughput, avg. and 5% UE throughput

- Beam switching latency

- RSRP distribution

- Outage / measures of robustness

Moreover, one of the benefits of reduced number of beam measurements for the UE is an energy saving, it is hence important to also evaluate such impacts during the SI. One example is to reuse the UE power consumption models for FR2 defined in [4].

Use case performance metrics could include metrics defined in [3], including a potential energy saving metric, for example using the models defined in [4].

# 6 Conclusions

In the previous sections we made the following observations:

Observation 1	Beam predictions are possible due to the inherent channel correlation in spatial or time-frequency domain

Observation 2	Prediction performance heavily depends on the simulation parameters (e.g. parameters that control LOS/NLOS or indoor/outdoor UE)

Observation 3	Large signal quality drop causing potential outage when transitioning from LOS to NLOS

Observation 4	Training/test dataset defined by simulation parameters. Dataset comprising a mix of scenarios should be possible

Based on the discussion in the previous sections we propose the following:

Proposal 1	The study item should investigate how beam predictions can improve robustness and reduce overhead associated to beam management

Proposal 2	Beam management should be evaluated based on the scenarios defined in [3], which are summarized in the appendix

Proposal 3	No need to introduce a reference AI/ML model for calibration

Proposal 4	Model validity area should be described, if the AI/ML model is trained for a cell (cell-specific), multiple-cells, or for the entire scenario (valid for all cells).

Proposal 5	Models are trained offline (no sequential update of the model)

Proposal 6	No need for field data for the study item BM use case

Proposal 7	Evaluation should provide a description of ML/AI model, model input/output, along with the training methodology

Proposal 8	Training methodology could for example include the training optimization function, loss function and/or number of training/test samples

Proposal 9	Evaluation results could include ML-performance metrics related to beam predictions, both comprising classification and regression performance metrics

Proposal 10	Use case performance metrics could include metrics defined in [3], including a potential energy saving metric, for example using the models defined in [4].

# References

RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

RP-193133, New WID: Further enhancements on MIMO for NR , Samsung , TSG RAN Meeting #86,

R1-2007151 Moderator summary for multi-beam enhancement: EVM

TR 38.840, Study on User Equipment (UE) power saving in NR

R1-2203280, “General aspects of AI PHY”, Ericsson, 3GPP TSG-RAN WG1 Meeting 109-e, May 16-27, 2022.

R1-2203284, “Discussion on AI Beam Management”, Ericsson, 3GPP TSG-RAN WG1 Meeting 109-e, May 16-27, 2022.

# Appendix

#### Potential Evaluation scenarios

Table 1 Baseline assumptions for SLS: common over training/test scenarios

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz - BW: 80 MHz |
| Channel model | TR 38.901 with spatial consistency and physical UE mobility modelled. Outdoor UEs may be in line-of-sight (LOS) or not line-of-sight (NLOS) and/or transition between these two states due to movement if modelled. Addition of Blockers according to TR 38.901 (e.g. model B) is not precluded. |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ Companies to explain TXRU weights mapping. Companies to explain beam selection. Companies to explain number of BS beams Other configuration is not precluded. |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Number/location of panels: 3 panels (left, right, and back) Panel structure: 1x4x2 or (M, N, P) = (1, 4, 2), dH = 0.5 λ Companies to explain TXRU weights mapping. Companies to explain beam and panel selection. Companies to explain number of UE beams Other configuration, e.g. isotropic UE antenna (i.e. only gNB beam management procedures), is not precluded. |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| Beam correspondence | Companies to explain beam correspondence assumptions (in accordance to the two types agreed in RAN4) |
| Link adaptation | Based on CSI-RS |
| Traffic Model | Full buffer |
| Inter-panel calibration for UE | Ideal, non-ideal following 38.802 (optional) – Explain any errors |
| Control and RS overhead | Companies report details of the assumptions |
| Control channel decoding | Ideal or Non-ideal (Companies explain how it is modelled) |
| UE receiver type | MMSE-IRC as the baseline, other advanced receiver is not precluded |
| BF scheme | Companies explain what scheme is used |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| Other simulation assumptions | Companies to explain serving TRP selection Companies to explain scheduling algorithm |
| Algorithm details (when applicable) | Companies to report: - Beam reporting mechanism - Beam metric L1-RSRP - Number of active panels |
| Other potential impairments | Not modelled (assumed ideal). If impairments are included, companies will report the details of the assumed impairments |
| Performance metrics | - CDF of UE throughput, avg. and 5% UE throughput - TCI state update (beam indication) signaling overhead (separate analysis from SLS) - Beam switching latency - RSRP distribution |
| ML performance metrics | - Classification metric - Regression metric - .. |

Table 2 Example Intra-cell mobility scenarios for training/testing

| Parameters | Values |
| --- | --- |
| Scenarios | Dense Urban (macro-layer only, TR 38.913), 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site), 100% outdoor: - One UE is dropped for each of the 21 sectors/cells with specified linear trajectory (see mobility description below). - Follow random dropping patterns and random velocity direction in TR 38.901 with RSRP based cell-selection. Other scenarios (e.g., Urban Macro 500 m ISD) are not precluded. |
| UE Speed | For Dense Urban: 60 km/h and 120 km/h |
| UE Mobility and trajectory handling | Linear trajectory, intra-cell mobility (constrained within one cell) - Trajectory sampling at most spaced by decorrelation distance Dense Urban – specific linear trajectory: For each of the 21 cells: One UE is dropped as follows: For the upper right sector/cell (can be extended analogously to the upper left and lower sectors/cells, see Appendix B) with d=30m, the UE starts at P and moves along the 120-deg line downward to Q. Variations of this specified linear trajectory are not precluded and should be reported if used. Each sector is a cell and that the cell association for intra-cell mobility is geographic and not RSRP based. Note: Optionally, if for some reason a company would like to simulate only one cell/sector, the company should clearly state this in the contribution, including the assumed interference model. Other configurations, e.g. different d values, orientation of P-Q trajectory and random starting point and direction along P-Q trajectory, are not precluded. |
| UE and panel orientation | Companies will describe the assumed UE and panel orientation relative to the direction of the trajectory, e.g. vertical UE placement (talking mode) with random UE xy-orientation. |

Overview of the ML-specific information is summarized in the table below

Table 3: Example ML-specific information to be provided in the evaluations

| Parameters | Potential Values |
| --- | --- |
| Dataset description (Training/Test data) | - Number of samples - Assumptions on mix of scenarios, e.g. training data includes multiple UE velocities and/or trajectories specifications |
| Model validity area | - Site specific - Multiple sites - Deployment specific (model is used for all evaluated sites) |
| Model description | - Model type (Neural network, random forest, linear regression ….) - Model hyperparameters |
| Model input description | - Historical observations (observation time) - Number of input parameters - Type of input parameters - … |
| Model output description | - Output description (e.g. predicted time/frequency/beam) |
| Training methodology | - loss function |
| Common KPIs | - Classification KPIs - Regression KPIs |
| Model complexity KPIs | - Number of parameters - Floating point operations |