3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203284

Online, May 16th – 27th, 2022

Agenda Item:	9.2.3.2

Source:	Ericsson

Title:	Discussion on AI Beam Management

Document for:	Discussion, Decision

# 1	Introduction

The study item Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN#94e [1]. It will be the first study of AI/ML technology in 3GPP RAN1. The study item will explore 3GPP frameworks to enable AI/ML including, for example, AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, and life cycle management. The study should quantify the performance, robustness, complexity, and potential specification impact of AI/ML based solutions.

One use case identified for the pilot study is beam management:

RP-213599 (SID): Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Radio channels in 3GPP network deployments exhibit correlations in time, frequency, and space. These correlations depend to a large extent on the radio equipment, local propagation environment, and UE mobility. While it is not feasible to analytically model every deployment in detail, it may be feasible to collect and curate datasets to train AI/ML models that exploit such correlations to improve robustness, throughput, latency, and overhead. For example,

- UE-side beam prediction AI/ML models may combine downlink reference signal measurements with other sensor data to reduce latency and save power by, for example, predicting RSRP values for unmeasured beams.

- NW-side beam prediction AI/ML models can adapt to the local propagation environment and UE mobility patterns to improve robustness, overhead, and throughput by, for example, predicting sudden RSRP drops due to UE mobility and blockages.

In this contribution, we discuss the beam management use case scope and potential specification impacts. General aspects on AI/ML functional frameworks, life cycle management (LCM), UE capabilities, and AI/ML model testing, which apply more widely than this use case, are discussed in [2]. Evaluation methodologies for the beam management use case are discussed in [3].

We view AI/ML model generation (design and training) as a proprietary endeavour with no standard impact. We believe that this use case study can focus on the following activities:

- UE-NW collaborations to enhance data collection for (offline) AI/ML model generation, with associated specification impacts (i.e., assistance information for offline AI/ML model generation).

- Configuration, activation, deactivation, monitoring, and enhancement of trained beam prediction AI/ML models, with associated specification impacts (i.e., assistance information for AI/ML model inference).

- Evaluations to quantify possible performance gains to, e.g., justify future normative work.

# 2	UE/NW Collaboration levels

AI/ML models for beam prediction can be deployed to gNBs and/or UEs. We foresee three scenarios [2]:

- NW-side AI: Single-sided AI/ML functionality in the NW.

- UE-side AI: Single-sided AI/ML functionality in the UE.

- Dual-sided joint AI: AI/ML functionality in both the UE and NW operating over the air interface (joint operation).

Note: The above categorization does not exclude the situation where both the NW and UE operate AI/ML functionality independently, and transparently, of one another.

We see potential benefits for single-sided AI in the UE and/or NW.

- NW-side AI: gNBs are static in the typical macro deployments, and, therefore, have opportunities to learn site-specific properties (such as blockers in the environment, and typical UE mobility patterns). The gNB typically has better computational, memory, and power capabilities than the UE.

- UE-side AI: UEs have access to measurements of downlink reference signals and external information sources (such as sensors) that can be used to understand and exploit the environment and/or UE movement.

We do not see a strong need to study dual-sided joint AI features for beam management, given the complexities of such solutions and their limited potential in this use case. Indeed, we believe that these difficulties may be exaggerated for beam management use cases as joint UE-NW training may require site-specific training data. Moreover, functional framework aspects of dual-sided AI/ML will be covered in the CSI and positioning use cases. A more detailed discussion of the complexities of dual-sided AI/ML is given in [2].

Study UE-sided AI enhancements for beam management.

Study NW-sided AI enhancements for beam management.

Do not study dual-sided joint AI enhancements for beam management.

The predictive power of AI/ML models depends to a large extent on the quality of the data on which they are trained and inferred. The current beam management framework, however, presents several challenges for reliable data collection for training and inference.  For example, the following issues will inhibit the NW’s ability to curate suitable datasets on which AI/ML models can be trained, verified, tested and deployed:

- The NW does not know if/when a UE has filtered RSRP measurements over multiple SSB bursts.

- The NW does not know if/when the UE has used narrow or wide beams per UE panel.

- The NW does not know how old the L1-RSRP measurement in an SSB beam report is (only that it is done within a time window defined by TL1-RSRP_Measurement_Period_SSB as specified in TS 38.113).

- The NW does not know if the UE has evaluated all candidate UE panels for an SSB beam report, or only a subset of them.

- The NW does not know if a reported beam has been received with a dual-polarized UE panel or a single polarized UE panel.

- The NW can only receive up to 4 best beams in a beam report which limits the knowledge of the channel between the NW and UE.

Proprietary beam management procedures executed on the UE side (resp. NW side) effect NW side (resp. UE side) data quality and, therefore, AI/ML model generation and performance.

A key benefit of AI/ML solutions is the ability to continuously develop and improve models based on observed performance. This advantage, however, presents a potential risk for beam management: Uncoordinated and transparent actions from AI/ML models operating in the UE and NW may make beam management and AI/ML model generation even more challenging. For example, it can be challenging to optimise legacy beam management procedures, or learn useful patterns, if one side is constantly changing its behaviour by updating its model.

Single-sided AI/ML models in the UE, which are transparent to the NW, may make beam management and AI/ML model generation more difficult.

For UE-side AI, it is important to identify potential impacts on NW performance and NW-side AI/ML model generation. For example, the NW may need to be aware that the UE performs ML functionality (such as RSRP predictions) to execute corresponding functionalities/operations on the NW side. It is also important to leverage the potentials of capable UEs performing beam predictions to improve the overall NW performance.

UE-side beam prediction AI/ML capability signalling can enable improving NW performance.

# 3	Potential Standardization impact

General aspects of AI/ML model generation and LCM can be discussed under the general agenda item. We believe that this use case study can focus on standardization impacts associated with the following:

- UE-NW collaborations to enhance data collection for (offline) AI/ML model generation (e.g. assistance information for offline model generation, for example, address the issue that NW does not know how old the L1-RSRP measurement in an SSB beam report is).

- UE-NW collaborations to detect network performance impacts due to drifts in ML model operations at the UE side

- Configuration, activation, deactivation, monitoring, and assistance information for model inference (potential information used when performing model training).

Study enhanced beam management procedures to aid data collection for (offline) single-sided UE/NW model generation, for example potential assistance information.

Study specification impacts for beam prediction AI/ML model configuration activation, deactivation, and monitoring.

We believe that capability information about the specific AI/ML model being executed in the UE, and the reliability of its outputs/predictions, is important for leveraging the potentials of capable UEs performing beam predictions. In addition to impact on RAN1 specifications, there might be impact on RAN4 specifications when it comes to test scenarios and performance requirements.

Study AI/ML model capability reporting associated with beam predictions from UE.

Study the benefit of signalling predicted values and associated confidence levels for beam management.

The beam management use case may be particularly well suited to site-specific AI/ML models trained on site-specific data. We propose to study requirements and potential solutions for generating and providing site-specific beam prediction AI/ML models to UEs. Such requirements might include, for example, the expected AI/ML model size and possibilities to dynamically reconfigure a single AI/ML model for different sites.

Quantify the benefits of site-specific beam prediction models AI/ML models trained on site-specific data.

Study requirements and solutions for enabling trained site-specific beam prediction AI/ML models to UEs

# 4 Conclusions

In the previous sections we made the following observations:

Observation 1	Proprietary beam management procedures executed on the UE side (resp. NW side) effect NW side (resp. UE side) data quality and, therefore, AI/ML model generation and performance.

Observation 2	Single-sided AI/ML models in the UE, which are transparent to the NW, may make beam management and AI/ML model generation more difficult.

Observation 3	UE-side beam prediction AI/ML capability signalling can enable improving NW performance.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Study UE-sided AI enhancements for beam management.

Proposal 2	Study NW-sided AI enhancements for beam management.

Proposal 3	Do not study dual-sided joint AI enhancements for beam management.

Proposal 4	Study enhanced beam management procedures to aid data collection for (offline) single-sided UE/NW model generation, for example potential assistance information.

Proposal 5	Study specification impacts for beam prediction AI/ML model configuration activation, deactivation, and monitoring.

Proposal 6	Study AI/ML model capability reporting associated with beam predictions from UE.

Proposal 7	Study the benefit of signalling predicted values and associated confidence levels for beam management.

Proposal 8	Quantify the benefits of site-specific beam prediction models AI/ML models trained on site-specific data.

Proposal 9	Study requirements and solutions for enabling trained site-specific beam prediction AI/ML models to UEs

# References

RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 – 17, 2021.

R1-2203280, General aspects of AI PHY framework, Ericsson, 3GPP TSG-RAN WG1 Meeting 109-e, May 16-27, 2022.

R1-2203283, Evaluation of AI beam management, Ericsson, 3GPP TSG-RAN WG1 Meeting 109-e, May 16-27, 2022.