3GPP TSG-RAN WG1 Meeting #109-e	Tdoc R1-2203285

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.2.4.1

Source:	Ericsson

Title:	Evaluations on AI-Pos

Document for:	Discussion, Decision

# Introduction

In RAN#94e, the new study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. This is the first AI/ML study for 3GPP RAN1, and explores the 3GPP framework for adopting AI/ML in the air interface. The study needs to investigate AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, life cycle management, etc. The investigation should also consider aspects such as performance, robustness, complexity, and potential specification impact.

One use case identified for the pilot study is positioning accuracy enhancements:

RP-213599 (SID): Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1] - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

In this contribution, we discuss the issues for establishing the framework for evaluating the potential enhancements achievable by AI/ML based positioning.

# Use Cases and Scenarios

5G system is expected to provide positioning services. TS22.261 provides 7 positioning services levels, see Appendix A.

For AI/ML study, the goal should be to achieve high positioning accuracy for scenarios where existing non-AI/ML methods struggle to satisfy the performance requirements. The existing methods may be inadequate due to:

- Demanding high reference signal overhead. For example, large bandwidth reference signals with several time-domain repetitions, where the reference signals are typically PRS for DL, SRS for UL;

- High signalling overhead between the UE and the network;

- Strict requirement on the availability of a certain number (e.g., >=3) of LOS links between the UE and neighbour gNBs;

- Difficulty to handle synchronization error between TRPs;

## Prioritized Use Cases

Industry IoT (IIoT) is expected to be a major enabler for revolutionizing the manufacturing industry. It provides an infrastructure to support extensive connectivity and communications in industrial production and logistics. Specifically, factory automation is expected to enable industrial mass production with high quality and cost-efficiency. It imposes strict requirements on the underlying communication infrastructure in terms of availability, latency, and localization services.

In terms of positioning, the smart factory use cases poses the most stringent positioning performance requirements. For example, horizontal accuracy of <30cm for 99.9% of the UEs, see TS 22.104.  On the other hand, the physical environment of indoor factory leads to radio propagation channel condition that make it difficult for existing positioning methods to satisfy the tightest requirements. For instance, the factory floor is scattered with metallic objects of irregular sizes; the clutters are large metallic machines which blocks the radio links and causes NLOS. This is challenging for the existing positioning methods (e.g., multi-RTT, DL-TDOA, UL-TDOA, DL-AoD, UL-AoA), which have the inherent assumptions of LOS links.

Thus the IIoT use case can be used as a primary example to test the effectiveness of AI/ML models for achieving high positioning accuracy.

AI/ML based positioning enhancements are most valuable for IIoT use cases, where existing methods struggle to satisfy the strict positioning accuracy requirements.

Various channel models have been developed in TR38.901 for the indoor smart factory use case, which reflect the various layout of the factory floor. For example:

- InF-SL:	 Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL:	 Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH: Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH: Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH: Indoor Factory with High Tx and High Rx (both elevated above the clutter)

From the above, two representative layouts can be selected for the evaluation. In our view, InF-SH and InF-DH can adequately reflect the variety of environments for the purpose of positioning evaluation.

Prioritize the use cases of indoor smart factory for the study of AI/ML based positioning enhancements.

The sub use cases include indoor factory floor (a) with sparse clutter and (b) with dense clutter.

## Deprioritized Use Cases

In our view, the study should deprioritize positioning UE in outdoor with high speed, e.g., automotive or railway use cases. For outdoor UEs, it’s likely that GNSS signal is hearable to the UE, and a UE equipped with GNSS receiver can be located with adequate positioning accuracy. Thus there is less urgency of resorting to AI/ML for improved positioning accuracy for outdoor UEs.

Deprioritize general outdoor commercial use cases for the study of AI/ML based positioning enhancements.

# Target Requirements

In TS 22.261, requirements of high-accuracy positioning for 5G system are provided, with service level 5-7 requiring sub-meter level horizontal accuracy. For verticals, TS 22.104 provides the positioning requirements for IIoT.

In our view, two demanding IIoT service levels can be selected to evaluate the capabilities of AI/ML based positioning. In Table 1, service level 5 and 6 are selected as performance targets. Service level 7 imposes <20cm accuracy for vertical as well as horizontal, thus not used. It’s noted that, in Table 1, physical layer latency target is derived as half of the E2E latency values. Positioning latency can be an optional performance metric to report.

The two scenarios provide two distinct level of positioning requirements, with (B) much more demanding than (A). This allows the evaluation to check which level of requirements AI/ML may be able to achieve, and the associated metrics, including UE position estimation delay, memory storage, and computation complexity. Compared to the Rel-17 study in TR38.857, (A) and (B) are more demanding due to the 99% and 99.9% availability. In contrast, 90% was used in TR38.857 as a performance target.

Table 1. Two IIoT scenarios for the evaluation of AI/ML based positioning enhancements.

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | E2E latency for position estimation of UE | Physical layer latency for position estimation | UE speed | Exemplary use case |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (A) | < 50 cm | < 3 m | 99 % | 1 s | < 500 ms | < 30 km/h | Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) |
| (B) | < 30 cm | < 3 m | 99.9 % | 10 ms | < 5 ms | < 30 km/h | Inbound logistics for manufacturing |

While some services use relative position accuracy (e.g., service level 7 in Table 7.3.2.2-1 of TS 22.261 in Appendix A), the evaluation study should focus on absolute accury only. Specifically, for Scenario (A) and (B) in Table 1, the horizontal and vertical accuracy requirements are defined for absolute UE position, not relative UE position. It is also observed that, vertical accuracy requirement (<3m) is intended to differentiate UEs located at different floors of a building, thus much more relaxed than horizontal accuracy requirement. Correspondingly, the evaluation should focus on achieving high horizontal accuracy (<1m), not high vertical accuracy.

The target performance requirements include sub-meter level horizontal accuracy and >=99% availability.

It is observed that the positioning requirements include other KPI than horizontal-vertical accuracy, for example, the latency of position estimation. Thus the AI/ML performance evaluation should consider aspects other than improved horizontal-vertical accuracy. The benefit of AI/ML can be reflected in the following manner while the horizontal-vertical accuracy is maintained:

- Improved radio resource efficiency via reduced reference signal overhead, e.g., PRS for DL, SRS for UL

- Reduced higher layer signaling overhead (e.g., RRC, LPP) between UE and network;

- Reduced higher layer signaling overhead (e.g., NRPPa) between gNB and LF;

- More robust estimation under varying channel conditions;

Positioning accuracy enhancements can be reflected in metrics other than sharper horizontal-vertical accuracy, e.g., improved radio resource efficiency, reduced higher layer signalling overhead, improved robustness.

# Channel Models and Simulation Assumptions

Indoor factory model (InF) is defined in TR 38.901. Two representative types were used in positioning study in TR 38.857 for FR1 and FR2:

- InF-SH, Indoor Factory with Sparse clutter and High base station height

- InF-DH, which reflects the Indoor Factory with Dense clutter and High base station height.

For Rel-18 AI/ML enabled positioning, it is suggested to reuse the simulation parameters in Table 6.1-1 of TR38.857, which is also copied in the Appendix.

Reuse the simulation parameters in Table 6.1-1 of TR38.857 for performance evaluation.

# Evaluation Methodology

## Baseline performance for comparison

NR already supports a variety of NR positioning methods, including multi-RTT, DL-TDOA, UL-TDOA methods. These existing methods provide baseline performance to compare and demonstrate the potential improvement from AI/ML model. In the evaluation, the existing methods may include the optional enhancements adopted in Rel-17 (e.g., LoS/NLoS indicator, TEG).

The baseline performance to compare and demonstrate the potential AI/ML gain are the existing NR positioning methods including Rel-17 enhancements.

## Data set and training

For the purpose of evaluation, it is proposed that synthetic data set generated according to the InF channel models in TR38.901 is used for model training, validation, and testing.

Synthetic data set generated according to the InF channel models in TR38.901 is used for model training, validation, and testing.

Regarding online vs offline training, offline training is assumed. No real-time online training is expected at least for the purpose of evaluation.

## UE-based and gNB-based positioning

Both UE-side and network-side AI/ML model should be studied. For UE-side, the target UE uses AI/ML inference to obtain its position or to improve the accuracy of measurements. For network side, the gNB can use AI/ML to improve the accuracy of measurement quantity. Also, the network node, LMF, can use AI/ML inference to estimate the location of the target UE.

In terms of collaboration level, it is the simplest to implement one sided AI/ML without collaboration (i.e., implementation based AI/ML; no change to air interface compared to existing Uu). On the other hand, it is more relevant to RAN1 standardization to evaluate AI/ML models with certain level of collaboration, e.g., an AI/ML model residing on one side is provided with assistance information from the other side.  The evaluation work should include various collaboration levels. Each participating company should report the detailed assistance information if UE-network collaboration is applied.

The study considers both UE-side and network-side AI/ML for positioning enhancement.

The study considers both UE-based and network-based position estimation.

The study considers various collaboration levels between UE and network, including “no collaboration” as a comparison.

Collaboration details (e.g., the assistance information details) of the evaluated AI/ML model is reported by participating companies.

## Input of the ML model

While it is known that non-RAT input (if available) can be incorporated to complement RAT input to achieve higher positioning availability and positioning accuracy, RAT-independent signals shall be excluded from the scope of this study item. That is, the input to the AI/ML model does not include non-RAT signals or measurements for improving the positioning accuracy, e.g., LTE, GNSS, sensor, camera, WiFi, Bluetooth, etc. This is due to the practical consideration of running simulation using NR signals, and the need to support a common UE which is not assumed to have special equipment.

The study only considers measurement of NR signals as input to the AI/ML model for positioning enhancement.

## ML model reporting

In addition to reporting evaluation results (e.g., position accuracy), we propose that AI/ML model and related procedure should be shared by the participating companies.

Along with evaluation results, the participating company should provide sufficient details about the experiment (e.g., data generation, feature extraction, AI/ML model design, training, validation, and testing) so that the whole experiment can be reproduced by other interested parties.

Specifically, the following details would typically need to be reported:

- A high-level description of the AI/ML model architecture (e.g., CNN structure). Sufficient details should be provided so that others can re-construct the AI/ML model.

- Simulation parameters used to generate synthetic datasets for training, validation, and testing. Sufficient detail should be provided to enable others to reproduce and align the most important characteristics of the dataset(s).

- Description of data preprocessing / feature extraction steps which are applied in generating the input to the ML model.

- Loss functions with detailed description. For example, regularization terms in the loss function.

Optionally, companies can share useful hyperparameters including, for example, the learning rate, learning rate decay, mini-batch size, optimizer details (e.g., ADAM), and any training techniques (such as dropout regularization). The latter may, for example, be important when studying AI/ML model generalizability.

# Key Performance Indicators

The Key Performance Indicators (KPI) or positioning metrics include:

- Horizontal accuracy. The percentile of positioning errors can be analyzed at: 50%, 80%, 90%, 99%.

  - (Optional) Vertical accuracy can be optionally evaluated and reported. Since vertical accuracy target is <3m for the scenarios (A) and (B), it’s not considered the bottleneck for positioning performance.

- Resource utilization. The time/frequency resources consumed by PRS and/or SRS is an important metric. For example, benefits of ML-based positioning can be demonstrated by achieving the same positioning accuracy as legacy methods, but consuming less time/frequency resources.

  - (Optional) Higher layer signaling overhead can be optionally evaluated. The signaling overhead include signaling between UE and gNB (e.g., RRC signalling), and signalling between UE/gNB and LMF.

  - (Optional) The amount of measurement required to reach a certain positioning accuracy can be estimated and reported, for example, if the AI/ML model has the benefit of reduced UE measurement. Less measurement leads to UE power saving, which is an important metric for UE performance.

- Complexity of the ML model. This includes the number of parameters that define the model (thus memory requirement), and computation complexity.

- Model generalization. An ML model that is valid over a range of deployment scenarios is much preferred than an ML model that is customized to a very specific deployment scenario. Hence, the description of the training set used during evaluations needs to be specified in order to generate comparable results. For example, the number of samples assumed in the training and also potential mix of scenarios while training. Mix of scenarios is important when quantifying the generalization performance of an ML-model. Similarly, another aspect on model generalization is whether it is trained for a specific cell, or valid for multiple cells, or all cells in the deployment. It can also be seen as the validity area of the model. Here the deployment scenario include: clutter distribution in the factory hall, gNB height, inter-site distance, gNB antenna configuration, UE antenna configuration, frequency range, SCS, PRS bandwidth, SRS bandwidth, etc.

Among the listed aspects above, companies are expected to report the underlined metrics, while other metrics are optional or qualitative in nature.

For evaluation results, participating companies are encouraged to report at least: (a) horizontal accuracy; (b) resource utilization; (c) complexity of the ML model; (d) model generalization.

# Candidate Functionalities Enabled by AI/ML

A variety of positioning related functionalities can be fulfilled by AI/ML models.  In general, the AI/ML model is designed to replace a traditional algorithm or functionality used for positioning.

## LOS/NLOS identification

Determining of a positioning related measurement is based on a line of sight (LOS) or non line of sight (NLOS) link is important for the quality of the positioning estimation step. Improving the LOS/NLOS identification will improve also the positioning accuracy. This is one area that could be studied and evaluated in the study item phase.

It is noted that legacy reporting exists in NR already, see the IE in 37.355 v17.0.0 below. Thus AI/ML enabled LOS/NLOS identification can be easily applied with little protocol change.

37.355 v17.0.0

–	LOS-NLOS-Indicator

The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver.

-- ASN1START

LOS-NLOS-Indicator-r17 ::= SEQUENCE {

indicator-r17			CHOICE {

soft-r17				INTEGER (0..10),

hard-r17				BOOLEAN,

...

},

...

}

-- ASN1STOP

LOS-NLOS-Indicator field descriptions

Indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver and has the following choices: - soft: This field specifies the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS. The value provides an estimate of the propability for a LOS propagation path between source and receiver. Scale factor 0.1; range 0 to 1. - hard: This field specifies whether the propagation path between source and receiver is estimated to be LOS (true) or NLOS (false).

## Timing estimation of detected paths

An important category of measurement that positioning methods rely on is timing detection, e.g., RSTD (Reference Signal Time Difference), RTT (Round Trip Time), Rx-Tx time difference. Timing based positioning methods use timing estimate reports from the gNB or the UE to calculate the UE position. Methods like multi-RTT, DL-TDOA, UL-TDOA have the potential to achieve sub-meter horizontal accuracy. Thus AI/ML models that can help improve the timing detection of radio paths are valuable for achieving high accuracy. While timing of the first detected path is important, it is helpful to include multipath channel as well, where timing of multiple detected paths are estimated.

In Rel-17, the concept of Timing Error Group (TEG) was introduced to indicate which timing measurements that have the same Tx or Rx timing error (within a certain margin). One area that could be exploited in the study item phase is if AI/ML could be used to either replace the Rel-17 TEG concept or further improve the performance when utilizing TEG.

## Spatial direction estimation

Another category of measurement that positioning methods rely on is spatial direction estimation, including angle-of-arrival (AoA) and angle-of-departure (AoD). Angle based positioning methods such as UL-AoA and DL-AoD use angular estimates from the gNB or UE to estimate the UE position.

A potential area to study is if the accuracy of angular reports can be improved using AI/ML models. On the other hand, in our understanding, angle detection accuracy alone is not adequate for achieving the sub-meter positioning accuracy. Also, the angle detection accuracy highly depends on the antenna configuration at the gNB and the UE.  Thus for the deployment scenario of interest (i.e., indoor factory), it is not productive to improve angle estimation by itself. Rather angle estimation can be used together with other information (e.g., timing detection) to improve positioning accuracy.

## Directly generated target UE position

The candidate functionalities discussed above all target to improve existing measurement reports used for positioning. Improving the accuracy of measurements will then lead to higher accuracy for the final positioning estimation. Another way to utilize AI/ML models is to use them to directly generate the target UE position estimation as the output of ML model, i.e., determine the UE position as an output of the AI/ML inference.

For a UE with inadequate LOS links, finger printing is a promising method to estimate the UE’s location. Finger printing is an exemplary method that directly generate the target UE’s position.

## Summary – functionalities for evaluation

As discussed above, the AI/ML model can be used to fulfil a variety of functionalities, which aim to directly or indirectly improve the position estimation of the target UE. For the evaluation purpose, it is necessary for RAN1 to focus on a small set of functionalities. This allows better concentration of simulation effort among the participating companies, and make it easier to calibrate and cross-check evaluation results among companies.

Based on the discussion above, for positioning evaluation, the most promising functionalities to be fulfilled by AI/ML are:

- LOS/NLOS identification

- Timing estimation of detected paths

Both functionalities can be performed at gNB and UE, thus allowing both gNB-side and UE-side ML models.  Thus it is proposed that RAN1 focuses the evaluation effort for these functionalise. It is up to individual company to decide if they evaluate ML performance for other functionalities (e.g., finger printing).

RAN1 study focuses on the evaluation of positioning enhancement where AI/ML models support these functionalities: LOS/NLOS identification, timing estimate of detected paths.

# Conclusion

In the previous sections we made the following observations:

Observation 1	AI/ML based positioning enhancements are most valuable for IIoT use cases, where existing methods struggle to satisfy the strict positioning accuracy requirements.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Prioritize the use cases of indoor smart factory for the study of AI/ML based positioning enhancements.

Proposal 2	The sub use cases include indoor factory floor (a) with sparse clutter and (b) with dense clutter.

Proposal 3	Deprioritize general outdoor commercial use cases for the study of AI/ML based positioning enhancements.

Proposal 4	The target performance requirements include sub-meter level horizontal accuracy and >=99% availability.

Proposal 5	Positioning accuracy enhancements can be reflected in metrics other than sharper horizontal-vertical accuracy, e.g., improved radio resource efficiency, reduced higher layer signalling overhead, improved robustness.

Proposal 6	Reuse the simulation parameters in Table 6.1-1 of TR38.857 for performance evaluation.

Proposal 7	The baseline performance to compare and demonstrate the potential AI/ML gain are the existing NR positioning methods including Rel-17 enhancements.

Proposal 8	Synthetic data set generated according to the InF channel models in TR38.901 is used for model training, validation, and testing.

Proposal 9	The study considers both UE-side and network-side AI/ML for positioning enhancement.

Proposal 10	The study considers both UE-based and network-based position estimation.

Proposal 11	The study considers various collaboration levels between UE and network, including “no collaboration” as a comparison.

Proposal 12	Collaboration details (e.g., the assistance information details) of the evaluated AI/ML model is reported by participating companies.

Proposal 13	The study only considers measurement of NR signals as input to the AI/ML model for positioning enhancement.

Proposal 14	Along with evaluation results, the participating company should provide sufficient details about the experiment (e.g., data generation, feature extraction, AI/ML model design, training, validation, and testing) so that the whole experiment can be reproduced by other interested parties.

Proposal 15	For evaluation results, participating companies are encouraged to report at least: (a) horizontal accuracy; (b) resource utilization; (c) complexity of the ML model; (d) model generalization.

Proposal 16	RAN1 study focuses on the evaluation of positioning enhancement where AI/ML models support these functionalities: LOS/NLOS identification, timing estimate of detected paths.

# References

RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” 3GPP RAN#94e, December 2021.

3GPP TS 22.261 V18.6.0 (2022-03), “Service requirements for the 5G system.”

3GPP TS 22.104 V18.3.0 (2021-12), “Service requirements for cyber-physical control applications in vertical domains.”

3GPP TR 38.901 V16.1.0 (2019-12), “Study on channel model for frequencies from 0.5 to 100 GHz.”

3GPP TR 38.857 V17.0.0 (2021-03), “Study on NR Positioning Enhancements.”

# Appendix A. Rel-18 Positioning Performance Requirements

TS 22.261 V18.6.0 (2022-03):

7.3	High-accuracy positioning

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

TS 22.104 V18.3.0 (2022-03):

5.7	Positioning performance requirements

Table 5.7.1-1: Positioning performance requirements

| Scenario | Horizontal accuracy | Vertical accuracy | Availability | Heading | Latency for position estimation of UE | UE speed | Corresponding Positioning Service Level in TS 22.261 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mobile control panels with safety functions (non-danger zones) | < 5 m | < 3 m | 90 % | n/a | < 5 s | n/a | Service Level 2 |
| Process automation – plant asset management | < 1 m | < 3 m | 90 % | n/a | < 2 s | < 30 km/h | Service Level 3 |
| Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1 m (relative positioning) | n/a | 99 % | n/a | 1 s | < 30 km/h | Service Level 3 |
| Augmented reality in smart factories | < 1 m | < 3 m | 99 % | < 0.17 rad | < 15 ms | < 10 km/h | Service Level 4 |
| Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | < 3 m | 99.9 % | < 0.54 rad | < 1 s | n/a | Service Level 4 |
| Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes) | < 50 cm | < 3 m | 99 % | n/a | 1 s | < 30 km/h | Service Level 5 |
| Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | < 3 m | 99.9 % | n/a | 10 ms | < 30 km/h | Service Level 6 |
| Inbound logistics for manufacturing (for storage of goods) | < 20 cm | < 20 cm | 99 % | n/a | < 1 s | < 30 km/h | Service Level 7 |

# Appendix B. Simulation Assumptions in TR 38.857 for IIoT Use Case

Table 6.1-1: Parameters common to InF scenarios

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be - (baseline) at least the convex hull of the horizontal BS deployment. - (optional) It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: - Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height - (Optional): {40%, 3m, 5m} - (Optional): {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |