3GPP TSG RAN WG1 Meeting #109-e	 R1-2203287

e-Meeting, May 9th – May 20th, 2022

Agenda item:		8.12.1

Source: 				Nokia, Nokia Shanghai Bell

Title: 	Remaining Issues on Reliability Improvements for RRC_CONNECTED UEs supporting MBS

Document for: 	Discussion and Decision

# Introduction

Discussions on the MBS WID have been made since several RAN1 meetings, and agreements were made regarding reliability improvements [1][2].

In this document, we provide explanations, observations and proposals for the open issues and items left FFS.

In section 2, we discuss open issues regarding multiplexing multiple NACK-only feedback and multiplexing HARQ-ACK feedback with other UCIs. In addition, we express our perspective on HARQ-ACK Type-1 codebook design. Moreover, we provide our view on the details of HARQ-ACK feedback for SPS.

In section 3, we conclude the document by presenting the summary of the main ideas.

# Improvements on Reliability Mechanisms for Multicast Transmission

## HARQ-ACK Multiplexing/Codebook

In the following sections, we express our view on open issues regarding multiplexing multicast HARQ-ACK feedback with other UCIs on PUCCH and PUSCH, in addition to details regarding Type-1 HARQ-ACK codebook.

### Multiplexing of Multiple NACK-only Feedback

Regarding the capacity problem of NACK-only feedback, the following agreements were made in the last meeting, where the open issues are highlighted with red:

Agreement

For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements):

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - FFS: how to determine PUCCH resource

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast.

    - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID.

      - FFS mapping details.

      - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook.

    - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs

  - Alt4 is not supported for more than 4 TBs

- FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR

- FFS: UE capability

Agreement

Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

Regarding the PUCCH resource to be used for Alt. 1, to transmit the multiplexed HARQ-ACK feedback, the UE should be configured with specific PUCCH resource(s) that is/are distinct from other NACK-only resources. If such kind of PUCCH resource(s) is/are not configured to transmit multiplexed HARQ feedback, since different UEs may be receiving different MBS services, and unicast PUCCH configurations would be different for different UEs, this would increase the complexity of the gNB scheduling orthogonal resources for the multiplexed HARQ feedback, i.e., the last PRI in the DCIs cannot be used for all the UEs to select an optimal PUCCH resource to transmit a multiplexed HARQ-ACK feedback (which can include HARQ feedback of different MBSs at each UE).

Observation 1: For Alt. 1, since different UEs may be receiving different MBS services, and unicast PUCCH configurations would be different for different UEs, it would introduce high complexity to the gNB to schedule orthogonal resources for the multiplexed HARQ feedback, i.e., the last PRI in the DCIs cannot be used for all the UEs to select an optimal PUCCH resource to transmit a multiplexed HARQ-ACK feedback (which can include HARQ feedback of different MBSs at each UE).

In addition, NACK-only PUCCH-config would only include PUCCH resources of type 0 and 1 that do not allow the UE to transmit more than 2 feedback bits. Therefore, PUCCH resource(s) that are dedicated to multiplexed NACK-only feedback should be configured to the UE (in a pre-configured manner, such as RRC signalling). Those resources can be located within the NACK-only or unicast PUCCH-config.

Proposal 1: For Alt. 1, to transmit the multiplexed HARQ-ACK feedback, the UE is configured by RRC signalling with specific dedicated PUCCH resource(s) that the UE utilizes to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI.

- Dedicated PUCCH resource(s) can be configured within the NACK-only or unicast PUCCH-config.

Figure 1 illustrates the necessary change needed in the NACK-only PUCCH-config.

Figure 1 Example PUCCH-config for NACK-only without / with PUCCH resource for multiplexed feedback.

Furthermore, after converting the NACK-only bits to ACK/NACK, concatenation can be based on the PUCCH resource IDs that are originally scheduled for NACK-only feedback. Compared to G-RNTI based concatenation method of ACK/NACK, the mechanism would introduce less complexity to the UE, since the HARQ bits need not to be mapped to RNTIs.

Proposal 2: For Alt. 1., to transmit the multiplexed HARQ-ACK feedback, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback.

Regarding Alt. 4, the UE should be configured with a specific mapping between the decoding results and PUCCH resource IDs to utilize. Naturally, the UE should only configure the TBs, whose NACK-only feedback is scheduled for that slot. Therefore, the FFS regarding k1 value in the “last DCI” does not make much sense.

In our view, mapping between NACK-only sequences to PUCCH resource IDs can be made per G-RNTI, since in Alt. 4, it is not feasible to have feedback for multiple G-RNTIs in the same slot, which would require all the UEs receiving one MBS scheduled with a G-RNTI to receive also the other MBS scheduled with the other G-RNTI(s).

Regarding whether Alt. 4 applies to a single G-RNTI or multiple G-RNTIs: if we agree on a single G-RNTI, then the UE can be scheduled to provide NACK-only feedback for different G-RNTIs only at different slots. Otherwise, if we agree on multiple G-RNTIs, then a similar restriction naturally arises as follows:

If one UE receives G-RNTI1 and G-RNTI2, and another UE receives G-RNTI2 and G-RNTI3. In that case, the UEs cannot be scheduled to provide feedback for all G-RNTIs in the same slot, because received TBs to PUCCH resources mapping would be different. This means, anyway there should be time multiplexing between PUCCH transmissions for different G-RNTIs.

Proposal 3: In Alt. 4, the UE is configured with a specific mapping between the NACK-only sequences of the decoding results of the scheduled TBs and PUCCH resource IDs to utilize per G-RNTI using RRC signalling.

In addition, the following was proposed regarding how a synchronization between the UE and the gNB can be achieved on the size of the multiple NACK-only feedback, without ensuing agreements:

Proposal 3.4.5

For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4),

- based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI.

  - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback.

  - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only.

  - Note3: Whether Alt4 is applied to a single G-RNTI or all configured G-RNTI can still be discussed further.

- FFS: based on the k1 set and TDRA table configured by higher layer signaling,

  - Note4: It is similar to Type-1 codebook size determination for ACK/NACK based feedback.

  - The number of HARQ-ACK bits consists of all G-RNTIs configured with NACK-only based feedback.

As mentioned by several other companies during the last meeting and indicated by Note2 above, DL DAI for Type-2 codebook is separately counted per G-RNTI for ACK/NACK feedback. This should also be the approach here to keep a unified framework between NACK-only and ACK/NACK.

When Alt.1 is used, then the UE can behave as in ACK/NACK feedback, such that the UE creates two (or more in case of more G-RNTIs) sub-codebooks based on the corresponding C-DAIs and concatenate them.

When Alt.4 is used, we have explained above that the UE anyway cannot be scheduled to provide feedback for multiple G-RNTIs in the same slot. Thus, there is no need for the summation of the last DL DAI values for each G-RNTI.

In addition, the goal of NACK-only feedback is to save PUCCH overhead. Therefore, the usage of Type-1-like behavior (as in FFS) should be avoided, as Type-1 codebook aims to be as reliable as possible with maximum redundancy.

Observation 2: The goal of NACK-only based HARQ feedback is to save PUCCH overhead, whereas, Type-1 codebook aims to be as reliable as possible with maximum amount of redundancy.

Proposal 4: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt. 1 or Alt. 4),

- The number of NACK-only bits to be transmitted is based on C-DAI included in the scheduling DL DCI.

  - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback.

  - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only.

  - Note3: For Alt. 1, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI, as for ACK/NACK feedback.

### Multiplexing of NACK-only Feedback with SR

In the last meeting, there were several rounds of discussions for multiplexing NACK-only feedback with SR without any ensuing agreements. The following proposals were made:

Proposal 3.2.1.4-1 (GTW)

When PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR for the same priority, support the following:

  - For up to two HARQ-ACK bits for NACK-only feedback is transformed into ACK/NACK HARQ bits and is multiplexed with SR as legacy procedure of Rel-15/16.

    - Alt1-1: PUCCH resources configured for unicast will be used.

      - Note: either the PUCCH resource for SR or for ACK/NACK is used per the legacy procedure of Rel-15/16.

      - the PUCCH resource is determined by the PRI in the last DCI for multicast;

    - Alt1-2: the PUCCH resource is determined by RRC configuration;

    - Alt1-3: the PUCCH resource for NACK-only feedback transmission is SR resource.

  - For the case of more than two TBs of NACK-only, FFS which alternative from the following is supported:

    - Alt2-1: SR is dropped

    - Alt2-2: NACK-only feedback is transformed into ACK/NACK HARQ bits and PF 2/3/4 is used for the transmission per the legacy SR and HARQ-ACK multiplexing rule of Rel-15/16.

    - Alt2-3: NACK-only is dropped

Proposal 3.2.1.4-2 (if proposal 3.2.1.4-1 is not agreeable)

Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17,

- It is up to UE implementation when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

We have been extensively discussing how multiplexing SR and NACK-only feedback would work since several meetings. However, even in the last meeting there were several alternatives and different companies were raising issues for each of those. In our view, NACK-only feedback would rarely occur together with an SR. Thus, at this late stage, we propose to agree that multiplexing of NACK-only feedback with SR is not supported in Rel-17. As indicated in the above proposal, the UE can decide either to transmit SR or the NACK-only feedback.

Proposal 5: Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17,

- It is up to UE implementation whether to drop SR or NACK-only feedback, when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

### Multiplexing NACK-only Feedback with Unicast/Multicast ACK/NACK Feedback

In the last meeting, the following agreement was made regarding multiplexing NACK-only feedback with ACK/NACK based feedback of unicast or multicast:

Agreement

For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the following Alt2 (from the previous agreement) is adopted:

- Alt1: the converted NACK-only bits are concatenated to the ACK/NACK feedback.

- Alt2: the codebook construction/concatenation is the same as when the feedback mode is ‘ACK/NACK’ for both G-RNTIs.

Based on the agreement, the codebook construction and concatenation are the same as when the feedback mode is ACK/NACK. However, in case of ACK/NACK feedback, there are several options on how to create a codebook based on the type of the HARQ-ACK codebooks configured for unicast and multicast. For example, if Type-1 codebook is configured for multicast and different G-RNTIs are to be ACK/NACK-ed in the same slot, the UE creates a unified Type-1 codebook. Whereas the behavior is different for Type-2, in which case the UE creates separate sub-codebooks and concatenates those. Thus, RAN1 should decide how Alt. 2 would work.

In our view, there can be several options:

- The UE constructs Type-2 kind of codebook for the NACK-only feedback, as RAN1 is most likely to agree on DL DAI-based Type-2 like solution for NACK-only scheduling. If this mechanism is agreed, then the concatenation with the other UCI (ACK/NACK based HARQ-ACK for multicast or unicast) can follow the already agreed mechanisms for ACK/NACK feedback.

- The UE is configured with type of codebook to be created for NACK-only feedback for such an occasion.

- The UE constructs the NACK-only codebook same way with ACK/NACK based unicast or multicast codebook.

To us, the option which would require minimum standardization effort and UE impact is (1).

Moreover, the UE should also follow the same behavior, when multiple NACK-only feedback is scheduled to be transmitted with other unicast UCI(s) or PUSCH, as the previous agreement was only covering a single NACK-only feedback case.

Proposal 6: For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the UE constructs Type-2-like codebook for the NACK-only feedback based on DL DAI and the concatenation follows the already agreed mechanisms for ACK/NACK multicast feedback.

Proposal 7: For multiplexing NACK-only feedback with CSI of unicast or PUSCH transmission, the UE constructs Type-2-like codebook for the NACK-only feedback based on DL DAI and follows the mechanisms defined for multicast ACK/NACK feedback.

### Multiplexing Multicast HARQ-ACK with PUSCH Transmission

The following agreement was made regarding how UL DAI is configured for Type-2 codebook for unicast and multicast that are scheduled in the sane PUSCH transmission:

Agreement

If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted:

- Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast.

    - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs.

        - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

          - Alt1-1: UL-DAI indicates the sum of DL-DAIs

          - Alt1-2: DL-DAIs have the same value.

          - Alt1-3: largest DL DAI value among the configured G-RNTIs.

          - Other alternatives are not precluded

In our view, Alt1-1 would create ambiguities in the size of the codebook, as only 2-bits can be used as the counter. For example, when 3 different G-RNTIs are to be multiplexed, if the summation is made over the last received DL DAI values for each G-RNTI, the UL DAI field would be in range [0-3], which could represent, [0-3], [4-7], [8-11] or [12-15]. In case the UE misses some DCIs, it may be hard for the UE to distinguish between these different intervals, which would lead to a size mismatch between the UE and the gNB. Also, we cannot understand the intention of Alt1-2, which would counteract Type-2 codebook scheduling.

Therefore, we prefer Alt1-3. The gNB can use the largest DL DAI value among the last received DCIs for each G-RNTI as the UL DAI value, so that the UE can create sub-codebooks for each G-RNTI with the size indicated as the UL DAI. In case the number of received TBs is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook.

Proposal 8: If Type-2 codebook is configured for unicast and multicast, 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast:

- The 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs.

- UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI.

- In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly.

The following agreement was made regarding how UL DAI is configured for the case where codebook for unicast and multicast are configured differently and they are scheduled to be multiplexed in the same PUSCH transmission:

Agreement

When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast.

- The UL-DAI for multicast is 1-bit for Type-1 codebook

    - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs.

- The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs.

    - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs.

      - Alt1-1: UL-DAI indicates the sum of DL-DAIs

      - Alt1-2: DL-DAIs have the same value.

      - Alt1-3: largest DL DAI value among the configured G-RNTIs.

      - Other alternatives are not precluded

  - FFS: details of the UL DAI field

For the FFS, based on our understanding, same mechanism with the previously discussed operation, where both unicast and multicast are configured with Type-2 codebooks should apply due to the aforementioned reasoning.

Proposal 9: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, the 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs.

- UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI.

- In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly

Moreover, one issue still remaining is how the UL DAI value is set, when the NACK-only feedback is configured for different G-RNTIs that are to be multiplexed with PUSCH transmission. We expect that Type-2-like behavior will be agreed also for NACK-only, where the DL DAI is used to indicate the number of NACK-only bits to be created. Therefore, a similar mechanism could easily be adopted, such that the parts for multicast Type-2 codebook of the previous agreements are agreed also for NACK-only feedback.

Proposal 10: When NACK-only feedback is to be multiplexed in PUSCH (with and without unicast HARQ-ACK codebook), the same UL DAI mechanisms that were previously agreed for the case of ACK/NACK based feedback with Type-2 codebook are followed by the UE.

### Multiplexing Multicast HARQ-ACK with Type-3 and Enhanced Type-3 Unicast HARQ-ACK

The following proposals were made in the last meeting on multiplexing multicast HARQ-ACK with Type-3 and enhanced Type-3 unicast codebooks:

Proposal 2.1.5-1

When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes including the ones used for multicast.

Proposal 2.1.5-2

When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide enhanced Type-3 codebook of smaller size in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

In our understanding, Type-3 codebook contains feedback for all HARQ processes that the UE is configured with. Also, same HARQ processes can be used by the gNB for unicast and multicast, e.g., UE can be scheduled for unicast data using HARQ process ID 1 and then for multicast data using the same HARQ process later on. Therefore, if the UE reports Type-3 codebook for unicast, it does not need to add an additional multicast codebook, since it reports feedback for all the HARQ processes.

- Therefore, RAN1 should agree on the proposal made by the feature lead in the previous meeting:

Proposal 11: When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes including the ones used for multicast.

- Regarding enhanced Type-3 codebook, it was mentioned by the feature lead that a similar agreement exists in Rel-17 URLLC WI. In our view, as agreed for enhanced Type-2 codebook for unicast, the UE could also generate separate sub-codebooks for unicast and multicast respectively and append the multicast HARQ-ACK sub-codebook to the unicast HARQ-ACK sub-codebook. However, we will also not oppose the existing proposal for the sake of progress.

Proposal 12: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide enhanced Type-3 codebook of smaller size in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

### Type-1 HARQ-ACK codebook

The following agreement was made in the last meeting for the case where the feedback is disabled for some of the G-RNTIs that the UE receives when configured with Type-1 codebook:

Agreement

For Type-1 codebook generation,

- if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot;

- For other cases, FFS between the 3 alternatives below:

  - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs.

  - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI

  - Other alternatives are not precluded

We prefer the first alternative. In our view, if the feedback for a G-RNTI is disabled by the network at a UE, the network would not need the decoding result from that UE at any time for any purpose. Companies are proposing Alt.2, such that the UE could report the actual decoding result, as the UE anyway produces a Type-1 HARQ-ACK codebook, however, this will add more complexity to UE when constructing the codebook and there would be nearly zero gain on the network side. Regarding Alt. 3, this is not meaningful, as the requirements of different MBSs can be different. So, some services may not even require any feedback from a UE, whereas the others do.

Proposal 13: For Type-1 codebook generation if at least one configured G-RNTI of the UE is with HARQ-ACK enabled, UE reports NACK for the G-RNTI(s) with HARQ-ACK disabled regardless of decoding results for that latter G-RNTI(s).

## SPS

Similar to Section 2.1.6, the following agreement was made in the last meeting for the case of SPS:

Agreement

If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

- FFS between the 4 alternatives below:

  - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

  - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signalling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling.

  - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC.

  - Other alternatives are not precluded.

As in case of dynamic scheduling, we prefer the first alternative. In our view, if the feedback for a G-CS-RNTI is disabled by the network at a UE, the network would not need the decoding result from that UE at any time for any purpose. Alt.2 will add more complexity to UE when constructing the codebook and there would be nearly zero gain on the network side. Alt. 3 is not meaningful, as the requirements of different MBSs can be different. So, some services may not even require any feedback from a UE, whereas the others do. In our view, Alt. 4 is the same with Alt. 1.

Proposal 14: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

  - UE reports NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

In our view, the above proposal shall also apply the case, where the enable/disable condition is set vice-versa:

Proposal 15: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast SPS and is configured with disabled HARQ feedback for dynamic multicast scheduling, for Type-1 codebook generation,

  - UE reports NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

# Conclusions

In this contribution, we have discussed various aspects of this WI. From those discussions we have the following observations:

Observation 1: For Alt. 1, since different UEs may be receiving different MBS services, and unicast PUCCH configurations would be different for different UEs, it would introduce high complexity to the gNB to schedule orthogonal resources for the multiplexed HARQ feedback, i.e., the last PRI in the DCIs cannot be used for all the UEs to select an optimal PUCCH resource to transmit a multiplexed HARQ-ACK feedback (which can include HARQ feedback of different MBSs at each UE).

Observation 2: The goal of NACK-only based HARQ feedback is to save PUCCH overhead, whereas, Type-1 codebook aims to be as reliable as possible with maximum amount of redundancy.

According to those observations we have the following proposals:

Proposal 1: For Alt. 1, to transmit the multiplexed HARQ-ACK feedback, the UE is configured by RRC signalling with specific dedicated PUCCH resource(s) that the UE utilizes to transmit multiplexed NACK-only bits, rather than selecting the PUCCH resource based on the PRI field of the last DCI.

- Dedicated PUCCH resource(s) can be configured within the NACK-only or unicast PUCCH-config.

Proposal 2: For Alt. 1., to transmit the multiplexed HARQ-ACK feedback, concatenation of the converted NACK-only bits is based on the ascending order of PUCCH resource IDs that are originally scheduled for NACK-only feedback.

Proposal 3: In Alt. 4, the UE is configured with a specific mapping between the NACK-only sequences of the decoding results of the scheduled TBs and PUCCH resource IDs to utilize per G-RNTI using RRC signalling.

Proposal 4: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt. 1 or Alt. 4),

- The number of NACK-only bits to be transmitted is based on C-DAI included in the scheduling DL DCI.

  - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback.

  - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only.

  - Note3: For Alt. 1, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI, as for ACK/NACK feedback.

Proposal 5: Multiplexing NACK-only based feedback for multicast with SR is not supported in Rel-17,

- It is up to UE implementation whether to drop SR or NACK-only feedback, when the PUCCH transmission for the NACK-only based feedback for multicast collides with PUCCH transmissions for SR.

Proposal 6: For multiplexing NACK-only feedback for the first G-RNTI with ACK/NACK based feedback for the second G-RNTI or for unicast, the UE constructs Type-2-like codebook for the NACK-only feedback based on DL DAI and the concatenation follows the already agreed mechanisms for ACK/NACK multicast feedback.

Proposal 7: For multiplexing NACK-only feedback with CSI of unicast or PUSCH transmission, the UE constructs Type-2-like codebook for the NACK-only feedback based on DL DAI and follows the mechanisms defined for multicast ACK/NACK feedback.

Proposal 8: If Type-2 codebook is configured for unicast and multicast, 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast:

- The 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs.

- UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI.

- In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly.

Proposal 9: When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, the 2-bit UL DAI for multicast that is set to largest DL DAI value among the configured G-RNTIs is applied for all configured G-RNTIs.

- UE creates sub-codebooks per each G-RNTI with the size indicated as the UL DAI.

- In case the largest received DL DAI is less than the indicated UL DAI value for some G-RNTIs, the UE could pad the corresponding sub-codebook, accordingly

Proposal 10: When NACK-only feedback is to be multiplexed in PUSCH (with and without unicast HARQ-ACK codebook), the same UL DAI mechanisms that were previously agreed for the case of ACK/NACK based feedback with Type-2 codebook are followed by the UE.

Proposal 11: When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes including the ones used for multicast.

Proposal 12: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide enhanced Type-3 codebook of smaller size in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

Proposal 13: For Type-1 codebook generation if at least one configured G-RNTI of the UE is with HARQ-ACK enabled, UE reports NACK for the G-RNTI(s) with HARQ-ACK disabled regardless of decoding results for that latter G-RNTI(s).

Proposal 14: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation,

  - UE reports NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

Proposal 15: If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast SPS and is configured with disabled HARQ feedback for dynamic multicast scheduling, for Type-1 codebook generation,

  - UE reports NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

# References

- 3GPP Tdoc R1-2200810 “Agreements for NR MBS up to RAN1#107bis Rapporteur (CMCC)”.

- 3GPP Tdoc RP-220407 “Status report for WI NR Multicast and Broadcast Services, Huawei, HiSilicon”.