3GPP TSG RAN WG1 #109-e	R1- 2203289

E-meeting, May 9th – May 20th, 2022

Source:	Panasonic

Title: 	Maintenance on Solutions for NR to support non-terrestrial networks (NTN)

Agenda Item:		8.4

Document for:	Discussion and decision

# Introduction

The work item on Non Terrestrial Networks (NTN) is considered. We discuss remaining issues for the maintenance on Solutions for NR to support non-terrestrial networks (NTN).

# N_TA at Initial access

The following working assumption was proposed for confirmation during RAN1#108-e:

- Working assumption: When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as: - Option 1: NTA=TA⋅16⋅642μ. Where, TA is the TAC field in msg2/msgB

While most companies supported the working assumption, there was an argument that during initial access, a PRACH transmission may arrive earlier than the start of a PRACH occasion and proposed to support negative N_TA values when  NTA is updated at the UE after receiving TAC (T_A) in msg2/msgB to avoid “unnecessarily tight requirement on GNSS accuracy”. The reason stated was, that conclusions for FR1 requirements do not necessarily extend to FR2.

The CTA polynomial and satellite ephemeris can be designed to allow for interpolation of values within desired approximation errors. Also, the network implementation can ensure, that the reception timing of PRACH is offset toward a positive direction. The amount of the offset can include the approximation errors caused by GNSS accuracy. Hence, we do not think that negative N_TA values are necessary.

Proposal 1: Confirm the working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

Where, TA is the TAC field in msg2/msgB

# Maintenance on TACommonDriftVariation value range

The following granularity and value ranges for common TA parameters were agreed during RAN1#107-e.

Agreement

Confirm the Working assumption on granularity and bits allocation for Common TA parameters: Value range, granularity, and bits allocation of Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are as follows:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
|  | 0 ...66485757 (i.e: 0… 270.73 ms) |  | 26 bits |
| TACommonDrift | - 261935… + 261935 (i.e: --53.33 … +-53.33 ) |  | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 ) |  | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

However, there was an observation during simulations that TACommonDriftVariation (equivalent to acceleration) would need to become negative due to perturbations such as the Earth’s gravitational field and orbit inclination.  To account for that, the addition of a single sign-bit was proposed, and we are supportive.

Proposal 2: Add 1 bit for supporting negative TACommonDriftVariation values for GEO.

# UE behaviour w.r.t Validity timer expiry

It was agreed during RAN1#106bis-e, that epoch time marks the beginning of the validity duration:

Agreement:

NTN ephemeris validity timer should be started/restarted with configured timer validity duration at the epoch time of the assistance information (i.e. serving satellite ephemeris data)

And, during RAN1#107-e and RAN1#108-e, it was also agreed to provide for implicit and explicit indication of epoch time:

Agreement:

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- Otherwise, when Epoch time is not explicitly indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the NTN-specific SIB SI message is transmitted.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

The normal operation of an NR-NTN system follows then as illustrated in Figure 3 from R1-2202098 (copied below). Here, while a first validity duration is active, UE receives a new SIB-NTN (=SIB19 in the latest draft of TS 38.331) and new assistance information located within the SI-window. UE derives epoch time from the end of the SI-window, at which point in time also the new assistance information takes effect, and the second validity duration starts.

Note that in R1-2202098, additional cases were considered:

- Case 1: New assistance information is not available before expiry of the UL validity timer

- Case 2: New assistance information is available but not within the associated validity duration

The description of "available", "not available" are ambiguous caused by the sender or the receiver. We should distinguish above two cases separately. Case 1a and case 2a are caused by the configuration of the network. Case 1b and 2b are caused by the radio channel conditions.

- Case 1a: New assistance information is not sent before expiry of the UL validity timer

- Case 1b: New assistance information is not received before expiry of the UL validity timer

- Case 2a: New assistance information is sent outside the associated validity duration

- Case 2b: New assistance information is is sent within the associated validity duration, but it is not received.

For case 1a and 2a, these are just badly configured operation. UE can expect such wrong configuration should not happen. The proper network configuration is new assistance information is available before the expiry validity duration given by the previous assistance information. TS 38.331 section 12 describes processing delay requirements for RRC procedures. Although there is no description on SIB reception, the order of the RRC processing delay is 10 to 16 ms. If new assistance information is received just before the expiry of the previous information, during the RRC processing delay of the decoding, old assistance information cannot be used. Therefore, we propose following.

Proposal 3: UE may expect that new assistance information is given by the NTN-specific SIB19 [X] seconds earlier than the expiry validity duration given by the previous assistance information.

- FFS: options for [X] are 1 sec, 100 ms, 10 ms, or the RRC processing delay. Or it can be defined within RAN4.

For case 1b and 2b, in RAN1#106bis-e, “The UE assumes that it has lost uplink synchronization if new or additional assistance information (i.e., serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration. FFS: details on how to acquire new or additional assistance information” was agreed. Therefore, our view is, if the situation of case 1b or 2b happens at UE, it should be same as lost uplink synchronization. In general, UE is not able to identify to lost uplink synchronization, but this case is able to identify by UE because of the UL validity timer and validity duration. We propose not to transmit anything when new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration.

Proposal 4: UE stops the transmission if new or additional assistance information is not received within the associated validity duration.

Figure 3 (from R1-2202098): Case 3: New assistance information is available before expiry of the UL validity timer

# Ambiguity in interpretation SFN indicating Epoch time

When epoch time is explicitly indicated jointly with satellite assistance information, RAN1 agreed that epoch time is the starting time of a DL sub-frame, indicated by an SFN and a sub-frame number. This causes ambiguity due to SFN wrap around and potential SFN difference between cells for epoch time of neighbor/target cell.

## Ambiguity due to SFN wrap around

The system frame number wraps around every 10,24 seconds. An extreme example illustrates the occuring issue. Assume that SIB19 is received during SFN 0 and epoch time is indicated as SFN 1023. RAN1 must decide if SFN 1023 points into the future of the current SFN cylce or into the past of the immediately preceding SFN cycle.

Based on the agreement for implicit signalling, that epoch time is implicitly known as the end of the SI window during which the NTN-specific SIB SI message is transmitted, a natural extension of this approach could be that explicit epoch time is always assumed to be in the future of the SIB-NTN transmission. However, this bears the risk that the delay between SIB-NTN reception and indicated epoch time becomes quite large. In the extreme example above, assistance information would become valid only after 10.24 seconds.

In addition, RAN1 lacks a discussion of when the satellite assistance information becomes valid. We see the following alternatives to deal with epoch time and validity duration:

Alternative 1: New assistance information takes effect at the timing of the epoch time and is valid during the new validity period. In case of explicit indication, the epoch time is always in the future.

In this case, initial access and handover can be delayed because UE needs to wait for sending PRACH until the assistance information being valid.

Alternative 2: New assistance information takes effect at the reception of the assistance information and valid from the timing of the reception until expiration of the new validity timer. In case of explicit indication, the epoch time is always in the future or the nearest timing to the SIB-NTN.

In this case, the validity period varies depending on the assistance information transmission timing because validity timer is semi-static. Because the values of the assistance information are determined such that the sufficient accuracy is kept within the valid period, if the validity duration varies, the accuracy may not be ensured.

Alternative 3: New assistance information takes effect at the timing of the epoch time and is valid during the new validity period. In case of explicit indication, the epoch time is always in the past.

In this case, the validity period may be slightly reduced because the validity period starts at the past timing.

In our understanding, Alternative 3 offers a good compromise. It is implied, that for implicit indication of epoch time, the assistance information in SIB19 would always become valid in the future of SIB19 transmission. And for explicit indication of epoch time, the assistance information in SIB19 or dedicated RRC signaling would always be valid instantly if epoch time is agreed to be in the past of SIB19 or dedicated RRC signaling transmission. The only downside of setting epoch time in the past is that the validity duration may be slightly reduced. We think, that it is acceptable compared to the network access delay for initial access or handover.

Proposal 5: The assistance information carried in SIB19 or dedicated RRC signaling becomes valid at epoch time.

Proposal 6: If indicated explicitly by SFN and subframe number, epoch time t_epoch is in the past when UE reads the SIB19 or dedicated RRC signaling at time t where 𝑡_𝑒𝑝𝑜𝑐ℎ ≤𝑡.

Another issue caused by the SFN wrap around is update frequency of the assistance information by the network. In principle, the network could be configured to embed SIB19 relatively frequently and with large validity periods, e.g., tens or hundreds of seconds, and the assistance information (satellite ephemeris and common timing advance parameters) carried by SIB19 do not need to be updated during the validity period. However, because epoch time needs to be updated at least every 10.24 seconds in order to allow UE to correctly identify the epoch time indicated using SFN which wrap-arounds every 10.24 seconds, a complete and frequent update of the assistance information (satellite ephemeris and common timing advance parameters) is required. Update of satellite ephemeris and common TA parameters requires interaction among satellite, NTN control center and gNB as mentioned in [4]. Even if once per 10.24 seconds, a frequent update could cause an undesired operational complexity especially for GEO where the validity duration is significantly long. This operational complexity should be corrected by adding to SIB19 a counter for the SFN-cycles which have elapsed since the first instance of the SIB-NTN in a given validity period. This enables to repeat the same assistance information in SIB19 during a validity period, which reduces the network complexity, and UEs are no longer subject to the SFN-cycle wrap-around. When the same assistance information in SIB19 is repeated during a validity period, only the SFN-cycle counter is updated, while the assistance information is simply repeated from the first instance. This makes gNBs knows all UEs to operate with an identical set of parameters during a validity period.

The largest validity period currently is 900s, equal to approximately 88 SFN-cycles. Hence a 7-bit counter would be sufficient.

Proposal 7: Add to SIB-NTN a counter with 7 bits for the SFN-cycles which have elapsed since the epoch time in the first instance of the SIB-NTN in each validity period.

## Ambiguity of epoch time of neighbor/target cell for measurement and handover

As mentioned in RAN2 LS related discussion [5], assistance information (i.e. satellite ephemeris and common TA parameters) of neighbor cell would need to be indicated to UE. Following is the copy of the relevant section from [4] for convenience.

For neighbor cell measurement and cell reselection in RRC_IDLE or RRC_INACTICE modes, necessary neighbor cell information for measurement and cell reselection is broadcasted via SIB2. For neighbor cell measurement and handover (HO) in RRC_CONNECTED, necessary information for the measurement and accessing the neighbor cell for HO is indicated via dedicated RRC signaling from the current serving cell and UE is not required to receive SIB of neighbor cell before the handover.

In RAN1, it was agreed that epoch time for satellite ephemeris and common TA parameters will be expressed in terms of system frame number (SFN) and subframe number. Considering that the neighbor cell may be served by different gNBs which can have their own SFN, it would be necessary to clarify which cell’s SFN and subframe number as well as reference point is used as the epoch time of neighbor cell ephemeris and common TA parameters.

Option 1: the epoch time for the neighbor cell is based on the SFN and subframe number in the respective neighbor cell.

In this option, the same contents of the assistance information as broadcasted in the respective neighbor cell can be forwarded from the neighbor gNB and indicate it to the UE as it is. The UE determines the epoch time based on the SFN obtained from the neighbor cell’s MIB. But, the UE may not be able to receive the neighbor cell’s MIB and therefore may not be able to understand the assistance information before receiving the MIB as illustrated in Figure 1. Therefore, one way would be that the serving gNB provides information on the relative SFN to the neighbor cell SFN together with the assistance information of the neighbor cell. The reference point of the epoch time is the reference point (i.e. uplink time synchronization point) for the respective neighbor cell.

Option 2: the epoch time for the neighbor cell is based on the SFN and subframe number in the current serving cell.

In this option, the assistance information of the respective neighbor cell is generated using the serving cell’s SFN and subframe number with the reference point in the serving cell at each gNB or NTN control center. The network complexity can be large because gNB needs to generate the assistance information of multiple satellites.

Proposal 8: Because epoch time is expressed by SFN and subframe number which can be different for the respective gNBs, it is necessary clarify which cell’s SFN and subframe number as well as reference point is used as the indication of the epoch time of the neighbor cell.

- Option 1: the epoch time for the neighbor cell is based on the SFN and subframe number in the respective neighbor cell. gNB provides relative information to the neighbor cell’s SFN.

- Option 2: the epoch time for the neighbor cell is based on the SFN and subframe number in the current serving cell.

Figure 1 Issue on the neighbor cell epoch time due to the SFN difference

# Conclusion

In this contribution, we discussed issues on UL timing and frequency synchronization for NTN. In summary, we propose the following:

Proposal 1: Confirm the working assumption:

When TAC (TA) in msg2/msgB is received, UE receives the first adjustment and NTA is updated as:

- Option 1: NTA=TA⋅16⋅642μ.

Where, TA is the TAC field in msg2/msgB

Proposal 2: Add 1 bit for supporting negative TACommonDriftVariation values for GEO.

Proposal 3: UE may expect that new assistance information is given by the NTN-specific SIB19 [X] seconds earlier than the expiry validity duration given by the previous assistance information.

- FFS: options for [X] are 1 sec, 100 ms, 10 ms, or the RRC processing delay. Or it can be defined within RAN4.

Proposal 4: UE stops the transmission if new or additional assistance information is not received within the associated validity duration.

Proposal 5: The assistance information carried in SIB19 or dedicated RRC signaling becomes valid at epoch time.

Proposal 6: If indicated explicitly by SFN and subframe number, epoch time t_epoch is in the past when UE reads the SIB19 or dedicated RRC signaling at time t where 𝑡_𝑒𝑝𝑜𝑐ℎ ≤𝑡.

Proposal 7: Add to SIB-NTN a counter with 7 bits for the SFN-cycles which have elapsed since the epoch time in the first instance of the SIB-NTN in each validity period.

Proposal 8: Because epoch time is expressed by SFN and subframe number which can be different for the respective gNBs, it is necessary clarify which cell’s SFN and subframe number as well as reference point is used as the indication of the epoch time of the neighbor cell.

- Option 1: the epoch time for the neighbor cell is based on the SFN and subframe number in the respective neighbor cell. gNB provides relative information to the neighbor cell’s SFN.

- Option 2: the epoch time for the neighbor cell is based on the SFN and subframe number in the current serving cell.

# References

- RP-201256, “WID: Solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN)”

- R1-2202908, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”, RAN1#108-e

- R1-2201387, “Enhancements on UL time and frequency synchronization”, Panasonic, RAN1#108-e

- R1-2201640, “Discussion on RAN2 LS on NR NTN Neighbor Cell and Satellite Information”, RAN1#108-e, Panasonic

- Draft TS38.331 https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/Specifications/202203_draft_specs_after_RAN_95