3GPP TSG RAN WG1 #109-e	R1-2203290

e-Meeting, May 9th - 20th, 2022

Title:	Remaining issues on initial access aspects for 52.6 to 71 GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.1

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on maintenance of initial access aspects for Rel-17 NR above 52.6GHz.

Discussion on initial access aspects

  - 2.1 CORESET#0 configuration for 96 RBs

In RAN1 #108 e-meeting, RAN1 has made progress on the CORESET#0 configuration. RAN1 has agreed to use a single table to define the CORESET#0 configuration for all supported SCSs in FR2-2, but there is still a remaining issue to be solved, that is whether/how to define a additional RB-level offset (candidate values {56, 76} if defined) for 96 RBs CORESET#0. The working assumption reached in RAN1 #108 e-meeting is as follows [1].

Working Assumption

- The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2.

  - FFS: whether/how to define X, if defined, candidate values {56, 76}

- Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed

- Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization.

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | X |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | X |
| 12 | 3 | 24 | 2 | -20 if -21 if > 0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if -21 if > 0 |
| 15 | 3 | 48 | 2 | 48 |

Based on LS [2] and the following agreements [3] from RAN4, the CORESET#0 configuration should be defined such that it can support floating channelization for licensed operation with an assumption that GSCN are sub-sampled by 3/12/6 for 120/480/960 kHz cases, respectively.

Agreement (RAN4 #101b-e): Consider the different channelization for licensed band(s) and unlicensed band(s) - Fixed sync raster for unlicensed bands - Fixed scheme should not be constrained by IEEE channel raster - Send LS to RAN1 to make sure that RAN1 accommodates the solution already now for both fixed and floating sync raster. - For the contiguous carrier aggregation, the channel spacing of adjacent channels should be multiple of the larger SCS, i.e., 960KHz, used by two channels/CCs - Floating sync raster for licensed bands - Stick to the agreement last meeting for 3 x 17.28MHz as the minimum granularity. - Refer to gap between adjacent GSCN values is not smaller than 3 x 17.28MHz - FFS: Unlicensed bands tries to use the sub-set of sync raster for licensed bands Agreement(RAN4 #102-e): GSCN for licensed band - GSCN step sizes - 3 for 120 kHz - 12 for 480 kHz - 6 for 960 kHz - Those values will be further checked after agreeing on the SU values. Agreement(RAN4 #102-e): for RF channel raster for licensed band, SCS based channel raster is used for licensed bands, and the step size for channel raster is: - 2 for 120kHz - 8 for 480kHz - 16 for 960kHz

For 96 RBs CORESET#0 configuration, defining offsets for right and left shifts is sufficient and feasible. So in addition to 0, we should also support for 76 RBs offset, which allows a better resource usage of RMSI PDSCH and avoids beam switching for transmitting SSB, CORESET#0 and RMSI PDSCH. While 56 RBs offset will cause a waste in RMSI PDSCH resource allocation.

Proposal 1: For 96 RBs CORESET#0 configuration, defining a additional offset: 76 RBs.

Proposal 2: Adopt TP#1 for 96 RBs CORESET#0 configuration.

TP#1:

Reason for change

Define a additional RB-level offset for 96 RBs CORESET#0

Summary of change

Define a additional offset 76 RBs for 96 RBs CORESET#0 in Table 13-10A

Consequences if not approved

96 RBs CORESET#0 only supports a offset 0 RB. SSB can only be aligned with lower edge of CORESET#0

=========================== Start of TP#1 for TS 38.213 =========================

13	UE procedure for monitoring Type0-PDCCH CSS sets

If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Table 13-10A for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix.

For operation with shared spectrum channel access in FR2-2 and for operation without shared spectrum channel access, a UE assumes that the offset in Tables 13-1 through 13-10A is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10, kSSB is defined in [4, TS 38.211].

< Unchanged parts are omitted >

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | 76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | 76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 13 | 3 | 24 | 2 | -24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 15 | 3 | 48 | 2 | -48 |

< Unchanged parts are omitted >

======================= End of TP#1 for TS 38.213 ============================

Conclusion

In this contribution, we discuss the remaining issues on initial access aspects in NR above 52.6 GHz band operation and have the proposal.

Proposal 1: For 96 RBs CORESET#0 configuration, defining a additional offset: 76 RBs.

Proposal 2: Adopt TP#1 for 96 RBs CORESET#0 configuration.

TP#1:

Reason for change

Define a additional RB-level offset for 96 RBs CORESET#0

Summary of change

Define a additional offset 76 RBs for 96 RBs CORESET#0 in Table 13-10A

Consequences if not approved

96 RBs CORESET#0 only supports a offset 0 RB. SSB can only be aligned with lower edge of CORESET#0

=========================== Start of TP#1 for TS 38.213 =========================

13	UE procedure for monitoring Type0-PDCCH CSS sets

If during cell search a UE determines from MIB that a CORESET for Type0-PDCCH CSS set is present, as described in clause 4.1, the UE determines a number of consecutive resource blocks and a number of consecutive symbols for the CORESET of the Type0-PDCCH CSS set from controlResourceSetZero in pdcch-ConfigSIB1, as described in Tables 13-1 through 13-10, for operation without shared spectrum channel access in FR1 and FR2-1, or as described in Tables 13-1A and 13-4A for operation with shared spectrum channel access in FR1, or as described in Table 13-10A for FR2-2, and determines PDCCH monitoring occasions from searchSpaceZero in pdcch-ConfigSIB1, included in MIB, as described in Tables 13-11 through 13-15. SFNc and nc are the SFN and slot index within a frame of the CORESET based on SCS of the CORESET and SFNSSB,i and nSSB,i are the SFN and slot index based on SCS of the CORESET, respectively, where the SS/PBCH block with index i overlaps in time with system frame SFNSSB,i and slot nSSB,i. The symbols of the CORESET associated with pdcch-ConfigSIB1 in MIB or with searchSpaceSIB1 in PDCCH-ConfigCommon have normal cyclic prefix.

For operation with shared spectrum channel access in FR2-2 and for operation without shared spectrum channel access, a UE assumes that the offset in Tables 13-1 through 13-10A is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10, kSSB is defined in [4, TS 38.211].

< Unchanged parts are omitted >

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | 76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | 76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 13 | 3 | 24 | 2 | -24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 15 | 3 | 48 | 2 | -48 |

< Unchanged parts are omitted >

======================= End of TP#1 for TS 38.213 ============================

Reference

- Draft_Minutes_report_RAN1#108-e_v010, March 2022

- R1-2200894(R4-2200081), LS for the channelization for up to 71 GHz, RAN4

- R4-2206535, WF on system parameters of FR2-2