3GPP TSG RAN WG1 #109-e	R1-2203292

e-Meeting, May 9th - 20th, 2022

Title:	Remaining issues on data channel enhancements for 52.6 to 71GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issues on PUSCH/PDSCH enhancement for Rel-17 above 52.6GHz.

Discussion

Clarification on “scheduled PXSCH”

- Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs?

- Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the [valid] PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

In the email discussion of RAN1 #108 e-meeting, clarification on whether “scheduled PXSCH” in previous agreements implies valid PxSCH or not was discussed [1] and majority companies prefer a common solution for Case 5 and Case 6. For Case 5, if “based on configured SLIVs” is supported, it may cause an incorrect HARQ-ACK information to be generated by Type1 HARQ-ACK codebook with time domain bundling since UE is scheduled by DCI 1 and DC2 to simultaneously receive two PDSCHs in the same slot as shown in Figure 1. However, compared with “based on configured SLIVs”, “based on valid SLIVs” can provide more scheduling flexibility. Further, it can be also used to provide a valid and reasonable rule for out-of-order scheduling and there is no additional impact on the current spec.

Figure 1 Type-1 HARQ-ACK codebook with time domain bundling

For Case 6, we prefer supporting “valid PDSCH(s)/PUSCH(s)” because only valid PDSCH(s)/PUSCH(s) may be actually scheduled and assigned HARQ process by gNB.

Proposal 1: For out-of-order scheduling, the rule for OOO scheduling determined is determined based on valid SLIVs.

Proposal 2:  For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

Type 2 HARQ-ACK codebook

There is a remaining issue about [configured or valid] SLIV to be resolved as follow:

Agreement (RAN1#107-e)

For multi-PDSCH scheduling with a single DCI

- Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell.

  - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell.

  - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell.

  - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method.

    - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured)

    - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case

  - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook.

  - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook,

    - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

The pros and cons of the configured and valid SLIV have been fully discussed in previous meetings. In some cases, valid SLIV can achieve better re-transmission efficiency and HARQ-ACK feedback efficiency. Configured SLIV can simplify the UE implementation. Considering the limited time left in this stage, for type-2 HARQ-ACK codebook with multi-PDSCH scheduling and HARQ-ACK time domain bundling, our preference is configured SLIV which is simple and valid. Therefore, the following working assumption should be confirmed.

- Workding assumption:

The TP#1 below for TS 38.213 section 9.1.3.1 is endorsed.

- TP#1 (Construction of bundling group based on “configured” SLIVs)

If a UE is provided numberOfHARQ-BundlingGroups and is not provided harq-ACK-SpatialBundlingPUCCH for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ−ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions on the serving cell c, the UE generates NHARQ−ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ−ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ−ACKCBG/TB,max=NHARQ−ACK,cTBG,max and C=NPDSCH,c. For a TBG with at least one PDSCH not overlapping with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, the UE assumes that a PDSCH in the TBG is correctly received if the PDSCH overlaps with an UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a TBG that includes only PDSCH(s) overlapping with UL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, if provided, NACK is generated for the TBG.

Proposal 3: Confirm the working assumption and the PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI should be allocated to the bundling groups for type 2 HARQ-ACK codebook.

Conclusion

In this contribution, we discussed the remaining issues on PUSCH/PDSCH enhancement.The following proposals are provided:

Proposal 1: For out-of-order scheduling, the rule for OOO scheduling determined is determined based on valid SLIVs.

Proposal 2:  For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI, only the valid PDSCHs scheduled by the first DCI are considered for definition of the corresponding timeline requirements.

Proposal 3: Confirm the working assumption and the PDSCHs corresponding to configured SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI should be allocated to the bundling groups for type 2 HARQ-ACK codebook.

Reference

- R1-2202339, Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)

- R1-2200263, Remaining issues on the data channel enhancements for 52.6 to 71GHz