3GPP TSG RAN WG1 #109-e	R1-2203293

e-Meeting, May 9th - 20th, 2022

Title:	Remaining issues on channel access for 52.6 to 71GHz

Source: 	ZTE, Sanechips

Agenda Item:	8.2.4

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss some clarifications and remaining issues on channel access for NR above 52.6GHz, as follows:

- EDT

- LBT type indication in Fallback DCI and RAR UL grant

- LBT type update within a COT

- Default LBT type

- DCI format 2_0 enhancement

- Multi-beam LBT

- Per-beam LBT failure indication

- Short Control Signalling for UL

- No LBT

- Editorial issues

Discussion

2.1 EDT

EDT determination when LBT is performed over a wider bandwidth

In RAN1#108-e meeting [1], the issue on how to determine EDT when LBT is performed over a wider bandwidth than the active BWP bandwidth and whether to distinguish gNB and UE’s behavior have been discussed but no consensus was reached. In our view, whether the device is allowed to perform LBT over a wider bandwidth can be left for the implementation. If the device is willing to perform LBT over a wider bandwidth, it is a natural way to determine EDT based on the actual bandwidth that LBT is performed, rather actual occupied bandwidth, or etc. However, it is worth noting that we should not allow uncontrolled high EDT at least from the perspective of coexistence with other existing system/device such as 802.11 ad/ay(node) .

Besides, we do not see a need to distinguish gNB and UE’s behavior for EDT determination for LBT.

Proposal 1: EDT is determined based on the actual bandwidth that LBT is performed when LBT is performed over a wider bandwidth than the active BWP bandwidth.

Proposal 2: To avoid uncontrolled high EDT, it is proposed that EDT should be not higher a specific threshold.

The definition of Pout for a sensing beam

In RAN1#108-e meeting, we have discussed EDT determination for a COT with SDM/TDM transmission of beam with beam switching and proposed conclusion [2] is as follows:

Proposal 2.2-2:

For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT

Majority of the companies support the above proposal 2.2-2 to realize the benefits of directional LBT, but a few companies think that all sensing beams should use a common EDT according to the previous agreement. In our view, it is unnecessary to consider which the EIRP of transmission is not intended for sensing beam when determining EDT.

Proposal 3: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

2.2 LBT type indication in Fallback DCI and RAR UL grant

In previous meetings, regarding whether to use 2 bits or 1 bit for indicating LBT type in fallback DCI and RAR UL grant has been fully discussed but there is no consensus. Wherein, the main divergence is whether to allow Cat2 LBT indicated in fallback DCI. In our view, although Cat2 LBT is seen as an optional feature, this does not mean that it is not allowed to be indicated by fallback DCI. Instead, Cat2 LBT can be indicated as long as UE has a capability of supporting Cat2 LBT and reports this capability. Further, we also don’t think that the use of the fallback DCI is just limited to the case before RRC connection establishment. With that in mind, we prefer to use 2 bits for indication LBT type in fallback DCI.

Proposal 4: For fallback DCI format 0_0 and 1_0 and RAR UL grant, it is proposed to use 2 bits for the ChannelAccess-Cpext field to indicate LBT type.

2.3 LBT type update within a COT

In Rel-16 NR-U, it was agreed to support LBT Type switching from Type 1 channel access procedures to Type 2A channel access procedure when the UE detects DCI format 2_0 and know its corresponding UL transmission is within the remaining channel occupancy. While for Rel-17 NR above 52.6GHz, we think that similar LBT switching mechanism should be supported. That is, support LBT type from Type 1 channel access procedures as described in Clause 4.4.1 to Type 2 channel access procedures as described in Clause 4.4.2 or Type 3 channel access procedures as described in Clause 4.4.3. Wherein, specific switch to Type 2 channel access procedures as described in Clause 4.4.2 or Type 3 channel access procedures as described in Clause 4.4.3 can be controlled by RRC parameter.

Proposal 5: Introduce a RRC parameter to control Type 2 channel access procedures or Type 3 channel access procedures will be used for the case where the UE later finds out the transmission is in a gNB COT.

2.4 Default LBT type

For RRC parameter “channelAccessMode2-r17”, we can observe in draft TS 38.331 that it is an optional IE. If “channelAccessMode2-r17” is not configured as “enabled”, corresponding field description states that the UE does not apply these channel access procedures. In our view, for the region where LBT is required or LBT is not required, if this parameter is not enabled, we need to further consider and determine the default LBT type.

Proposal 6: If “channelAccessMode2-r17” is not enabled, it is necessary to further clarify what the default LBT is for the region where LBT is required or LBT is not required.

2.5 DCI format 2_0 enhancement

In Rel-15 NR, DCI format 2_0 only contained SFI. While in Rel-16 NR-U, several additional fields were introduced in DCI format 2_0, including CO duration, available RB set, search space group switching. However, in Rel-17 NR above 52.6GHz frequency band, transmissions are expected to be highly directional. In order to improve the probability of channel access, directional LBT is introduced to allow the transmitter to choose an intended beam direction to perform the channel access procedure. Based on this, when DCI format 2_0 is transmitted, we need to further discuss and determine whether CO duration, SFI, available RB set, search space group switching indicated in DCI format 2_0 should be associated with the directional beam. In our view, if the gNB performs directional LBT successfully in a beam direction, then it is a natural way that DCI format 2_0 is transmitted to the intended UEs in this beam direction and allow these UE to share this COT initiated by gNB with a beam direction. While for other UEs outside gNB sensing beam, this beam-specific COT is not allowed to be shared. Accordingly, search space group switching is also beam specific. While for available RB set, due to the concept is not introduced in Rel-17 NR above 52.6GHz, it cannot be configured. Besides, for SFI, it is reasonable to be configured as beam agnostic.

Proposal 7: If directional LBT is adopted, it is a natural way to support CO duration, search space group switching in a beam-specific manner in FR2-2.

2.6 Multi-beam LBT

In RAN1#107 e-meeting, the agreement on LBT of multi-beam COT in TDM/SDM manner had been achieved. Wherein, per beam LBT is supported but there is no consensus on how to implement per-beam LBT at the starting of COT. The potential alternatives are as follows:

- Alt A: The per-beam LBT for different beams is performed in TDM/SDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

For Alt A-1, it needs more time to complete LBT procedure for all sensing beams in turn. If LBT procedure on one of the beams is blocked, LBT on other beams will not start to be performed, this will further increase delay for all subsequent transmissions. For Alt A-2, the transmission with beam is transmitted immediately after the sensing is finished for this beam. For Alt A-3 and Alt B, their common ground is both support eCCA performed in different beams simultaneously. The difference point is Alt A-3 need to perform eCCA in different beams in turn, which will further increase LBT overhead, while Alt B can support like-type A multi-channel channel access method. Based on this, we propose Alt B if the device has the capability to simultaneously sense in different beams. Otherwise, we prefer Alt A-2 or Alt A-3. For Alt B, we think that multi-beam LBT can use similar method as multi-channel channel access procedure.

Proposal 8: If the device has the capability to simultaneously sense in different beams, Alt B that“The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams” can be supported for the transmission with multiple beams.

Proposal 9: If the node has no the capability to simultaneously sense in different beams, Alt A-3 that “The node performs eCCA of the different beams simultaneous, round robin between different beams” can be considered for the transmission with multiple beams.

  - 2.7 Per-beam LBT failure indication

In RAN2 #116bis e-meeting[3], it was agreed that per-beam LBT failure indication does not need to be provided from RAN1 to RAN2. However, from perspective of RAN1, we think that per-beam LBT failure indication is necessary to be indicated to RAN2 especially when directional LBT is used and performed as a failure. For this case, if such indications are not provided to RAN2, we would like to know how MAC counts LBT failure and triggers LBT failure recovery procedure.

RAN2 #116bis e-meeting: - 4: From RAN2 point of view there is no need that PHY provides per-beam LBT failure indications to MAC in Rel-17. No need to send LS to RAN1 unless they request RAN2 view.

Proposal 10: If directional LBT is used, it is recommended that per-beam LBT failure indication is supported in FR2-2 to better align the directional beam transmission characteristics and be compatible with the existing mechanisms.

  - 2.8 Short Control Signalling for UL

In RAN1 #105 e-meeting, Contention Exempt Short Control Signaling rules for UL was further discussed and reached the following agreement for msg1 and Msg A, but there is still no consensus on whether 10% limitation is per UE or per Cell.

Agreement : Contention Exempt Short Control Signalling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signalling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In order to avoid the misuse of contention exempt short control signalling rules and consider fair and friendly coexistence with other nodes from other system, we think that it is reasonable to apply 10ms limitation within a 100ms observation period for all UEs in a cell, which responds to Alt 1 listed in above agreement.

Proposal 11: Adopt Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

Proposal 12: Adopt TP1 into Section 4.4.5 of TS 37.213:

Reason for change:

Clarify “Contention Exempt Short Control Signalling rules” for UL.

Summary of change:

Add “all UEs in a Cell” into the last paragraph of clause 4.4.5 in TS 37.213.

Consequences if not approved:

Unclear “Contention Exempt Short Control Signalling rules” for UL.

*** <Beginning of Text Proposal 1 of TS 37.213> ***

4.4.5	Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel:

-	Transmission(s) of the discovery burst by the gNB

-	Transmission(s) of the first message in a random access procedure by the UE

When the gNB/all UEs in a cell transmit(s) the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/all UEs in a cell shall not occupy the corresponding channel more than  over any  interval.

*** <Ending of Text Proposal 1 of TS 37.213> ***

Another issue to be clarified is: if the transmission of DL/UL channels/signals considered as Short Control Signalling is in a COT initiated by gNB or UE and LBT is performed before Short Control Signalling transmission, in our understanding, it should not be counted into 10ms limitation within the 100ms observation period.

Proposal 13: For the case of the transmission of DL/UL channels/signals considered as Short Control Signalling is in a COT initiated by gNB or UE and LBT is performed before Short Control Signalling transmission, it is suggested that such transmission should not be counted into 10ms limitation within the 100ms observation period.

The last issue to be clarified is: whether to introduce a new RRC parameter to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed. If such RRC parameter is introduced, it will be beneficial to avoid the misuse of Contention Exempt Short Control Signaling rule and false statistics on UE side and provide more flexibility to enable/disable this functionality on gNB side.

Proposal 14: It is proposed to introduce a new RRC parameter to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed.

2.9 No LBT

In RAN1 #102 e-meeting, we have reached a basic consensus that both LBT and No LBT are supported for gNB/UE to initiate a channel occupancy. But there is still no consensus on the use cases to use No LBT and whether it is necessary to introduce some restrictions when No LBT is used, such as the duration of a transmission corresponding to No LBT, and the triggering conditions for No LBT fallback to LBT.

On use cases to use No LBT, the following can be considered: (1) specific areas such as ITU region 2 and 3 in which No LBT is not required to be used for unlicensed carrier. (2) interference controlled environment. (3) a common case that NR-U and NR-U coexistence scenario and the absence of any other systems can be guaranteed. For instance, the node1 belongs to operator 1 while the node2 is served for operator 2. If the transmitted beams of node 1 and node 2 do not overlap or transmission of two nodes is not interfered each other, then the transmission for the node preparing to transmit will not affect that for another node even if LBT is not performed for the node preparing to transmit.

Proposal 15: No LBT can be considered to be used in the following use cases:

- Specific areas such as ITU region 2 and 3.

- Interference controlled environment.

- The transmission beams of nodes of different operators in the same system (e.g., NR-U) have little interference with each other.

Although we observe that No LBT can be applied in some specific cases and even can be extended to more cases in the future. In our understanding, no LBT should be workable only if some interference elimination mechanisms are applied on top of it, e.g. Automatic Transmit Power Control (ATPC), Dynamic frequency selection (DFS), duty cycle. If no LBT is supported, the spec impact of introducing such enhancement should be further studied and evaluated.

Observation 1: No LBT should be workable only if some interference elimination mechanisms are applied on top of it. If no LBT is supported, the spec impact of introducing such enhancement should be further studied and evaluated.

On the restriction of the duration of a transmission for using No LBT, we think that similar restriction as defined in Type 2C channel access procedure in TS 37.213 can also introduced in above 52.6GHz NR-U frequency band but the length of a transmission can be relaxed. On the contrary, if there is no any limitations on the duration of a transmission for using No LBT, it may lead to unfair the opportunities of channel access/occupancy and also violate the basic principle of friendly and fair coexistence, e.g., the transmission of some nodes is continuously blocked, or the effect of persistent interference on devices that have occupied the channel in advance.

Proposal 16: Similar restriction as defined in Type 2C channel access procedure in TS 37.213 can also introduced in above 52.6GHz NR-U frequency band but the length of a transmission can be relaxed.

- The duration of the corresponding DL transmission is at most [Y] symbols or ms.

Furthermore, it is necessary to support certain mechanism to allow the fallback from no LBT to LBT, if the above mentioned condition on No LBT is not satisfied. This can be triggered by gNB explicitly or implicitly, e.g. based on the interference level or correctly decoding rate.

Proposal 17: Conditions for No LBT fallback to LBT should be further studied, e.g., based on the interference level or correctly decoding rate.

Editorial issues

TS 37.213

Reason for change:

Misalignment of higher-layer parameter name between TS 37.213 and TS 38.331.

Summary of change:

Change  “ChannelAccessMode-r16” to “channelAccessMode-r16”

Consequences if not approved:

Misaligned parameter name between different specs.

*** <Beginning of Text Proposal 2 of TS 37.213_h10> ***

4.1	Downlink channel access procedures

An eNB operating LAA Scell(s) on channel(s) and a gNB performing transmission(s) on channel(s) shall perform the channel access procedures described in this clause for accessing the channel(s) on which the transmission(s) are performed.

In this clause,  for sensing is adjusted as described in clause 4.1.5 when applicable.

A gNB performs channel access procedures in this clause unless the higher layer parameter CchannelAccessMode-r16 is provided and CchannelAccessMode-r16 =' semiStatic'.

4.2	Uplink channel access procedures

A UE performing transmission(s) on LAA Scell(s), an eNB scheduling or configuring UL transmission(s) for a UE performing transmission(s) on LAA Scell(s), and a UE performing transmission(s) on channel(s) and a gNB scheduling or configuring UL transmission(s) for a UE performing transmissions on channel(s) shall perform the procedures described in this clause for the UE to access the channel(s) on which the transmission(s) are performed.

In this clause, transmissions from a UE are considered as separate UL transmissions, irrespective of having a gap between transmissions or not, and  for sensing is adjusted as described in clause 4.2.3 when applicable.

A UE performs channel access procedures in this clause unless the higher layer parameter CchannelAccessMode-r16 is provided and CchannelAccessMode-r16 =' semiStatic'.

If a UE fails to access the channel(s) prior to an intended UL transmission to a gNB, Layer 1 notifies higher layers about the channel access failure.

4.2.3	Energy detection threshold adaptation procedure

A UE accessing a channel on which UL transmission(s) are performed, shall set the energy detection threshold () to be less than or equal to the maximum energy detection threshold .

is determined as follows:

-	If the UE is configured with higher layer parameter maxEnergyDetectionThreshold-r14 or maxEnergyDetectionThreshold-r16,

-	 is set equal to the value signalled by the higher layer parameter;

-	otherwise

-	the UE shall determine  according to the procedure described in clause 4.2.3.1;

-	if the UE is configured with higher layer parameter energyDetectionThresholdOffset-r14 or energyDetectionThresholdOffset-r16

-	 is set by adjusting  according to the offset value signalled by the higher layer parameter;

-	otherwise

-	the UE shall set .

The UE is not expected to be configured with ul-toDL-COT-SharingED-Threshold-r16 when the UE is provided with CchannelAccessMode-r16 ='semiStatic',

< Unchanged parts are omitted >

4.3	Channel access procedures for semi-static channel occupancy

Channel access procedures based on semi-static channel occupancy as described in this Clause, are intended for environments where the absence of other technologies is guaranteed e.g., by level of regulations, private premises policies, etc.

If a gNB provides UE(s) with higher layer parameters CchannelAccessMode-r16 ='semiStatic' by SIB1 or dedicated configuration for a serving cell, a periodic channel occupancy can be initiated by the gNB on a channel(s) within the bandwidth of the serving cell every  within every two consecutive radio frames, starting from the even indexed radio frame at  with a maximum channel occupancy time , where  period in , is a higher layer parameter provided in SemiStaticChannelAccessConfig and . A duration of   at the end of a period is referred to as the idle duration of that period.

< Unchanged parts are omitted >

*** <Ending of Text Proposal 2 of TS 37.213_h10> ***

TS 38.212

Reason for change:

Misalignment of higher-layer parameter name between TS 38.212 and TS 38.331.

Summary of change:

Change “ChannelAccessMode-r16 ='semistatic'” to “channelAccessMode-r16 = semiStatic”

Consequences if not approved:

Misaligned parameter name between different specs.

*** <Beginning of Text Proposal 3 of TS 38.212_h10> ***

7.3.1.1.1	Format 0_0

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

-< Unchanged parts are omitted >

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise.

< Unchanged parts are omitted >

-	TPC command for scheduled PUSCH – 2 bits as defined in Clause 7.1.1 of [5, TS 38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bit otherwise

< Unchanged parts are omitted >

Table 7.3.1.1.1-4A: Channel access type & CP extension if CchannelAccessMode-r16 = "semisStatic" is provided

| Bit field mapped to index | Channel Access Type | The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211] | Initiator of the channel occupancy associated with the UL transmission as described in Clause x.x in TS 37.213 |
| --- | --- | --- | --- |
| 0 | No sensing as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 1 | No sensing as defined in Clause 4.3 in TS 37.213 | 2 | gNB |
| 2 | Sensing within a 25us interval as defined in Clause 4.3 in TS 37.213 | 0 | gNB |
| 3 | Sensing as defined in Clause 4.3.1.2 in TS 37.213 | 0 | UE |
| Note: Row index 3 is only applicable if ue-SemiStaticChannelAccessConfig is provided. Otherwise, the row is reserved. |  |  |  |

7.3.1.1.2	Format 0_1

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-1 or in Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 or Table 7.3.1.1.2-35A are configured by the higher layer parameter ul-AccessConfigListDCI-0-1.

< Unchanged parts are omitted >

7.3.1.1.3	Format 0_2

DCI format 0_2 is used for the scheduling of PUSCH in one cell.

The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-0-2 or in Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ul-AccessConfigListDCI-0-2.

< Unchanged parts are omitted >

7.3.1.2.1	Format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell.

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; 0 bits otherwise

-	Reserved bits – 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise

< Unchanged parts are omitted >

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit

< Unchanged parts are omitted >

7.3.1.2.2	Format 1_1

DCI format 1_1 is used for the scheduling of one or multiple PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-1-1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-1 or in Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 or Table 7.3.1.2.2-6A are configured by the higher layer parameter ul-AccessConfigListDCI-1-1.

< Unchanged parts are omitted >

7.3.1.2.3	Format 1_2

DCI format 1_2 is used for the scheduling of PDSCH in one cell.

The following information is transmitted by means of the DCI format 1_2 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI:

< Unchanged parts are omitted >

-	ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ul-AccessConfigListDCI-1-2 or in Table 7.3.1.1.1-4A if CchannelAccessMode-r16 = "semisStatic" is provided, for operation in a cell with shared spectrum channel access; otherwise 0 bit. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter ul-AccessConfigListDCI-1-2.

< Unchanged parts are omitted >

*** <Ending of Text Proposal 3 of TS 38.212_h10> ***

TS 38.213

Reason for change:

Misalignment of higher-layer parameter name between TS 38.213 and TS 38.331.

Summary of change:

Change “ChannelAccessMode-r16'” to “channelAccessMode-r16”.

Consequences if not approved:

Misaligned parameter name between different specs.

*** <Beginning of Text Proposal 4 of TS 38.213_h10> ***

8.2	Random access response - Type-1 random access procedure

In response to a PRACH transmission, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in clause 10.1, that is at least one symbol, after the last symbol of the PRACH occasion corresponding to the PRACH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set as defined in clause 10.1. If NTA,adjUE or NTA,adjcommon, as defined in [4, TS 38.211], is not zero, the window starts after an additional TTA+kmac msec where TTA is defined in [4, TS 38.211] and kmac is provided by K-Mac or kmac=0 if K-Mac is not provided.The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

< Unchanged parts are omitted >

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if CchannelAccessMode-r16 = "semiStatic" is provided.

< Unchanged parts are omitted >

8.2A	Random access response - Type-2 random access procedure

< Unchanged parts are omitted >

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

< Unchanged parts are omitted >

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if CchannelAccessMode-r16 = "semiStatic" is provided

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

< Unchanged parts are omitted >

11.1.1	UE procedure for determining slot format

This clause applies for a serving cell that is included in a set of serving cells configured to a UE by slotFormatCombToAddModList and slotFormatCombToReleaseList, availableRB-SetsToAddModList and availableRB-SetsToReleaseList, switchTriggerToAddModList and switchTriggerToReleaseList, or co-DurationsPerCellToAddModList and co-DurationsPerCellToReleaseList.

< Unchanged parts are omitted >

If neither CO-DurationPerCell-r16 nor SlotFormatCombinationsPerCell are provided and if CchannelAccessMode-r16 = "semiStatic" is provided, the procedures in this clause apply with assuming a channel occupancy time defined in clause 4.3 of [15, TS 37.213] is the remaining channel occupancy duration if a DL transmission burst(s) is detected within the channel occupancy time.

< Unchanged parts are omitted >

*** <Ending of Text Proposal 4 of TS 38.213_h10> ***

Conclusion

In this contribution, we share some our views on remaining issues of channel access mechanism for 52.6 GHz to 71GHz and provide the following observations and proposals:

Proposal 1: EDT is determined based on the actual bandwidth that LBT is performed when LBT is performed over a wider bandwidth than the active BWP bandwidth.

Proposal 2: To avoid uncontrolled high EDT, it is proposed that EDT should be not higher a specific threshold.

Proposal 3: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

Proposal 4: For fallback DCI format 0_0 and 1_0 and RAR UL grant, it is proposed to use 2 bits for the ChannelAccess-Cpext field to indicate LBT type.

Proposal 5: Introduce a RRC parameter to control Type 2 channel access procedures or Type 3 channel access procedures will be used for the case where the UE later finds out the transmission is in a gNB COT.

Proposal 6: If “channelAccessMode2-r17” is not enabled, it is necessary to further clarify what the default LBT is for the region where LBT is required or LBT is not required.

Proposal 7: If directional LBT is adopted, it is a natural way to support CO duration, search space group switching in a beam-specific manner in FR2-2.

Proposal 8: If the device has the capability to simultaneously sense in different beams, Alt B that“The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams” can be supported for the transmission with multiple beams.

Proposal 9: If the node has no the capability to simultaneously sense in different beams, Alt A-3 that “The node performs eCCA of the different beams simultaneous, round robin between different beams” can be considered for the transmission with multiple beams.

Proposal 10: If directional LBT is used, it is recommended that per-beam LBT failure indication is supported in FR2-2 to better align the directional beam transmission characteristics and be compatible with the existing mechanisms.

Proposal 11: Adopt Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell.

Proposal 12: Adopt TP1 into Section 4.4.5 of TS 37.213:

Reason for change:

Clarify “Contention Exempt Short Control Signalling rules” for UL.

Summary of change:

Add “all UEs in a Cell” into the last paragraph of clause 4.4.5 in TS 37.213.

Consequences if not approved:

Unclear “Contention Exempt Short Control Signalling rules” for UL.

*** <Beginning of Text Proposal 1 of TS 37.213> ***

4.4.5	Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel:

-	Transmission(s) of the discovery burst by the gNB

-	Transmission(s) of the first message in a random access procedure by the UE

When the gNB/all UEs in a cell transmit(s) the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/all UEs in a cell shall not occupy the corresponding channel more than  over any  interval.

- *** <Ending of Text Proposal 1 of TS 37.213> ***

Proposal 13: For the case of the transmission of DL/UL channels/signals considered as Short Control Signalling is in a COT initiated by gNB or UE and LBT is performed before Short Control Signalling transmission, it is suggested that such transmission should not be counted into 10ms limitation within the 100ms observation period.

Proposal 14: It is proposed to introduce a new RRC parameter to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed.

Proposal 15: No LBT can be considered to be used in the following use cases:

- Specific areas such as ITU region 2 and 3.

- Interference controlled environment.

- The transmission beams of nodes of different operators in the same system (e.g., NR-U) have little interference with each other.

Observation 1: No LBT should be workable only if some interference elimination mechanisms are applied on top of it. If no LBT is supported, the spec impact of introducing such enhancement should be further studied and evaluated.

Proposal 16: Similar restriction as defined in Type 2C channel access procedure in TS 37.213 can also introduced in above 52.6GHz NR-U frequency band but the length of a transmission can be relaxed.

- The duration of the corresponding DL transmission is at most [Y] symbols or ms.

Proposal 17: Conditions for No LBT fallback to LBT should be further studied, e.g., based on the interference level or correctly decoding rate.

Reference

- Draft_Minutes_report_RAN1#108-e_v010

- R1-2202876, FL summary for channel access of 52.6GHz to 71GHz band_ver03

- Draft_R2-116bise_Meeting_Report_v1