3GPP TSG RAN WG1 #109-e		R1-2203302

e-Meeting, May 9th – 20th, 2022

Agenda Item:     8.1.2

Source:	Spreadtrum Communications

Title:	             Remaining issues on multi-TRP deployment

Document for:	Discussion and decision

Introduction

In this paper, we provide one text proposal on HST-SFN deployment.

Text proposal

- Enhancement on HST-SFN deployment

Reason for changes

In RAN1#106b-e [1], regarding the combination of PDCCH and PDSCH, after extensive discussion, we have the following agreement. We notice that it has not been captured into spec.

Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH This is optional UE feature Note: The support of such combination scheme is for URLLC use-case only.

Summary of changes

If PDCCH is configured with ‘sfnSchemeA’, and UE not supporting combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH, then two TCI states are indicated by any codepoint for  DCI format 1_1/1_2.

If PDCCH is configured with ‘sfnSchemeB’, then two TCI states are indicated by any codepoint for  DCI format 1_1/1_2.

Consequences if not approved:

The agreement is not captured.

Proposal 1: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#1 for TS 38.214--------------------------------------

- 5.1	UE procedure for receiving the physical downlink shared channel

-----------------------------Unchanged part omitted--------------------------

When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of sfnSchemeA for PDCCH only, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2.

------------------------------------------End of Text Proposal#1 for TS 38.214--------------------------------------

Conclusion

In this contribution, we present one text proposal on HST-SFN deployment.

Proposal 1: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#1 for TS 38.214--------------------------------------

- 5.1	UE procedure for receiving the physical downlink shared channel

-----------------------------Unchanged part omitted--------------------------

When a UE is configured with both sfnSchemePdsch and sfnSchemePdcch, the UE shall expect that sfnSchemePdsch and sfnSchemePdcch are set to the same scheme, either 'sfnSchemeA' or 'sfnSchemeB'.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeA' for a DL BWP and activated with two TCI states by MAC CE, and the UE does not report its capability of sfnSchemeA for PDCCH only, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2.

If a UE is configured with sfnSchemePdcch set to 'sfnSchemeB' for a DL BWP and activated with two TCI states by MAC CE, the UE does not expect to be indicated with one TCI state in a codepoint of the DCI field 'Transmission Configuration Indication' in DCI format 1_1/1_2.

------------------------------------------End of Text Proposal#1 for TS 38.214--------------------------------------

Reference

- Draft Report of 3GPP TSG RAN WG1 #106b-e

- 3GPP TS 38.214: "NR; Physical layer procedures for data", h10