3GPP TSG RAN WG1 #109-e                                                   R1-2203303

e-Meeting, May 9 – May 20, 2022

Agenda Item:     8.1.3

Source:	Spreadtrum Communications

Title:	Remaining issues on SRS in Rel-17

Document for:	Discussion and decision

Introduction

In this contribution, we give our views on the remaining issue of SRS in Rel-17.

Text proposal

- Collision handling for aperiodic SRSs

Reason for changes

Within Rel-17 SRS framework, collisions among aperiodic SRS resource set become frequent after introducing flexible aperiodic SRS indication. Besides, this issue could be more severe under CA case considering the limitation of UE capabilities.

Though scheduling is a way to minimize the collisions, the flexibility limitation is also not negligible. It is necessary and beneficial to introduce new dropping rules for aperiodic SRS collisions.

Summary of changes

Execute dropping based on SRS resource set ID and its associated CC ID. Specifically, lower ID’s priority is higher than higher ID.

Consequences if not approved:

Following current specification, UE doesn’t know how to handle aperiodic SRS collisions.

Proposal 1: Adopt the following text proposal for TS 38.214.

/************************ Start of Text Proposal **************************/

6.2.1	UE sounding procedure

/************************ Unchanged parts omitted**************************/

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

For the overlapping SRS resources with the higher layer parameter resourceType in SRS-Resource set to 'aperiodic', and at least one resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource, a UE can transmit one or more SRS resources subject to UE's capability, where

−	the SRS resource (s) correspond lowest SRS-ID(s)  with lowest CC-ID(s).

/************************ Unchanged parts omitted**************************/

/************************ End of Text Proposal **************************/

Conclusion

In this contribution, we give our views on the remaining issue of SRS in Rel-17 with the following text proposal.

Proposal 1: Adopt the following text proposal for TS 38.214.

/************************ Start of Text Proposal **************************/

6.2.1	UE sounding procedure

/************************ Unchanged parts omitted**************************/

In case a SRS resource with resourceType set as 'aperiodic' is triggered on the OFDM symbol(s) configured with periodic/semi-persistent SRS transmission, the UE shall transmit the aperiodic SRS resource and only the periodic/semi-persistent SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic/semi-persistent SRS symbol(s) that are not overlapped with the aperiodic SRS resource are transmitted. In case a SRS resource with resourceType set as 'semi-persistent' is triggered on the OFDM symbol(s) configured with periodic SRS transmission, the UE shall transmit the semi-persistent SRS resource and only the periodic SRS symbol(s) overlapping within the symbol(s) are dropped, while the periodic SRS symbol(s) that are not overlapped with the semi-persistent SRS resource are transmitted.

For the overlapping SRS resources with the higher layer parameter resourceType in SRS-Resource set to 'aperiodic', and at least one SRS resource set is configured with parameter availableSlotOffset across all configured BWPs in a component carrier except when SRS is configured with the higher layer parameter SRS-PosResource, a UE can transmit one or more SRS resources subject to UE's capability, where

−	the SRS resource (s) correspond lowest SRS-ID(s)  with lowest CC-ID(s).

/************************ Unchanged parts omitted**************************/

/************************ End of Text Proposal **************************/