3GPP TSG RAN WG1 #109-e			R1-2203309

e-Meeting, May 9th – 20th, 2022

Agenda item:	8.8.2

Source: 	Spreadtrum Communications

Title: 	Discussion on joint channel estimation for PUSCH&PUCCH

Document for:	Discussion and decision

Introduction

This contribution provides some thoughts on the remaining issues of joint channel estimation for PUSCH&PUCCH.

Discussion

Dynamic PUCCH repetition factor indication for HARQ-ACK of first SPS PDSCH associated with the activation DCI and SPS release DCI

Due to the PUCCH resource provided by n1PUCCH-AN or SPS-PUCCH-AN-List is used to feedback HARQ-ACK for the first SPS PDSCH activated by activation DCI in [108-e-NR-CRs-02], the remaining issue of whether or not dynamic PUCCH repetition factor indication for HARQ-ACK of first SPS PDSCH can be indicated by the activation DCI can be reopen in this meeting.

Since PRI in the activation DCI is ignored, the repetition number configured within the PUCCH resource also cannot be applied now. Thus, the dynamic PUCCH repetition factor indication mechanism does not apply to HARQ-ACK corresponding to the first SPS PDSCH after activation. And the dynamic PUCCH repetition factor indication mechanism can apply to HARQ-ACK corresponding to the SPS release DCI.

Therefore, we have the following proposal:

- In NR Rel-17, the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e does not apply to HARQ-ACK for SPS PDSCH including the first SPS PDSCH associated with the activation DCI except for the HARQ-ACK corresponding to the SPS release DCI

Power control with PUSCH/PUCCH repetitions

TPC commands with DCI format 2_2 was still open. Among the below options, if it is clearly stated that CG-DG PUSCH interference is not included in the DMRS bundle, we are supportive of Option 3/3a in Proposal 8. Otherwise, Proposal 9 dynamic event is a clean solution. Because if gNB want to keep the current actual window, it would restrain the transmission of DCI format 2_2. While gNB prefer to apply a power boosting other than remain the DMRS joint channel estimation, the instant TPC command would be preferred at this case. So considering, the limited benefit and controllable TPC adjustment, TPC command is treated as an event is a good forward.

Proposal 8: Agree in principle to the following in order to capture the working assumption on group common TPC commands with format 2_2 for accumulate TPC commands for CG-PUSCH. - Option 1: Legacy definition of KPUSCHi is preserved for PUSCH transmissions without DM-RS bundling. Redefine KPUSCHi for PUSCH transmissions within a nominal TDW in case of DM-RS bundling. e.g., KPUSCHi is a number of symbols from K symbols before the start of the first repetition within the nominal time domain window including the transmission occasion i and before a first symbol of the transmission occasion i. - FFS: the value of K, e.g., K is “a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c”. - Option 2a: Modify the TPC command value set Di, e.g. if transmission occasion i is not the first transmission occasion within a nominal time domain window, then any TPC command values received via DCI format 2_2 contained in the set Di are deleted and added to the set Dj where j is a transmission occasion occurring after the end of the nominal time domain window and j>0 is the smallest integer for which KPUSCH(i) symbols before PUSCH transmission occasion i is earlier than KPUSCH(j) symbols before PUSCH transmission occasion j. - Option 3: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that would take effect for the transmission occasions occurring after transmission occasion i1 and no later than transmission occasion k (i.e. including occasion k itself). - Option 3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands, - For a transmission occasion i occurs within a nominal time domain window, fi=f i1, where transmission occasion i1 is a first transmission occasion within the nominal time domain window. - For the first transmission occasion k occurring after the nominal time domain window, fk=fi1+δj, where δj is all the TPC command values that are received between KPUSCHi1 symbols before transmission occasion i1 and KPUSCHk symbols before transmission occasion k. - Note: If a UE is configured with DMRS bundling, for a given PUSCH power control adjustment state l in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission occasion m in symbol p and is configured with any CG PUSCH transmission occasion n ending before symbol p, the UE is not expected to receive a TPC command by DCI format 2_2 arrived later than KPUSCHm symbols before transmission occasion m but no later than KPUSCHn symbols before transmission occasion n. - Combined Option 1&3a: For group common TPC commands with format 2_2, if UE is configured to accumulate TPC commands and configured with DMRS bundling, - For a transmission occasion i occurs within a nominal time domain windowKPUSCHi for PUSCH transmissions occasion i from KPUSCHi1 symbols before the start a first symbol of the transmission occasion i1 to a first symbol of the transmission occasion i where transmission occasion i1 is a first transmission occasion within the nominal time domain window for the same transport block as transmission occasion i and KPUSCHi1 reuses the same Rel-15/16 definition of KPUSCHx. - The definition of set for in Rel-15/16 TS 38.213 is reused.. Proposal 9: - The action of group common TPC commands with format 2_2 [indicating different values of power control adjustment] is regarded as an event. - The following working assumption is not confirmed. Working assumption: - The action of group common TPC commands with format 2_2 does not constitute an event that violates power consistency and phase continuity. - If UE is configured to accumulate TPC commands, - If UE receives TPC commands that would take into effect during a configured TDW, UE accumulates TPC commands without taking effect during the current configured TDW. TPC commands take effect after the current configured TDW. - If UE is not configured to accumulate TPC commands - the last TPC command that would take effect within a configured TDW supersedes all previous TPC commands that take effect within that configured TDW and only the last TPC command is applied by the UE after the current configured TDW. - FFS: no more than 1 TPC command is expected to take effect during a configured TDW..

- Option 3/3a in Proposal 8 is preferred if CG-DG PUSCH interference is not supported. Otherwise, Proposal 9 TPC command is treated as an event.

Conclusion

In this contribution, we made the following proposals.

- In NR Rel-17, the dynamic PUCCH repetition factor indication mechanism agreed in RAN1 106e does not apply to HARQ-ACK for SPS PDSCH including the first SPS PDSCH associated with the activation DCI except for the HARQ-ACK corresponding to the SPS release DCI

- Option 3/3a in Proposal 8 is preferred if CG-DG PUSCH interference is not supported. Otherwise, Proposal 9 TPC command is treated as an event.

References

Draft Report of 3GPP TSG RAN WG1 #108-e.

R1-2202753	FL summary #3 of PUCCH coverage enhancement	Moderator (Qualcomm)

R1-2202870 Summary of email discussion on joint channel estimation for PUSCH Moderator (China Telecom)