3GPP TSG RAN WG1 #109-e			R1-2203313

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	8.11.2

Source:		Spreadtrum Communications

Title:		Remaining issues on inter-UE coordination in sidelink resource allocation

Document for:		Discussion and decision

Introduction

In this contribution, we discuss the remaining issues of inter-UE coordination in mode 2 for NR sidelink communication.

Discussion

The timeline of the resource selection window used for IUC information transmission

Reason for change:

In RAN1#108-e [1], the following agreement was made for inter-UE coordination scheme 1:

Agreement - Notations: - (n+T_1) – Start slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement - (n+T_2) – End slot of resource selection window for determining the set of resources - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered - For inter-UE coordination information triggered by UE-B’s explicit request - Alt 1-1: - X1 ≤ (n’+T’_1) - (n’+T’_2) ≤ X2 - For inter-UE coordination information triggered by a condition other than explicit request reception, - Alt 2-2: - (n’+T’_2) < X3 FFS: Values for X1, X2, X3

There was a remaining issue about the values for X1, X2, and X3 in the last meeting. First of all, we should clarify the meaning of (n+T_1). (n+T_1) is an absolute starting location of the resource selection window for determining the set of resources, which is directly provided by IUC request or up to UE-A’s implementation. And we do not need to separately consider the meaning of n and T_1, where n in (n+T_1) may not be the time location for resource (re-)selection triggering. Based on the above, we should consider the timeline between (n+T_1), (n+T_2), (n’+T’_1), and (n’+T’_2) rather than n, (n’+T’_1), and (n’+T’_2).

For request-based inter-UE coordination, we think IUC information is transmitted before the resource selection window of UE-B, otherwise, the preferred/non-preferred resource set may be outdated. Hence, X2 should be the starting location of the resource selection window for determining the set of resources, i.e., X2 = (n+T_1). For (n’+T’_1). The starting location of the resource selection window used for IUC information transmission should be later than the starting location of the resource selection window for IUC request transmission at least.

For condition-based inter-UE coordination, the (n+T_1) and (n+T_2) are up to UE-A’s implementation. Hence, the timeline between the resource selection window used for IUC information transmission and for determining the set of resources can also be achieved by UE-A’s implementation.

Summary of change:

Clarify the ending location of the resource selection window used for IUC information transmission should be earlier than the starting location of the resource selection window of UE-B, for request-based inter-UE coordination.

Consequences if not approved:

If the timeline of the resource selection window used for IUC information transmission is not clarified, UE-A may provide the outdate IUC information.

Based on the above reason for change, summary of change, and consequences if not approved, the following TP for TS 38.214 is proposed.

Proposal 1: Adopt the following Text Proposal#2 in TS 38.214.

------------------------------------------------------Begin text proposal for 38.214-------------------------------------------------------

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

<Unchanged parts omitted>

When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows:

-	The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A, and the starting time location of the resource selection window is later than the n+T_2.

-	The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A.

<Unchanged parts omitted>

------------------------------------------------End text proposal for 38.214--------------------------------------------------

How to deal with the different IUC information decoded by SCI format 2-C and MAC CE

The following agreements about the container of inter-UE coordination (IUC) information were achieved in the previous meetings:

Agreement For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives: - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. Agreement The following working assumption is confirmed with modification in RED. - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3] Agreement MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B. - For the indication of resource set, the following is supported: - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request. - First resource location of each TRIV is separately indicated by the inter-UE coordination information - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used. - FFS: UE capability details - 2nd SCI is UE RX optional - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

According to the agreements, SCI format 2-C and MAC CE could be used as the container simultaneously for carrying IUC information. Considering a special case, IUC information decoded from SCI format 2-C and MAC CE may be different, it is necessary to clarify which IUC information should be used by UE-B. From our point of view, considering the higher reliability of SCI format 2-C, IUC information decoded from SCI format 2-C should be used when the IUC information decoded from SCI format 2-C and MAC CE are different.

Proposal 2: When IUC information decoded from SCI format 2-C and MAC CE are different, IUC information in SCI format 2-C should be considered by UE-B.

Conditions for supporting only using preferred resource set for resource selection by UE-B

In the last meeting, the following draft proposal is reached in the FL summary:

Draft proposal 3-17: For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met: - Option 1: UE-B does not have a capability of performing sensing/resource exclusion. - Option 2: UE-B performs random resource selection. - Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation. - It is allowed only in resource pool (pre)configured with random resource selection

The above draft proposal was obtained by the email discussion in the last meeting, which considers the conditions for UE-B to use the preferred resource set for resource selection without sensing procedure. When UE-B has no capability for performing sensing/resource exclusion or/and only supports performing random resource selection, using the preferred resource set for resource selection only is necessary in order to avoid conflict with other UE’s transmissions. Besides, option 3 is beneficial for sidelink power saving. Hence, we support the above draft proposal.

Proposal 3: For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

  - It is allowed only in resource pool (pre)configured with random resource selection.

UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As

In RAN1#108-e, the following agreement was made for inter-UE coordination scheme 1:

Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection Agreement - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As, - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set. - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As. - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline) - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As, - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

There is a remaining issue about UE-B’s behavior when it receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As in the above agreement. In our opinion, before discussing the solution of the above issue, many aspects should be considered, e.g., whether the UE-A is the intended receiver of UE-B (the resource set provided by the intended receiver should be taken into account firstly), the trigger type of the preferred and non-preferred resource sets (the provided resource set triggered by request should be taken into account firstly), and whether the preferred/non-preferred resource sets provided is expired (considering chronological order), etc. Hence, considering the limited time, we prefer to up to UE-B implementation to use one or multiple of them in its resource (re)selection.

Proposal 4: when UE-B receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

Conclusions

In this contribution, we discuss the remaining issues of inter-UE coordination in mode 2 for NR Sidelink.

For the timeline of the resource selection window used for IUC information transmission, we give the TP in Proposal 1.

Proposal 1: Adopt the following Text Proposal#2 in TS 38.214.

------------------------------------------------------Begin text proposal for 38.214-------------------------------------------------------

8.1.4A	UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission

<Unchanged parts omitted>

When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows:

-	The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A, and the starting time location of the resource selection window is later than the n+T_2.

-	The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A.

<Unchanged parts omitted>

------------------------------------------------End text proposal for 38.214--------------------------------------------------

For the following Proposal 2 and Proposal 3, as they are related with RAN2 specification, we suggest sending an LS about RAN1 conclusions to RAN2.

Proposal 2: When IUC information decoded from SCI format 2-C and MAC CE are different, IUC information in SCI format 2-C should be considered by UE-B.

Proposal 3: For Scheme 1, Option B can be used for preferred resource set when at least one of following condition is met:

- Option 1: UE-B does not have a capability of performing sensing/resource exclusion.

- Option 2: UE-B performs random resource selection.

- Option 3: UE-B has a capability of performing sensing/resource exclusion, but UE-B determines not to perform sensing/resource exclusion by its implementation.

  - It is allowed only in resource pool (pre)configured with random resource selection.

For the following proposal 4, as it can be an UE implementation issue, no specification is expected.

Proposal 4: when UE-B receives both preferred resource set and non-preferred resource set from the same UE-A or different UE-As, it is up to UE-B implementation to use one or multiple of them in its resource (re)selection.

References

RAN1 Chairman’s Notes, RAN1#108-e, e-Meeting.