3GPP TSG RAN WG1 Meeting #109-e                            	        R1-2203316

E-Meeting, May 9th – 20th, 2022

Agenda Item:	8.14

Source:	Spreadtrum Communications

Title:	Maintenance on enhancements to time and frequency synchronization for IOT NTN

Document for:	Discussion and decision

Introduction

In this contribution, the remaining issues of the time and frequency synchronization enhancements to NB-IoT/eMTC over NTN are discussed.

Long UL transmission on PUSCH and PRACH

In RAN1#107e, the following agreement on long UL transmission on PUSCH and PRACH has been made [1].

Agreement For UL Segmented transmission during RRC_CONNECTED: - If a segment duration is configured, the UE is expected to adjust the value for pre-compensation for a segment. - FFS: UL transmission segment duration for NPDCCH ordered NPRACH/NPUSCH for NB-IoT and PDCCH ordered PRACH/PUSCH/PUCCH for eMTC is configurable by dedicated RRC Signalling - For UE pre-compensation per segment, further discuss how the following options apply from one segment to the next segment, and potential down-selection among the options: - Option 1: Skip / drop / insert samples - Option 2: puncture OFDM symbols - Option 3: Blanking subframes/slots where UE skip a slot or a subframe - FFS whether this can be left to UE implementation or if specification impact is needed

In this section, we share our views on specification impact of method(s) to drop / insert samples, blanking subframes / slots for UL Segmented transmission.

Text proposals for long UL transmission on PUSCH

Reason for change: In RAN1#107e, it was agreed that UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by drop / Insert samples / Puncture OFDM symbols or blank subframes / slots where UE skip a slot or a subframe.

Agreement UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples / Puncture OFDM symbols - UE may blank subframes / slots where UE skip a slot or a subframe The total transmission time is not changed UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot The method used for the UE pre-compensation is known to the eNB by a single UE capability - UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing) FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

For the methods that UE may blank subframes / slots where UE skip a slot or a subframe, it needs to be captured by the specs since resource mapping may be affected. Therefore, the procedure of inserting gap should be specified in 36.211 using similar description as legacy transmission gap.

Summary of change: Add the description of the resource mapping for PUSCH.

Consequences if not approved: No support of NB-IoT/eMTC support for Non-Terrestrial Networks.

Proposal 1: Adopt text proposal 1 in section 2.1.

------------------------------------------ Start of Text Proposal 1------------------------------------------ < Unchanged parts are omitted > For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331. BL/CE UL subframes within the gap of Ngapprecompensation time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe , one slot, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. For BL/CE UEs in CEModeB, for PUSCH transmission not associated with Temporary C-RNTI, for frame structure type 1, after a transmission duration of time units (which may include subframes that are not BL/CE UL subframes), a gap of time units shall be inserted, according to the UE capability ue-CE-NeedULGaps, as specified in 3GPP TS 36.331 [9]. BL/CE UL subframes within the gap of time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH. < Unchanged parts are omitted > -------------------------------------------- End of Text Proposal 1-----------------------------------------

Text proposals for long UL transmission on PRACH

Reason for change: In RAN1#107e, the following agreements on long UL transmission on PRACH had been made [1].

Agreement For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples - UE may blank subframe / repetition unit where UE drops a subframe / repetition unit The total transmission time is not changed FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit FFS Specification impact Agreement For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble. - The total transmission time is not changed - FFS Details of method(s) to drop / insert samples

For the methods that UE may blank subframe / repetition unit where UE drops a subframe / repetition unit, it needs to be captured by the specs since resource mapping may be affected. Therefore, the procedure of inserting gap should be specified in 36.211 using similar description as legacy transmission gap.

Summary of change: Add the description of the resource mapping for PRACH.

Consequences if not approved: No support of NB-IoT/eMTC support for Non-Terrestrial Networks.

Proposal 2: Adopt text proposal 2 in section 2.2.

< Unchanged parts are omitted > -------------------------------------------- Start of Text Proposal 2---------------------------------------- For a UE communicating over NTN, after transmissions and/or postponements due to NPRACH of Nsegmentprecompensation time units, for frame structure type 1, a gap of Ngapprecompensation time units shall be shall be counted for the NPUSCH resource mapping but not used for transmission of the PUSCH according to the UE capability ue-NBIOT-NeedSegmentedPrecompensationGaps , as specified in 3GPP TS 36.331. UL slots within the gap of Ngapprecompensation time units shall be counted for the NPUSCH resource mapping but not used for transmission of the NPUSCH. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is one subframe, one slot or 2 slots, or one symbol may be configured by network based on the quantity supported by the UE if indicated by UE in ue-CE-NeedSegmentedPrecompensationGaps. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. NPRACH gaps as defined in clause 10.1.6.1 are not part of the NPRACH resource. For frame structure type 2, the valid uplink subframes which are not used for NPRACH transmission when it is not possible to map G symbol groups back-to-back are not part of the NPRACH resource. The mapping of is then repeated until slots have been transmitted. After transmissions and/or postponements due to NPRACH of time units, for frame structure type 1, a gap of time units shall be inserted where the NPUSCH transmission is postponed. The portion of a postponement due to NPRACH which coincides with a gap is counted as part of the gap. < Unchanged parts are omitted > -------------------------------------------- End of Text Proposal 2-----------------------------------------

Conclusion

As summary, we have the following proposals.

Proposal 1: Adopt text proposal 1 in section 2.1.

Proposal 2: Adopt text proposal 2 in section 2.2.

Reference

3GPP RAN1#107-e Chairman’s Notes, November 11th – 19th, 2021.