3GPP TSG RAN WG1 #109-e		R1-2203317

e-Meeting, May 9th – 20th, 2022

Agenda Item:  8.16.3

Source:	Spreadtrum Communications

Title:	UE features for enhanced IIoT and URLLC

Document for:	Discussion and decision

Introduction

In this contribution, we gives our considerations on the UE features for enhanced IIoT and URLLC, based on the updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e [3].

Discussion

25-1: SPS HARQ-ACK deferral in case of TDD collision

- Prerequisite FG

  - Prerequisite feature groups should include FG 5-18 and 12-6, or FG 12-6 only.

    - Agree with the intention that SPS HARQ-ACK deferral only useful when SPS PDSCH using small periodicity. So FG 12-6 should be prerequisite. In additional, we are fine for choose FG 5-18 and 12-6, or FG 12-6 only.

- Type

  - Per UE.

25-2 to 25-3b: PUCCH Repetition enhancements

- Prerequisite FG

  - For FG 25-2, prerequisite feature group is 4-23

    - Since FG4-23 is the basic feature of slot based repetition, though it is for PUCCH format 1/3/4

  - For FG 25-3, prerequisite feature group are 4-23 and 11-3

  - For FG 25-3a, prerequisite feature group are 25-3 and 30-5

  - For FG 25-3b, prerequisite feature group are 25-3

- Type

  - For FG 25-2, our preference is per FS, although its prerequisite FG 4-23 is per UE. However, PUCCH format 0/2 is short PUCCH duration, the application cases are limited, such as it make sense to do repetition for URLLC traffic. However, it could not make sense to support this for UL coverage enhancement. This is also the reason that PUCCH format 0/2 does not support repetition in Rel-15, due to the limitation of this application.

  - Per FS for 25-3/3a/3b, to align with the granularity of the prerequisites, because FG 11-3 is per FS.

25-4 to 25-7: Retransmission of cancelled HARQ-ACK

FG 25-4:

- Prerequisite FGs are 10-16 and 11-1

- Type: Per band, to keep consistence with FG 10-16.

FG 25-5:

- Prerequisite FGs are 10-16 and 11-4

- Type: Per FS, to keep consistence with FG 11-4.

FG 25-6:

- Type: Per band, to keep consistence with FG 10-16.

FG 25-7:

- Prerequisite FGs: N/A

- Type:

  - FG 25-7: Per band. Similar as enhanced Type 3 HARQ-ACK CB, HARQ-ACK re-transmission also need to be per band.

25-8: Semi-static HARQ-ACK codebook for sub-slot PUCCH

- Prerequisite FGs: 4-11 and 11-3

- Type: per FS, to keep consistence with FG 11-3

25-9 to 25-10a: PUCCH cell switching

- Components:

  - According to [or FGs 22-6 or 22-6a] part, it can be confirmed. Because FG 22-6 and 22-6a support up to three or four different numerologies of one PUCCH cell group. Considering PUCCH cell switching operation, since it is only configured to do switching between two cell, thus FG 22-6 and 22-6a have similar UE implementation as FGs 6-9 and 6-9a.

  - According to FFS whether/how to indicate the capability for support of PUCCH cell switch in two PUCCH groups, we prefer to have separate FGs. The original FG 25-9 and 25-10/10a are for PUCCH cell switching in one PUCCH group, not support switching within two PUCCH groups. The new separate FGs, such as named as FG 25-9a, FG 25-10b, FG 25-10c, they indicate supporting of PUCCH cell switching in two PUCCH groups.

    - FG 25-9a: Semi-static PUCCH cell switching in two PUCCH groups, if support 22-7

    - FG 25-10b: PUCCH cell switching based on dynamic indication for same length of overlapping PUCCH slots/sub-slots in two PUCCH groups.

    - FG 25-10c: PUCCH cell switching based on dynamic indication for different length of overlapping PUCCH slots/sub-slots in two PUCCH groups

25-14 to 25-15: PHY prioritization of overlapping DG-PUSCH and CG-PUSCH with different priorities

- Prerequisite FGs: 12-1.

- Type: per FS, as prerequisite FG.

25-16: Intra-UE multiplexing with different priorities

- Prerequisite FGs: 11-3. We do not think 12-1 is a prerequisite. At least for capability 1, a UE does not need to partial cancel LP PUCCH/PUSCH.

- Type: per FS, to keep consistence with FG 11-3

Conclusions

In this contribution, we give our considerations for UE features of enhanced IIoT and URLLC.

References

R1-2202928	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2201514	Summary on UE features for enhanced IIoT and URLLC	Moderator (NTT DOCOMO, INC.)