3GPP TSG RAN WG1 #109-e		R1-2203318

e-Meeting, May 9th – 20th, 2022

Agenda Item:  8.16.8

Source:	Spreadtrum Communications

Title:	UE features for NR coverage enhancement

Document for:	Discussion and decision

Introduction

In this contribution, we gives our considerations on the UE features for NR coverage enhancement, based on the UE feature list was approved during RAN1 #108-e meeting [1].

Discussion

30-1 to 30-3a

- FG 30-2 Component

  - According to [K], we think it can be change into K>1, and remove the [], based on the below agreement achieved in RAN1#108e.

Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected.

- Prerequisite

  - FG 30-2: Regarding [11-6, 30-1], they can be removed. Because neither 11-6 nor 30-1 is the necessary requirement for available slot based repetition.

  - FG 30-3: FG 11-6. Because TBoMS can be indicated by TDRA in DCI which TDRA table is configured with TBoMS resource allocation

  - FG 30-3a: FG 30-3.

30-4 to 30-4h: DM-RS bundling

- Type:

  - FGs 30-4 is per band. Since DM-RS bundling cannot be implemented for some band combination.

  - FGs 30-4x: per band per BC. Agree with the explanation that DMRS bundling can be done in some bands, but not in these band combinations.

- Prerequisite

  - FG 30-4: N/A

  - FG 30-4a: FG 30-4 and one of {5-14, 5-17, 5-16}.

    - Do not need FG 30-1 or 30-2. DM-RS bundling can be supported without larger PUSCH repetition number, or available slot.

  - FG 30-4b: FG 30-4 and 11-5.

    - Do not need 30-1. Because FG 30-1 is for PUSCH repetition type A, it is not related with PUSCH repetition type B.

  - FG 30-4c: confirm FG 30-4 and 30-3

  - FG 30-4d: confirm FG 30-4 and 4-23

  - FG 30-4e: confirm 30-4a or 30-4b, or 30-4c

  - FG 30-4f: confirm 30-4d

Conclusions

In this contribution, we give our considerations for UE features of UL coverage enhancement.

References

R1-2202928	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2202929	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e including remaining RAN1 issues	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2201517	Summary on UE features for NR coverage enhancement	Moderator (NTT DOCOMO, INC.)