3GPP TSG RAN WG1 #109-e		  R1- 2203323

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.1.3.1

Source:	Spreadtrum Communications

Title:	Discussion on increased number of orthogonal DMRS ports

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new WI on MIMO Evolution for Downlink and Uplink in Rel-18 has been approved [1]. In this WI, increasing the maximum number of DMRS ports is part of the objectives, which can be found below.

Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS

In this contribution, we provide our view on how to increase the number of orthogonal DMRS ports.

Discussion

According to the WID, increasing the maximum number of DMRS ports should not increase to DMRS overhead, and the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS. Therefore, Rel-18 DMRS enhancement by increasing the maximum number of DMRS symbols is out of scope.

In our views, there are two possible solutions to increase DMRS ports based on modification of current DMRS design. The first one is considered as FDM solution which is to equally divide the current CDM group into two new CDM groups. In each new CDM group, the RE pattern for each antenna port remains comb structure. The illustration of the FDM solution can be found in Figure 2-1 and Figure 2-2 as below.

(a) Current CDM groups vs. (b) New CDM groups

Figure 2-1. FDM solution for DMRS Type 1.

(a) Current CDM groups vs. (b) New CDM groups

Figure 2-2. FDM solution for DMRS Type 2.

The second solution is considered as FD-CDM4 solution which is to introduce length-4 FD-CDM for each of the current CDM group. For DMRS Type 2, there are 4 REs within a PRB for each CDM group, it is straight forward to apply the length-4 FD-CDM. While for DMRS Type 1, there are 6 REs within a PRB for each CDM group. Therefore, it is not possible to apply length-4 FD-CDM within a PRB. The possible way is to divide the REs of a pair of PRBs for each CDM group into 3 RE groups. The length-4 FD-CDM can be applied to the 4 REs within each RE group. The illustration of the FD-CDM4 solution can be found in Figure 2-3 and Figure 2-4 as below

- FD-CDM2 vs. (b) FD-CDM4

Figure 2-3. FD-CDM4 solution for DMRS Type 1.

- FD-CDM2 vs. (b) FD-CDM4

Figure 2-4. FD-CDM4 solution for DMRS Type 2.

Before we do the down selection, there are several aspects which needs to be further studied:

- Performance aspect: FDM solution increases the number of DMRS ports by reducing the number of REs for each DMRS port, it can be expected that the performance will degraded. On the other hand, FD-CDM4 solution multiplexes more DMRS ports by introducing FD-CDM4, the large CDM operation will also bring negative impact on the performance. Evaluation is needed to identify which solution has the better performance.

- Signaling aspect: Both solutions will increase the bit length of antenna port(s) field in DCI. Comparing with FD-CDM4 solution, FDM solution will introduce more CDM groups. For the same DMRS port combination, more candidate values will be added into the antenna port(s) table to indicate different number of CDM group(s) without data. Thus, the signaling overhead of antenna port(s) field for FDM solution may be larger than that for FD-CDM4 solution.

- Backward compatibility: For FDM solution, the legacy UE and Rel-18 UE can be scheduled with antenna ports from different CDM groups. For FD-CDM4 solution, the legacy UE and Rel-18 UE can also be scheduled with antenna ports from different CDM groups. Besides, the FD-CDM4 sequences [+1 +1 +1 +1] and [+1 -1 +1 -1] can be used for legacy UE by legacy signaling. Therefore, FD-CDM4 solution may have better backward compatibility than FDM solution.

- Other aspects:

  - Since FDM solution will increase the number of CDM groups, if it is combined with low PAPR DMRS sequence specified in Rel-16, some further enhancement is needed.

  - Since FD-CDM4 solution for DMRS Type 1 will perform FD-CDM4 across PRB, how to pair the RBs within the scheduled PDSCH needs further study. Besides, there may be some potential issue if the number of RBs within the scheduled PDSCH is odd.

Proposal 1: For DMRS enhancement, support either FDM solution or FD-CDM4 solution.

Conclusion

In this contribution, we gave our views on the potential issues on multi-beam enhancements. The following proposals are achieved:

Proposal 1: For DMRS enhancement, support either FDM solution or FD-CDM4 solution.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)