3GPP TSG RAN WG1 #109-e                                                   R1-2203324

e-Meeting, May 9 – May 20, 2022

Agenda Item:     9.1.3.2

Source:	Spreadtrum Communications

Title:	Discussion on SRS enhancement targeting TDD CJT and 8 TX operation

Document for:	Discussion and decision

Introduction

In the WID of Rel-18 MIMO Evolution for Downlink and Uplink, the objectives concerning SRS enhancement are list below:

Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32 Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this contribution, we provide our view on SRS enhancement.

SRS capacity enhancement

In Rel-18, CJT is one of the main objectives for MIMO evolution. Compared with NCJT, CJT can achieve higher performance due to the deep cooperation among TRPs. The studies of CJT/NCJT can date back to LTE era though not specified. The network can rely on SRS to estimate its relevant DL channels in TDD. Consequently, increasing the performance of receiving SRS can also ameliorate CJT. Thus, it is beneficial to enlarge the SRS capacity and minimize the inter-TRP SRS interference.

Currently, the SRS capacity depends on KTC and nSRScs,maxas shown in Table.1. With the premise of reuse existing SRS comb structure, one way is to increase the number of nSRScs,max. We recommend to study increasing  nSRScs,max for each value of  KTC, e.g., increase nSRScs,max to 12 for both KTC=2, 8.

Another method is to further explore the code domain enhancement, e.g., TD-OCC.  Without consuming more time-frequency resources, more SRSs belonging to same/different UEs can be simultaneously delivered via TD-OCC. However, some issues needs to be carefully studied. First, though TD-OCC can increase SRS capacity, the interference also becomes more severe which may degrade CJT performance. Second, for high speed scenario the performance of TD-OCC should be justified. Third, legacy SRS hopping and SRS TD-OCC seems to be difficult to co-exist.

Table.1 Maximum number of cyclic shifts nSRScs,max as a function of KTC

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

Proposal 1: Study the increase of nSRScs,max for SRS.

Proposal 2: Support to introduce SRS TD-OCC for SRS capacity enhancement, and meanwhile investigate the relevant impact with the following aspects:

- Interference impact

- High speed scenario

- Co-existence with hopping

SRS interference randomization

Though Rel-15/16/17 SRS is UE-specifically designed, it can hardly handle the inter-TRP SRS interference under mTRP. There are two main categories to minimize the inter-TRP SRS interference.

The first orientation to create randomization on sequence level. The SRSs transmitted toward different TRP can have different sequence randomizing parameters. The other orientation inherits the idea of LTE CRS, where CRS belonging to different serving cell can have different shifts. Similarly, in Rel-18 we can study the TRP-specific SRS shift which instead creates randomization for SRS locations among TRPs.

Proposal 3: Per-TRP SRS sequence-randomization/shifts can be studied in Rel-18.

8 Tx UL operation

The Rel-18 SRS enhancement also involves increasing SRS maximum Tx number from 4 to 8. From our perspective, the maximum port per-symbol should be maintained which is 4 per-symbol which could be a baseline assumption in Rel-18 SRS enhancement. If we want to map SRS ports>4 in one symbol, the inter-port interference increases and the number of REs per-port also decreases. Both would bring negative effect to SRS reception/estimation. Therefore, we want to maintain the 4 SRS-port per symbol as a baseline assumption. Additionally, such constraint is also beneficial for co-existence with legacy SRS resources.

Proposal 4: Maintain the maximum ports per-symbol for 8Tx SRS, i.e., 4 SRS-port per symbol.

Conclusion

In this contribution, we provide our view on SRS enhancement in Rel-18. The following proposals are achieved:

Proposal 1: Study the increase of nSRScs,max for SRS.

Proposal 2: Support to introduce SRS TD-OCC for SRS capacity enhancement, and meanwhile investigate the relevant impact with the following aspects:

- Interference impact

- High speed scenario

- Co-existence with hopping

Proposal 3: Per-TRP SRS sequence-randomization/shifts can be studied in Rel-18.

Proposal 4: Maintain the maximum ports per-symbol for 8Tx SRS, i.e., 4 SRS-port per symbol.

References

[1] RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)