3GPP TSG RAN WG1 #109-e		R1-2203325

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.1.4.1

Source:	Spreadtrum Communications

Title:	             Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and decision

Introduction

In light of R18 MIMO WID [1], the following objective is included in the scope:

6. Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In this paper, we will present our opinions on simultaneous multi-panel UL transmission for both S-DCI and M-DCI based M-TRP operation.

Discussion

S-DCI based M-TRP

PUSCH

In Rel-17, to improve the reliability of PUSCH, S-DCI based M-TRP TDMed PUSCH transmission is supported for both FR1 and FR2. In details, two SRS resource sets would be configured, each associated with one TRP. Two SRI(s), two TPMI(s), and two TPC(s) are given by the DCI fields to realize transmission parameters determination for each TRP separately, to better match with the communication link.

In Rel-18, since simultaneous multi-panel UL transmission for higher UL throughput/reliability can be assumed, SDMed PUSCH transmission can be considered, where higher throughput can be expected.

Proposal 1: For S-DCI based M-TRP operation, SDMed PUSCH transmision can be considered.

Regarding FDMed transmission, it seems to be that lower latency can be achieved compared with TDMed transmission. However, the benefit may be marginal, for the sake of PUSCH repetition TypeB also supported in Rel-17. On the other hand, the performance gain is not clear if comparing with SDMed PUSCH transmission. The justification of supporting FDMed PUSCH transmisison is needed to be further verified.

PUCCH

In Rel-17, one PUCCH resource can be configured with two spatial information or two sets of power control parameters, to enable TDMed PUCCH repetition transmission across M-TRP.

In Rel-18, since simultaneous PUCCH transmission across multiple panel is allowed, it is possible to support SDMed and FDMed PUCCH transmission. From our side, since PUCCH is one-port transmission, unlike multiple layer transmission for SDMed PUSCH, SDMed transmission for PUCCH can be thought to be PUCCH repetition in spatial domain. If so, SDMed PUCCH transmission can be considered, where resource overhead can be reduced while the reliability does not loss compared with TDMed transmisison.

Proposal 2: SDMed PUCCH transmission across multiple panels can be considered.

Regarding FDMed transmission, compared with TDMed transmission, indeed lower latency can be achieved. However, the benefit may be marginal, for the sake of intra-slot PUCCH repetition also supported in Rel-17. For PUCCH resource with PUCCH format0/1/4, only one PRB is allocated, and related OCC code is defined based on the length of one PRB. Thus, it is not clear about how to support FDMed PUCCH transmission for one PUCCH resource.

M-DCI based M-TRP

For M-DCI based M-TRP, only simultaneous PUCCH transmission or PUSCH transmission across multiple panels are allowed in Rel-18. However, since the backhaul between multiple TRPs may not be ideal, the cases that different types of channels/signals overlap in time could take place. One illustration is shown below in Figure 1, where PUCCH, PUSCH and SRS for different TRPs are overlapped in time domain.

Figure 1 one illustration on channels/signals for different TRPs overlapping

Thus, how to select one channel/signal to transmit and drop another for overlapping cases should be considered. For example, learning from Rel-15/Rel-16/Rel-17, in our opinion the following principles can be considered:

  - Priority index of channel/signal: The larger the priority index is, the higher the priority is. For example, for case (a) in Figure 1, assuming the priority index of PUCCH is 0 and the priority index of PUSCH is 1, then PUCCH can be dropped and PUSCH would be transmitted.

  - The type of channel/signal. For example, for case (b) in Figure 1, assuming the priority index of PUSCH is 0, SRS can be dropped.

  - The information of the channel carrying. For example, for case (c) in Figure 1, assuming priority index for overlapping channel/signal is the same, drop AP SRS when PUCCH carrying HARQ-ACK, link recovery request and/or SR; transmit AP SRS when PUCCH carrying semi-persistent/periodic CSI report(s) or semi-persistent/periodic L1-RSRP report(s) only, or only L1-SINR report(s).

With what we have said, other principles are also not precluded. The solutions are use case specific.

Proposal 3: When different types of channels/signals each for different TRP are overlapping in time domain, the dropping mechanisms should be enhanced.

Figure 2 one illustration of two PUCCHs or PUSCHs each for different TRP overlapping

In Rel-15/Rel-16/Rel-17, for one PUCCH/PUSCH, the transmisssion power is calculated according to the same power control formulations in clause 7 of [2], which would not exceed the maximum allowed transmission power. However, as shown in Figure 2, when two PUCCHs each for different TRP overlap in time domain, the transmission power of each PUCCH can be achieved independently as the legacy way. However, there is possibility that the sum power of the two overlapping PUCCHs or PUSCHs may exceed the maximal allowed transmission power for one UE. Thus, enhancement on power control for M-DCI based M-TRP operation should be considered.

Proposal 4: When the total power from two overlapping PUCCHs or PUSCHs each for one different TRP exceeds the maximal allowed transmission power for UE, power control enhancement should be considered.

Conclusion

In this contribution, we provide our opinions on UL precoding indication for multi-panel transmission:

For S-DCI based M-TRP:

Proposal 1: For S-DCI based M-TRP operation, SDMed PUSCH transmision can be considered.

Proposal 2: SDMed PUCCH transmission across multiple panels can be considered.

For M-DCI based M-TRP:

Proposal 3: When different types of channels/signals each for different TRP are overlapping in time domain, the dropping mechanisms should be enhanced.

Proposal 4: When the total power from two overlapping PUCCHs or PUSCHs each for one different TRP exceeds the maximal allowed transmission power for UE, power control enhancement should be considered.

Reference

- RP-213598, MIMO Evolution for Downlink and Uplink

- 3GPP TS 38.213: "NR; Physical Layer Procedures for control".