3GPP TSG RAN WG1 #109-e		R1-2203326

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.1.4.2

Source:	Spreadtrum Communications

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and decision

Introduction

In the WID of Rel-18 MIMO Evolution for Downlink and Uplink, the objective concerning SRI/TPMI enhancement for enabling 8 TX UL transmission is list below [1]:

Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In this paper, we will present our opinions on SRI/TPMI enhancement for enabling 8 TX UL transmission.

Discussion

SRI enhancement for enabling 8 TX UL transmission

The bit width of SRS resource indicator in DCI format 0_1 and 0_2 is  log2k=1minLmax, NSRSNSRSk and  log2k=1minLmax, NSRS,0_2NSRS,0_2k for non-codebook based transmission according current specification, respectively [2]. Obviously, the maximum bit width is 4 bits when Lmax is 4 and NSRS or NSRS,0_2 is 4.

When 8 TX UL transmission is enabled, the maximum value of both Lmax and NSRS or NSRS,0_2 could be 8, then follow the design principles of existing specification, namely, all possible combinations of no more than Lmax SRS resources are selected from NSRS or NSRS,0_2 SRS resources configured in the SRS resource set, we can simply calculate that the maximum bit width will be 8 bits.

Observation 1: Extending the SRI indication for non-codebook directly will introduces additional 4bit DCI overhead.

Considering the high overhead introduced by direct enhancement, further study is needed to reduce DCI overhead for non-codebook-based transmission when 8 TX UL transmission is enabled. Based on the possible deployment of UE antenna, e.g. multi-panel for 8 TX, a feasible method is limit the selection of SRS resource for part of layers having large spatial differences, such as limiting the first four layers to the first four SRS resources. DCI overhead reduction for SRS resource indicator should be considered and the potential methods to reduce DCI overhead should further study.

Proposal 1：Potential methods to reduce DCI overhead should further study.

TPMI enhancement for enabling 8 TX UL transmission

For codebook based transmission in NR system, the design principle or reference of precoding matrix is different for different waveforms or number of antenna ports. For example, the precoding matrix for 2/4 antenna ports using DFT-s-OFDM is based on single-layer precoding matrix using 2/4 antenna ports in LTE UL transmission, while the precoding matrix for 2 antenna ports using CP-OFDM is based on two-layer precoding matrix using 2 antenna ports in LTE DL transmission. In addition, the precoding matrix for 4 antenna ports using CP-OFDM is based on NR type I codebook for DL transmission [3].

Codebook design principle for 8 antenna ports targeting CPE/FWA/vehicle/Industrial devices should be considered firstly when TPMI enhancement is carried out. In our view, LTE householder matrix Wn=I-2ununH/unHun, NR DL Type I Single-Panel codebook for 8-layer transmission Wl,l',l",m,m',m",n8=18Pvl,mφnvl,mvl,m-φnvl,mvl',m'φnvl',m'vl',m'-φnvl',m'vl",m"vl",m"vl",m"-vl",m"vl",m"vl",m"vl",m"-vl",m", or other possible combinations of existing precoding matrices, e.g. W4W4φW4-φW4 can be considered, where W4 can be NR UL precoding matrix for 4-layer transmission.

The design of UL codebook should focus on low transmission overhead and less design work. Generally speaking, the overhead of LTE householder matrix is low but more codebook design work is required to expansion to 8 antenna ports, while reuse NR DL codebook, even the Type I Single-Panel codebook, will introduce high overhead and some simplifications are needed. In addition, codebook based on the possible combination of existing precoding matrices needs evaluate the performance. Different codebook design methods are open for us and need to be future evaluated or improved.

Based on the above discussion, our proposal is as follows.

Proposal 2: The following codebook design principle for 8 antenna ports can be considered:

- Option 1: based on LTE householder matrix

- Option 2: based on NR DL codebook for 8-layer transmission

- Option 3: other possible combination of existing precoding matrices

Conclusion

In this contribution, we discussed SRI/TPMI enhancement for enabling 8 TX UL transmission. The following proposals are achieved:

Observation 1: Extending the SRI indication for non-codebook directly will introduces additional 4bit DCI overhead.

Proposal 1：Study the potential methods to reduce DCI overhead, e.g. limit the selection of SRS resource for part of layers.

Proposal 2: The following codebook design principle for 8 antenna ports can be considered:

- Option 1: based on LTE householder matrix

- Option 2: based on NR DL codebook for 8-layer transmission

- Option 3: other possible combination of existing precoding matrices

Reference

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, RAN#94, Samsung (Moderator)

- 3GPP TS 38.212 v17.0.0.

- 3GPP TS 38.211 v17.0.0.