3GPP TSG RAN WG1 #109-e                            R1-2203328

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.3.2

Source:	Spreadtrum Communications

Title:	Discussion on subband non-overlapping full duplex

Document for:	Discussion and decision

Introduction

In RAN#94e, SI on evolution of NR duplex operation was approved [1]. As described in the SID, study on subband non-overlapping full duplex will be conducted in Rel-18 as shown below.

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss the possible solutions related to subband non-overlapping full duplex (SBFD).

Discussion

Subband non-overlapping full duplex is a key scheme of duplex enhancement in Rel-18 to provide uplink latency reduction, coverage enhancement and spectrum efficiency improvement. Severe co-channel cross link interference (CLI) introduced in full overlapping full duplex is replaced by subband CLI in SBFD with the help of separation of downlink subband and uplink subband. Considering different subband configurations of SBFD, CLI type varies and CLI analysis is necessary for different scenarios. During Rel-15, the CLI interference mitigation schemes for duplexing flexibility were initially discussed. In Rel-16, UE-to-UE CLI measurement on SRS-RSRP and CLI-RSSI was introduced [2] and for gNB-to-gNB CLI slot configurations sharing among gNBs were provided via backhaul signaling in CLI handling and RIM for NR[3]. Regarding the new feature of SBFD, relevant CLI enhancement should be studied in Rel-18. In sum, we think the study of for SBFD should focus on aspects like subband configuration, CLI analysis for different scenarios and CLI handling.

Proposal 1: Focus on the study of the following aspects of SBFD

- Subband non-overlapping duplex schemes

- CLI analysis

- CLI handling (gNB/UE)

Subband non-overlapping duplex schemes

In this section, we provide some technical aspects of subband non-overlapping full duplex schemes.

Slot configuration and dynamic/flexible slot format in SBFD

Currently, dynamic TDD on an unpaired spectrum allows dynamic changing of the transmission direction in time domain. Specifically, DL and UL transmission directions can be dynamically assigned in a TDMed manner via SFI indication or grants in DCI. In Rel-18, duplex enhancement is only deployed at the gNB side and half duplex operation is still assumed at the UE side.

For example, SBFD is used at gNB side with DUD subband configuration in slot 1-3. However, due to the half-duplex limitation UE1 receives downlink signal in slots 0/1/2 and transmit uplink signal in slot 3/4 according to slot configuration DDDUU and UE2 transmits uplink signal in slot 1, 2 and 4 and receives downlink signal in slot 0 and 3 according to slot configuration DUUDU shown in Figure 1. In this case, gNB can simultaneously scheduling downlink UE in downlink subbands and uplink UE in uplink subband with SBFD configuration on slots 1/2/3.

Figure 1: dynamic/flexible TDD slot configuration for gNB and UEs

Therefore, legacy slot configuration or with enhancement should be taken into account for SBFD configuration.

Observation 1: Legacy slot configuration and dynamic slot format can be used to support SBFD as the starting point.

Transparent or non-transparent SBFD for UE

Obviously, gNB is aware of all DL/UL directions on DL and UL subbands, one question is whether a UE knows the deployment of SBFD or not. According to this issue, there are two options.

- Option 1: Transparent SBFD for UE. UE only knows its own direction

- Option 2: Non-transparent SBFD for UE. UE knows all the DL/UL directions on DL and UL subbands.

For Option 1, it is a straightforward solution from the UE’s point of view. First, it can reuse all the DL receptions and UL transmission collisions defined in 38.213. There is no change from the UE side. Such as if configured with or scheduled the DL reception, UE only do the DL channel reception on the DL BWP. It does care about whether there is some UL transmission from another UE meanwhile within the frequency domain resource of its DL BWP. Second, this option requires the gNB to take care of the scheduling cautiously, such as there would be no obscure DL/UL judgement, i.e. UE does not expect to do semi-static DL reception and semi-static UL transmission simultaneously, or dynamic DL reception and dynamic UL transmission in at least one overlapping symbol. Fortunately, those two ambiguous cases are not allowed in the current specification, it can reuse this limitation for SBFD. For the remaining collisions, there would be no problem based on the priority between DL and UL direction. So SBFD can be realized transparently to UE via scheduling

For Option 2, the non-transparent method is beneficial for the scheduling flexibility from the gNB side of view. Such as it allows the scheduling partial overlapping with DL subband and UL subband, which is not expected even from gNB side of view. Thus, UE has to cancel the DL reception or UL transmission under this new SBFD DL/UL collision. However, it needs all subband configurations with transmission direction are informed to UEs, UE is possible to decide to transmit or receive in the relative subband. But it may consume large amount of overhead considering the potential frequent change of information for subband configuration, e.g., subband number, subband size, etc. Especially, it brings new UE behaviors when deciding DL or UL direction.

Observation 2: Transparent SBFD for UE is the most straightforward scheme. Non-transparent SBFD for UE requires new UE behavior of DL/UL direction determination.

Concept of DL subband and UL subband

Regarding the concept of subband, it needs study that a new definition of subband can be introduced or reuse the existence definitions such as DL BWP and UL BWP. Thus, there would be two alternatives for the concept of DL/UL subband.

- Alt1: Not define DL/UL subband for UE, reuse the DL/UL BWP in SBFD scheme

- Alt2: New concepts of DL/UL subband are defined for UE

For Alt1, subband is only a virtual frequency resource region from gNB point of view, but UE does not know any ideal of these subbands division. UE only knows DL/UL BWP pair configurations as same as Rel-15 and DL/UL BWP pair still share the same center frequency as shown in Figure 2. In addition, the UL/DL BWP size is not dynamic changed according to the use of SBFD.

Figure 2: Reuse DL/UL BWP

For Alt2, besides DL/UL BWP pair, DL/UL subband are defined additionally. Based on this new concept, UE can derive the exact region of DL and UL frequency resource. Therefore, the DL and UL regions can be different from DL/UL BWP, which means it provides more DL/UL direction information to UE. However, the necessary of this new concept should be provided first. In Figure 3, DL/UL subbands are defined in SBFD region, in this case UE can use the exact frequency resource in the corresponding subband region according to its slot configuration.

Figure3: Define DL/UL subband in SBFD

Proposal 2: Whether DL/UL subband is defined or not can be further studied for SBFD scheme.

Divisional DL and UL subbands

Non-overlapping subbands are one part of frequency resources within a carrier from gNB perspective. It need to study the divisional granularity. Such as the subband size, the number of DL and UL switching points in the frequency and time domain.

Regarding the subband size, it can be RB granularity, such as the continuous RBs for UL subband. A similar concept to subband is the RB-set in NR-U, which could be further studied whether some specific size should be used for subband of SBFD.

For the number of DL/UL switching points in the frequency and time domain, it needs to study some restrictions to simplify the SBFD implementation. For example, whether allowed one or two switching points for a carrier if DL/UL subbands are known to UE. The advantage of the restriction is it can simplify the CLI analysis. And it can achieve the objective of UL coverage, UL latency reduction. Next, it can reduce the payload of signaling. In addition, it can lower UE implementation for SBFD. Last, this can improve spectrum efficiency considering the unusable resources between DL/UL subbands.

Proposal 3: Subband size and the number of DL and UL switching points in the frequency and time domain need further study.

Figure 4: Different number of switch points in frequency and time domain

Guard band between DL/UL subbands

To alleviate the impact of intra/inter-subband CLI, guard band is necessary to effectively mitigate the interference from another transmission direction. Similar discussion concerning guard band has been done in NR-U where guard band is to separate different LBT channel bandwidth. The major difference is the guard band of NR-U can still be used to transmit data where such case might not be allowed in SBFD. Anyway, we can consider the configuration of NR-U guard band as a starting point.

Proposal 4: The Guard band between DL/UL subbands needs further study.

CLI analysis for SBFD

According to interference handling at gNB, there are self-interference (denoted as SI), co-channel gNB-to-gNB inter/intra subband CLI and adjacent channel gNB-to-gNB CLI related to SBFD. The different types of inter-gNB CLI are shown in Figure 5.

Figure 5: Interference at gNB side

- SI is the interference from DL subband transmission to UL subband reception in the same gNB. SI mitigation capability is the prerequisite of SBFD and should be evaluated in the study.

- Co-channel gNB-to-gNB inter subband CLI exists when two gNBs are configured with the same subband configuration which is the interference from DL subband transmission of one gNB to UL subband reception of another gNB. Referring to subband non-overlapping full duplex, DL subband and UL subband are FDMed and this kind of interference is called inter subband CLI.

- Co-channel gNB-to-gNB intra subband CLI exists when two gNBs are configured with different subband configurations or in the co-existence case between SBFD and legacy operation as shown in Figure 5 (3) and (3) b, respectively. This kind of CLI only occurs when the UL subband and DL subband are overlapped in different gNBs and it is the interference from DL (subband) transmission of one gNB to UL (subband) reception of another gNB on the overlapping resource in both subbands.

- One gNB suffers adjacent-channel gNB-to-gNB CLI in the co-existence case of gNB from another operator deployed in adjacent channel. Adjacent channel CLI is studied in Rel-16.

In SBFD, the interfered resources is in one subband instead of in the whole BWP. The degradation of performance caused by subband adjacent channel CLI should be evaluated in Rel-18.

At UE side, there are co-channel UE-to-UE inter/intra subband CLI and adjacent channel CLI in SBFD scenarios. Half duplex operation is assumed at UE side, and thus no SI is taken into account.

- Reception in DL subband of one UE suffers interference from transmission of UL subband of another UE when the subband configuration of these two UEs is the same shown in Figure 6 (1) called Co-channel UE-to-UE inter subband CLI.

- Overlapping of DL subband and UL subband between two UEs causes intra subband CLI when two cells are configured with different subband configurations or in the co-existence case between SBFD and legacy operation. Specifically, it only exists in inter cell UEs.

- Adjacent channel UE-to-UE CLI is the interference suffered in downlink signal reception of legacy UE from uplink signal transmission in uplink subband of SBFD UE belonging to another operator in adjacent channel.

Figure 6: Interference at UE side

Taking adjacent channel CLI modeling for example, ACLR/ACS were used to model the interference at Tx/Rx side. ACLR/ACS are defined as the attenuation over one bandwidth of adjacent frequency bands. But for SBFD, given the different locations of subband, the interference modeling may be relative to the distance between resources of aggressor and victim. As shown in Figure 7, the influence of adjacent channel CLI to UL subband-0 and UL subband-1 may be different.

Figure 7: Adjacent channel interference caused by different subband

Observation 3: gNB-to-gNB SI, inter-subband CLI modeling and adjacent channel subband CLI modeling should be studied.

Observation 4: Interferences in dynamic/flexible TDD scenarios is a subset of that in SBFD scenarios, a unified CLI mechanism for dynamic/flexible TDD and SBFD can be studied.

For co-channel co-existence case in Figure 8 (a), one cell is deployed with legacy downlink dominant slot configuration such as DDDSU and SBFD is used in the neighboring cell.

Figure 8: Interference in co-existence case with SBFD

With different slot and subband configuration, there will have interference from the downlink slot of gNB2 to the uplink subband of gNB1, as described before, the interferences are inter-gNB inter subband and intra subband CLI. The SBFD operation has no impact on the legacy gNB. As to UE side, there will have inter subband and intra subband CLI from the uplink subband of SBFD UE to downlink slot of legacy UE in another cell. Similar to the case for adjacent channel, inter-gNB inter/intra subband CLI and inter cell UE-to-UE inter/intra subband CLI are replaced by subband adjacent channel CLI shown in Figure 8 (b).

Observation 5: For the impact on legacy operation, there is no impact to the legacy gNB, only impact to legacy UE should be considered in co-existence cases.

CLI handling (gNB/UE)

Although, there is no UE-side full duplex in Rel-18, however, there is some new inter-UE CLI cases due to SBFD. Thus, inter-UE CLI enhancement can also studied in order to support gNB-side full duplex. Currently, the UE-to-UE CLI measurement/report can be event-triggered or periodic. However, such CLI measurement/report is done via high layer signaling which causes too much delay. For SBFD with or without dynamic TDD, the interference model is even more complex and rapidly varies. A faster CLI measurement/report mechanism may be beneficial to handle this issue. Therefore, in Rel-18 the L1-based CLI measurement/report should be taken into account.

Proposal 5: L1-based CLI measurement/report for SBFD/dynamic TDD needs further study.

In Rel-16, gNB-to-gNB CLI handling has been discussed and only TDD slot configuration is shared among gNBs. In Rel-18, the SBFD scenario is different from legacy TDD and may require more information sharing among gNBs. Similar to our view on UE-to-UE CLI handling, dynamic CLI handling among gNBs can be studied involving measurement resource definition, CLI measurement, CLI report, etc.

Proposal 6: Subband-level information can be considered for gNB-to-gNB’s information sharing.

Proposal 7: Dynamic CLI measurement/report among gNBs needs further study.

Conclusion

Based on the analyses and discussions on SBFD, following observations and proposals are given:

Observation 1: Legacy slot configuration and dynamic slot format can be used to support SBFD as the starting point.

Observation 2: Transparent SBFD for UE is the most straightforward scheme. Non-transparent SBFD for UE requires new UE behavior of DL/UL direction determination.

Observation 3: gNB-to-gNB SI, inter-subband CLI modeling and adjacent channel subband CLI modeling should be studied.

Observation 4: Interferences in dynamic/flexible TDD scenarios is a subset of that in SBFD scenarios, a unified CLI mechanism for dynamic/flexible TDD and SBFD can be studied.

Observation 5: For the impact on legacy operation, there is no impact to the legacy gNB, only impact to legacy UE should be considered in co-existence cases.

Proposal 1: Focus on the study of the following aspects of SBFD

- Subband non-overlapping duplex schemes

- CLI analysis

- CLI handling (gNB/UE)

Proposal 2: Whether DL/UL subband is defined or not can be further studied for SBFD scheme.

Proposal 3: Subband size and the number of DL and UL switching points in the frequency and time domain need further study.

Proposal 4: The Guard band between DL/UL subbands needs further study.

Proposal 5: L1-based CLI measurement/report for SBFD/dynamic TDD needs further study.

Proposal 6: Subband-level information can be considered for gNB-to-gNB’s information sharing.

Proposal 7: Dynamic CLI measurement/report among gNBs needs further study.

References

- RP-213591, Study on Evolution of NR Duplex Operation, CMCC, RAN#94-e, December 6-17, 2021

- TR 38.215-g40, Physical layer measurements

- R1-1901374, Summary#3 of Issues on UE-UE CLI measurements and Network Coordination Mechanism, LG Electronics, RAN1#Ad-Hoc 1901, January 21– 25, 2019