3GPP TSG RAN WG1 #109-e                            R1-2203329

e-Meeting, May 9 – May 20, 2022

Agenda Item:	9.3.3

Source:	Spreadtrum Communications

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Document for:	Discussion and decision

Introduction

In RAN#94e, SI on evolution of NR duplex operation is approved [1]. As described in the SID, study on potential enhancements on dynamic/flexible TDD will be conducted in Rel18 shown as below

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we discuss the possible solutions related to potential enhancements on dynamic/flexible TDD.

Discussion

Dynamic TDD has been supported since Rel-15. However, it was not well implemented in practice, due to lack of solutions for inter-gNB CLI. A lot of previous work was done on dynamic/flexible TDD including:

- The feasibility of dynamic/flexible TDD in different deployment scenarios. Performance evaluation on indoor hotspot, dense urban and urban macro with/without cross-link interference mitigation were given and mitigation schemes including advanced receiver, hybrid dynamic/static resource assignment, coordination-based schemes and sensing based schemes were investigated.

- CLI handling for inter-cell UE-to-UE CLI from the Rel-16 SI and subsequent WI on CLI handling and RIM for NR. Long term UE-to-UE measurement based on L3 measurement/reporting

- Network coordination mechanisms for CLI was supported and specified. gNB side exchanging of DL/UL transmission direction configuration was supported via backhaul signaling.

Therefore, repeated work would not be preferred. Enhancements on dynamic/flexible TDD can focus on inter-gNB CLI mitigation and further signaling design, to better support this existing feature to be implemented in the reality, based on the current specified dynamic/flexible TDD mechanism.

Observation 1: Enhancements on dynamic/flexible TDD can focus on inter-gNB CLI mitigation with high priority and further signaling design, to better support this existing feature to be implemented.

In this section, it briefly discusses the scenarios for dynamic TDD enhancement, and then analyzes the CLI in different scenarios, at last proposes some considerations on CLI handling schemes.

Proposal1: Focus on the following aspects of dynamic/flexible TDD enhancement

- CLI analysis

- CLI handling (gNB/UE)

CLI analysis

Macro & indoor factory scenario focuses on the scenario of indoor factory with heavy uplink traffic load while the macro cell deployed outside with heavy downlink traffic. In order to solve the asymmetry of uplink and downlink services, UL dominant TDD configuration is used in factories and DL dominant TDD configuration is reserved in Macro cell shown in Figure-1. Thus, there will be some additional interferences.

Co-located scenarios

For co-located scenarios, since more UL slots are used in factory, the UL reception of factory gNB suffers severe co-channel interference from DL transmission of Macro gNB shown in Figure 1(a). And the UEs in

Figure 1 Macro & indoor factory scenario in co-channel

Macro cell suffer CLI from UEs transmitting UL signal in factory while receiving DL data in DL slot shown in Figure 1 (b). Co-channel CLI was evaluated in the scenarios of indoor hotspot, dense urban and urban macro in Rel-15 NR flexible duplex [2]. Considering the penetration loss between outdoor and indoor, the impact of co-channel CLI decreases making this HetNet scenario a reasonable scenario for dynamic TDD enhancement.

Adjacent channel scenarios

When the factory is deployed by one operator and the macro cell outside is deployed by another operator in adjacent channel, this scenarios become an adjacent channel co-existence case shown in Figure 2.

Figure 2 Macro & indoor factory scenario in adjacent-channel

With different UL-DL slot configurations, there will be adjacent channel interference from the downlink slot of Macro gNB to the uplink slot of the factory gNB shown Figure 2 (a). UEs of downlink slot in Macro cell also suffer adjacent channel interference from the uplink slot of factory UEs in Figure 2 (b). CLI in adjacent channel was studied in Rel-16 CLI and RIM, and performance evaluations were given by RAN4 [3] and UE-to-UE CLI measurements by SRS-RSRP/CLI-RSSI were standardized. Further study may be required for CLI handling between the gNBs to enable the dynamic TDD in commercial networks.

Flexible/dynamic TDD co-existence with legacy operation

According to co-existence case, from the discussion above, we can see there is no impact on the legacy gNB (Macro gNB) in co-channel and adjacent-channel co-existence case. Only impact to legacy UEs (in Macro cell) should be taken in account in co-existence case.

Observation 2: When the legacy gNB uses DL dominant TDD UL-DL configuration and Rel-18 duplex gNB uses UL dominant TDD UL-DL configuration, there is no impact to the legacy gNB, only impact to legacy UE should be considered.

Observation 3: Interferences in dynamic/flexible TDD scenarios is a subset of that in SBFD scenarios, a unified CLI mechanism for dynamic/flexible TDD and SBFD can be studied.

CLI handling

Cross link interference mitigation schemes based on advanced receiver, coordination, power control, link adaption, sensing and many more were investigated in Rel-15[2].

Firstly, gNB-to-gNB CLI handling should be discussed due to lacking of inter-gNB CLI handling, the larger Tx power and greater inference level. The prerequisite for such interference suppression mechanism is accurate interference measurement. Suitable RS design and timing alignment between wanted signal and interference makes the measurement more accurate. Information exchange between gNBs by some enhanced schemes, e.g., OTA signaling, should also be considered in NR specification.

Secondly, UE-to-UE CLI measurement can be enhanced based on the outcome of discussion in Rel-16. L1 based short term CLI measurement and reporting can be considered to adapt to the dynamic change in TDD configuration.

Proposal2: gNB-to-gNB CLI management should be firstly studied and UE-to-UE CLI measurement/reporting can also be enhanced.

Conclusion

In this contribution, we discussed the CLI in scenarios for dynamic/flexible TDD enhancement in macro & indoor scenarios and possible CLI handling solutions. The observations and proposals are given below:

Observation 1: Enhancements on dynamic/flexible TDD can focus on inter-gNB CLI mitigation with high priority and further signaling design, to better support this existing feature to be implemented.

Observation 2: When the legacy gNB uses DL dominant TDD UL-DL configuration and Rel-18 duplex gNB uses UL dominant TDD UL-DL configuration, there is no impact to the legacy gNB, only impact to legacy UE should be considered.

Observation 3: Interferences in dynamic/flexible TDD scenarios is a subset of that in SBFD scenarios, a unified CLI mechanism for dynamic/flexible TDD and SBFD can be studied.

Proposal1: Focus on the following aspects of dynamic/flexible TDD enhancement

- CLI analysis

- CLI handling (gNB/UE)

Proposal2: gNB-to-gNB CLI management should be firstly studied and UE-to-UE CLI measurement/reporting can also be enhanced.

- References

- RP-213591, Study on Evolution of NR Duplex Operation, CMCC, RAN#94-e, December 6-17, 2021

- TR 38.802-e30, Study on New Radio Access Technology Physical Layer Aspects

- TR 38.828-g10, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR