3GPP TSG RAN WG1 #109-e                                                   R1-2203330

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.3.4

Source:	Spreadtrum Communications

Title:	Further considerations on duplex operation

Document for:	Discussion and decision

Introduction

NR duplex operation SI had been approved at RAN#94 [1]. Comparing with the conventional TDD operation, the subband non-overlapping full duplex (SBFD) at the gNB side within a conventional TDD band will be studied in R18.

In this contribution, we present some observations in terms of SBFD operation coexistence with legacy network including RedCap deployment.

Discussion

For SBFD operation, one key issue is to handle gNB-to-gNB adjacent-channel CLI and/or co-channel-CLI of the same or different operators. Two SBFD patterns in Figure 1 can be potential candidates for SBFD operation, which embed a small UL band into DL slots or allocate a small UL band at the edge carrier of DL slots. The candidate subband patterns are beneficial to reduce the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE, as the rest DL band part of DL slots can act as guard band for those new UL band in the original DL slots, which is analyzed in our companion contribution [2].

Figure 1: The candidate subband patterns for SBFD operation

According to the approved Rel-18 NR duplex SID, adjacent-channel co-existence with the legacy operation should study the feasibility of and impact on RF requirements. As RedCap operation has been specified in Rel-17, from our perspective, good coexistence performance should be studied not only between SBFD capable UE and legacy non-RedCap UE, but also between SBFD capable UE and R17 RedCap UE.

During R17 RedCap discussion, one of the most important coexistence issues is to solve PUSCH resource fragmentation, which will cause UL peak data rate reduction for non-RedCap UEs not supporting non-contiguous UL resource allocation. Finally, network is possible to configure RedCap UL and DL BWP at the edge of channel bandwidth to avoid PUSCH resource fragmentation, while can still maintain TDD DL-UL center frequency alignment.

Observation 1: Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location should be carefully designed.

As well known, UE power consumption is tightly depended on DL BWP bandwidth based on the power consumption model in 38.840 [3]. Although non-RedCap UEs support non-contiguous DL resource allocation, DL resource fragmentation introduced by NR duplex operation may not be friend to non-RedCap UEs from UE power consumption perspective. Figure 2 below is an example of DL resource fragmentation caused by SBFD operation.

Figure 2: An example of DL resource fragmentation caused by SBFD operation

Observation 2: DL resource fragmentation caused by NR duplex operation may not be friend to legacy UEs from UE power consumption perspective.

The main RedCap market is focused on use cases with relatively low cost/complexity, low energy consumption, and low data rate requirements. Therefore, SBFD operation is not recommended for RedCap UE, as it will increased the complexity for RedCap UE and otherwise, it will complicate the whole system design.

To avoid or minimize the DL resource fragmentation problem introduced by NR duplex operation, one possible solution is to configure SBFD band close to the DL BWP of R17 RedCap configured at the carrier edge. One of most benefit of this solution is that UL SBFD can use RedCap DL BWP as guard band to mitigate the inter-operator CLI, while can maximally avoid DL resource fragmentation for legacy non-RedCap UEs. Another potential solution is to configure SBFD band at the opposite side of edge carrier when R17 RedCap DL BWP is configured at the carrier edge. These two example solutions are depended on operators’ network deployments, which are illustrated in Figure 3.

Figure 3: Example deployments for SBFD and R17 RedCap

Proposal 1: The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation.

Conclusion

In this contribution, we discuss SBFD operation coexistence with legacy network including RedCap deployment with the following proposal and observations:

Proposal 1: The introduction of SBFD operation should study co-existence performance with the legacy NR operation including RedCap deployment, which allow network to configure SBFD and RedCap BWP in a legacy NR carrier simultaneously, meanwhile minimizing both UL resource fragmentation and DL resource fragmentation.

Observation 1: Both for R18 duplex operation enhancement and R17 RedCap, to achieve good co-existence performance with legacy network/UE, the bandwidth part location should be carefully designed.

Observation 2: DL resource fragmentation caused by NR duplex operation may not be friend to legacy UEs from UE power consumption perspective.

Reference

- RP-220633, “Revised SID: Study on evolution of NR duplex operation”, CMCC, RAN#95e, March 17th – 23th, 2022

- R1-2203328, “Discussion on subband non-overlapping full duplex”, Spreadtrum communications, RAN1#109-e, May 9th – 20th, 2022

- TR 38.840 V16.0.0, “Study on User Equipment (UE) power saving in NR (Release 16)”, June 2021