3GPP TSG RAN WG1 #109-e			R1- 2203331

e-Meeting, May 9th – 20th, 2022

Agenda Item: 	9.4.1.1

Source:		Spreadtrum Communications

Title:		Discussion on channel access mechanism of sidelink on unlicensed spectrum

Document for:		Discussion and decision

Introduction

According to the WID [1] of RAN #94, Rel-18 NR sidelink evolution will take sidelink on unlicensed spectrum into account for enhancement, and the objective for sidelink on unlicensed spectrum is as following:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we discuss the feasible channel access mechanism of sidelink on unlicensed spectrum.

Discussion

Channel access procedures for sidelink on unlicensed spectrum

In NR SL, UE obtains transmission resources through resource allocation mode 1 and mode 2. After obtaining the transmission resources, UE can complete the SL transmission. In mode 2, UE should perform resource sensing and resource selection. In the unlicensed spectrum, UE needs to perform channel access through LBT to obtain transmission opportunities. In order to support SL-U, mode 2 resource allocation and LBT mechanism need to be considered together. From our point of view, considering simplicity, we should not make too many enhancements to the resource allocation mode 2 and LBT mechanism. Therefore, a simple consideration is that for the resources obtained by resource allocation mode 2, UE needs to further perform LBT before transmission.

Proposal 1: LBT is supported for the selected resources of resource allocation mode 1 and mode 2.

In NR-U, if the absence of any other technology sharing a channel can be guaranteed on a long-term basis (e.g. by level of regulation), channel access procedures for semi-static channel occupancy can be supported. In SL-U, there are similar scenes that the absence of any other technology sharing a channel, such as factories. Therefore, channel access procedures for semi-static channel occupancy should also be supported in SL-U.

Proposal 2: Channel access procedures for semi-static channel occupancy should be supported in SL-U.

In NR-U, there are the following four channel access types. Obviously, the channel access types of NR-U should be as the baseline for the study in SL-U.

- Type 1: The time duration spanned by the sensing slots that are sensed to be idle before a transmission(s) is random.

- Type 2A: The transmission may be transmitted immediately after sensing the channel to be idle for at least a sensing interval 25μs.

- Type 2B: The transmission may be transmitted immediately after sensing the channel to be idle within a duration of 16μs.

- Type 2C: When the gap between transmission(s) is smaller than 16μs, channel sensing does not need to be performed.

Proposal 3: Type 1 and type 2(type 2A, type 2B and type 2C) channel access procedures of NR-U should be the baseline.

For channel access type 1, UE can adjust the contention window according to HARQ feedback in NR-U. In SL, HARQ feedback is enabled and disabled per resource pool configuration. When SL HARQ feedback is enabled, the contention window adjustment of NR-U can be reused. But when SL HARQ feedback is disabled, how to adjust the contention window should be further discussed.

In SL, CBR (Channel Busy Ratio) is introduced to indicate the portion of sub-channels in the resource pool whose SL RSSI measured by the UE exceeds a (pre-)configured threshold sensed over a CBR measurement window. It can roughly reflect the resource occupancy of the resource pool. So, when SL HARQ feedback is disabled, the SL CBR can be considered for contention window adjustment.

Proposal 4: When sidelink HARQ feedback is enabled, contention window can be adjusted according to SL HARQ feedback. When sidelink HARQ feedback is disabled, how to adjust the CWS should be further discussed.

COT sharing between UEs

In NR-U, gNB can share a COT and indicate the COT information to a number of UEs, and UE can also share a COT to gNB. During the shared COT, if the gap between two transmissions is smaller than 16μs, the channel access procedure does not need to be performed. So, COT sharing can provide UE more opportunities to access channels and improve the fairness of the coexistence with other RAT devices. In SL-U, COT sharing between UEs should also be supported. As shown in Figure 1, UE1 obtains COT1 after type 1 channel access procedure, and then UE1 can share COT1 with other UEs. If UE2 discovers the COT1 shared by UE1 before transmission, and the gap between the transmission of UE1 and UE2 is less than 16μs, UE2 can transmit directly without performing the channel access procedure. And UE2 can also share the COT1 to other UEs.

Figure 1 COT sharing between UEs

Proposal 5: COT sharing between UEs should be supported in SL-U.

Conclusions

Proposal 1: LBT is supported for the selected resources of resource allocation mode 1 and mode 2.

Proposal 2: Channel access procedures for semi-static channel occupancy should be supported in SL-U.

Proposal 3: Type 1 and type 2(type 2A, type 2B and type 2C) channel access procedures of NR-U should be   the baseline.

Proposal 4: When sidelink HARQ feedback is enabled, contention window can be adjusted according to SL HARQ feedback. When sidelink HARQ feedback is disabled, how to adjust the CWS should be further discussed.

Proposal 5: COT sharing between UEs should be supported in SL-U.

References

RAN #90e RP-202846.