3GPP TSG RAN WG1 #109-e                                                      R1-2203335

E-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.3

Source:	Spreadtrum Communications

Title:	Considerations on potential solutions for SL positioning

Document for:	Discussion and decision

Introduction

The study on sidelink positioning is approved in [1]. Several aspects need to be discussed in the SI, including requirements, evaluation methodology and positioning techniques for sidelink positioning.

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the uses cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we share our initial views on positioning techniques for sidelink positioning.

Positioning methods for sidelink positioning

- TDOA based positioning

In NR, the UE measures the time-difference-of-arrival (TDOA) between downlink positioning reference signals (PRS) from several TRPs to determine its location. The UE performs RSTD measurements using assistance data provided by network and the resulting measurements based on positioning reference signals PRS are used to locate the UE in relation to the TRPs. The measurement accuracy of TDOA will be affected by the synchronization accuracy between TRPs.

In sidelink, the synchronization accuracy between UEs may be not very ideal. If the synchronization error between UEs is relatively large, the accuracy of sidelink positioning based on TDOA is difficult to meet the requirements of some application scenarios. For absolute positioning, the position of RSUs are static. We can assume that the synchronization accuracy between RSUs is ideal. In this case, TDOA based location method can obtain high positioning accuracy. In our view, as a classic and simple location method, TDOA based location method needs to be considered and supported in sidelink. Furthermore, some methods to eliminate the problem of low positioning accuracy caused by synchronization error between UEs can to be considered, for example, setting the reference UE.

- RTT based positioning

In NR, the UE measures the time difference between the arrival time of the downlink positioning reference signal sent from the TRP and the time when the UE sends SRS (i.e., UE Rx-Tx time difference). The TRP measures the time difference between the arrival time of the SRS sent by the UE and the time when TRP sends PRS (i.e., TPR Tx-Rx time difference). The positioning server can determine the distance between the UE and the TRP according to UE Rx-Tx time difference and TPR Tx-Rx time difference, as shown in Figure 1.

Compared with TDOA based positioning technology, RTT based positioning technology does not rely on the strict synchronization between TPRs and the measurement accuracy will not be affected by the synchronization accuracy between TRPs, but the UE needs to know the exact time when the signal starts transmission. In sidelink, the synchronization accuracy between UEs is not very ideal. RTT based positioning technology should be considered, especially for some ranging-based services which only require the distance measurement.

Figure 1: RTT based positioning

- Angle-based positioning

gNB with an Rx antenna array can estimate the direction of the received signal and with beamformed Tx antenna array it can estimate the direction of the transmitted signal.

For UL-AoA, There are many algorithms for estimating UL AoA. The simple method is that the TRP directly uses the direction of the received beam as the UL AoA. The angle estimation resolution of this simple method is low. The method with high resolution is that the TRP receives UL SRS through the receiving antenna array, and decomposes the observation space into two subspaces through effective algorithms (such as MUSIC, ESPRIT, etc.), that is, signal subspace and noise subspace. The TRP estimates the arrival direction UL AoA of the SRS signal from the signal subspace. Once the UL AoA is obtained, the existing algorithm can be used to calculate the location of the UE.

For DL-AoD, the UE can measures the downlink reference signal reception power corresponding to each beam of the TPR, and then sends the measurement report to the location server. The location server determines the AoD according to the DL RSRP of each beam, and then estimates the UE position according to the AoD.

In NR, both TPR and UE will be equiped with more antennas, which can provide larger array aperture. Thus, angular measurement in NR is expected to play more important role to achieve accurate position estimation. In sidelink, the measurement accuracy of AoA and AoD are impacted by the number of Rx/Tx antennas of UE. UEs are generally not equipped with large-scale antennas. Therefore, in sidelink, high-precision AoA and AoD measurement may be difficult to achieve. In TS 22.261, ranging-based services are the applications utilizing the distance between two UEs and/or the direction of one UE from the other one. In 3D case, direction includes horizontal direction and elevation direction. However, for short-range ranging scenarios, such as smart home, it does not need high-precision AOA and AoD measurement for the determination of direction of UE. In order to support the applications utilizing the direction of one UE from the other one, angle-based positioning should be considered in SL positioning.

Proposal 1: RAN1 should consider the following positioning techniques for SL positioning: RTT based positioning, TDOA based positioning, AOA/D based positioning.

Sidelink reference signals for positioning

Positioning reference signals design

In the current sidelink transmission, some reference signals are introduced, such as S-SSB, PSSCH DMRS, PT-RS and CSI-RS which always within the bandwidth of PSSCH. These signals’ main purpose is to serve communication framework and thus these signals may not be suitable for positioning measurements and may impose certain constraints on either communication or positioning frameworks. In genreal, PSCCH bandwidth is difficult to meet the accuracy requirements of sidelink positioning. Furthermore, for positioning reference signals pattern design, good auto and cross correlation properties in the presence of time and frequency offsets and Doppler effect should be considered. The existing sidelink reference signal pattern cannot meet the positioning requirements. In our view, sidelink positioning reference signals need to be introduced for sidelink positioning. In NR, the PRS and SRS are used for positioning. The sidelink positioning reference signal design can follow the PRS / SRS of the existing NR.

Proposal 2: Sidelink positioning reference signals should be introduced for sidelink positioning.

Resource allocation for positioning reference signals

In NR, the parameter structure of PRS is a three-tier structure of positioning frequency layer, PRS resource set and PRS resource, that is, one layer can contain multiple resource sets and one resource set can contain multiple resources. Some PRS parameters are layer level, that is, the parameters of this level are applied to all resources contained in this layer. Some parameters are at the resource set level, and the parameters at this level are applied to all resources in the set. Other parameters are resource level and only indicate the configuration of the resource. Each TRP is configured with dedicated PRS transmission resources. For SRS transmission in NR, the network will also configure UE with dedicated resources for SRS transmission.

NR V2X supports two resource allocation modes: mode1 and mode2.

- Mode 1: The gNB schedules sidelink resources to the UE for transmission.

- Mode 2: UE independently selects sidelink resources for sidelink transmission.

In resource allocation mode 2, the high layer can request the UE to determine a subset of resource and select resource for PSSCH/PSCCH transmission from the subset of resource determined by UE.

For in-coverage and partial-coverage, the gNB can schedules sidelink resources for sidelink PRS transmission, i.e., resource allocation mode 1. In this case, UE independently selects sidelink resources (i.e., resource allocation mode 2) for sidelink PRS transmission can also be supported. For out-of-coverage, since there is no coverage of the gNB, the UE can only adopt the resource allocation mode 2 for sidelink PRS transmission.

Proposal 3: For SL PRS transmission, the resource allocation mode 1 and the resource allocation mode 2 can be considered.

Energy efficiency for position reference signals transmission

In SL positioning, UE may need to transmit sidelink positioning reference signals and receive sidelink positioning reference signals sent by other UEs. If the UE blindly sends or receives sidelink positioning reference signals, it will bring serious power consumption. In NR V2X, there is no “always on” reference signals to perform measurements. Furthermore, TS22.261 points out that the 5G system shall support energy efficient UE ranging operation. Therefore, energy efficiency for sidelink positioning reference signals transmission should be considered. For example, the target UE can trigger other UEs to send sidelink positioning reference signals or stop sidelink positioning reference signals through sidelink transmission.

Proposal 4: Energy efficiency for sidelink positioning reference signals transmission should be considered.

Conclusion

In this contribution, we provided initial views on sidelink positioning techniques and design enhancements. In summary, we have following proposals:

Proposal 1: RAN1 should consider the following positioning techniques for SL positioning: RTT based positioning, TDOA based positioning, AOA/D based positioning.

Proposal 2: Sidelink positioning reference signals should be introduced for sidelink positioning.

Proposal 3: For SL PRS transmission, the resource allocation mode 1 and the resource allocation mode 2 can be considered.

Proposal 4: Energy efficiency for sidelink positioning reference signals transmission should be considered.

Reference

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel (Email discussion moderator).