3GPP TSG RAN WG1 #109-e	                                         	R1-2203338

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.6.1

Source:	Spreadtrum Communications

Title:	Discussion on potential solutions to further reduce UE complexity

Document for:	Discussion and decision

Introduction

When the foundation has been laid in R17, enhancements can be considered to improve the support for the current and new use cases. To further expand the market for RedCap use cases with relatively low cost, low energy consumption, and low data rate requirements, e.g., industrial wireless sensor network use cases, some further complexity reduction enhancements should be considered.

According to the R18 RedCap SID [1], at least the following solutions should be studied.

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1] - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact - Potential solutions, which may complement each other, for reducing device complexity are focusing on: - UE bandwidth reduction to 5MHz in FR1, - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI - reduced UE peak data rate in FR1, - Possibly including restricted bandwidth for PDSCH and/or PUSCH - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

In our understanding, according to the master-subordinate relationship, the solutions listed in the SID can be divided into two categories, a) main solutions, and b) secondary/supplementary solutions:

  - Main solutions

    - UE bandwidth reduction to 5MHz in FR.

    - Reduced UE peak data rate in FR1.

  - Secondary/supplementary solutions

    - Relaxed UE processing timeline.

In this contribution, we discuss the potential solutions to further reduce UE complexity based on the above categories. For simplify, we call the Rel.18 RedCap as eRedCap in this contribution.

Main solutions

UE bandwidth reduction to 5MHz in FR1

Description

According to R17 RedCap WID [2], a R17 RedCap UE is required to support 20 MHz in FR1. With 20MHz bandwidth capability, the supported peak data rate (~80Mbps) is considerably higher than the requirements of some RedCap use cases, e.g., sensors. In order to match the requirements and the capability, further reduce the UE bandwidth to 5MHz is a possible way.

In this section, we present the analysis on 5MHz bandwidth for eRedCap. In our analysis, we assume the same maximum UE bandwidth in a band applies to both RF and baseband and applies to both data and control channels. Furthermore, this maximum UE bandwidth is assumed for both DL and UL.

Analysis of UE complexity reduction

The complexity evaluation assumptions and methodology can be found in our contribution [3]. As evaluated in R17, BW reduction may reduce the cost of BB components, e.g., ADC/DAC, FFT/IFFT, post-FFT data buffering, receiver processing block and so on.

However, for further reduce UE complexity, it is worth noticing that when performance requirements /capabilities are reduced to a certain level, the relationship between the performance requirements/capabilities and the cost is non-linear, this means that there is limited room for further cost reduction. Based on R17 evaluation methodology, the additional cost reductions from 20MHz to 5MHz are 2.14% for FDD and 1.33% for TDD. While compared with the R17 simplest RedCap UE, the cost reductions are about 6.63% for FDD and 4.73% for TDD.

Observation 2.1-1: Compared with R17 simplest RedCap UE, the additional cost reductions from 20MHz to 5MHz are about 6.63% for FDD and 4.73% for TDD.

Analysis of SSB/SIB acquisition impacts

- SSB reception

For SSB reception, if the SCS is 15KHz, the total span of SSB in frequency domain is 3.6MHz, which is smaller than 5MHz. While for 30Khz SCS, the SSB span is 7.2MHz, which is wider than 5MHz. However, the PSS/SSS only occupies 127 SCs [4], 5MHz eRedCap can receive the entire PSS/SSS and a part of PBCH (puncturing of PBCH). As the code rate of PBCH is quite small, then the PBCH can still be successful decoded. A contribution [5] discussed similar issues for <5MHz dedicated spectrum item.

Observation 2.1-2: For 5MHz BW, the SSB can be received for 15/30KHz SCS.

- CORESET#0 reception

After reception of SSB, the UE needs to determine the frequency position of CORESET#0 based on the SCS of SSB, PDCCH-ConfigSIB1 and subCarrierSpacingCommon. Different SCS combinations corresponding to different reference tables (table 13-1 to 13-10 in 38.213[6]) for CORESET#0.

For {SS/PBCH block, PDCCH} SCS is {15, 15} kHz and for the frequency bands with minimum channel bandwidth 5 MHz or 10 MHz, table 13-1(copied as below, 38.213 [6]) can be used for 5MHz eRedCap. However, only the first six index are available, since for other index, the total span of CORESET#0 is larger than 5MHz.

TS 38.213: Table 13-1: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 2. 2; 1; 24; 2; 4. 3; 1; 24; 3; 0. 4; 1; 24; 3; 2. 5; 1; 24; 3; 4. 6; 1; 48; 1; 12. 7; 1; 48; 1; 16. 8; 1; 48; 2; 12. 9; 1; 48; 2; 16. 10; 1; 48; 3; 12. 11; 1; 48; 3; 16. 12; 1; 96; 1; 38. 13; 1; 96; 2; 38. 14; 1; 96; 3; 38. 15; Reserved.

Further, when either the SCS of SSB or the SCS of CORESET#0 is equal to 30KHz, the total span of CORESET#0 is larger than 5MHz (Min 8.6MHz), take table 13-2 as an example. The SCS of PDCCH is 30KHz, the minimum number of NRBCORESET is 24, the BW of CORESET#0 is 8.64MHz.

TS 38.213: Table 13-2: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {15, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 5. 1; 1; 24; 2; 6. 2; 1; 24; 2; 7. 3; 1; 24; 2; 8. 4; 1; 24; 3; 5. 5; 1; 24; 3; 6. 6; 1; 24; 3; 7. 7; 1; 24; 3; 8. 8; 1; 48; 1; 18. 9; 1; 48; 1; 20. 10; 1; 48; 2; 18. 11; 1; 48; 2; 20. 12; 1; 48; 3; 18. 13; 1; 48; 3; 20. 14; Reserved. 15; Reserved.

Therefore, based on the current spec, 30KHz SCS cannot be supported by 5MHz RedCap. If 30Khz needs to be supported, new pattern(s) of CORESET#0 may be needed, great spec impacts were expected.

Observation 2.1-3: For 5MHz BW, the CORESET#0 can be received only when a) the SCS of CORESET#0 and SSB are equal to 15KHz, and b) the index of table 13-1 are limited to the first six index.

Observation 2.1-4: For 5MHz BW, if 30KHz needs to be supported, new pattern(s) of CORESET#0 may be needed, great spec impacts were expected.

- SIB reception

For SIB reception, the only observed impact is that the SIB-PDSCH needs to be scheduled within 5MHz BW.

Observation 2.1-5: For 5MHz BW, the SIB-PDSCH needs to be scheduled within 5MHz BW.

Analysis of performance impacts

- Data rate

Further bandwidth reduction results in a further reduction in the achievable peak data rate. Take a 1Rx+1MIMO+64QAM-FD-FDD RedCap as an example, if the BW is reduced from 20MHz to 5MHz, the supported peak data rate is reduced from 82Mbps/87.6Mbps in DL/UL to 17.7Mbps/18.9Mbps in DL/UL. For HD-FDD and TDD, the supported peak data rates are related to the ratio of DL and UL symbol/slots, and they will be smaller than those of FD-FDD.

The target data rate of R18 RedCap is 10Mbps, and it is aimed to meet the data requirements of low-end use cases, e.g, sensors, which data rate requirement is less than 2 Mbps. Therefore, 5MHz BW is enough.

Observation 2.1-6: 5 MHz maximum UE bandwidth in FR1 can fulfil the data rate requirements of low-end RedCap use cases.

- Coverage

For PDCCH coverage, one important aspect is whether the larger aggregation levels (AL), e.g. 8 and 16, can be supported after bandwidth reduction. In FR1, UE bandwidth 20 MHz is enough for supporting AL 16 for CORESET configuration. While for 5MHz, the RB number is reduced a lot. For 15KHz SCS, the RB number is reduced to 25, then the Max AL is limited to 8 if 3 symbols are assumed. For 30KHz SCS, the RB number is reduced to 11, the Max AL is limited to 4. This limitation may lead to a PDCCH coverage loss.

Observation 2.1-7: Reduce the maximum UE bandwidth to 5MHz in FR1 will reduce the coverage due to the limitation of max AL of PDCCH.

- PDCCH blocking and latency

Bandwidth reduction will increase the PDCCH block rate since the number of RBs is reduced, i.e., the number of PDCCH candidates decreased. With the increased PDCCH blocking rate, the scheduling latency may also be increased.

Observation 2.1-8: Reduce the maximum UE bandwidth to 5MHz in FR1 will increase the PDCCH blocking rate and latency .

Analysis of coexistence with legacy/R17 RedCap UEs

Regarding the coexistence issue, it is related to the initial BWP configuration. As a separate initial BWP structure was already established in R17, then, there are two possible options for R18 eRedCap:

  - Option 1: Shared with R17 RedCap

  - Option 2: Separate initial BW for R18 5MHz eRedCap

For option 1, the separate initial BWP is configured for both R17 RedCap and R18 5MHz eRedCap, then the BW of such initial BWPs should no wider than 5MHz. Obviously, option 1 may bring some limitations to R17 RedCap, but very small or even no spec impacts were observed(i.e., reuse the current structure, procedure, and IEs).

For option 2, another separate initial BWP is need to be introduced. That is to say, for a cell, there may have three kinds of initial BWPs, one for Non-RedCap, one for R17 RedCap and the other is for R18 5MHz eRedCap. There will be no restriction to R17 RedCap, but new IEs, new procedures are needed, and even NCD-SSB, PUCCH related issues need to be discussed again..

Observation 2.1-9: If the separate initial BWP introduced in R17 is shared by R17 RedCap and 5MHz eRedCap, small or even no spec impacts were expected, but limitation to R17 RedCap was observed .

Observation 2.1-10: If another separate initial BWP is introduced for R18 5MHz RedCap, great negative impacts to UE implementation, spec, standardization workload were expected.

Analysis of random access impacts

If separate 5MHz BWP is configured, the random access can be initialled in the separate BWP. However, it is worth noticing that with 5MHz, the flexibility of RACH configuration is quite limited.

Observation 2.1-11: With 5MHz BW, the flexibility of RACH configuration is quite limited.

Analysis of spec impacts

As we analysed in above, if 30Khz SCS and another separate initial BWPs for 5MHz RedCap are needed, huge spec impacts are expected. On the contrary, restrictions were observed, e.g., limited to 15KHz SCS only and limitations on the initial BWP for R17 RedCap.

Observation 2.1-12: For 5MHz BW, either great spec impacts or great limitations were observed.

Reduced UE peak data rate in FR1

Descriptions

In addition to reduce the BW to 5MHz, reduce the supported UE peak data rate while keep the BW capability at 20MHz is another possible way. According to the previous discussion (in R18 RWS) and in our understanding, there are at least three detailed ways to reduce the UE peak data rate, and they are listed as follows:

- Restricted bandwidth for PDSCH and/or PUSCH

- TBS restriction.

- Scaling factor

Although the last two methods are not explicitly listed in the SID, but in our understating, they are included in the main bullet of reduced UE peak data rate in FR1. The last two methods may have smaller spec impacts compared to the first one, so we think they can be studied in study item.

Observation 2.2-1: For reduced UE peak data rate, it was observed that there are at least three detailed methods can be considered.

- Restricted bandwidth for PDSCH and/or PUSCH

- TBS restriction.

- Scaling factor

In general, the essence of these three methods is TBS limitation within a TTI. In this section, we present the analysis on reduced UE peak data rate in FR1 for eRedCap. In our analysis, we assume the same maximum UE bandwidth is 20MHz in a band applies to both RF and baseband, i.e., the baseband BW for both control channel and data channel is 20MHz. Furthermore, 20MHz bandwidth is assumed for both DL and UL. In addition, according to the SID [1], the target peak data rate for R18 RedCap is around 10Mbps, then the TBS is restricted to 5000bits for 30KHz SCS is assumed for the following analysis.

Observation 2.2-2: The essence of these three methods for reduced UE peak data rate is TBS limitation within a TTI.

Analysis of UE complexity/cost reduction

The complexity evaluation assumptions and methodology can be found in our contribution [3]. Actually, we discussed the maximum TBS restriction for DL and UL in R17 phase [7], TBS restriction is mainly reduce the cost of data processing related components in BB, e.g., Receiver processing block, LDPC decoding, Memory(HARQ  buffer), UL processing block.

Our initial evaluation results shows that the additional cost reductions for reduced UE peak data rate are 1.75% for FDD and 1.2% for TDD, while compared with the R17 simplest RedCap UE, the cost reduction are about 5.44% for FDD and 4.27% for TDD.  The cost reductions are similar to BW reduction.

Observation 2.2-3: Compared with R17 simplest RedCap UE, the additional cost reductions for peak data reduction are about 5.44% for FDD and 4.27% for TDD.

Observation 2.2-4: The cost reductions of peak data reduction are comparable to those of reduce the maximum BW to 5MHz.

Analysis of SSB/SIB acquisition

- SSB and CORESET#0 reception

Based on our assumptions, i.e., same maximum UE bandwidth is 20MHz in a band applies to both RF and baseband, no issues were observed for the reception of SSB and CORESET#0, the current procedure/configuration for SSB and CORESET#0 can be reused.

Observation 2.2-5: For reduced peak data rate, no issues were observed for the reception of SSB and CORESET#0.

- SIB reception

The payload size of SIB is 2976bits [9], which is smaller than 5000bits. Then, from the perspective of TBS, the SIB can be received by the eRedCap. However, if the method of restricted bandwidth for PDSCH and/or PUSCH is adopted, there will be a limitation for SIB scheduling, i.e., the scheduled RB number for SIB should smaller than the restricted number. In addition, if a very low code rate is required for SIB PDSCH, the required RB number will be relatively large. That is to say, if the RB number is restricted too small, the SIB cannot be scheduled. The other two methods will not bring such impacts.

Observation 2.2-6: If the method of restricted bandwidth for PDSCH and/or PUSCH is adopted, there will be a limitation for SIB scheduling.

Observation 2.2-7: With the method of TBS restriction and scaling factor, no issues were observed for SIB reception.

Analysis of performance impacts

- Data rate

The target data rate of R18 RedCap is 10Mbps, this is the assumption/target for the solution of reduced UE peak data rate in FR1. This data rate is enough for the low-end RedCap use cases.

Observation 2.2-8: Reduced UE peak data rate to 10Mbps in FR1 can fulfil the data rate requirements of low-end RedCap use cases.

- Coverage

With 20Mhz BW capability, the max AL(coverage of PDCCH) keep the same with R17 RedCap.

Observation 2.2-9: As the control channel BW is 20MHz, the coverage performance is similar to R17 RedCap

- PDCCH blocking and latency

With 20Mhz BW capability, the PDCCH blocking rate is similar to R17 RedCap. Then, the scheduling latency from the perspective of PDCCH blocking is also same to R17 RedCap. However, as the TBS is restricted, the latency may be increased if the package payload is huge.

Observation 2.2-10: As the control channel BW is 20MHz, the PDCCH blocking rate is similar to R17 RedCap

Observation 2.2-11: As the TBS is restricted, the latency may be increased (compared to R17 RedCap).

Analysis of random access impacts

If the peak data rate is reduced to 10Mbps, the supported TBS within a TTI is around 5000bits (30KHz SCS), then the reception of common information during the random access phase may be affected (before capability reporting). As Msg1 is a sequence, there is no need to compare it.

- Paging (DL)

According to the current spec [8][9], the max paging record is 32, then the max payload of paging is no more than 3000bits, which is less than 5000bits.

- Msg 2 (DL)

According to the current spec [8][9], the max number of RAR within a PDSCH is 64, then the max payload of paging is around 4100bits, which is less than 5000bits.

- Msg 3 (UL)

According to the current spec [6], when a PUSCH is scheduled by a RAR, the UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [10]. based on [10], the max Qm is16 QAM and the max code rate is 0.64, then the peak data rate of Msg 3 is around 20.6Mbps, then the max payload size of Msg 3 is 10300 bits, which is higer than 5000bits.

However, the information carried by Msg3 during the initial access procedure in RRC IDLE state is RRCSetupRequest, according to [9], the payload size of RRCSetupRequest is quite small (less than 100bits). Other information that possibly carried by Msg3 are include RRCResumeRequest or RRCResumeRequest1, RRCReestablishmentRequest, RRCSystemInfoRequest, C-RNTI MAC CE+ RRCReconfigurationComplete, C-RNTI MAC CE, the payload size of these IEs are all small(less than 100bits) except for RRCReconfigurationComplete. Nevertheless, the RRCReconfigurationComplete is only used in connected mode, the gNB knows the TBS of eRedCap, and then the restriction can be applied.

- Msg 4 (DL)

There is no limitation for Msg4 in PHY spec, but according to L2 spec [8][9], Msg 4 is used to carry the RRC set up and contention resolution. The max PDCP size is 9K bytes and the contention resolution size is 6 bytes, in addition, there is an MAC header 1 byte. Therefore, Msg 4 can be up to 9007bytes = 72056bits, this bit size is a max value from spec perspective, obviously, it is higher than the allowed TBS within a TTI for the R17 RedCap even without any relaxation.

According to the above analysis, the reception of Msg4 may be impacted. Then we checked three different Msg 4 logs from current 5G network in China, and found that the max size of Msg4 is around 1320bits.

Table 2.2-1: Msg 4 logs from current 5G network in China

| Logs 1 | Logs 2 | Logs 3 |
| --- | --- | --- |

Based on network implementation, the transmission/reception of Msg3/Msg4 can be done.

Observation 2.2-12:  With reduced peak data rate, the reception of Msg4 may be impacted in principle. However, according to the logs from current 5G network, the payload of Msg4 is smaller than 5000bits.

Analysis of coexistence with legacy/R17 RedCap UEs

With 20MHz BW capability, the separate initial BWP structure introduced in R17 can be reused for eRedCap, that is to say, there is no new IE and procedure are needed, and also there is no limitation on R17 RedCap.

Observation 2.2-13: With reduced peak data rate, the initial BWP for R17 RedCap can be resued by R18 eRedCap, e.g., there is no need to introduce another initial BWP for R18 RedCap, and no limitation on R17 initial BWP.

Analysis of spec impacts

For spec impacts, as these are three different detailed methods, the possible spec impacts can be divided into common impacts and individual impacts.

Common impacts

- Enhancement or restriction for Msg4 transmission may needed. For example, the UE needs to report its capability before Msg4 transmission. In addition to early capability indication, we think this can be resolved by gNB implementation, e.g., the gNB can always restrict the payload size of Msg4 for all RedCap, or if the feedback of Msg4 from a RedCap is NACK, the gNB can change the payload size for Msg4.

Individual impacts

- For restricted bandwidth for PDSCH and/or PUSCH, our understanding is that this method is to restrict the scheduled RB number of PDSCH and PUSCH in frequency domain and maybe combine with the restriction on the number symbols in time domain. Based on this detailed method, the data BW(RB Number) determination is needed, and then a new IE may also needed for capability reporting after initial access.

- For TBS restriction, as the TBS is calculated by serval factors, e.g., modulation order, code rate, the number of MIMO layers and the number of REs. TBS restriction can be achieved by single factor restriction or restriction on factor combinations. Then, the factor(s) to be restricted need to be confirmed.  Further, the simplest way is to specify a max TBS for eRedCap, and then detailed restriction (restrict to which factor or factor combinations) can up to gNB implementation. Similar to method one, a new IE may needed for capability reporting after initial access.

- For scaling factor, this method was discussed in the later phase of R17 WI, and the details for scaling factor can be found in our contributions [11][12]. The scaling factor is given by higher layer parameter scalingFactor (in UE capability IEs) and can take the values 1, 0.8, 0.75, and 0.4. The UE capability signaling (scalingFactor and supportedModulationOrderDL) allows UE to flexibly indicate the data rate it supports, and indicate the maximum TBS it supports implicitly [13]. The only impacts observed for scalingFactor is constraint relaxation, as the current IE can be reused.

We summarized individual impacts in the following table

Table 2.2-2: possible spec impacts of different methods

| Restricted bandwidth for PDSCH and/or PUSCH | TBS restriction. | Scaling factor |
| --- | --- | --- |
| - BW(RB Num) determination; - Capability reporting (New IE); | - Determination on TBS and/or factors to be restricted; - Capability reporting (New IE or reuse); | - Constraint relaxation; - Reuse the current IE(SF) |

The expected impacts of these method are very small, especially for scaling factor based method.

Observation 2.2-14: The expected spec impacts of the detailed methods of reduced peak data rate are very small.

Observation 2.2-15: The scaling factor based method was discussed in the later phase of R17 WI, and the only impact observed for scalingFactor is constraint relaxation.

Comparison of main solutions.

Based on the analyses in the chapter 2.1 and 2.2, the summary and comparison of UE bandwidth reduction to 5MHz and reduced UE peak data rate are listed in the following table

Table 2.3-1: Comparison of UE bandwidth reduction to 5MHz and reduced UE peak data rate

| Possible impacts | Further BW reduction to 5MHz | Reduced UE peak data rate |
| --- | --- | --- |
| Cost reduction | FDD: 6.63%, TDD: 4.73% | FDD: 5.44%, TDD: 4.27% |
| SSB/SIB acquisition | Limited to 15KHz or great spec impacts | No/small impact |
| Data rate | Meet the requirements for the low-end RedCap use cases. | Meet the requirements for the low-end RedCap use cases. |
| Coverage (PDCCH) | Reduced (compared to R17 RedCap) | No impact (compared to R17 RedCap) |
| PDCCH blocking rate | Increased (compared to R17 RedCap) | No impact (same to R17 RedCap) |
| Latency | May be reduced | May be reduced |
| Random access | Limited configuration (Limited to 5MHz) | Msg 4 reception (may be resolved by gNB) |
| Coexistence | Share the initial BWP with R17(restriction of R17 RedCap) or great spec impacts | Almost no impact (Share the initial BWP with R17) |
| Specification | Either great spec impacts or great limitations | Small |

Observation 2.3-1: Compared with further BW reduction to 5MHz, reduced UE peak data rate brings less spec impacts, performance loss and limitations.

Secondary solution

Relaxed UE processing timeline

UE processing time relaxation was widely studied in Rel.17 study item, and all the possible impacts were captured in TR 38.875 [14]. Due to the R17 TU limits, this solution was not introduced in R17.

Description of feature

The relaxed UE processing time mainly focus on N1 and N2, N1 is the number of OFDM symbols required for UE processing from the end of PDSCH reception to the earliest possible start of the corresponding ACK/NACK transmission from UE perspective. N2 is the number of OFDM symbols required for UE processing from the end of PDCCH containing the UL grant reception to the earliest possible start of the corresponding PUSCH transmission from UE perspective. Both of N1 and N2 are related to the timing from DL to UL, since the timing needs to be aligned between UE and gNB.

In R17 study item, the values of N1 and N2 to be studied are assumed to be doubled compared to those of capability 1 (listed below, [10]).

TS:38.214 PDSCH processing time for PDSCH processing capability PDSCH processing capability 1; PDSCH processing capability 2. μ; PDSCH decoding time N1 [symbols]; PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ pos0 in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured; dmrs-AdditionalPosition = pos0 in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 8; N1,0 (13 or 14, according to the PDSCH DM-RS position); 3. 1; 10; 13; 4.5. 2; 17; 20; 9 for frequency range 1. 3; 20; 24; NA. PUSCH preparation time for PUSCH timing capability PUSCH timing capability 1; PUSCH timing capability 2. μ; PUSCH preparation time N2 [symbols]; PUSCH preparation time N2 [symbols]. 0; 10; 5. 1; 12; 5.5. 2; 23; 11 for frequency range 1. 3; 36; NA.

In R18, the target relaxed values for N1 and N2 can be the same as R17.

Observation 3.1-1: For relaxed UE processing timeline, the doubled N1 and N2 compared to those of capability 1 were studied in R17.

Analysis of UE complexity/cost reduction

The complexity evaluation assumptions and methodology can be found in our contribution [3]. Processing time relaxation may reduce the cost of BB components, e.g., Receiver processing block, LDPC decoding, DL control processing & decoder and UL processing block. Our evaluation results shows that the additional cost reductions for relaxed UE processing timeline are 1% for FDD and 0.83% for TDD, while compared with R17 simplest RedCap UE, the cost reduction are about 3.15% for FDD and 2.97% for TDD..

Observation 3.1-2: Compared with Rel-17 simplest RedCap UE, the additional cost reductions are about 3.15% for FDD and 2.97% for TDD.

Analysis of SSB/SIB acquisition

- SSB, CORESET#0 and SIB reception

As introduced in chapter 3.1.1, both N1 and N2 are related to the timing from DL to UL, then the reception of SSB, CORESET#0 and SIB will not be impacted.

Observation 3.1-3: With relaxed UE processing timeline, no issues were observed for the reception of SSB, CORESET#0 and SIB.

Analysis of random access impacts

In random access procedure, N1/N2 are related to the timing requirements such as the time from Msg2 to Msg3, and PUCCH timing of Msg4. Then, the relaxed N1/N2 may impact the scheduling of Msg3 and PUCCH of Msg4. Take Msg3 as an example, the time gap between RAR UL grant and Msg3 for normal UE is N1+N2+0.5ms [6], while the time gap for a UE with relaxed processing time is 2*N1+2*N2+0.5ms. If the gNB cannot know the processing time capability of the UE in advance, in order to ensure the access procedure, the gNB is always need to select a larger scheduling delay. It is workable, but some negative impacts to the access latency were observed. However, if early identification of this kind of RedCap UE prior to Msg2 scheduling is supported, this impact seems no more exist.

Observation 3.1-4: With relaxed UE processing timeline, some negative impacts to the access latency were observed.

Analysis of other impacts

If the N1/N2 values to be studied in R18 are keep the same to those in R17 study item, then the other possible impacts captured in TR 38.875 [14] (listed below) can be used as reference.

### 7.5.3 Analysis of performance impacts Coverage: No coverage impact is expected from a more relaxed UE processing time in terms of N1 and N2. Network capacity and spectral efficiency: Depending on the gNB scheduler implementation, there may be no or minor impact on network capacity or spectral efficiency from a more relaxed UE processing time in terms of N1 and N2. Data rate: No impact on instantaneous peak data rate is expected from a more relaxed UE processing time in terms of N1 and N2, but the UE throughput may be reduced if the HARQ round trip time is extended. The throughput requirements identified for the RedCap use cases are still expected to be fulfilled. Latency and reliability: Relaxed UE processing time in terms of N1 and N2 has impact on latency. For downlink transmission, relaxed N1 value impacts how fast HARQ-ACK feedback can be sent after the reception of PDSCH. For uplink transmission, relaxed N2 value impacts how fast PUSCH can be scheduled with respect to the UL grant. How significant the impact on latency is depends on use cases and scheduled number of retransmissions. Power consumption: Relaxed UE processing time in terms of N1 and N2 may allow for processing with lower clock frequency and lower voltage which may help reducing the UE power consumption. The impact on power consumption of relaxed UE processing time depends on implementation and traffic characteristics. ### 7.5.4 Analysis of coexistence with legacy UEs In scenarios where RedCap UEs coexist with legacy UEs, relaxed UE processing time capability in terms of N1 and N2 may for RedCap UEs increase the complexity for the scheduling. The relaxed UE processing time capability in terms of N1 and N2 may cause potential coexistence issues with legacy UEs during initial access if early identification of RedCap UEs prior to Msg2 scheduling is not supported or conservative scheduling is not possible. If gNB schedules all UEs according to relaxed timing relationships for RedCap UEs, legacy UEs may experience an increase in control plane latency. ### 7.5.5 Analysis of specification impacts A new UE processing time capability needs to be defined if relaxed UE processing time in terms of N1 and N2 is introduced. New values of N1 and N2, as well as how the PDSCH processing time and PUSCH preparation time are determined by N1 and N2, need to be defined. Depending on the degree of relaxation of the N1 and N2 values, specification details on scheduling timing related to the default TDRA tables and HARQ-ACK timing range may also need to be updated.

According to the above, the impacts of processing time relaxation are relatively small.

Observation 3.1-5: The other impacts of processing time relaxation are relatively small.

Solution combinations

For solution combinations, we understand that it is unnecessary to combine the two main solutions, i.e., combine the UE bandwidth reduction to 5MHz and reduced UE peak data rate in FR1, since a similar effect can be achieved by both solutions.

Further, based on the SID, the following combinations can be studied

- Combination 1: Combination of 5MHz BW and processing time relaxation

- Combination 2: Combination of reduced peak data rate and processing time relaxation

In this chapter, we analysis the cost impacts of these two combinations, the other impacts can be found in previous chapter.

Combination of 5MHz BW and processing time relaxation

- Analysis of UE complexity/cost reduction

The complexity evaluation assumptions and methodology can be found in our contribution [3]. When combine 5MHz BW and processing time relaxation, the impacted components include the following:

- ADC / DAC

- FFT/IFFT

- Post-FFT data buffering

- Receiver processing block

- LDPC decoding

- Memory(HARQ  buffer)

- DL control processing & decoder

- UL processing block

Based on R17 evaluation methodology, the additional cost reductions for combination of 5MHz BW and processing time relaxation are 2.99% for FDD and 1.99% for TDD. While compared with Rel-17 simplest RedCap UE, the cost reduction can be roughly calculated as 8.87% for FDD and 7.08% for TDD.  The cost reductions are relatively attractive compared to single solution.

Observation 4.1-1: Compared with Rel-17 simplest RedCap UE, the cost reductions for combination 1 are 8.87% for FDD and 7.08% for TDD.  The cost reductions are relatively attractive compared to single solution.

Combination of reduced peak data rate and processing time relaxation

- Analysis of UE complexity/cost reduction

The complexity evaluation assumptions and methodology can be found in our contribution [3]. When combine reduced peak data rate and processing time relaxation, the impacted components increased, include the following:

- ADC / DAC

- FFT/IFFT

- Post-FFT data buffering

- Receiver processing block

- LDPC decoding

- Memory(HARQ  buffer)

- DL control processing & decoder

- UL processing block

Based on R17 evaluation methodology, the additional cost reductions for combination of reduced peak data rate and processing time relaxation are 2.35% for FDD and 1.71% for TDD. While compared with Rel-17 simplest RedCap UE, the cost reduction can be roughly calculated as 7.31% for FDD and 6.08% for TDD.  The cost reductions are relatively attractive compared to single solution, and almost the same compared to combination of 5MHz BW and processing time relaxation

Observation 4.2-2: Compared with Rel-17 simplest RedCap UE, the cost reductions for combination 2 are 7.31% for FDD and 6.08% for TDD.  The cost reductions are relatively attractive compared to single solution.

Observation 4.2-3: The additional cost reductions for the combination 1 (i.e., reduced peak data rate and processing time relaxation) are comparable to those for the combination2 (i.e., 5MHz BW and processing time relaxation).

Cost views for solutions

According to the previous observations for cost reduction, we can found that the cost reduction are not that attractive based on the Rel-17 evaluation methodology. The reason is that Rel-17 evaluation methodology is only focus on RF and BB, and due to the non-linear relationship between the components performance and the cost, there is no much room for cost reduction in RF and BB.

However, it is worth noticing that there are some other components (e.g., memory, different from RF and BB) will be affect by R18 features. BW reduction to 5MH and reduced peak data rate are both lead to a lower peak data rate, which means lower memory requirements, then the less cost of memory. Based on the product experience, if the peak data rate can be reduced from ~80Mbps to 10Mbps, the memory cost can be reduced by around 10% in overall. The cost reduction for memory is the other important motivation for R18 RedCap.

In order to illustrate the principle of R18 cost reduction, and to convey correct and positive information to vertical industries, we think it is necessary to capture the impact of R18 features on memory cost in the corresponding TR.

Observation 5-1: There are some other components (e.g., memory) will be affect by R18 features.

Obserbation 5-2: BW reduction to 5MH and reduced peak data rate are both lead to a lower peak data rate, which means lower memory requirements, then the less cost of memory.

Obserbation 5-3: The cost reduction for memory is the other important motivation for R18 RedCap.

Summary and preference

In chapter 2, we analyzed and compared two main solutions (i.e., BW reduction to 5MHz and reduced peak data rate) for R18 RedCap. For reduced peak data rate, we think there are at least three-detailed methods can be further considered, i.e., a) restricted bandwidth for PDSCH and/or PUSCH, b) TBS restriction. c) Scaling factor, and we think the essence of these three methods is the same. Further, we observed that compared with further BW reduction to 5MHz, reduced UE peak data rate brings less spec impacts, performance loss and limitations, while achieve the similar cost reductions.

In chapter 3, we analyzed a supplementary solution (i.e., processing time relaxation) for R18 RedCap. First, we think the values to be considered in R18 can be the same to those assumed in R17, i.e., the values for N1/N2 are assumed to be doubled compared to those of capability 1. Based on the doubled N1/N2, we observed that processing time relaxation brings relatively small impacts while brings  considerable benefits to UE implementation (complexity reduction and power saving).

In chapter 4, we combined the main solutions and the supplementary solution, and we analyzed the complexity reduction of the combinations. We observed that the cost reductions of the combinations are relatively attractive compared to single solution.

In chapter 5, we explained the additional cost reduction due to the reduced requirements of memory, and we see the necessity to consider and reflect the impact of the memory cost in R18.

Based on the above, we have the following proposals:

Proposal 1: Adopt the solution of reduced UE peak data rate for R18 RedCap, the detailed methods to reduce the UE peak data rate can be determined in the next meeting or in the WI.

Proposal 2: For reduced UE peak data rate, at least the following detailed methods should be considered.

- Restricted bandwidth for PDSCH and/or PUSCH

- TBS restriction.

- Scaling factor

Proposal 3: Adopt the solution of processing time relaxation for R18 RedCap, and the value of N1/N2 are assumed to be doubled compared to those of capability 1.

Proposal 4: The memory cost should be considered in Rel.18 and capture the impact of R18 features on memory cost in the corresponding TR.

Conclusion

Observations

Based on the analyses and discussions, we have the following observations:

- Observations for UE bandwidth reduction to 5MHz in FR1 (chapter 2.1)

Observation 2.1-2: For 5MHz BW, the SSB can be received for 15/30KHz SCS.

Observation 2.1-3: For 5MHz BW, the CORESET#0 can be received only when a) the SCS of CORESET#0 and SSB are equal to 15KHz, and b) the index of table 13-1 are limited to the first six index.

Observation 2.1-4: For 5MHz BW, if 30KHz needs to be supported, new pattern(s) of CORESET#0 may be needed, great spec impacts were expected.

Observation 2.1-5: For 5MHz BW, the SIB-PDSCH needs to be scheduled within 5MHz BW.

Observation 2.1-6: 5 MHz maximum UE bandwidth in FR1 can fulfil the data rate requirements of low-end RedCap use cases.

Observation 2.1-7: Reduce the maximum UE bandwidth to 5MHz in FR1 will reduce the coverage due to the limitation of max AL of PDCCH.

Observation 2.1-8: Reduce the maximum UE bandwidth to 5MHz in FR1 will increase the PDCCH blocking rate and latency .

Observation 2.1-9: If the separate initial BWP introduced in R17 is shared by R17 RedCap and 5MHz eRedCap, small or even no spec impacts were expected, but limitation to R17 RedCap was observed .

Observation 2.1-10: If another separate initial BWP is introduced for R18 5MHz RedCap, great negative impacts to UE implementation, spec, standardization workload were expected.

Observation 2.1-11: With 5MHz BW, the flexibility of RACH configuration is quite limited.

Observation 2.1-12: For 5MHz BW, either great spec impacts or great limitations were observed.

- Observations for Reduced UE peak data rate in FR1 (chapter 2.2)

Observation 2.2-1: For reduced UE peak data rate, it was observed that there are at least three detailed methods can be considered.

- Restricted bandwidth for PDSCH and/or PUSCH

- TBS restriction.

- Scaling factor

Observation 2.2-2: The essence of these three methods for reduced UE peak data rate is TBS limitation within a TTI.

Observation 2.2-3: Compared with R17 simplest RedCap UE, the additional cost reductions for peak data reduction are about 5.44% for FDD and 4.27% for TDD.

Observation 2.2-4: The cost reductions of peak data reduction are comparable to those of reduce the maximum BW to 5MHz.

Observation 2.2-5: For reduced peak data rate, no issues were observed for the reception of SSB and CORESET#0.

Observation 2.2-6: If the method of restricted bandwidth for PDSCH and/or PUSCH is adopted, there will be a limitation for SIB scheduling.

Observation 2.2-7: With the method of TBS restriction and scaling factor, no issues were observed for SIB reception.

Observation 2.2-8: Reduced UE peak data rate to 10Mbps in FR1 can fulfil the data rate requirements of low-end RedCap use cases.

Observation 2.2-9: As the control channel BW is 20MHz, the coverage performance is similar to R17 RedCap

Observation 2.2-10: As the control channel BW is 20MHz, the PDCCH blocking rate is similar to R17 RedCap

Observation 2.2-11: As the TBS is restricted, the latency may be increased (compared to R17 RedCap).

Observation 2.2-12:  With reduced peak data rate, the reception of Msg4 may be impacted in principle. However, according to the logs from current 5G network, the payload of Msg4 is smaller than 5000bits.

Observation 2.2-13: With reduced peak data rate, the initial BWP for R17 RedCap can be resued by R18 eRedCap, e.g., there is no need to introduce another initial BWP for R18 RedCap, and no limitation on R17 initial BWP.

Observation 2.2-14: The expected spec impacts of the detailed methods of reduced peak data rate are very small.

Observation 2.2-15: The scaling factor based method was discussed in the later phase of R17 WI, and the only impact observed for scalingFactor is constraint relaxation.

- Observations for summary and Comparison of main solutions (chapter 2.3)

Observation 2.3-1: Compared with further BW reduction to 5MHz, reduced UE peak data rate brings less spec impacts, performance loss and limitations.

- Observations for relaxed UE processing timeline (chapter 3.1)

Observation 3.1-1: For relaxed UE processing timeline, the doubled N1 and N2 compared to those of capability 1 were studied in R17.

Observation 3.1-2: Compared with Rel-17 simplest RedCap UE, the additional cost reductions are about 3.15% for FDD and 2.97% for TDD.

Observation 3.1-3: With relaxed UE processing timeline, no issues were observed for the reception of SSB, CORESET#0 and SIB.

Observation 3.1-4: With relaxed UE processing timeline, some negative impacts to the access latency were observed.

Observation 3.1-5: The other impacts of processing time relaxation are relatively small.

- Observations for Solution combinations (chapter 4)

Observation 4.1-1: Compared with Rel-17 simplest RedCap UE, the cost reductions for combination 1 are 8.87% for FDD and 7.08% for TDD.  The cost reductions are relatively attractive compared to single solution.

Observation 4.2-2: Compared with Rel-17 simplest RedCap UE, the cost reductions for combination 2 are 7.31% for FDD and 6.08% for TDD.  The cost reductions are relatively attractive compared to single solution.

Observation 4.2-3: The additional cost reductions for the combination 1 (i.e., reduced peak data rate and processing time relaxation) are comparable to those for the combination2 (i.e., 5MHz BW and processing time relaxation).

- Observations for other cost views for the solutions (chapter 5)

Observation 5-1: There are some other components (e.g., memory) will be affect by R18 features.

Obserbation 5-2: BW reduction to 5MH and reduced peak data rate are both lead to a lower peak data rate, which means lower memory requirements, then the less cost of memory.

Obserbation 5-3: The cost reduction for memory is the other important motivation for R18 RedCap.

Proposals

Based on the analyses and above observations, we have the following proposals:

Proposal 1: Adopt the solution of reduced UE peak data rate for R18 RedCap, the detailed methods to reduce the UE peak data rate can be determined in the next meeting or in the WI.

Proposal 2: For reduced UE peak data rate, at least the following detailed methods should be considered.

- Restricted bandwidth for PDSCH and/or PUSCH

- TBS restriction.

- Scaling factor

Proposal 3: Adopt the solution of processing time relaxation for R18 RedCap, and the value of N1/N2 are assumed to be doubled compared to those of capability 1.

Proposal 4: The memory cost should be considered in Rel.18 and capture the impact of R18 features on memory cost in the corresponding TR.

Reference

- RP-213661, New SID on Study on further NR RedCap UE complexity reduction. RAN#94e, December 6 - 17, 2021.

- RP-220966, Revised WID on support of reduced capability NR devices. RAN#95e, 17th -23rd March 2022.

- R1-2203339 Discussion on evaluation needs and assumptions for eRedCap, RAN1#109e, May 9th – 20th, 2022.

- TS 38.211-h10, Physical channels and modulation-Release 17, V17.1.0 (2022-03).

- RWS-210121, Railway, Smart Grid and PPDR support in Rel-18. Nokia, RAN Rel-18 Workshop, 28th June – 2nd July 2021.

- TS 38.213-h10, Physical layer procedures for control-Release 17, V17.1.0 (2022-03).

- R1-2008100, Discussion on potential UE complexity reduction features, RAN1#103e, October 26th – November 13th, 2020.

- TS 38.321, Medium Access Control (MAC) protocol specification- Release 17, V17.0.0 (2022-03)

- TS 38.331, Radio Resource Control (RRC) protocol specification-Release 17, V17.0.0 (2022-03)

- TS 38.214, Physical layer procedures for data-Release 17, V17.1.0 (2022-03).

- R1-2107385, Discussion on scaling factor for RedCap, Spreadtrum Communications, Apple, CEPRI, RAN1#106e. August 16th – 27th, 2021

- R1-2109837, Discussion on L2 buffer size reduction for RedCap, Spreadtrum Communications, CAICT, CATT, CEPRI, China Unicom, October 11th – 19th, 2021.

- TS 38.306, User Equipment (UE) radio access capabilities-Release 17, V17.0.0 (2022-03)

- TR 38.875, Study on support of reduced capability NR devices- Release 17, V17.0.0 (2021-03).