3GPP TSG RAN WG1 #109-e                                                       R1-2203342

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.7.2

Source:	Spreadtrum Communications

Title:	Discussion on network energy saving techniques

Document for:	Discussion and decision

# Introduction

In [1], one objective of network energy saving is to study and identify techniques.

- Study and identify techniques on the gNB and UE side to improve network energy savings in terms of both BS transmission and reception, which may include: - How to achieve more efficient operation dynamically and/or semi-statically and finer granularity adaptation of transmissions and/or receptions in one or more of network energy saving techniques in time, frequency, spatial, and power domains, with potential support/feedback from UE, and potential UE assistance information [RAN1, RAN2] - Information exchange/coordination over network interfaces [RAN3] Note: Other techniques are not precluded

This contribution discusses the network energy saving techniques in high level.

To identify the benefits for techniques, we also provide initial evaluation results based on the energy consumption model listed in our companion contribution [2]. In our evaluations, we assume the capacity and coverage keep unchanged for a given scenario, because the energy consumption in constant capacity and coverage can represent the energy efficiency.

# Time domain techniques

## Candidate techniques

For time domain techniques, there are some candidates.

- Reduction of common signal/channel: SSB can be muted and UE can achieve timing reference from other cells, or SSB is only sent when needed;

- Switching on/off of BS: BS can be switched on/off based on some factors, e.g. UE distribution, packet arrival;

## Initial evaluation results

### Reduction of common signal/channel

The pre-assumption for reduction of common signal/channel is that the common signal/channel can be switched off in some cases, e.g. no UE camps on a specific cell. Thus, we only evaluate a simple scenario shown as follows.

- Scenario:

  - 1 micro cell

  - Empty loading (e.g. 10% loading including SSB transmission)

  - 20 slots duration @15kHz SCS, 20ms duration

- Baseline: SSB with 20ms periodicity; 2 slots contain SSB which have 10% loading; 6 slots are transition slots; 12 slots are slots in the light sleep state

  - Total: E_micell_10p * 2 + E_micell_lighttrns + E_micell_lightslp * 12 = 750 * 2 + 400 + 200 * 12 = 4300, where E_micell_10p is the energy consumption of micro cell BS in 10% loading, E_micell_lighttrns is the transition energy of micro cell BS in the light sleep, E_micell_lightslp is the energy consumption of micro cell BS in the light sleep.

- SSB-less: 6 slots are transition slots; 14 slots are slots in the light sleep state

  - Total: E_micell_lighttrns + E_micell_lighslp * 14 = 400 + 200 * 14 = 3200

  - Gain: (4300 – 3200) / 4300 ~= 26%

The nominator of the above gain can be written as (E_micell_10p – E_micell_lightslp) * 2. Thus, (E_micell_10p – E_micell_lightslp) is important for the gain, which means the difference between empty loading and the light sleep for micro cell BS.

Observation 1: The energy difference between empty loading and the light sleep is important for energy saving gain of reduction of common signal/channel.

### Switching on/off of BS

Directly switching a BS off can provide the energy saving gain. However, it is not feasible to directly switch a BS off with loss of capacity if we want to exploit the energy efficiency. Thus, we need to study a more complicated scenario where the total capacity is maintained by shifting loading from micro cells to a macro cell.

- Scenario:

  - 1 macro cell plus 3 micro cells

  - Low loading (30% loading)

  - 20 slots duration @15kHz SCS, 20ms duration

- Baseline:

  - 1 macro cell: 20 slots has 30% loading

  - 3 micro cells: 20 slots has 30% loading

  - Total: E_macell_30p * 20 + E_micell_30p * 20 * num_micell = 1875 * 20 + 1250 * 20 * 3 = 37500 + 75000 = 112500, where E_macell_30p is the energy consumption of macro cell BS in 30% loading, E_macell_30p is the energy consumption of micro cell BS in 30% loading and num_micell is the number of micro cell BS.

- Switching off of the micro cell BS and shifting loading to the macro cell:

  - 1 macro cell: 10 slots has 30% loading; 10 slots has 60% loading (assume having the same energy consumption as 50% loading)

  - 3 micro cell: 10 slots has 30% loading; 6 slots are transition slots; 4 slots are in the light sleep state

  - Total: E_macell_30p * 10 + E_macell_60p * 10 + (E_micell_30p * 10 + E_micell_lighttrns + E_micell_lightslp * 4) * num_micell = 1875 *10 + 3375 * 10 + (1250 *10 + 400 + 200 * 4) * 3 = 52500 + 41100 = 93600

  - Gain: (112500 – 93600) / 112500 ~= 17%

The nominator of the above gain can be written as (E_macell_30p - E_macell_60p) * 10 + (E_micell_30p * 10 - E_micell_lighttrns - E_micell_lightslp * 4) * num_micell, and (E_macell_30p - E_macell_60p) is negative part, and (E_micell_30p * 10 - E_micell_lighttrns - E_micell_lightslp * 4) is positive part. Thus, (E_micell_30p * 10 - E_micell_lighttrns - E_micell_lightslp * 4) is important for the gain, which means the difference between low loading and the light sleep (including transition energy) for micro cell BS.

Observation 2: The energy difference between low loading and the light sleep (including transition energy) is important for energy saving gain of switching on/off of BS.

# Frequency domain techniques

## Candidate techniques

For frequency domain techniques, there are some candidates.

- Bandwidth adaptation: The used bandwidth can be enlarged or reduced based on some factors, e.g. UE distribution, packet arrival;

## Initial evaluation results

### Bandwidth adaptation

Directly reducing the bandwidth of a BS can provide the energy saving gain obviously. However, it is not a feasible scenario to directly reduce the bandwidth of a BS with loss of capacity. Thus, we need to study a more complicated scenario where the total capacity is maintained by shifting loading from micro cells to a macro cell.

- Scenario:

  - 1 macro cell plus 3 micro cells

  - Low loading (30% loading)

  - 20 slots duration @15kHz SCS, 20ms duration

- Baseline:

  - 1 macro cell: 20 slots has 30% loading

  - 3 micro cells: 20 slots has 30% loading

  - Total: E_macell_30p * 20 + E_micell_30p * 20 * num_micell = 1875 * 20 + 1250 * 20 * 3 = 37500 + 75000 = 112500

- Reducing the used bandwidth of the micro cell BS to half, and shifting loading to the macro cell:

  - 1 macro cell: 10 slots has 30% loading; 10 slots has 50% loading

  - 3 micro cell: 10 slots has 30% loading; 10 slots has 10% loading

  - Total: E_macell_30p * 10 + E_macell_50p * 10 + (E_micell_30p * 10 + E_micell_10p * 10) * num_micell = 1875 *10 + 3375 * 10 - (1250 *10 + 750 * 10) * 3 = 52500 + 60000

  - Gain: (112500 – 112500) / 112500 = 0%

The nominator of the above gain can be written as (E_macell_30p - E_macell_50p) * 10 + (E_micell_30p * 10 – E_micro_10p * 10) * num_micell, and (E_macell_30p - E_macell_50p) is negative part, and (E_micell_30p * 10 – E_micell_10p * 10) is positive part. However, it seems the positive part is cancelled by the negative part. The reason could be the energy consumption of bandwidth scaling is proportional to that of loading scaling, and if the total loading is unchanged, the bandwidth adaptation cannot improve the energy efficiency.

Observation 3: If the energy consumption of bandwidth scaling is proportional to that of loading scaling, and if the total loading is unchanged, the bandwidth adaptation cannot improve the energy efficiency.

# Spatial domain techniques

## Candidate techniques

For spatial domain techniques, there are some candidates.

- Reduction of beams of common signal/channel: A part of SSB beams can be muted based on some factors, e.g. UE distribution, packet arrival;

- TRX (antenna ports) adaptation: A part of TRX can be switched off based on some factors, e.g. UE distribution, packet arrival;

## Initial evaluation results

### Reduction of beams of common signal/channel

The pre-assumption for reduction of beams of common signal/channel is that the common signal/channel can be switched off in some cases, e.g. no UE camps on some beam areas in a specific cell. Thus, we only evaluate a simple scenario shown as follows.

- Scenario:

  - Micro cell

  - Empty loading (e.g. 10% loading including SSB transmission)

  - 20 slots duration @15kHz SCS, 20ms duration

- Baseline: SSB with 20ms periodicity; 2 slots contain SSB which have 10% loading; 6 slots are transition slots; 12 slots are slots in the light sleep state

  - Total: E_micell_10p * 2 + E_micell_lighttrns + E_micell_lightslp * 12 = 750 * 2 + 400 + 200 * 12 = 4300

- Closing half of SSB beams: SSB with 20ms periodicity; 2 slots contain the half SSB beam; 6 slots are transition slots; 12 slots are slots in the light sleep state

  - Total: E_micell_10p * 50% * 2 + E_micell_lighttrns + E_micell_lightslp * 12 = 750 * 5% * 2 + 400 + 200 * 12 = 3550

  - Gain: (4300 – 3550) / 4300 ~= 17%

The nominator of the above gain can be written as (E_micell_10p * 50% * 2). Thus, E_micell_10p is important for the gain, which means empty loading for micro cell BS.

Observation 4: The energy for empty loading is important for energy saving gain of reduction of beams of common signal/channel.

### TRX adaptation

For simplification, we assume the simple scenario that micro cell BS use only a part of TRX (beams) to serve UEs while the used bandwidth is increasing. In this scenario, micro cell BS should know which TRX (beams) can be suitable to serve the UEs. In fact, in this scenario, the used bandwidth is exchanged for the number of TRX.

- Scenario:

  - Micro cell

  - Low loading (e.g. 30% loading)

  - 20 slots duration @15kHz SCS, 20ms duration

- Baseline: 20 slots has 30% loading

  - Total: E_micell_30p * 20 = 1250 * 20 = 25000

- TRX adaptation: Micro cell BS closes half of TRX. 20 slots has 50% loading (the loading increase represents the bandwidth increase), but with half of TRX

  - Total: E_micell_50p * 50% * 20 = 2250 * 50% * 20 = 22500

  - Gain: (25000 – 22500) / 250000 ~= 10%

The nominator of the above gain can be written as (E_micell_30p – E_micell_50p * 50%) * 20. Thus, (E_micell_30p – E_micell_50p * 50%) is important for the gain, which means the difference between empty loading and medium loading with TRX reduction for micro cell BS. In other words, if the gain is positive, reducing the number of TRX is more energy efficient than increasing the used bandwidth in our energy model.

Observation 5: The energy difference between empty loading and medium loading with TRX reduction is important for energy saving gain of TRX adaptation.

# Summary of our evaluation results for the candidate techniques

From above our evaluations, we summarize our simple evaluation results for the candidate techniques in the following table.

Table 1: The summary of our evaluation results for the candidate techniques

|  | Reduction of common signal/channel | Switch on/off of BS | Bandwidth adaptation | Reduction of beams of common signal/channel | TRX adaptation |
| --- | --- | --- | --- | --- | --- |
| Scenario | Homo-Net; Empty loading | Het-Net; Low loading | Het-Net; Low loading | Homo-Net; Empty loading | Homo-Net; Low loading |
| Technique details | SSB is muted in some cases | Switch off the BS and transfer the loading | Reduce the used bandwidth and transfer the loading | Switch off a part of SSB beams | Switch off a part of TRX |
| Energy saving gain (indeed it reflect energy efficiency gain) | Yes; Reducing the transmission occasions | Yes; Shifting loading to the BS with higher energy efficiency | Maybe No; Bandwidth is proportional to loading | Yes; Reducing the transmission beams | Yes Shifting loading to a part of beams |
| Scenario | Homo-Net; Empty loading | Het-Net; Low loading | Het-Net; Low loading | Homo-Net; Empty loading | Homo-Net; Low loading |
| Priority | High | High | Low | High | High |

According above summary table, we have the following proposal.

Proposal 1: Consider the following techniques:

- Time domain: Reduction of common signal/channel, switch on/off of BS

- Frequency domain: Bandwidth adaptation

- Spatial domain: Reduction of beams of common signal/channel, TRX adaptation

Proposal 2: The following techniques have higher priority:

- Time domain: Reduction of common signal/channel, switch on/off of BS

- Spatial domain: Reduction of beams of common signal/channel, TRX adaptation

# Conclusion

As the conclusion, we have the following observations.

Observation 1: The energy difference between empty loading and the light sleep is important for energy saving gain of reduction of common signal/channel.

Observation 2: The energy difference between low loading and the light sleep (including transition energy) is important for energy saving gain of switching on/off of BS.

Observation 3: If the energy consumption of bandwidth scaling is proportional to that of loading scaling, and if the total loading is unchanged, the bandwidth adaptation cannot improve the energy efficiency.

Observation 4: The energy for empty loading is important for energy saving gain of reduction of beams of common signal/channel.

Observation 5: The energy difference between empty loading and medium loading with TRX reduction is important for energy saving gain of TRX adaptation.

We have the following proposals.

Proposal 1: Consider the following techniques:

- Time domain: Reduction of common signal/channel, switch on/off of BS

- Frequency domain: Bandwidth adaptation

- Spatial domain: Reduction of beams of common signal/channel, TRX adaptation

Proposal 2: The following techniques have higher priority:

- Time domain: Reduction of common signal/channel, switch on/off of BS

- Spatial domain: Reduction of beams of common signal/channel, TRX adaptation

# Reference

- RP-213554, “New SI: Study on network energy savings for NR”, RAN#94e, Huawei, Dec.  6th - 17th, 2021.

- R1-2203341, “Discussion on performance evaluation of network energy savings”, RAN1#109e, Spreadtrum, May 9th - 20th, 2022.