3GPP TSG RAN WG1 #109-e			R1-2203345

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.9.2

Source: 	Spreadtrum Communications

Title: 	Discussion on UE support for two overlapping CRS rate matching patterns

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new Work Item was agreed to support the following objectives.

The following objectives shall be included for improvement of NR spectrum efficiency for LTE-NR co-existence (RAN1): - Study and if needed specify NR PDCCH reception in symbols with LTE CRS REs. [RAN1] - Investigate enabling LTE CRS to puncture NR PDCCH, including the impact to NR PDCCH DMRS if there is the performance gain from the additional PDCCH resources. - Allow a UE to support, and be configured with, two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP [RAN1, RAN2]

This contribution provides some considerations on UE supporting for two overlapping CRS rate matching patterns regardless of support or configuration of multi-TRP.

Discussion

In the current RAN1 spec, two CRS rate matching patterns overlapping in frequency is only supported if PDSCH transmission scheme is multi-DCI based M-TRP. As shown below in yellow highlight quoted from 38. 214 clause 5.1.4.2.

A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE in RRC_IDLE and RRC_INACTIVE modes for broadcast reception, REs indicated by 'RateMatchPatternLTE-CRS' in PDSCH-Config-MCCH or PDSCH-Config-MCCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - Each RateMatchPatternLTE-CRS configuration contains v-Shift consisting of LTE-CRS-vshift(s), nrofCRS-Ports consisting of LTE-CRS antenna ports 1, 2 or 4 ports, carrierFreqDL representing the offset in units of 15 kHz subcarriers from (reference) point A to the LTE carrier centre subcarrier location, carrierBandwidthDL representing the LTE carrier bandwidth, and may also configure mbsfn-SubframeConfigList representing MBSFN subframe configuration. A UE determines the CRS position within the slot according to Clause 6.10.1.2 in [15, TS 36.211], where slot corresponds to LTE subframe. - If the UE is configured by higher layer parameter PDCCH-Config with two different values of coresetPoolIndex in ControlResourceSet and is also configured by the higher layer parameter lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 in ServingCellConfig, the following REs are declared as not available for PDSCH: - if the UE is configured with crs-RateMatch-PerCoresetPoolIndex, REs indicated by the CRS pattern(s) in lte-CRS-PatternList1-r16 if the PDSCH is associated with coresetPoolIndex set to '0', or the CRS pattern(s) in lte-CRS-PatternList2-r16 if PDSCH is associated with coresetPoolIndex set to '1'; - otherwise, REs indicated by lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16, in ServingCellConfig.

The lte-CRS-PatternList1-r16 and lte-CRS-PatternList2-r16 are defined in 38331 as below.

lte-CRS-PatternList1 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH. The LTE CRS patterns in this list shall be non-overlapping in frequency. The network does not configure this field and lte-CRS-ToMatchAround simultaneously.

lte-CRS-PatternList2 A list of LTE CRS patterns around which the UE shall do rate matching for PDSCH scheduled with a DCI detected on a CORESET with CORESETPoolIndex configured with 1. This list is configured only if CORESETPoolIndex configured with 1. The first LTE CRS pattern in this list shall be fully overlapping in frequency with the first LTE CRS pattern in lte-CRS-PatternList1, The second LTE CRS pattern in this list shall be fully overlapping in frequency with the second LTE CRS pattern in lte-CRS-PatternList1, and so on. Network configures this field only if the field lte-CRS-ToMatchAround is not configured and there is at least one ControlResourceSet in one DL BWP of this serving cell with coresetPoolIndex set to 1.

Thus, it is clear that the CRS pattern in one list cannot be overlap with each other. And the corresponding CRS pattern in list 1 and list 2 are fully overlapping because those two lists are for two TRP. However, the cell edge UEs need to avoid serious interference from two LTE cells. Therefore, it would be good for UE to be configured with two CRS pattern even if it is without mTRP operation. Thus the restriction should be removed.

In this case, there can be two alternatives to achieve this purpose:

Alt1: CRS patterns in one list to support overlapping. Nevertheless, it is against the main intention of the CRS patterns in one list is only for one cell. This alternative would make the whole structure more difficult to indicate which CRS patterns REs are reserved for PDSCH reception.

Alt2: introduce another CRS pattern list in Rel-18. CRS patterns in the new list can be overlapped with original list even without mTRP configuration. E.g. CRS pattern in lte-CRS-PatternList3 can be overlap with CRS pattern in lte-CRS-PatternList1, without the restriction of multi-TRP configuration. This alternative is straight forward and have good backward compatibility with Rel-16 DSS and MIMO. So we support Alt 2 and have the following proposal.

- For two overlapping CRS rate matching patterns, introduce another CRS pattern list (lte-CRS-PatternList3), which can be configured  regardless of multi-TRP

  - CRS patterns in lte-CRS-PatternList3 can be overlap with CRS patterns in lte-CRS-PatternList1

The second issue is when a UE is configured with two overlapping CRS patterns, the REs in both lists are non-available for PDSCH or can be selected only one for rate matching, like multiple TRP case which list is reserved depend on the coresetPoolIndex. From our understanding, since the main intention is to mitigate the interference of LTE CRS from neighbouring LTE cells to NR cells. This condition is static, so no strong technical reason to support dynamic indication between those two CRS pattern lists. PDSCH rate matching around those two CRS patterns can based on RRC configuration, i.e. if the lte-CRS-PatternList3 is configured, then REs indicated by lte-CRS-PatternList1 and lte-CRS-PatternList3 are considered to be reserved.

- REs indicated by lte-CRS-PatternList1 and lte-CRS-PatternList3 are considered to be reserved for PDSCH, once lte-CRS-PatternList3 is configured.

  - Without dynamic indicate which one CRS Pattern List is used for a PDSCH.

Next, if the third CRS pattern list is configured, it should clarify whether it can be used together with lte-CRS-PatternList1, or with lte-CRS-PatternList1 and lte-CRS-PatternList2 simultaneously. Our understanding is multi-DCI in mTRP is target for cell edge UE, too. There is a possibility that one UE is deployed with mTRP and also need to do interference from the neighbour LTE cell. We are open for the discussion.

The last but not least is the maximum number of LTE CRS overlapping rate matching patterns within NR carrier should be maintained. This aligns with the principle when CRS pattern lists introduced in Rel-16 DSS and MIMO. The FGs are listed in the appendix. It can keep the complexity of UE implementation for CRS rate matching.

Similarly, it can introduce a new UE capability reporting for this feature. And the number of the additional CRS rate matching patterns of Rel-18 is accounted in the total number of rate matching pattern reported by the UE for Rel-15.

- Maximum number of LTE CRS overlapping rate matching patterns within NR carrier should be maintained

- Introduce a new UE capability report for two LTE CRS overlapping rate matching patterns without mTRP, and the number of the additional CRS rate matching patterns is accounted in the total number of rate matching pattern reported by the UE.

Conclusion

In this contribution, we made the following observation and proposals.

- Maximum number of LTE CRS overlapping rate matching patterns within NR carrier should be maintained

- For two overlapping CRS rate matching patterns, introduce another CRS pattern list (lte-CRS-PatternList3), which can be configured  regardless of multi-TRP

  - CRS patterns in lte-CRS-PatternList3 can be overlap with CRS patterns in lte-CRS-PatternList1

- REs indicated by lte-CRS-PatternList1 and lte-CRS-PatternList3 are considered to be reserved for PDSCH, once lte-CRS-PatternList3 is configured.

  - Without dynamic indicate which one CRS Pattern List is used for a PDSCH.

- Introduce a new UE capability report for two LTE CRS overlapping rate matching patterns without mTRP, and the number of the additional CRS rate matching patterns is accounted in the total number of rate matching pattern reported by the UE.

References

RP-213575 New WI: Enhancement of NR Dynamic spectrum sharing (DSS)

- Appendix

| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | 1) Maximum number of LTE-CRS rate matching patterns in total within a NR carrier using 15 kHz SCS 2) Maximum number of LTE-CRS non-overlapping rate matching patterns within a NR carrier using 15 kHz SCS | 5-28 (Rate-matching around LTE CRS) | multipleRateMatchingEUTRA-CRS-r16 { maxNumberPatterns-r16, maxNumberNon-OverlapPatterns-r16} | BandNR | n/a | n/a (FR1 only) | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot UE reporting component 1 for 14-1 also reports component 2. Reporting of values of Component 1 larger than two is only applicable when reporting values of Component 2 larger than one. | Optional with capability signalling Component 1:{2, 3, 4, 5, 6} Component 2: {1, 2, 3} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14-1a | Two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz overlapping with a LTE carrier | 1. Support of two LTE-CRS overlapping rate matching patterns within a part of NR carrier using 15 kHz SCS overlapping with a LTE carrier | 14-1 | overlapRateMatchingEUTRA-CRS-r16 | BandNR | n/a | n/a (FR1 only) | For DSS The number of the additional CRS rate matching patterns reported in Rel-16 is accounted in the total number of rate matching pattern reported by the UE for Rel-15 by using pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot and pdsch-RE-MappingFR1-PerSymbol/pdsch-RE-MappingFR1-PerSlot | Optional with capability signaling |