3GPP TSG RAN WG1 #109-e			R1-2203346

e-Meeting, May 9th – 20th, 2022

Agenda item:	9.10.1

Source: 	Spreadtrum Communications

Title: 	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion and decision

Introduction

In RAN#94 meeting, a new Work Item was agreed to support the following objective.

1. Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1] - Identify the maximum number of cells that can be scheduled simultaneously - Consider both intra-band and inter-band CA operation - Consider both FR1 and FR2 - The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

This contribution provides some considerations on multi-cell PUSCH/PDSCH scheduling with a single DCI.

Discussion

Scenarios

During WID discussion in RAN#94 meeting, there is no limitation on the scenarios that can be used for multi-cell PUSCH/PDSCH scheduling via one single DCI, all the possible combinations were suggested to be decided in WI stage. Thus, there should be some studies on the applicable scenarios to check whether those cases should be involved in the scope.

Table 1: The multi-cell combinations allowed for one single DCI scheduling

| Scenarios | The multi-cell combinations via one single DCI scheduling |
| --- | --- |
| 1 | FR1 and FR2 - Whether scheduling and scheduled cells are in the same frequency range or can be in the different frequency ranges - Whether both FR2-1 and FR2-2 are included by FR2 |
| 2 | Intra-band and inter-band - Whether scheduling and scheduled cells are intra-band or can be inter-band. |
| 3. | PCell scheduled by sSCell in FR2 - Whether this Rel-17 DSS enhancement is in the scope, - If so, the Rel-17 DSS structure is reused, or unified solution of Rel-18 multi-cell PUSCH/PDSCH scheduling with a single DCI is applied. |
| 4 | Licensed and unlicensed band - Whether unlicensed band is supported - If so, whether scheduling and scheduled cells are separately allowed in licensed band and unlicensed band, or otherwise. |
| 5. | Same and different TAG - Whether scheduling and scheduled cells are allowed in same TAG or can be in different TAG. |
| 6. | Same and different PUCCH group - Whether scheduling and scheduled cells are allowed in same PUCCH group or can be in different PUCCH group |
| 7 | Different priority scheduling - Whether multi-cell PUSCHs/PDSCHs scheduling with a single DCI are with same priority or can be different priority, e.g. different priority HARQ-ACK for PDSCHs, and HP and LP PUSCH scheduling with one DCI |

Considering the limited TU allocation, it would be good to do down selection among all those multi-cell combinations to save time, such as to exclude some scenarios directly from the first beginning. Therefore, we have the following proposal.

- It is suggested to study and decide the scope of multi-cell combinations via one single DCI scheduling, to do down select among all those conditions:

  - FR1 and FR2

  - Intra-band and inter-band

  - PCell scheduled by sSCell in FR2

  - Licensed and unlicensed CCs

  - Belong to Different TAG cells

  - CCs from same PUCCH group or different PUCCH group

  - Different priority scheduling

For Scenario#1, Scenario#2, and Scenario5, they can be in the scope. One simple example is when cross carrier scheduling, the PDCCH and its scheduling PDSCH/PUSCH can be in the same frequency range or can be in the different frequency ranges, and they can be intra-band cells or inter-band cells, furthermore, there is no TAG limit when configure the cross carrier scheduling but with the TA impact included in the processing time. There is no frequency range and band restrictions. Therefore, for multi-cell scheduling, they share the similarly implementation when considering the relationship between PDCCH and PDSCH, the difference is there would more PDSCHs or PUSCHs associated with one PDCCH.

For Scenario#3 PCell scheduled by sSCell in FR2, we are not sure the benefit comparing with PCell scheduled by sSCell in FR1. Because FR2 has limited coverage, especially for PDCCH transmission, it is a big problem. Before sufficient justification, we prefer to preclude this out of scope, or with low priority, unless the final structure can be immediate used for PCell scheduled by sSCell in FR2 without any change.

For Scenario#6, if the CCs scheduled by one DCI are all belong to same PUCCH group can use one PUCCH resource for HARQ-ACK feedback. Otherwise, different PUCCH resource would be indicated by the DCI, which is not preferred. Thus we recommend to limit multiple cells scheduled by one DCI belonging to the same PUCCH group for easy HARQ-ACK on PUCCH report.

For Scenario#7 Different priority scheduling, such as one DCI scheduling both high priority PUSCH and Low priority PUSCH on different cells, it will lead to complex configuration and DCI design. First, HP and LP have different reliable requirement for PDCCH, it does make sense to use the single DCI always targeting the high priority BLER even for low priority scheduling. Second, this HP and LP scheduling have different delay requirement. For example, HP HARQ-ACK and LP HARQ-ACK may be configured with different sub slot length. The PDSCH to HARQ-ACK feedback timing will be very complex and with many issue problems to be handled, like which PDSCH is used as the reference time for HARQ-ACK, the HP or LP or the latest ending symbol. So we prefer to limited to same priority scheduling for this feature, at least in Rel-18.

- Scenario#3 PCell scheduled by sSCell in FR2 can be with lower priority

- For Scenario#7 Different priority scheduling can be with lower priority

- Multiple cells scheduled by one DCI should belong to the same PUCCH group

Maximum number of cells that can be scheduled simultaneously

In Rel-17 DSS, up to two cells PDSCH were discussed, and found that it is too limited for the one DCI scheduling multiple cells. From this point, more number of cells such as more than 2 can be considered to be supported in this WI, for better feasibility and usable of this feature. Another understanding is when the supported cell number by one DCI is increased, the DCI size would be rapidly multiplied, which requires larger aggregation level. So the simultaneous scheduling cell number should not be too big to beyond the maximum DCI sizes which fit to AL16.

- At least Support up to 4 cells scheduling with a single DCI.

Framework for multi-cell scheduling

In Rel-17 CA, self-carrier scheduling and cross-carrier scheduling are supported. When self-carrier scheduling is used, the PDCCH and PDSCH are on the same cell. While cross-carrier scheduling is used for one serving cell, the PDCCH is on another serving cell and PDSCH is on this cell. Before Rel-17 DSS, either self-carrier scheduling or cross-carrier scheduling is configured for one cell. In Rel-17 DSS, PSCell can be configured to both of self-carrier scheduling and cross-carrier scheduling.

Relationship with self-carrier scheduling and cross-carrier scheduling

According to multi-cell scheduling, the first thing is to define its framework, including the relationship with the current self-carrier scheduling and cross-carrier scheduling.

The Table 2 gives all possible implementation of multi-cell scheduling. If the column and row with the same content setting, it means one cell only use one type of scheduling, self-carrier scheduling or cross-carrier scheduling. If they are with different content, it means this cell can use different type of scheduling combination, such as self-carrier scheduling and cross-carrier scheduling introduced in Rel-17 DSS.

Therefore, there can be three type of PDCCH location according to multi-cell scheduling:

- Case 0 one cell’s scheduling only from multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling

- Case 1, one cell is configured as self-carrier scheduling, meanwhile can be multi-cell scheduling by another cell.

- Case 2, one cell is configured as cross-carrier scheduling, meanwhile can be multi-cell scheduling by another cell.

Table 2: Relationship with the current self-carrier scheduling and cross-carrier scheduling

| Combinations | Self-carrier scheduling | Cross-carrier scheduling | Multi-cell scheduling |
| --- | --- | --- | --- |
| Self-carrier scheduling | Rel-15 |  |  |
| Cross-carrier scheduling | Rel-17 DSS (PSCell) | Rel-15 |  |
| Multi-cell scheduling | Case 1, One Cell is configured as self-carrier scheduling, meanwhile can be multi-cell scheduling by another cell. | Case 2, One Cell is configured as cross-carrier scheduling, meanwhile can be multi-cell scheduling by another cell. | Case 0 One Cell’s scheduling only from multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling |

For example, the Figure 1 and Figure 2 below show an example of Case 0, Case 1 and Case 2 in the Table 1. Case 0 is the simplest configuration, so at least case 0 should be supported. In addition, case 1 and case 2 can provide additional scheduling flexibility, it can easily fall back to self or cross-carrier scheduling.

Figure 1: An example for Multi-cell scheduling Case 0

Figure 2: An example for Multi-cell scheduling Case 1 and Case 2

- At least support Case 0 multi-cell scheduling, i.e one Cell’s scheduling only from multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling

- Further study the other type of multi-cell scheduling, e.g. combination of self/cross-carrier scheduling.

In addition, the maximum number of scheduling cells can do multi-cell schedule one cell should be studied. Such as only one scheduling cell is allowed to do multi-cell scheduling another cell in Rel-18. The reason is it can simplify the BD/CCE handling. If multiple scheduling cell can do multi-cell scheduling for one scheduled cell, it would be very difficult to define its BD and CCE on every multi-cell scheduling configuration.

- It is recommended to give a restriction for the maximum number of scheduling cells when multi-carrier scheduling is configured for a scheduled cell.

PDCCH BD and CCE handling for multi-cell scheduling

The PDCCH BD and CCE handling including the PDCCH overbook for multi-cell scheduling largely depend on the above Case 0/1/2.

For Case 0, if one cell’s scheduling only from multi-cell scheduling, all the BD and CCE of this cell do not divide into several parts. Such as CC2 can only be multi-cell scheduled from CC0 in Figure 1, the BD and CCE for the scheduled cell are calculated as the scheduling cell, which is CC0 for this example.

For Case 1 and 2, since one cell is configured as self-carrier scheduling and multi-cell scheduling by another cell, the BD and CCE are divided into two parts, one is BD/CCE for self/cross-carrier scheduling part, and the other is for multi-cell scheduling part. Scaling factor in Rel-17 DSS can be used as a starting point.

- For Case 0, the BD and CCE for the scheduled cell are calculated as the scheduling cell. For BD and CCE handling of Case 1 and 2, scaling factor in Rel-17 DSS can be used as a starting point.

DCI format and DCI design

For one stage DCI, it is similar as the current DCI design, contains all the scheduling information in one PDCCH. Regarding the fields in the one stage DCI for payload reduction, there are can be three types, which were also proposal in RAN1#104e in the summary by the FL.

FL Proposal#2 in RAN1 104e, R1-2102138: For the two-cell scheduling DCI, if supported, study below options for payload reduction: - All the fields of the DCI can be divided into three types: - First type field: common to the two PDSCHs - Second type field: separate to the two PDSCHs - Third type field: common or separate to the two PDSCHs dependent on RRC configuration - Other solutions are not precluded, e.g., using 2-stage DCI to schedule two PDSCHs on two carriers.

The DCI payload size is main concern, because with the increase of DCI size, the performance of PDCCH is impacted, such as reliability of PDCCH and coding/decoding complexity. We think the solution in above proposal can be directly extended into multi-cell scheduling, such as for the multi-cell scheduling DCI, in order to payload reduction, all the fields of the DCI can be divided into three types:

- First type field: common to the multi-cell PDSCHs/PUSCHs

- Second type field: separate to the multi-cell PDSCHs/PUSCHs

- Third type field: common or separate to the multi-cell PDSCHs/PUSCHs dependent on RRC configuration

Through these two type fields, multi-cell scheduling can share common fields, it can use comparable field length in DCI of single cell scheduling for multi-cell scheduling, to avoid multiplied DCI size increment with the number cells.

Another benefit is the introduction of the configurable of common/separate field type (named Flexible field), which support the unified solution for different cell combinations of scheduling and scheduled cells. When the scheduled cells share more similar transmission condition or scheduling status, there can be more common fields, otherwise, more separate fields.

As shown in the Figure 3 below, it is an example of single stage DCI design for multi-cell scheduling.

- For common field, such as Identifier for DCI formats field, it is still one bit applied for all the scheduled cells, only PDSCH scheduling or PUSCH scheduling.

- By default, the separate field in the DCI contains independent sub-fields for each scheduled cell. Such as Time domain resource assignment, if this DCI schedule two cells, it provides two sub-fields, the first sub-field is for the 1st scheduled cell, and the second sub-field is for the 2nd scheduled cell. The length of each sub-fields depend on its TDRA table of corresponding cell.

- Regarding flexible field, it can be common field or separate field based on RRC configuration, then it is same as the above. For example, Carrier Indicator field if configured as common field, the field includes the information for all the scheduled cell, such as carrier indicators.

Figure 3: single stage DCI design for multi-cell scheduling

Clearly, when use one stage DCI, the exactly DCI size should be known before head, in order to avoid large number of padding bits, the limited scheduled cells group should be carefully configured. Furthermore, one stage DCI can provide less latency scheduling comparing with two stage DCI, and it is still carried by one PDCCH, which can largely reuse the current scheduling design.

- At least one stage DCI can be applied for multi-cell scheduling with a single DCI.

- For the multi-cell scheduling DCI, in order for payload reduction, all the fields of the DCI can be divided into three types:

- First type field: common to the multi-cell PDSCHs/PUSCHs

- Second type field: separate to the multi-cell PDSCHs/PUSCHs

- Third type field: common or separate to the multi-cell PDSCHs/PUSCHs dependent on RRC configuration

UE PDSCH reception preparation time with multi-cell scheduling

According to UE PDSCH reception preparation time with multi-cell scheduling, if the SCS of scheduled cell is different from the scheduling cell, the gap between PDCCH end symbol and the starting position of PDSCH should be applied, as same as cross carrier scheduling. The value of Npdsch PDCCH symbols defined in 38.214 clause 5.5 can be reused here.

- The gap between PDCCH end symbol and the starting position of PDSCH defined in 38.214 should be applied for multi-cell scheduling with a single DCI when the SCS of scheduled cell is different from the scheduling cell.

Conclusion

In this contribution, we made the following proposals.

- It is suggested to study and decide the scope of multi-cell combinations via one single DCI scheduling, to do down select among all those conditions:

  - FR1 and FR2

  - Intra-band and inter-band

  - PCell scheduled by sSCell in FR2

  - Licensed and unlicensed CCs

  - Belong to Different TAG cells

  - CCs from same PUCCH group or different PUCCH group

  - Different priority scheduling

- Scenario#3 PCell scheduled by sSCell in FR2 can be with lower priority

- For Scenario#7 Different priority scheduling can be with lower priority

- Multiple cells scheduled by one DCI should belong to the same PUCCH group

- At least Support up to 4 cells scheduling with a single DCI.

- At least support Case 0 multi-cell scheduling, i.e one Cell’s scheduling only from multi-cell scheduling, not configured as self-carrier nor cross-carrier scheduling

- Further study the other type of multi-cell scheduling, e.g. combination of self/cross-carrier scheduling.

- It is recommended to give a restriction for the maximum number of scheduling cells when multi-carrier scheduling is configured for a scheduled cell.

- For Case 0, the BD and CCE for the scheduled cell are calculated as the scheduling cell. For BD and CCE handling of Case 1 and 2, scaling factor in Rel-17 DSS can be used as a starting point.

- At least one stage DCI can be applied for multi-cell scheduling with a single DCI.

- For the multi-cell scheduling DCI, in order for payload reduction, all the fields of the DCI can be divided into three types:

- First type field: common to the multi-cell PDSCHs/PUSCHs

- Second type field: separate to the multi-cell PDSCHs/PUSCHs

- Third type field: common or separate to the multi-cell PDSCHs/PUSCHs dependent on RRC configuration

- The gap between PDCCH end symbol and the starting position of PDSCH defined in 38.214 should be applied for multi-cell scheduling with a single DCI when the SCS of scheduled cell is different from the scheduling cell.

References

RP-220834 Revised WID on Multi-carrier enhancements