3GPP TSG RAN WG1 #109-e		R1-2203348

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.11.1

Source:	Spreadtrum Communications

Title:	Discussion on XR specific power saving techniques

Document for:	Discussion and decision

Introduction

In RAN#94, a new SI for XR was approved, and the objectives include XR specific power saving techniques, as shown below.

Objectives on XR-specific Power Saving (RAN1, RAN2):[1] - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement.

In this paper, we present our opinions on XR specific power saving techniques.

Discussion

C-DRX enhancement

DRX Pattern periodicity enhancement

For a typical XR DL frame generation rate (per second) of 60 (baseline), 90, 120 fps, the packet arrival periodicity is 16.67ms, 11.11ms, 8.33ms. Obviously, the periodicity is non-integer. The configuration values in TS 38.331[2] of R15/16 DRX long cycle are 10, 20, 32ms, etc. And the DRX short cycle are 2, 3, 4ms, etc. Thus, all the related DRX cycle values are integers. Hence, for whichever DRX cycle value chose from 38.331, there is a mismatch between the non-integer XR DL frame arrival periodicity and the integer DRX cycle value. Figure 1 illustrates the mismatch between the XR DL frame generation rate with 60fps and the DRX cycle with 16ms. From the figure, it can be observed that after several DRX cycles, the XR DL burst arrives at the outside DRX Active Time. It will cause unendurable transmission delay or packet loss if the packet delay budget (PDB) limitation is exceeded.

Figure 1. The mismatch between the XR DL frame generation rate and the DRX cycle

Observation 1: There is a mismatch between the XR DL frame arrival periodicity and the C-DRX cycle values, which will lead to unendurable transmission delay or packet loss if the PDB limitation is exceeded.

In order to guarantee the XR burst transmissions satisfy PDB limits, optimized DRX mechanism providing matching DRX pattern with XR traffic periodicity need to be studied. A potential method is to shorten the DRX cycle, make the UE wake up more frequently, and set a longer On Duration timer, which will increase the power consumption by staying longer in awake state. Besides these power consumption methods, semi-static DRX patterns adjustments or combinations multiple DRX cycles to match XR traffic is more preferred. As shown in Figure 2, the DRX cycles of three cycles are 16 / 16 / 18ms, respectively. These DRS cycle combination can just in line with XR burst periodicity, which is friendlier from the power saving perspective. Thus, we propose to study this semi-static DRX pattern enhancement for XR traffic.

Figure 2. Combine the DRX patterns to match the XR traffic

Proposal 1: It is recommended adopting semi-static DRX patterns adjustments or combinations multiple DRX cycles to match XR traffic.

DRX pattern design with jitter

The impact of jitter on UE receiving DL transmission should also be considered, since jitter will lead to random arrival time of XR DL traffic. If jitter makes the arrival time of XR DL traffic early as depicted in Figure 3(a), the packet transmission will be held until UE wakes up, so it will increase the packet transmission delay, which is not ideal considering the strict PDB requirement. If the XR DL traffic arrives late as depicted in Figure 3(b), the wake-up time of the UE may be prolonged, which will increase the power consumption.

Figure 3. The impact of jitter on (a) early XR burst arrival time; (b) late XR burst arrival time

Observation 2: Considering the impact of jitter, the XR DL traffic arrival time is random. Early DL traffic arrival will increase the packet transmission delay, and late DL traffic arrival will increase the UE power consumption.

For Late XR burst arrival, the PDCCH skipping or search space group switching can help to save some power consuming during the extended the active time. However, there is no efficient solution for early arrival. To solve the problem related to jitter, dynamic DRX pattern adjustment should be studied, such as early trigger or postponing the DRX starting position, as shown in Figure 4.

In addition, to solve the issues about the combination of non-integer periodicity and jitter, UE can be configured with multiple DRX configurations associated with different DRX parameters, such as DRX cycle and/or On Duration.

Figure 4. Dynamic adjust DRX pattern, such as early trigger or postpone the DRX starting position

We also evaluated the power consumption of UE. The power saving gain (PSG) is determined from A: the enhanced CDRX mechanisms which allow adjusting DRX On duration start time offset to be aligned with each XR DL traffic arrival time and B: the power consumption of baseline (AlwaysOn) case; PSG = (B-A)/B×100%.

From our evaluation results, the PSG provided by the enhanced CDRX mechanism is shown in Table 1. The specific parameters can be found in the appendix.

Table 1: Evaluation results

| DRX configuration |  |  | PSG |  |
| --- | --- | --- | --- | --- |
| DRX cycle (ms) | On duration timer (ms) | Inactivity timer (ms) | Mean (%) | Range (%) |
| 16 | 10 | 4 | 13.71 | 5.5~27.5 |

Proposal 2: Dynamic DRX pattern adjustment should be studied, such as early trigger or postponing the DRX starting position.

Proposal 3: To solve the issues about the non-integer periodicities and jitter, UE can be configured with multiple DRX configurations associated with different DRX parameters, such as DRX cycle and/or On Duration.

PDCCH monitoring enhancement

According to PDCCH monitoring enhancement for XR power saving, the search space group switching and PDCCH skipping introduced in Rel-17 power saving can be used as the start point. Search space group switching can be used for dynamic switching between dense traffic and sparse traffic cases. PDCCH monitoring skipping can stop PDCCH monitoring for several controllable periods, which can provide up to 20% power saving compared with legacy baseline.

However, the balance between power saving and PDCCH flexibility/low latency should also be guaranteed, because XR traffic including DL and UL both have strict PDB. The scheduling delay is a relatively important character in XR design. From this point, we provide several PDCCH monitoring enhancements for further study.

First, considering the XR periodicity, for example video stream can be generated at 60, 90 or 120 frames per second, which are {16.67ms, 11.11ms, 8.33ms}. However, PDCCH monitoring periodicity is configured from {1slot, 2slot, 4slot, 5slot, 8slot…}. Thus PDCCH periodicity and XR periodicity do not match. When PDCCH monitoring occasion is not aligned with the traffic periodicities, larger delay will be introduced. In order to satisfy the PDB, more frequent PDCCH occasions i.e. small PDCCH period should be configured. Obviously, it will increase power consumption. So new PDCCH monitoring periodicity which are customized for XR periodicity should be studied.

Observation 3: Mismatch of XR periodicity and PDCCH monitoring occasions will lead to more frequent on duration period or longer on duration timer.

Proposal 4: According to the mismatch of XR periodicity and PDCCH monitoring occasions, semi-static PDCCH monitoring periodicity and PDCCH monitoring skipping should be studied to match XR traffic.

Second, for the impact of jitter, according to the previous RAN1 study, it can be modeled as truncated Gaussian distribution with limited duration [-4, 4] ms. Jitter impacts will lead to uncertainty of traffic arrival. When it arrives early, there is no PDCCH occasion, more waiting time is required. On the other hand, if it is late arrival, there would be more useless PDCCH blind detections due to empty PDCCH scheduling and still need to wait for the next one. Therefore, jitter impact would be more severe and complex compared with misaligned periodicity. PDCCH skipping is a good method.  For example, gNB can dynamic indicate to skip some PDCCH monitoring according to traffic arrival condition. And gNB can also fast resume monitoring if there is a traffic.

Observation 4: More power consumption is expected, as more frequent PDCCH monitoring is needed for the jitter in DL XR traffic.

Proposal 5: Rel-17 PDCCH skipping can be a starting point to solve XR jitter effects.

Uplink data transmission enhancement

XR traffic includes periodic downlink and uplink data bursts. It is good for power consuming if the uplink and downlink data could be transmitted via Uu interface at the same time. However, the period of uplink data is 4ms, and the PDB is 10ms, which is much shorter than the period of downlink data, so it is difficult to always transmit uplink data and downlink data at the same time [3].

For the non-XR session, if the PDB of this uplink data is longer than the period of XR downlink data, UE could wait for the dynamic uplink grant which is assigned in the XR burst window, and transmit the data. This waiting is beneficial for power saving.

Observation 5: The uplink data of XR session shall not be aligned with the downlink XR data.

Observation 6: The uplink data of non-XR session shall be aligned with the downlink XR data.

Conclusion

In this contribution, we discussed XR-specific power saving techniques to accommodate XR service characteristics. The following proposals are achieved:

Observation 1: There is a mismatch between the XR DL frame arrival periodicity and the C-DRX cycle values, which will lead to unendurable transmission delay or packet loss if the PDB limitation is exceeded.

Observation 2: Considering the impact of jitter, the XR DL traffic arrival time is random. Early DL traffic arrival will increase the packet transmission delay, and late DL traffic arrival will increase the UE power consumption.

Observation 3: Mismatch of XR periodicity and PDCCH monitoring occasions will lead to more frequent on duration period or longer on duration timer.

Observation 4: More power consumption is expected, as more frequent PDCCH monitoring is needed for the jitter in DL XR traffic.

Observation 5: The uplink data of XR session shall not be aligned with the downlink XR data.

Observation 6: The uplink data of non-XR session shall be aligned with the downlink XR data.

Proposal 1: It is recommended adopting semi-static DRX patterns adjustments or combinations multiple DRX cycles to match XR traffic.

Proposal 2: Dynamic DRX pattern adjustment should be studied, such as early trigger or postponing the DRX starting position.

Proposal 3: To solve the issues about the non-integer periodicities and jitter, UE can be configured with multiple DRX configurations associated with different DRX parameters, such as DRX cycle and/or On Duration.

Proposal 4: According to the mismatch of XR periodicity and PDCCH monitoring occasions, semi-static PDCCH monitoring periodicity and PDCCH monitoring skipping should be studied to match XR traffic.

Proposal 5: Rel-17 PDCCH skipping can be a starting point to solve XR jitter effects.

Reference

- RP-213587, New SID “Study on XR Enhancements for NR”.

- TS 38.331, “NR, Radio Resource Control (RRC) protocol specification”

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR

- TR 38.840, Study on User Equipment (UE) power saving in NR

Appendix

The simulation assumptions are listed below:

Table 2: Simulation assumptions

| FR |  |  | FR1 |
| --- | --- | --- | --- |
| System Bandwidth |  |  | 100 MHz |
| Numerology |  |  | 30 KHz SCS, 0.5 ms slot duration |
| DL stream | Downlink Traffic Model | Packet size: | Truncated Gaussian |
|  |  |  | Mean = Avg. Data Rate / Frames Per Second; STD, Max, Min = 10.5%, 150%, 50% of Mean |
|  |  | Jitter: | Truncated Gaussian |
|  |  |  | Mean: 0 ms; STD 2 ms; Range [-4, 4] ms |
|  | Bit rates (Mbps) |  | 30 |
|  | # of DL streams |  | 1 |
|  | stream packet generation rate, (fps or Hz) |  | 60 |
|  | Mean Packet Size (byte) |  | 62500 |
| TBS related parameters | The number of subcarriers in a physical resource block NSCRB |  | 12 |
|  | The number of symbols of the PDSCH allocation within the slot |  | 10 |
|  | The number of REs for DM-RS per PRB |  | 4 |
|  | The overhead configured by higher layer parameter xOverhead in PDSCH-ServingCellConfig NohPRB |  | 0 |
|  | The total number of allocated PRBs nPRB [4] |  | 52 |
|  | Modulation Order Qm |  | 8 |
|  | Target code Rate R |  | 0.93 |
|  | The UE shall use the number of layers ʋ |  | 4 |
| DRX configuration | DRX cycle (ms) |  | 16 |
|  | On duration timer (ms) |  | 10 |
|  | Inactivity timer (ms) |  | 4 |