3GPP TSG RAN WG1 #109-e		R1-2203349

e-Meeting, May 9th – 20th, 2022

Agenda Item:     9.11.2

Source:	Spreadtrum Communications

Title:	XR capacity consideration

Document for:	Discussion and decision

Introduction

The capacity of XR session has been discussed in Rel-17 with some agreements in TR38.838 [1]. In this contribution, some details are discussed continuously as according to the approved Rel-18 XR SID [2]

Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements

Discussion

The capacity enhancements in [1] could be divided into 3 categories: decrease the amount of data, guarantee the important/emergency packet, and transmit more packets, as illuminated in figure 1:

Figure 1: Division of capacity enhancement

For these 3 categories, assistance information is important. For the downlink data, assistance information could come from core network, as in Figure 2, which includes the burst arrival pattern, the volume of burst, the importance and dependency in the packets of one burst [2]. With this assistance information, gNB could decrease the amount of data via network coding, transmit the important/emergency/useful packets with higher priority, and drop the other packets if needed. In order to generate this assistance information, some modifications are necessary for the core network, which is difficult for SA and XR server. For XR server, this modification is out of the scope of 3GPP. For the node of core network, it need distinguish the importance of every packet, and generate this assistance information. However, this DPI is not proper for 3GPP network, and could not work if the session is encrypted by XR server.

The other origin of assistance information is coming from UE. For this solution, the application layer in UE receives the packets, deduces the missing packets, decides the importance of these packets, and requests the gNB to transmit or retransmit only some packets. Besides this solution, UE application layer could also predict the importance of future packets and indicate to gNB, which is illustrated in Figure 2.

Figure 2: Assistance information from UE

- Step 1: six packets are delivered from core network to gNB

- Step 2: gNB decides packet #2 is not important, so only five packets are delivered to UE if capacity is a problem, but packet #4 and packet #6 fail in transmission.

  - Step 2b: if wireless resource is not a problem, gNB may also transmit all the 6 packets to UE.

- Step 3: Three packets are delivered to UE’s application layer: packet #1, #3, and #5. Application layer deduces only mission packet #4 is needed to be retransmitted

- Step 4: UE’s access layer indicates gNB to retransmit the packet #4.

  - Step 4b: UE could also indicate gNB to retransmit the packet #4 and packet #6, with different priority. Packet #4 is assigned higher priority in retransmission than the packet #6.

- Step 5: Packet #4 is retransmitted to UE.

  - Step 5b: gNB may retransmit both packet #4 and packet #6, if possible.

For step 4, there are two alternatives:

- Alternative A: UE indicates retransmission via legacy ACK/NACK.

For this alternative, UE shall not generate ACK/NACK based on the TB decoding result, but based on the indication from higher layer.

- Alternative B: UE indicates retransmission via new indication.

For this alternative, the legacy ACK/NACK procedure is not modified, but gNB shall not retransmit the packet based on this ACK/NACK. In addition, a new indication is introduced to trigger the retransmission. UE generates the legacy ACK/NACK based on the TB decoding result, and generates the new indication based on the indication from higher layer.

With these two assistance information, only the packets which are important and needed indeed shall be transmitted/retransmitted via Uu interface in case of network congestion, so the capacity of XR is enhanced. These two types of assistance information could work together, or independently.  For example, PUCCH can be the channel to carry this assistance information.

For alternative A, since the legacy HARQ-ACK needs less time compared with this new indication, larger gap between PDSCH slot and PUCCH slot may be needed, e.g. larger K1 contained in the DCI. In this case, it can reuse all the HARQ-ACK feedback mechanisms, including HARQ-ACK codebook generation, and PUCCH resource selection. However, the impact on the legacy HARQ-ACK should be considered.

For alternative B, the advantage is it has no effect to legacy HARQ-ACK, new indication from higher layer is introduced. Because the new indication needs more processing time, there is a possibility that it cannot be within the same PUCCH resource with legacy HARQ-ACK. Under this situation, the new PUCCH resource for this new indication needs to be triggered. Such as DL DCI trigger an aperiodic indication can be studied.

Proposal 1: Assistance information from higher layer to physical layer should be considered for XR capacity enhancement.

Proposal 2: The legacy HARQ-ACK mechanism and its enhancement should be investigated when assistance information from higher layer to physical layer is introduced for XR capacity enhancement.

Conclusion

In this contribution, we discuss the procedure of improving transmission efficiency by reducing low priority packet transmission with assistance information with the following proposals.

Proposal 1: Assistance information from higher layer to physical layer should be considered for XR capacity enhancement.

Proposal 2: The legacy HARQ-ACK mechanism and its enhancement should be investigated when assistance information from higher layer to physical layer is introduced for XR capacity enhancement.

Reference

- TR 38.838, Study on XR (Extended Reality) Evaluations for NR

- R1-2203219, LS on UE Power Saving for XR and Media Services	SA2, Nokia