3GPP TSG RAN WG1 #109-e                                                     R1-2203350

E-Meeting, May 9th – 20th, 2022

Agenda Item:	9.12.1

Source:	Spreadtrum Communications

Title:	Consideration on coverage enhancement for NR NTN

Document for:	Discussion and decision

Introduction

In RAN#94_e, the study on coverage enhancement to NR over NTN is approved in [1].

4.1.1 Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The work needs to cover the use case of voice and low-data rate services using commercial smartphones with more realistic assumptions on antenna gains instead of 0dBi currently assumed for link budget analysis for non-terrestrial networks. The specific realistic antenna gain assumption will be determined at the working group level. The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density. Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope): - Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4] - VoIP and low-data rate services for commercial handset terminals The following items are shown as examples of areas to consider in the next step of the study. The actual items for study will be based on the evaluation of coverage issues specific to NTN identified above. - NTN-specific repetitions enhancements beyond techniques covered in Rel-17 CovEnh WI for the relevant channels - NTN-specific techniques for improved diversity and/or reduced polarization loss - Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR - NOTE: Intent is not to introduce a new codec. RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be updated.

In this contribution, we share our initial views on coverage enhancements to NR over NTN.

Smart phones in NTN

Smart phones can achieve global NR operation anywhere on earth through NTN network access, which is a very valuable application scenario. It can expand the application scenarios of NTN network. However, there is a certain difference between handheld UE characteristics for system level simulations in [2] and smartphone. For smart phone, it is assumed up to -5dbi antenna gain, which is up to 5 dB less compared to handheld UE in NTN. For smartphone in NTN, with realistic -5dBi TX antenna gain, there is a large coverage gap that needs to be compensated, especially for UL transmission.

Table 6.1.1.1-3: UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) | Handheld | Other (Note 1) |
| --- | --- | --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) | S band (i.e. 2 GHz) | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter | (1, 1, 2) with omni-directional antenna element | Directional (M,N,P,Mg,Ng) = (TBD,TBD,2,1,1); (dV,dH) = (TBD, TBD)λ with directional antenna element (HPBW=65 deg) |
| Polarisation | circular | Linear : +/-45°X-pol | Linear : +/-45°X-pol |
| Rx Antenna gain | 39.7 dBi | 0 dBi per element | TBD dBi per element |
| Antenna temperature | 150 K | 290 K | TBD K |
| Noise figure | 1.2 dB | 7 dB | TBD dB |
| Tx transmit power | 2 W (33 dBm) | 200 mW (23 dBm) | [TBD W (TBD dBm)] |
| Tx antenna gain | 43.2 dBi | 0 dBi per element | TBD dBi per element |
| Note 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. Note 2: VSAT terminal characteristics could be implemented with phased array antenna |  |  |  |

Link Budget Analysis for smart phone in NTN

In TR 38.821, the formula for CNR calculation is

CNR dB=EIRP [dBW]+GT[dB/K]-k [dBW/K/Hz]-PLFS [dB]-PLA [dB]-PLSM [dB]-PLSL[dB]-PLAD [dB]-B [dBHZ]

where EIRP is effective isotropic radiated power (EIRP), GT is antenna-gain-to-noise-temperature, k is Boltzmann constant and equals to -228.6 dBW/K/Hz, PLFS is free space path loss, PLA is atmospheric path loss due to gases and rain fades, PLSM is shadowing margin, PLSL[dB] is scintillation loss, PLAD is additional loss, for example degradation due to feeder links in case of non-regenerative systems, and B is channel bandwidth. According to the link budget calculation formula and the antenna gain of the smart phone (i.e., -5dBm), we calculate the CNR of the uplink and downlink channels for smart phone in NTN.

For DL channel, it can be observed that the worst CNR for the GEO/LEO-1200/LEO-600 satellites are given by -10.2 dB/-3.8 dB/-1.4dB. For GEO, the worst CNR values are lower than the SNR requirements for downlink physical channels.

For UL channel, it can be observed that the worst CNR for the GEO/LEO-1200/LEO-600 satellites are given by -20.7 dB/-13.6 dB/-8.2dB. These CNR values are lower than the SNR requirements for uplink physical channels with the Rel-17 coverage enhancement.

Observation 1: For GEO, the worst CNR values are lower than the SNR requirements for downlink physical channels.

Observation 2: For uplink physical channels, the worst CNR values of the GEO/LEO-1200/LEO-600 satellites are lower than the SNR requirements with the Rel-17 coverage enhancement.

NTN-specific repetitions enhancements

- Repetitions enhancements for 2-Step RACH

In NTN, there is a large propagation delay between UE and gNB. The 4-step random access process will bring large access delay in NTN for smart phone. In order to reduce the access delay, R16 has introduced a two-step random access process. In our view, 2-Step RACH should be supported for smartphone in NTN to reduce access delay. However, for smart phone, it is assumed up to -5dbi antenna gain, which is up to 5 dB less compared to handheld UE in NTN. There is a large coverage gap that needs to be compensated, especially for UL transmission. Rel-17 does not provide coverage enhancement for 2-step random access and the uplink coverage enhancement in R18 also does not include 2-step random access. Therefore, coverage enhancement for 2-Step RACH need to be considered in R18 for smart phones in NTN.

Proposal 1: Repetitions enhancements for 2-Step RACH should be considered in R18 for smart phones in NTN.

- Larger aggregation factor for PDSCH transmission

In RAN1#106_e [3], the following agreement on enhancing the performance of PDSCH transmission had been achieved.

Agreement: The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] - FFS: X = 8, 16 or 32

The small antenna gain in smart phones leaves gap to downlink channel SNR requirements in GEO. For enhancing the performance of PDSCH transmission, solutions with less spec impacts should be considered firstly. In our view, larger aggregation/repetition factor can be considered.

Proposal 2: Larger aggregation factor for PDSCH transmission should be considered in R18 for smart phones in NTN.

Conclusion

As summary, we have the following proposals.

Observation 1: For GEO, the worst CNR values are lower than the SNR requirements for downlink physical channels.

Observation 2: For uplink physical channels, the worst CNR values of the GEO/LEO-1200/LEO-600 satellites are lower than the SNR requirements with the Rel-17 coverage enhancement.

Proposal 1: Repetitions enhancements for 2-Step RACH should be considered in R18 for smart phones in NTN.

Proposal 2: Larger aggregation factor for PDSCH transmission should be considered in R18 for smart phones in NTN.

Reference

- RP-213690, “New WI: NR NTN (Non-Terrestrial Networks) enhancements”, RAN Vice-Chair (AT&T), Thales.

- TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”.

- 3GPP RAN1#106_e Chairman’s Notes, August 16th – 27th, 2021.