3GPP TSG RAN WG1 #109	R1-2203355

e-Meeting, 09 – 20 May 2022

Agenda item:		9.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on L1/L2 signaling for side control information of Network-Controlled Repeater

Document for:		Discussion and Decision

# Introduction

Rel-18 study on network-controlled repeaters has the following objectives:

The study on NR network-controlled repeaters is to focus on the following scenarios and assumptions:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands, while during the study FR2 deployments may be prioritized for both outdoor and O2I scenarios.

- For only single hop stationary network-controlled repeaters

- Network-controlled repeaters are transparent to UEs

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

NOTE1: Cost efficiency is a key consideration point for network-controlled repeaters.

Study and identify which side control information below is necessary for network-controlled repeaters including assumption of max transmission power [RAN1]

- Beamforming information

- Timing information to align transmission / reception boundaries of network-controlled repeater

- Information on UL-DL TDD configuration

- ON-OFF information for efficient interference management and improved energy efficiency

- Power control information for efficient interference management (as the 2nd priority)

Study and identify L1/L2 signaling (including its configuration) to carry the side control information [RAN1]

Study the following aspects of network-controlled repeater management

- Identification and authorization of network-controlled repeaters [RAN2, RAN3]

NOTE2: Coordination with SA3 may be needed.

This contribution discusses considerations on control channel design for NCR operation.

# NCR Architecture Considerations

An exemplary diagram of a typical NCR deployment scenario is shown in Figure 1.

Figure 1: Example deployment scenario for network topology employing NCR

In this diagram the amplify-and-forward links, F_bh-X and F_access-X, are both sides of the analog repeater link intended to support connectivity for both data and control for access UEs to the gNB via the NCR while in the coverage area of the NCR.  While some parameters, such as resource allocation, and TDD config must be managed jointly for F_bh and F_access other parameters such as beam management related parameters are managed separately and so the links are identified as distinct interfaces.

The control link C-NCR is the control interface between the gNB and NCR intended to support appropriate NCR configuration to enable F-link connectivity.  To maintain proper configuration of the NCR the following features must be supported for the maintenance of the C-NCR link:

- Timing synchronization for C-NCR

- TDD config for C-NCR link

- High-level configuration of L1/L2 channels for C-NCR

- Beam management for C-NCR physical channels

Additionally, the following features are necessary to support proper maintenance of the F_bh and F_access link for UEs connected to the parent gNB via the NCR:

- Timing synchronization for F_bh, and F_access

- TDD configuration F_bh and F_access links

- On/off control of F_bh and F_access links

- Beam management of F_bh, and F_access links for each UE connected via NCR

- Power/gain control for F_bh àF_access links in DL and F_accessàF_bh in UL

While enhancements may be necessary, an existing framework exists to manage several of these features for UEs connected through direct access to gNBs in NR RAN. To the extent possible, re-using existing signaling to support proper NCR maintenance can reduce specification impact of the NCR study and limit network overhead through redundant signaling

Observation 2.1:	A legacy signaling framework exists for TDD configuration, beam management, and timing synchronization in NR RAN.

Observation 2.2:	Re-use of existing signaling framework to support NCR management limits redundant specification effort and avoids unnecessary network overhead.

With this motivation in mind, the design of L1/2 control channels for NCR maintenance should prioritize supporting existing signals and information elements specified in legacy NR specifications.

Proposal 2.1:	L1/L2 control channel design should study the benefits of using existing signals and information elements specified in legacy NR specifications.

While multiple options could be considered for NCR higher layer management, the NCR as a static network node should not require a fully flexible stack implementation.  Rather, to limit device cost and complexity and avoid significant specification impact, it should be sufficient to configure NCR control channels via OAM signaling.

Proposal 2.2:	L1/2 control channel and additional higher layer config of NCR is managed via OAM signaling

# L1 Control Channel Design

The following existing DL L1 signals (i.e., PDCCH) may be useful for NCR management:

- DCI format 0_0, 0_1, 0_2:	Provide relevant information for dynamically scheduled PUSCH including TDRA, FDRA, TCI state ID, and associated TPC commands.  An NCR capable of receiving DCI format 0_X for each UE connected its parent gNB via the NCR may be capable of autonomously configuring UL beam config for PUSCH, on/off config for UL resources where PUSCH may be schedules.

- DCI format 1_0, 1_1, 1_2: Provide relevant information for dynamically scheduled PDSCH including TDRA, FDRA, and TCI state ID.  An NCR capable of receiving DCI format 1_x for each UE connected to its parent gNB via the NCR may be capable of autonomously configured DL beam config for PDSCH, and on/off config for DL resources where PDSCH may be scheduled

- DCI format 2_0: Provides DL/UL indication for symbols semi-statically configured as flexible. An NCR capable of receiving DCI format 2_0 could support dynamic TDD for all UEs connected to the parent gNB via the NCR.

Beam management may be supported by semi-static configuration for most channels and signals, excepting dynamically scheduled PDSCH and PUSCH, which would require indication of the desired beam configuration at the time a scheduling decision was made.  Dynamic beam management for PDSCH/PUSCH could be supported by explicit signaling from the parent gNB indicating the desired beam config at the desired time.  This approach could however increase latency and signaling overhead.

Observation 3.1: Explicit signaling of desired beam configuration for PDSCH and PUSCH could increase in scheduling latency and add signaling overhead.

Alternatively, an NCR may be capable of autonomously configuring its beam state by decoding the PDCCH transmission of UEs connected to its parent gNB via the NCR.  In this scenario a capable NCR may determine the beam state for a PDSCH by identifying the associated TCI state ID from the DCI format 0_X or 1_X that scheduled it.  This approach would limit signaling overhead and reduce latency but increase complexity and cost of the NCR itself.

Observation 3.2: 	An NCR capable of decoding DCI format 0_X and 1_X could autonomously configure its beam state and on/off configuration with reduced latency and signaling overhead but would increase the cost and complexity of the NCR.

Proposal 3.1:	Determine if an NCR is expected to determine its beam state for PDSCH and PUSCH transmission autonomously or beam management of the NCR will be under explicit control of the parent gNB.

Observation 3.3:	An NCR capable of autonomously configuring its beam state for DL and UL transmissions must be capable of receiving DCI format 1_X and DCI format 0_X respectively for each UE connected to the parent gNB via the NCR.

In addition to dynamic indication of beam configuration for PDSCH and PUSCH, dynamic indication of PDCCH may also be needed to appropriately configure the beam state of the NCR when transmitting DCIs intended for UEs connected to the parent gNB via the NCR.  CORESETs configured on different TCI state IDs may overlap on symbols.  In these cases there is potential ambiguity in the appropriate beam state of the NCR that is only resolved when a scheduling decision is made by the gNB on which UE is intended to receive the DCI.  After the scheduling decision is made but prior to transmission of the DCI an indication of the appropriate beam state must be provided to the NCR.

Proposal 3.2:	To support CORESET configurations on different beams that overlap in time resources, early indication of beam configuration for PDCCH transmission must be provided to the NCR from the parent gNB.

Dynamic on/off configuration of the NCR may likewise be either autonomously determined by the NCR itself or left under explicit control of the parent gNB.  Just as with beam management for PDSCH and PUSCH, autonomous on/off control of the NCR could make use of DCI format 0_X, and 1_X to identify time resources in which no transmissions are scheduled and the NCR could therefore be put into an off state.  If capability is not supported, explicit signaling of the on/off state for F_bh and F_access links must be provided from the parent gNB which could increase signaling overhead.

Observation 3.4:	An NCR capable of autonomously managing the on/off state of its F_bh and F_access links must be capable of receiving DCI format 0_X and DCI format 1_X for each UE connected to the parent gNB via the NCR.

TDD config for NR RAN can be managed through both semi-static configuration and dynamic indication.  Symbols in each slot are configured as either DL, UL, or flexible, where flexible symbols require a dynamic indication of their link direction via L1 signal (i.e., DCI format 2_0).  To simplify design of the NCR it may be sufficient to limit TDD config to only semi-static DL and UL configuration which would avoid the need for the NCR to be capable of receiving DCI format 2_0.

Proposal 3.3:	An NCR capable of supporting dynamic TDD for UEs connected to its parent gNB via the NCR must be capable of receiving DCI format 2_0.

L1 UL control channel (i.e., PUCCH) may carry HARQ ACK for DL transmissions, scheduling requests for desired UL transmissions and L1 measurement reports for RRM.  HARQ ACK is necessary to support L1 acknowledgment of DL transmissions and can be used to trigger re-transmission via HARQ. Alternatively HARQ ACK can also be carried in PUSCH transmissions. Scheduling request is used to dynamically indicate a UE’s desire to schedule a PUSCH transmission.  Alternatively, PUSCH can also be configured semi-statically.  L1 measurement reporting is carried in PUCCH for the purposes of monitoring link quality supporting beam management.  Rel-18 NCR study has agreed that the NCR deployment is limited to “single hop stationary” links.  In these scenarios it may be assumed that L1 measurement reporting may not be required frequently.  Alternatively, L1 measurement reporting can also be transmitted in PUSCH.  It may be possible given the static deployment of NCR that PUCCH is not needed depending on additional design decisions related to F_bh beam management, L2 control channel design, and higher layer config.  Information typically transmitted in PUCCH may be transmitted in SPS PUSCH or not supported at all.  Supporting NCR without PUCCH capability would reduce flexibility of the NCR and may limit some configuration options but would have the benefit of reducing cost and complexity of the NCR.

Proposal 3.4:	Determine if an NCR must be capable of supporting PUCCH transmissions.

# L2 Control Channel Design

Similar to L1 control channel design, it may not be necessary to support UL L2 signals to manage the NCR.  If L2 signals are not necessary, it may be possible to avoid functionality for support PUSCH and PUCCH transmission in addition to PRACH and RACH procedure generally.  This could reduce cost and complexity of NCR deployment.

Proposal 4.1: 	Determine if L2 UL signals are supported by NCR.

Even when supporting UL signaling it may be reasonable to expect that UL synchronization can be managed with reduced capability relative to UEs.  NCR deployment is assumed to be stationary, supporting single hop links, and should therefore not expect changes in UL timing reference once deployed.  For this reason it is not necessary for an NCR to be capable of receiving TA MAC-CE to support UL timing synchronization.

Proposal 4.2: 	Uplink timing synchronization can be managed by implementation and does not require L2 signaling.

As mentioned in Section 3, autonomous management of F_access beams and on/off configuration may be an effective way to means of reducing signaling overhead but requiring increased cost and complexity for NCR deployment.  While TCI states and DL RS IDs may be configured without enhancement to support the F_access beam management, TCI state activation and TCI state indication for UE-specific PDCCH are existing MAC CE signals that can be provided to a UE for the purposes of beam management.  For the NCR to properly configure F_access beams for UE specific physical channels, the NCR must be aware of dynamically indicated TCI state activation for each of the UEs connected to the parent gNB through the NCR, as well as dynamically indicated TCI states for UE-specific PDCCH.

Proposal 4.3: If autonomous beam management and/or on/off configuration is supported by the NCR, the NCR must be aware of TCI State Activation/Deactivation MAC-CE and TCI state Indication for UE-specific PDCCH MAC-CE sent to UEs connected to the parent gNB via the NCR.

Additionally, while it may be possible to configure semi-persistent schedules for the transmission of SRS, CSI-RS, PDSCH and PUSCH, dynamic L2 indication is also supported for the activation/deactivation of these semi-persistent schedules.  If dynamic activation of SP SRS, CSI-RS, PDSCH, and/or PUSCH is supported, NCR must be aware of the activation/deactivation of these configured schedules for UEs connecting to the parent gNB via the NCR.

Proposal 4.4: 	Activation of SP SRS, CSI-RS, PDSCH, and PUSCH for UEs connected to the parent gNB via the NCR may need to be provided to the NCR to support autonomous beam management and on/off configuration.

UEs may be configured with several static physical channels and signals that are not expected to be active when a UE is in a DRx cycle.  For this reason, knowledge of DRX commands for each of the UEs connected to the parent gNB via the NCR may provide relevant information to the NCR that would be necessary to optimally configure its on/off state.  Since DRX commands for the UEs are provided via MAC CE, it would be expected that L2 signaling to the NCR indicating DRX commands for the UEs connected to the parent gNB via the NCR may be useful in supporting autonomous on/off configuration by the NCR.

Proposal 4.5: 	Knowledge of DRx commands for UEs connected to the parent gNB via the NCR may provide useful information for optimum on/off control.

# Conclusion

Observations and proposals for design of NCR management interface are:

Observation 2.1:	A legacy signaling framework exists for TDD configuration, beam management, and timing synchronization in NR RAN.

Observation 2.2:	Re-use of existing signaling framework to support NCR management limits redundant specification effort and avoids unnecessary network overhead.

Proposal 2.1:	L1/L2 control channel design should study the benefits of using existing signals and information elements specified in legacy NR specifications.

Proposal 2.2:	L1/2 control channel and additional higher layer config of NCR is managed via OAM signaling

Observations and proposals for L1 control channel design are:

Observation 3.1: Explicit signaling of desired beam configuration for PDSCH and PUSCH could increase in scheduling latency and add signaling overhead.

Observation 3.2: An NCR capable of decoding DCI format 0_X and 1_X could autonomously configure its beam state and on/off configuration with reduced latency and signaling overhead but would increase the cost and complexity of the NCR.

Observation 3.3:	An NCR capable of autonomously configuring its beam state for DL and UL transmissions must be capable of receiving DCI format 1_X and DCI format 0_X respectively for each UE connected to the parent gNB via the NCR.

Observation 3.4:	An NCR capable of autonomously managing the on/off state of its F_bh and F_access links must be capable of receiving DCI format 0_X and DCI format 1_X for each UE connected to the parent gNB via the NCR.

Proposal 3.1:	Determine if an NCR is expected to determine its beam state for PDSCH and PUSCH transmission autonomously or beam management of the NCR will be under explicit control of the parent gNB.

Proposal 3.2:	To support CORESET configurations on different beams that overlap in time resources, early indication of beam configuration for PDCCH transmission must be provided to the NCR from the parent gNB.

Proposal 3.3:	An NCR capable of supporting dynamic TDD for UEs connected to its parent gNB via the NCR must be capable of receiving DCI format 2_0.

Proposal 3.4:	Determine if an NCR must be capable of supporting PUCCH transmissions.

Proposals for L2 control channel design are:

Proposal 4.1: 	Determine if L2 UL signals are supported by NCR.

Proposal 4.2: 	Uplink timing synchronization can be managed by implementation and does not require L2 signaling.

Proposal 4.3: If autonomous beam management and/or on/off configuration is supported by the NCR, the NCR must be aware of TCI State Activation/Deactivation MAC-CE and TCI state Indication for UE-specific PDCCH MAC-CE sent to UEs connected to the parent gNB via the NCR.

Proposal 4.4: 	Activation of SP SRS, CSI-RS, PDSCH, and PUSCH for UEs connected to the parent gNB via the NCR may need to be provided to the NCR to support autonomous beam management and on/off configuration.

Proposal 4.5: 	Knowledge of DRx commands for UEs connected to the parent gNB via the NCR may provide useful information for optimum on/off control.

# References

- RP-213700, “New SI: Study on NR Network-controlled Repeaters”, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.